IEEE P802.11
Wireless LANs

| LongNAV Text Proposal Submission to address Letter Ballot 84 Comment CID782 Coexistance |  |  |  |  |
| --- | --- | --- | --- | --- |
| The text changes attempt to clarify the meaning of the description of LongNAV section. Date: 2006-09-15 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Joseph Levy | InterDigital Communications Corporation | 2 Huntington Quadrangle 4th Floor, South Wing Melville, NY 11747 | +1 631 622 4139 | joseph.levy@interdigital.com |

Abstract

This document contains a text proposed change to the P802.11n-D1.03 Draft to address CID782.  The change rewords and restuctures the LongNAV section to clarify the intent of the section.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# Introduction:

## Motion to Adopt

A motion to adopt the changes defined in this submission means that the editing instructions and any changed or added material are actioned in the current TGn Draft.  This introduction, is not part of the adopted material.

Editing instructions formatted like this are intended to be copied into the TGn Draft (i.e. they are instructions to the 802.11 editor on how to merge the TGn amendment with the baseline documents).

TGn Editor:  Editing instructions preceded by “TGn Editor” are instructions to the TGn editor to modify existing material in the TGn draft.   As a result of adopting the changes, the TGn editor will execute the instructions rather than copy them to the TGn Draft.

Summission Note: Notes to the reader of this submission are not part of the motion to adopt.  These notes are there to clarify or set in context the response to the omission.

# TEXT Proposal:

TGn Editor: Change section 9.13.5.2, which begins on page 95 line 61  in draft D1.03 as indicated by the following redline changes:

9.13.5.2 LongNAV

Under the rules of LongNAV operation, when a STA holds a TXOP, it may set a NAV value intended to protect multiple PPDUs using a single MAC layer protection exchange.

The duration of the NAV may be set either by:

— The STA accurately predicting the duration of the multiple PPDUs

to be protected and then setting the NAV duration to the predicted value.  This method may provide maximum MAC effiency.

Or

—

The STA may estimate the required duration, allowing the duration time to include estimates for the multiple PPDUs to be sent and to account for:

— An estimation of the time needed for retries of failed transmissions in the current exchange;

— variation in the total estimated time caused by the adaptation of transmit parameters by training feedback during the current exchange;

- the possible transmission of MSDUs that may arrive at the MAC Data SAP during the current exchange.

This method allows for flexibility in which frames may be transmitted during the TXOP and for flexibility in the rates at which these frames will be transmitted.

LongNAV protection selects a duration value, limited by the remaining duration in the current TXOP, and setting the NAV to this value using one of the MAC layer protection techniques. The Duration/ID field in frames within an A-MPDU shall contain the remaining duration of TXOP (referenced to the end of the PPDU carrying the frame). All frames sent in the TXOP by Initiator or Responder shall contain the remaining duration of TXOP in the Duration/ID field.

Figure n36 shows an example of the use of LongNAV with implicit Block Ack. In this example, an RTS/CTS exchange establishes NAV protection of the TXOP. Then there follows a sequence of transmissions of AMPDUs and BlockAck responses from the responder.