IEEE P802.11
Wireless LANs

| Make Before Break or Tentative Reassociation (Replace) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-09-18 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Clint Chaplin | Symbol Technologies | 6480 Via Del Oro, San Jose, CA, 95119-1208, USA | +1-408-528-2766 | clint.chaplin@symbol.com |

Abstract

In order to minimise or eliminate any gap in data connectivity while roaming, it is proposed to let a STA make a partial connection with a new AP without dropping the connection with the old AP.  The STA can then negotiate with the new AP, including EAPOL messages, to set up the correct conditions for data connectivity, while still using the old AP for data connectivity.  Once the correct conditions are set up, the STA can then break the connection with the old AP, and start using the new AP for data connectivity.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# Overview

In the original 802.11 standard, successful association meant that the AP would notify the DS of the new STA to AP mapping, and that was it.  As more complex features have been added to the standard, more and more negotiations, actions, and handshakes have been added to the association process.  These new necessary actions delay the start of possible data communication through the AP.  If these two steps could be reversed, the gap in data communication during a roam could be reduced or eliminated.

The Make Before Break scheme splits reassociation triggered actions into two parts: the notification to the DS of the STA to AP mapping is one part (called “STA-AP Mapping Notification” in this document), and the rest of the actions is the other part (called “Reassociation Triggered Actions” in this document). By splitting this functionality, association can then be done in two steps, rather than a single indivisible step.

For Make Before Break, the first step in the process is the Reassociation Triggered Actions, and is triggered by the STA sending a Tentative Reassociation Request to the new AP.  Once the new AP responds to this request, the new AP and the STA would then be free to do any and all of the actions, handshakes, and negotiations that are currently part of the association process; the only action that would not take place would be the STA-AP Mapping Notification, and the STA would not be allowed to send data packets to any entity other than the AP it is associated with.  Since the STA-AP Mapping Notification has not happened, the STA is free to continue data communications through the old AP.  Also, since the Tentative Reassociation Request/Response triggers everything that the regular Association Request would trigger (with the exception of the STA-AP Mapping Notification), no changes would have to be explicitly made to these mechanisms.

The STA would have to be capable of carrying on conversations with two different APs simultaneously, and these two APs will probably be on different channels.  Existing power save mechanisms can be used to allow the STA to communicate to both APs on the different channels, thus keeping the two conversations from affecting one another and preventing either AP from dropping the STA while the STA is off channel.  Alternatively, other mechanisms (existing or new) could be used to allow the STA to maintain communication with both APs.

Note that the existing EAPOL security communications are, strictly speaking, between the STA and the AP; the AP proxies these messages to the DS using it’s own MAC address, rather than the MAC address of the STA.  Using the MAC address of the STA to the DS at the new AP is not yet allowed, since the STA-AP Mapping Notification has not happened.

Once all negotiations, actions, and handshakes have taken place and the STA is satisfied, it can send a Complete Reassociation Request to the new AP.  In response, the new AP sends a Complete Reassociation Response to the STA, and also performs the STA-AP Mapping Notification to the DS.  At this point, reassociation is complete, and data communications may go through the new AP.

The AP has an age-out mechanism in place for the Tentative Reassociation.  This age-out time is conveyed from the AP to the STA in the Tentative Reassociation response; the time is in seconds.   If the age-out timer times out, the AP will disassociate the STA, sending a Disassociate message to the STA as it does so.  The AP is also free to send a Disassociate message to the STA before the age-out timer times out, if it needs to free up the resources that are being taken up by the Tentative Reassociation.

New reassociation requests and reassociation reply messages were defined, rather than attempting to reuse the existing Reassociation Request/Reassociation Reply messages.  It is hoped that, by doing so, changes to the existing state machines to add Make Before Break can be minimised.

The existing reassociation mechanism is not replaced by Make Before Break; rather, Make Before Break is a optional feature in addition to the existing reassociation mechanism.  If a STA wants to use Make Before Break but cannot find an AP that supports it, it can always drop back to the existing reassociation mechanism (and take the break in data communications as a penalty).

The STA is allowed to have Tentative Reassociations simultaneously open with multiple APs.  The STA would have to complete the reassociation with only one of the APs.

# Management Frame Changes

Two new requests are defined that are used by the STA to signal to the new AP what it wants.  One request is defined to be a request to the new AP to let the STA open communications with it; this request is called Tentative Reassociation Request.  The other request is defined to be a request to the new AP to complete the association; this request is called the Complete Reassociation Request.

Since the number of possible management frame types is so limited, these new requests are implemented by modifying the existing Reassociation Request/Reassociation Response management frames.  To do so, a new information element is defined: Make Before Break Reassociation.  This information element is defined in the following table:

|  | Element ID | Length | MIC Control | MIC | Reassociation Type | Tentative Reassociation Lifetime | ANonce | SNonce | GTK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 2 | 16 | 2 | 2 | 16 | 16 | 16 |

The Element ID will be the next available Element ID as assigned by the IEEE 802.11 number czar.

Length gives the number of octets in the information field of the Information Element..

The MIC Control field is two octets as shown below.

|  | MIC Algorithm | Reserved | Information Element Count |
| --- | --- | --- | --- |
| Bit: | B0-B3 | B4-B7 | B8-B15 |

The MIC algorithm is one of the values shown below:

| Value | Algorithm description |
| --- | --- |
| 0 | Reserved |
| 1 | HMAC-MD5. IETF RFC 2104 and IETF RFC 1321 together define this function |
| 2 | HMAC-SHA1-128 |
| 3-15 | Reserved |

The Information Element Count of the MIC Control field contains the number of information elements that are included in the MIC calculation. A value of zero indicates no MIC is present.

The MIC field contains a Message Integrity Check, calculated using the algorithm selected by the MIC algorithm field of the MIC Control field.

Reassociation Type gives the type of Reassociation being requested, according to the values in the following table:

| Reassociation Type value | Meaning |
| --- | --- |
| 0 | Tentative Reassociation |
| 1 | Complete Reassociation |
| 2-65535 | reserved |

Tentative Reassociation Lifetime is zero in the Reassociation Request, and in the Reassociation Response is the tentative reassociation age-out time, in seconds.

The ANonce field contains a nonce value chosen by the AP. It is encoded following the conventions from 7.1.1.

The SNonce field contains a nonce value chosen by the STA. It is encoded following the conventions from 7.1.1.

GTK contains the Group Key; it is encrypted (see procedures in 8A.5.4), and is defined in Figure 143 of

8.5.2.

This new Make Before Break Reassociation IE is optionally inserted into the Reassociation Request/Reassociation Response management frames.  Use of this information element is made optional in order to allow backwards compatibility with existing implementations.

If the STA is capable of Make Before Break, and the new AP has advertised that it is capable of Make Before Break, then the STA may put the Make Before Break Reassociation IE in the Reassociation Request management frame, if the STA wants to perform Make Before Break.  Otherwise, the STA will leave out the Make Before Break Reassociation IE, and will reassociate in the older way.

If the Make Before Break Reassociation IE is present in the Reassociation Request management frame, and the AP implements Make Before Break, the AP must put the Make Before Break Reassociation IE in the reassociation Response management frame.  In all other cases, the Make Before Break Reassociation IE is not inserted into the Reassociation Reply management frame.

# Advertisement of Make Before Break Capability.

If the AP is capable of Make Before Break, it advertises that fact by including the Make Before Break information element in the beacons and probe responses.

# Completing the Association.

If a STA does a tentative association and RSN is being used, the STA must still do something when completing the association to prove it is who it says it is.  For this purpose the Complete Reassociation Request and Complete Reassociation Response is used, with security information in the Make Before Break Reassociation information element.  This security information currently is the ANonce, the SNonce, and optionally the current GTK, all MICed.  Also, the RSNIE is included in the Complete Reassociation Request.  The use of the MIC acts to protect this frame from being spoofed or replayed.

ReassociationRequest(Make Before Break Reassociation(MIC, Complete Reassociation , ANonce, SNonce,0), RSNIE)

ReassociationResponse(Make Before Break Reassociation(MIC, Complete Reassociation, Anonce, SNonce, GTK), RSNIE)

# Indirect Make Before Break

Direct Make Before Break does have some negative characteristics.  A client needs to manage two conversations with two different APs that may be on different channels; using possible power save techniques.  This may not be viable for some applications.

This proposal specifes an indirect Make Before Break solution that allows the client to carry on a conversation with a target AP through the current AP using the DS as a communication channel.  This is accomplished by defining and using a new Ethertype and then defining an Ether Subtype field and using a single value of the Subtype at this time.  By doing this, future needs for Ethernet Types in IEEE 802.11 can use the Ether Subtype under this new EtherType, and IEEE 802.11 can control the use of the Ether SubTypes through the IEEE 802.11 ANA.

The frame used for Indirect Make Before Break is defined in the following table:

|  | Destination Address | Source Address | EtherType | Ether SubType | IEEE 802.11 Frame |
| --- | --- | --- | --- | --- | --- |
| Octet Count | 6 | 6 | 2 | 1 | 14-??? |
|  |  |  | XX-XX | 01 |  |

The encapsulated IEEE 802.11 Frame is defined in Section 7, with limits as detailed below:

The encapsulated IEEE 802.11 Frame Control field is similar to Figure 19 in 7.1.3.1, with differences as detailed in the following table:

|  | B0_B1 | B2_B3 | B4_B7 | B8 | B9 | B10 | B11 | B12 | B13 | B14 | B15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | Reserved | Type | Subtype | Reserved | Reserved | More Frag | Retry | Reserved | Reserved | Protected Frame | Reserved |
| Bits | 2 | 2 | 4 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |

For the encapsulated IEEE 802.11 Frame, the following table lists the valid Type and Subtype values:

| Type Value B3 B2 | Type Description | Subtype Value B7 B6 B5 B4 | Subtype Description |
| --- | --- | --- | --- |
| 00 | Management | 0000 | Association request |
| 00 | Management | 0001 | Association response |
| 00 | Management | 0010 | Reassociation request |
| 00 | Management | 0011 | Reassociation response |
| 00 | Management | 0100 | Reserved |
| 00 | Management | 0101 | Reserved |
| 00 | Management | 0110-0111 | Reserved |
| 00 | Management | 1000 | Reserved |
| 00 | Management | 1001 | Reserved |
| 00 | Management | 1010 | Disassociation |
| 00 | Management | 1011 | Authentication |
| 00 | Management | 1100 | Deauthentication |
| 00 | Management | 1101 | Action |
| 00 | Management | 1110-1111 | Reserved |
| 01 | Reserved | 0000-1111 | Reserved |
| 10 | Data | 0000 | Data |
| 10 | Data | 0001-0011 | Reserved |
| 10 | Data | 0100 | Null (no data) |
| 10 | Data | 0101-1111 | Reserved |
| 11 | Reserved | 0000-1111 | Reserved |

Given the limitations as outlined above, and IEEE 802.11 frame may be sent from a STA through the current AP to a target AP.  Thus, authentication, QoS negotiation, 4 way handshake, or other protocols between the STA and the target AP can take place.

There are some limitations that need to be put into place.  Full Association is not allowed, but Tentative Association is encouraged.  Also, the fragmentation timeout needs to be increased, because the communication medium is inherently slower than directly over the air.

Section 2 TBD

### Section 3 TBD

### Section 4 TBD

### TGr Editor: Keep 7.1.3.1.8.

### TGr Editor: Replace 7.2.3.1 with the following:

7.2.3.1 Beacon frame format

Change the last row of Table 8 as follows:

## Table 8—Beacon frame body

| Order | Information | Notes |
| --- | --- | --- |
| 28 | Make Before Break Information | The Make Before Break IE is present in a Beacon only when dot11MakeBeforeBreakEnabled is true |
| Last | Vendor Specific | One or more vendor specific information elements may appear in this frame. This information element follows all other information elements. |

### TGr Editor: Replace 7.2.3.6 with the following:

7.2.3.6 Reassociation request frame format

Insert the following row into Table 12:

## Table 12—Reassociation request frame body

| Order | Information | Notes |
| --- | --- | --- |
| 11 | Make Before Break Reassociation | The Make Before Break Reassociation IE is present in an Reassociation Request only when dot11makeBeforeBreakEnabled is true |

### TGr Editor: Replace 7.2.3.7 with the following:

7.2.3.7 Reassociation response frame format

Insert the following row into Table 13:

## Table 13—Reassociation response frame body

| Order | Information | Notes |
| --- | --- | --- |
| 7 | ###### Make Before Beak Reassociation | The Make Before Break Reassociation IE is present in an Reassociation Response only when dot11MakeBeforeBreakEnabled is true and the Make Before Break Reassociation IE was present in the associated Reassociation Request frame |

### TGr Editor: Replace 7.2.3.9 with the following:

7.2.3.9 Probe response frame format

#### Change the last two rows of Table 15 (Vendor Specific and Requested Information elements) as follows:

## Table 15—Probe response frame body

| Order | Information | Notes |
| --- | --- | --- |
| 26 | Make Before Break Information | The Make Before Break IE is present in a Probe response only when dot11MakeBeforeBreakEnabled is true |
| Last-1 | Vendor Specific | One or more vendor-specific information elements may appear in this frame. This information element follows all other information elements except the Requested Information elements. |
| Last | Requested Information Elements | Elements requested by the Request information element of the Probe Request frame |

### TGr Editor: Delete 7.2.3.10.

### TGr Editor: Delete 7.3.1 and all subsections.

### TGr Editor: Replace 7.3.2 with the following:

7.3.2 Information elements

Change the last three rows of Table 26 as follows:

## Table 26—Element IDs

| Information Element | Element ID |
| --- | --- |
| Make Before Break Information | 54 |
| Make Before Break Reassociation | 55 |
| Reserved | 56-220 |
| Vendor Specific | 221 |
| Reserved | 222-255 |

### TGr Editor: Delete 7.3.2.25 and all subsections.

### TGr Editor: Delete 7.3.2.28.

### TGr Editor: Replace 7.3.2.42 with the following:

7.3.2.42 Make Before Break information element

The Make Before Break information element contains the Make Before Break Capability.  The Access Point uses the Make Before Break Information Element to advertise its support for Make Before Break capability. The format for this information element is given in Figure 113M.

|  | Element ID | Length | Make Before Break Capability |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 |

## Figure 113M – Make Before Break information element

The Length field shall be set to 1.

The Make Before Break Capability is one octet.  The Make Before Break Capability value is defined in Figure 113N.

|  | Make Before Break Capability | Make Before Break Direct Communication | Make Before Break Indirect Communication | Reserved |
| --- | --- | --- | --- | --- |
| Bit: | B0 | B1 | B2 | B3 B4 B5 B6 B7 |

## Figure 113N – Make Before Break Capability value

Make Before Break Capability is set if the AP supports Make Before Break

Make Before Break Direct Communication is set if the AP supports direct communication for setting up a Make Before Break.

Make Before Break Indirect Communication is set if the AP supports indirect communication for setting up a Make Before Break.

### TGr Editor: Replace 7.3.2.43 with the following:

7.3.2.43 Make Before Break Reassociation information element

The Make Before Break Reassociation information element is used during a Reassociation Request and Reassociation Response frame to specify information during a Make Before Break.

|  | Element ID | Length | MIC Control | MIC | Reassociation Type | Tentative Reassociation Lifetime | ANonce | SNonce | GTK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 2 | 16 | 2 | 2 | 16 | 16 | 16 |

## Figure 113O – Make Before Break Reassociation information element

The Length field for this element indicates the length of the information field, as defined below.

.

The MIC Control field is two octets, and is defined in Figure 113P.

|  | MIC Algorithm | Reserved | Information Element Count |
| --- | --- | --- | --- |
| Bit: | B0-B3 | B4-B7 | B8-B15 |

## Figure 113P – MIC Control Field

The MIC algorithm is one of the values from Table 43B:

## Table 43B – MIC Control Field

| Value | Algorithm description |
| --- | --- |
| 0 | Reserved |
| 1 | HMAC-MD5. IETF RFC 2104 and IETF RFC 1321 together define this function |
| 2 | HMAC-SHA1-128 |
| 3-15 | Reserved |

The Information Element Count of the MIC Control field contains the number of information elements that are included in the MIC calculation. A value of zero indicates no MIC is present.

The MIC field contains a Message Integrity Check, calculated using the algorithm selected by the MIC algorithm field of the MIC Control field.

Reassociation Type gives the type of Association being requested, according to the values in Figure 113Q:

| Reassociation Type value | Meaning |
| --- | --- |
| 0 | Tentative Reassociation |
| 1 | Complete Reassociation |
| 2-65535 | reserved |

## Figure 113Q – Reassociation Type value

Tentative Reassociation Lifetime gives the lifetime, in seconds, of the tentative reassociation.

The ANonce field contains a nonce value chosen by the AP. It is encoded following the conventions from 7.1.1.

The SNonce field contains a nonce value chosen by the STA. It is encoded following the conventions from 7.1.1.

GTK contains the Group Key; it is encrypted and is defined in Figure 143 of 8.5.2.

### TGr Editor: Delete 7.3.2.44 through 7.3.2.47.

### TGr Editor: Delete 7.4 and all subsections.

### TGr Editor: Delete 8.4 and all subsections.

### TGr Editor: Delete 8.5 and all subsections.

### TGr Editor: Delete 8.5A and all subsections.

### TGr Editor: Delete Section 8A and all subsections.

### TGr Editor: Delete 10.3.4 and all subsections.

### TGr Editor: Replace 10.3.7 and subsections with the following:

10.3.7 Reassociate

10.3.7.1 MLME-REASSOCIATE.request

10.3.7.1.2 Semantics of the service primitive

Change the text of 10.3.7.1.2 as follows:

The primitive parameters are as follows:

MLME-REASSOCIATE.request		(

NewAPAddress,

ReassociateFailureTimeout,

CapabilityInformation,

ListenInterval,

SupportedChannels,

RSN,

QoS Capability,

Content of Make Before Break Reassociation Information Element,

VendorSpecificInfo

)

Insert the following row (Content of Make Before Break Reassociation Information Element) as the second to last entry of the table in 10.3.7.1.2:

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Content of Make Before Break Reassociation Information Element | Information Element | As defined in 7.3.2.43 | Present only when dot11MakeBeforeBreakEnabled is true. If an Make Before Break Reassociation information element is to be present in the Reassociate Frame, then that Make Before Break Reassociation information element is here; otherwise this is null. |

10.3.7.2 MLME-REASSOCIATE.confirm

10.3.7.2.2 Semantics of the service primitive

Change the text of 10.3.7.2.2 as follows:

The primitive parameters are as follows:

MLME-REASSOCIATE.confirm		(

ResultCode,

CapabilityInformation,

AssociationID,

SupportedRates,

EDCAParameterSet,

Content of Make Before Break Reassociation Information Element,

VendorSpecificInfo

)

Insert the following row (Content of Make Before Break Reassociation Information Element) as the second to last entry of the table in 10.3.7.2.2:

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Content of Make Before Break Reassociation Information Element | Information Element | As defined in 7.3.2.43 | Present only when dot11MakeBeforeBreakEnabled is true. If an Make Before Break Reassociation information element is to be present in the Reassociate Frame, then that Make Before Break Reassociation information element is here; otherwise this is null. |

10.3.7.3 MLME-REASSOCIATE.indication

10.3.7.3.2 Semantics of the service primitive

Change the text of 10.3.7.3.2 as follows:

The primitive parameters are as follows:

MLME-REASSOCIATE.indication		(

PeerSTAAddress,

CurrentAPAddress CapabilityInformation,

ListenInterval,

SSID,

SupportedRates,

RSN,

QoSCapability,

Content of Make Before Break Reassociation Information Element,

VendorSpecificInfo

)

Insert the following row (Content of Make Before Break Reassociation Information Element) as the second to last entry of the table in 10.3.7.3.2:

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Content of Make Before Break Reassociation Information Element | Information Element | As defined in 7.3.2.43 | Present only when dot11MakeBeforeBreakEnabled is true. If an Make Before Break Reassociation information element was present in the Reassociate Frame, then that Make Befotre Break Reassociation information element is reflected here; otherwise this is null. |

10.3.7.4 MLME-REASSOCIATE.response

10.3.7.4.2 Semantics of the service primitive

Change the text of 10.3.7.4.2 as follows:

The primitive parameters are as follows:

MLME-REASSOCIATE.response		(

PeerSTAAddress,

ResultCode,

CapabilityInformation,

AssociationID,

Content of Make Before Break Reassociation Information Element,

VendorSpecificInfo

)

Insert the following row (Content of Make Before Break Reassociation Information Element) as the second to last entry of the table in 10.3.7.4.2:

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Content of Make Before Break Reassociation Information Element | Information Element | As defined in 7.3.2.43 | Present only when dot11MakeBeforeBreakEnabled is true. If an Make Before Break Reassociation information element was present in the Reassociate Frame, then that Make Before Break Reassociation information element is reflected here; otherwise this is null. |

### TGr Editor: Delete 10.3.31 and all subsections.

TGr Editor: Delete 10.3.32 and all subsections.

### TGr Editor: Add the following as 11.3:

#### Replace 11.3 with the following:

A STA keeps two state variables for each STA with which direct communication via the WM is needed:

- Authentication state: The values are unauthenticated and authenticated.

- Association state: The values are unassociated, tentatively associated, and fully associated.

These two variables create four local states for each remote STA:

- State 1: Initial start state, unauthenticated, unassociated.

- State 2: Authenticated, not associated.

- State 3a: Authenticated and tentatively reassociated.

- State 3b: Authenticated and fully associated.

The relationships between these station state variables and the services are given in Figure 199.

Figure 199—Relationship between state variables and services

The current state existing between the source and destination station determines the IEEE 802.11 frame types that may be exchanged between that pair of STAs (see Clause 7). The state of the sending STA given by Figure 8 is with respect to the intended receiving STA. The allowed frame types are grouped into classes and the classes correspond to the station state. In State 1, only Class 1 frames are allowed. In State 2, either Class 1 or Class 2 frames are allowed. In State 3a and State 3b, all frames are allowed (Classes 1, 2, and 3). The frame classes are defined as follows:

- Class 1 frames (permitted from within States 1, 2, 3a, and 3b):

  - Control Frames

    - Request to Send (RTS)

    - Clear to Send (CTS)

    - Acknowledgement (ACK)

    - Contention-Free (CF)-End+ACK

    - CF-End

  - Management Frames

    - Probe request/response

    - Beacon

    - Authentication:
Successful authentication enables a STA to exchange Class 2 frames. Unsuccessful authentication leaves the STA in State 1.

    - Deauthentication:
Deauthentication notification when in State 2, State 3a, or State 3b changes the STA’s state to State 1. The STA shall become authenticated again prior to sending Class 2 frames. Deauthentication notification when in State 3a or 3b implies disassociation as well.

    - Announcement traffic indication message (ATIM)

    - Spectrum Management Action 
Within an IBSS, action frames are class 1.

  - Data Frames

    - Data:
Data frames between STAs in an IBSS with frame control (FC) bits “To DS” and “From DS” both false.

- Class 2 frames (if and only if authenticated; allowed from within States 2, 3a, and 3b only):

  - Management Frames

    - Association request/response

- Successful association enables Class 3 frames.

- Unsuccessful association leaves STA in State 2.

- Unsuccessful tentative reassociation leaves STA in State 2.

- Unsuccessful full association leaves STA in State 3a.

    - Reassociation request/response

- Successful reassociation enables Class 3 frames.

- Unsuccessful reassociation leaves the STA in State 2 (with respect to the STA that was sent the reassociation message).

- Unsuccessful tentative reassociation leaves STA in State 2.

- Unsuccessful full reassociation leaves STA in State 3a.

- Reassociation frames shall only be sent if the sending STA is already associated in the same ESS.

    - Disassociation

- Disassociation notification when in State 3a or 3b changes a STA’s state to State 2. This STA shall become associated again if it wishes to utilize the DS.

If STA A receives a Class 2 frame with a unicast address in the Address 1 field from STA B that is not authenticated with STA A, STA A shall disallow the received Class 2 frame and send a deauthentication frame to STA B.

- Class 3 frames (if and only if tentatively or fully associated; allowed only from within State 3a and 3b):

  - Data Frames

    - Data subtypes: Data frames allowed. That is, either the “To DS” or “From DS” FC bits may be set to true to utilize the DSS.
In State 3a, only Data frames between STA A and STA B are allowed.  In State 3b, all Data frames are allowed.

    - QoS data subtypes allowed to/from non-AP QSTA(s) that are associated with QAP(s).

    - Data frames between QSTAs in a QBSS with FC bits “To DS” and “From DS” both false.

  - Management frames

    - QoS, DLS, and Block Ack Action.

  - Control frames

    - Power save (PS)-Poll

    - Action:
Within an infrastucture BSS, action frames are class 3.

    - Block Ack (BlockAck)

    - Block Ack Request (BlockAckReq)

If STA A receives a Class 3 frame with a unicast address in the Address 1 field from STA B that is authenticated but not tentatively or fully associated with STA A, STA A shall disallow the received Class 3 frame and send a disassociation frame to STA B.

If STA A receives a Class 3 frame with a unicast address in the Address 1 field from STA B that is not authenticated with STA A, STA A shall disallow the received Class 3 frame and send a deauthentication frame to STA B.

(The use of the word “receive” in this subclause refers to a frame that meets all of the filtering criteria specified in Clause 8 and Clause 9.)

TGr Editor: Delete 11.3.1 and all subsections.

### TGr Editor: Replace 11.3.2 and subsections with the following:

Change 11.3.2 and subsections as shown below:

11.3.2 Association, reassociation, and disassociation

This subclause defines how a STA associates and reassociates with an AP and how it disassociates from it.

The states used in this description are those defined in 11.3.

11.3.2.1 STA association procedures

Upon receipt of an MLME-ASSOCIATE.request primitive a STA shall associate with an AP via the following procedure:

- The STA shall transmit an Association Request frame to an AP with which that STA is authenticated.  If the MLME-ASSOCIATE.request primitive contained an RSN information element with only one pairwise cipher suite and only one authenticated key suite, this RSN information element shall be included in the Association Request frame.

- If an Association Response frame is received with a status value of “successful,” the STA is now associated with the AP. The state variable shall be set to State 3b, and the MLME shall issue an MLME-ASSOCIATE.confirm primitive indicating the successful completion of the operation.

- If an Association Response frame is received with a status value other than “successful” or the AssociateFailureTimeout expires, the STA is not associated with the AP. The MLME shall issue an MLME-ASSOCIATE.confirm primitive indicating the failure of the operation. The Status Code returned in the Association Response frame indicates the cause of the failed association attempt. Any misconfiguration or parameter mismatch, e.g., data rates required as Basic Rates that the STA did not indicate as supported in the STA's Supported Rates information element, shall be corrected before the SME issues an MLME-ASSOCIATE.request for the same AP. If the Status Code indicates the association failed because of a reason that is not related to configuration, e.g., the AP is unable to support additional associations, the SME shall not issue an MLME-ASSOCIATE.request for the same AP, until a period of at least 2 seconds has elapsed.

- The SME shall establish an RSNA, or it shall enable WEP by calling MLME.SETPROTECTION.request primitive with ProtectType set to “Rx_Tx,” or it shall do nothing if it does not wish to secure communication.

The STA’s SME shall delete any PTKSA and temporal keys held for communication with the indicated STA by using MLME-DELETEKEYS.request primitive (see 8.4.10) before invoking MLME-ASSOCIATE.request primitive.

11.3.2.2 AP association procedures

When an Association Request frame is received from a STA, the AP shall associate with the STA using the following procedure:

- If the STA is not authenticated, the AP shall transmit a Deauthentication frame to the STA and terminate the association procedure.

- In an RSNA, the AP shall check the values received in the RSN information element, to see if the values received match the AP’s security policy. If not, the association shall not be accepted.

- Upon receipt of an MLME-Associate.response service primitive, the AP shall transmit an Association Response with a status code as defined in 7.3.1.9. If the status value is “successful,” the association identifier assigned to the STA shall be included in the response.

- When the status value of the association is not successful, the AP shall indicate a specific reason for the failure to associate in the Status Code of the Association Response frame. If any Status Code value from Table 23 in 7.3.1.9 is an appropriate reason for the failure to associate, the AP shall indicate that Status Code value. The use of the unspecified reason value of the Status Code shall indicate the association failed for a reason that is unrelated to every other defined Status Code value in Table 23.

- When the Association Response with a status value of “successful” is acknowledged by the STA, the STA is considered to be associated with this AP. The state variable for the STA shall be set to State 3b.

- The SME shall establish an RSNA, or it shall enable WEP by calling MLME.SETPROTECTION.request primitive with ProtectType set to “Rx_Tx,” or it shall do nothing if it does not wish to secure communication.

- The SME will inform the DS of the new association.

The STA’s SME shall delete any PTKSA and temporal keys held for communication with the indicated STA by using MLME-DELETEKEYS.request primitive (see 8.4.10) upon receiving a MLME-ASSOCIATE.indication primitive.

11.3.2.3 STA reassociation procedures

Upon receipt of an MLME-REASSOCIATE.request primitive that does not contain a Make Before Break Reassociation Information Element, a STA shall reassociate with an AP via the following procedure:

- If the state variable is in State 1, the STA shall inform the SME of the failure of the reassociation by issuing an MLME-REASSOCIATE.confirm primitive.

- The STA shall transmit a Reassociation Request frame to the new AP. If the MLME-REASSOCIATE.request primitive contained an RSN information element with only one pairwise cipher suite and only one authenticated key suite, this RSN information element shall be included in the Reassociation Request frame.

- If a Reassociation Response frame is received with a status value of “successful,” the STA is now associated with the new AP. The state variable shall be set to State 3b, and the MLME shall issue an MLME-REASSOCIATE.confirm primitive indicating the successful completion of the operation.

- If a Reassociation Response frame is received with a status value other than “successful” or the AssociateFailureTimeout expires, the STA is not associated with the AP. The MLME shall issue an MLME-REASSOCIATE.confirm primitive indicating the failure of the operation. The Status Code returned in the Reassociation Response frame indicates the cause of the failed reassociation attempt. Any misconfiguration or parameter mismatch, e.g., data rates required as Basic Rates that the STA did not indicate as supported in the STA's Supported Rates information element, shall be corrected before the SME issues an MLME-REASSOCIATE.request for the same AP. If the Status Code indicates the reassociation failed because of a reason that is not related to configuration, e.g., the AP is unable to support additional associations, the SME shall not issue an MLME-REASSOCIATE.request for the same AP, until a period of at least 2 seconds has elapsed.

- The SME shall establish an RSNA, or it shall enable WEP by calling MLME.SETPROTECTION.request primitive with ProtectType set to “Rx_Tx,” or it shall do nothing if it does not wish to secure communication.

The STA’s SME shall delete any PTKSA and temporal keys held for communication with the indicated STA by using MLME-DELETEKEYS.request primitive (see 8.4.10) before invoking MLME-REASSOCIATE.request primitive without a Make Before Break Reassociation Information Element.

11.3.2.4 STA tentative reassociation procedures

Upon receipt of an MLME-REASSOCIATE.request primitive that contains a Make Before Break Reassociation Information Element and the Reassociation Type is set to Tentative Association, a STA shall tentatively reassociate with an AP via the following procedure:

- If the state variable is in State 1, the STA shall inform the SME of the failure of the tentative reassociation by issuing an MLME-REASSOCIATE.confirm primitive.

- The STA shall transmit a Reassociation Request frame to the new AP.  TheMake Before Break Reassociation Information Element that is in the MLME-REASSOCIATE.request primitiveshall be included in the Association Request frame.  If the MLME-REASSOCIATE.request primitive contained an RSN information element with only one pairwise cipher suite and only one authenticated key suite, this RSN information element shall be included in the Reassociation Request frame.

- If a Reassociation Response frame is received with a status value of “successful,” the STA is now tentatively associated with the new AP. The state variable shall be set to State 3a, and the MLME shall issue an MLME-REASSOCIATE.confirm primitive indicating the successful completion of the operation.

- If a Reassociation Response frame is received with a status value other than “successful” or the AssociateFailureTimeout expires, the STA is not tentatively associated with the AP. The MLME shall issue an MLME-REASSOCIATE.confirm primitive indicating the failure of the operation. The Status Code returned in the Reassociation Response frame indicates the cause of the failed tentative reassociation attempt. Any misconfiguration or parameter mismatch, e.g., data rates required as Basic Rates that the STA did not indicate as supported in the STA's Supported Rates information element, shall be corrected before the SME issues an MLME-REASSOCIATE.request for the same AP. If the Status Code indicates the reassociation failed because of a reason that is not related to configuration, e.g., the AP is unable to support additional associations, the SME shall not issue an MLME-REASSOCIATE.request for tentative association for the same AP, until a period of at least 2 seconds has elapsed.

- The SME shall establish an RSNA, or it shall enable WEP by calling MLME.SETPROTECTION.request primitive with ProtectType set to “Rx_Tx,” or it shall do nothing if it does not wish to secure communication.

The STA’s SME shall delete any PTKSA and temporal keys held for communication with the indicated STA by using MLME-DELETEKEYS.request primitive (see 8.4.10) before invoking MLME-REASSOCIATE.request primitive containing a Make Before Break Reassociation Information Element and the Reassociation Type is set to Tentative Association.

11.3.2.5 STA full reassociation procedures

Upon receipt of an MLME-REASSOCIATE.request primitive containing an Make Before Break Reassociation Information Element and the Reassociation Type is set to Full Association, a STA shall fully associate with an AP via the following procedure:

- If the state variable is in State 1, the STA shall inform the SME of the failure of the full reassociation by issuing an MLME-REASSOCIATE.confirm primitive.

- The STA shall transmit a Reassociation Request frame to the new AP.  The Make Before Break Reassociation Information Element that is in the MLME-REASSOCIATE.request primitiveshall be included in the Association Request frame.  If the MLME-REASSOCIATE.request primitive contained an RSN information element with only one pairwise cipher suite and only one authenticated key suite, this RSN information element shall be included in the Reassociation Request frame.

- If a Reassociation Response frame is received with a status value of “successful,” the STA is now fully associated with the new AP. The state variable shall be set to State 3b, and the MLME shall issue an MLME-REASSOCIATE.confirm primitive indicating the successful completion of the operation.

- If a Reassociation Response frame is received with a status value other than “successful”, the STA is not fully associated with the AP. The MLME shall issue an MLME-REASSOCIATE.confirm primitive indicating the failure of the operation. The Status Code returned in the Reassociation Response frame indicates the cause of the failed full reassociation attempt. Any misconfiguration or parameter mismatch, e.g., data rates required as Basic Rates that the STA did not indicate as supported in the STA's Supported Rates information element, shall be corrected before the SME issues an MLME-REASSOCIATE.request for the same AP. If the Status Code indicates the reassociation failed because of a reason that is not related to configuration, e.g., the AP is unable to support additional associations, the SME shall not issue an MLME-REASSOCIATE.request for full association for the same AP, until a period of at least 2 seconds has elapsed.

11.3.2.46 AP reassociation procedures

Whenever a Reassociation Request frame that does not contain a Make Before Break Reassociation Information Element is received from a STA, the AP uses the following procedure to support reassociation:

- If the STA is not authenticated, the AP shall transmit a Deauthentication frame to the STA and terminate the reassociation procedure.

- In an RSNA, the AP shall check the values received in the RSN information element, to see whether the values received match the AP’s security policy. If not, the association shall not be accepted.

- Upon receipt of an MLME-Associate.response service primitive, the AP shall transmit a Reassociation Response frame with a status code as defined in 7.3.1.9. If the status value is “successful,” the association identifier assigned to the STA shall be included in the response.

- When the Reassociation Response frame with a status value of “successful” is acknowledged by the STA, the STA is considered to be associated with this AP. The state variable for the STA shall be set to State 3b.

- When the status value of the reassociation is not successful, the AP shall indicate a specific reason for the failure to reassociate in the Status Code of the Reassociation Response frame. If any Status Code value from Table 23 in 7.3.1.9 is an appropriate reason for the failure to reassociate, the AP shall indicate that Status Code value. The use of the unspecified reason value of the Status Code shall indicate the reassociation failed for a reason that is unrelated to every other defined Status Code value in Table 23.

- The SME shall establish an RSNA, or it shall enable WEP by calling MLME.SETPROTECTION.request primitive with ProtectType set to “Rx_Tx,” or it shall do nothing if it does not wish to secure communication.

- The SME will inform the DS of the new association.

The STA’s SME shall delete any PTKSA and temporal keys held for communication with the indicated STA by using MLME-DELETEKEYS.request primitive (see 8.4.10) upon receiving a MLME-REASSOCIATE.indication primitive that does not contain a Make Before Break Reassociation Information Element.

11.3.2.7 AP tentative reassociation procedures

Whenever a Reassociation Request frame contains an Make Before Break Reassociation Information Element and the Reassociation Type is set to Tentative Association is received from a STA, the AP uses the following procedure to support tentative reassociation:

- If the STA is not authenticated, the AP shall transmit a Deauthentication frame to the STA and terminate the reassociation procedure.

- In an RSNA, the AP shall check the values received in the RSN information element, to see whether the values received match the AP’s security policy. If not, the tentative association shall not be accepted.

- Upon receipt of an MLME-Associate.response service primitive, the AP shall transmit a Reassociation Response frame with a status code as defined in 7.3.1.9.  The Make Before Break Reassociation Information Element that is in the MLME-REASSOCIATE.response primitive shall be included in the Reassociation Response frame. If the status value is “successful,” the association identifier assigned to the STA shall be included in the response.

- When the Reassociation Response frame with a status value of “successful” is acknowledged by the STA, the STA is considered to be tentatively associated with this AP. The state variable for the STA shall be set to State 3a.

- When the status value of the reassociation is not successful, the AP shall indicate a specific reason for the failure to reassociate in the Status Code of the Reassociation Response frame. If any Status Code value from Table 23 in 7.3.1.9 is an appropriate reason for the failure to tentatively reassociate, the AP shall indicate that Status Code value. The use of the unspecified reason value of the Status Code shall indicate the tentative reassociation failed for a reason that is unrelated to every other defined Status Code value in Table 23.

- The SME shall establish an RSNA, or it shall enable WEP by calling MLME.SETPROTECTION.request primitive with ProtectType set to “Rx_Tx,” or it shall do nothing if it does not wish to secure communication.

The STA’s SME shall delete any PTKSA and temporal keys held for communication with the indicated STA by using MLME-DELETEKEYS.request primitive (see 8.4.10) upon receiving a MLME-REASSOCIATE.indication primitive that contains an Make Before Break Reassociation Information Element and the Reassociation Type is set to Tentative Association.

11.3.2.8 AP full reassociation procedures

Whenever a Reassociation Request frame contains a Make Before Break Reassociation Information Element and the Reassociation Type is set to Full Association is received from a STA, the AP uses the following procedure to support full reassociation:

- If the STA is not authenticated, the AP shall transmit a Deauthentication frame to the STA and terminate the reassociation procedure.

- Upon receipt of an MLME-Associate.response service primitive, the AP shall transmit a Reassociation Response frame with a status code as defined in 7.3.1.9.  The Make Before Break Reassociation Information Element that is in the MLME-REASSOCIATE.response primitive shall be included in the Reassociation Response frame. If the status value is “successful,” the association identifier assigned to the STA shall be included in the response.

- When the Reassociation Response frame with a status value of “successful” is acknowledged by the STA, the STA is considered to be fully associated with this AP. The state variable for the STA shall be set to State 3b.

- When the status value of the reassociation is not successful, the AP shall indicate a specific reason for the failure to reassociate in the Status Code of the Reassociation Response frame. If any Status Code value from Table 23 in 7.3.1.9 is an appropriate reason for the failure to fully reassociate, the AP shall indicate that Status Code value. The use of the unspecified reason value of the Status Code shall indicate the full reassociation failed for a reason that is unrelated to every other defined Status Code value in Table 23.

- The SME shall establish an RSNA, or it shall enable WEP by calling MLME.SETPROTECTION.request primitive with ProtectType set to “Rx_Tx,” or it shall do nothing if it does not wish to secure communication.

- The SME will inform the DS of the full association.

The STA’s SME shall delete any PTKSA and temporal keys held for communication with the indicated STA by using MLME-DELETEKEYS.request primitive (see 8.4.10) upon receiving a MLME-REASSOCIATE.indication primitive that contains a Make Before Break Reassociation Information Element and the Reassociation Type is set to Full Association.

11.3.2.59 STA disassociation procedures

Upon receipt of an MLME-DISASSOCIATE.request primitive, an associated STA shall disassociate from an AP using the following procedure:

- The STA shall transmit a Disassociation frame to the AP with which that STA is associated.

- The state variable for the AP shall be set to State 2 if and only if it was not State 1.

- The MLME shall issue an MLME-DISASSOCIATE.confirm primitive indicating the successful completion of the operation.

The STA’s SME shall delete any PTKSA and temporal keys held for communication with the indicated STA by using the MLME-DELETEKEYS.request primitive (see 8.4.10) and by invoking MLME-SETPROTECTION.request(None) before invoking the MLME-DISASSOCIATE.request primitive.

11.3.2.610 Non-AP STA disassociation receipt procedure

Upon receipt of a Disassociation frame, a STA shall operate as follows:

- The MLME shall issue an MLME-DISASSOCIATE.indication with the ReasonCode parameter set to the value of the Reason Code received in the Disassociation frame.

- The state variable for the AP shall be set to State 2 if and only if it was not State 1.

- If the Reason Code indicates a configuration or parameter mismatch as the cause of the disassociation, the STA shall not attempt to associate or reassociate with the AP sending the Disassociation frame, until the configuration or parameter mismatch has been corrected.

- If the Reason Code indicates the STA was disassociated for a reason other than configuration or parameter mismatch, the STA shall not attempt to associate or reassociate with the AP sending the Disassociation frame until a period of 2 seconds has elapsed.

The STA’s SME shall delete any PTKSA and temporal keys held for communication with the indicated STA by using the MLME-DELETEKEYS.request primitive (see 8.4.10) and by invoking MLME-SETPROTECTION.request(None) before invoking the MLME-DISASSOCIATE.request primitive.

11.8.711 AP disassociation initiation procedure

Upon receipt of a Disassociation frame from an associated STA, the AP shall disassociate the STA via the following procedure:

- The state variable for the STA shall be set to State 2.

- The MLME shall issue an MLME-DISASSOCIATE.indication primitive to inform the SME of the disassociation.

- The SME will update the DS.

The STA’s SME shall delete any PTKSA and temporal keys held for communication with the indicated STA by using the MLME-DELETEKEYS.request primitive (see 8.4.10) and by invoking MLME-SETPROTECTION.request(None) upon receiving a MLME-DISASSOCIATE.indication primitive.

TGr Editor: Delete 11.4.1.

### TGr Editor: Replace 11.4.3 with the following:

11.4.3 TS lifecycle

Replace figure 200, entitled “TS Lifecycle (Informative)” with the following:

Insert after the second paragraph of clause 11.4.3 (second paragraph starts “Initially a TS is inactive ...”):

A TS may be established by a TS Setup while in a Tentative Reassociation. Such a TS is created in the Accepted state. If the STA transitions to the Full Reassociation state with this AP, then the TS becomes Active. If the STA disassociates without transitioning to the Full Reassociation State, then the TS becomes Inactive.

Change the fourth paragraph of clause 11.4.3:

While the TS is aActive or Accepted, the parameters of the TSPEC characterizing the TS can be renegotiated, when the renegotiation is initiated by the non-AP QSTA. This negotiation can succeed, resulting in a change to the TSPEC, or can fail, resulting in no change to the TSPEC.

Annex A TBD

Annex D

(normative)

ASN.1 encoding of the MAC and PHY MIB

TGr editor: add the following changes to the draft:

Change the “Station Management (SMT) Attributes” of “Major Sections” of the MIB as follows:

--**********************************************************************

--* Major sections

--**********************************************************************

--

-- Station Management (SMT) Attributes

-- DEFINED AS “The SMT object class provides the necessary support

-- at the station to manage the processes in the station such that

-- the station may work cooperatively as part of an IEEE 802.11

-- network.

dot11smt OBJECT IDENTIFIER ::= { ieee802dot11 1 }

-- dot11smt GROUPS

-- dot11StationConfigTable ::= { dot11smt 1 }

-- dot11AuthenticationAlgorithmTable ::= { dot11smt 2 }

-- dot11WEPDefaultKeysTable ::= { dot11smt 3 }

-- dot11WEPKEYMappingsTable ::= { dot11smt 4 }

-- dot11PrivacyTable ::= { dot11smt 5 }

-- dot11SMTnotification ::= { dot11smt 6 }

-- dot11MultiDomainCapabilityTable ::= { dot11smt 7 }

-- dot11SpectrummanagementTable ::= { dot11smt 8 }

-- dot11RSNAConfigTable ::= { dot11smt 9 }

-- dot11RSNAConfigPairwiseCiphersTable ::= { dot11smt 10 }

-- dot11RSNAConfigAuthenticationSuitesTable ::= { dot11smt 11 }

-- dot11RSNAStatsTable ::= { dot11smt 12 }

-- dot11RegulatoryClassesTable ::= { dot11smt 13 }

-- dot11RadioResourceManagement ::= { dot11smt 14 }

-- dot11MakeBeforeBreakTable ::= { dot11smt 15 }

--

Change the “Dot11StationConfigEntry” of the “dotStationConfig TABLE” as follows:

-- *********************************************************************

-- * dotStationConfig TABLE

-- *********************************************************************

Dot11StationConfigEntry ::=

SEQUENCE {

dot11StationID					MacAddress,

dot11MediumOccupancyLimit			INTEGER,

dot11CFPollable					TruthValue,

dot11CFPeriod					INTEGER,

dot11CFPMaxDuration				INTEGER,

dot11AuthenticationResponseTimeOut		Unsigned32,

dot11PrivacyOptionImplemented			TruthValue,

dot11PowerManagementMode			INTEGER,

dot11DesiredSSID					OCTET STRING,

dot11DesiredBSSType				INTEGER,

dot11OperationalRateSet				OCTET STRING,

dot11BeaconPeriod					INTEGER,

dot11DTIMPeriod					INTEGER,

dot11AssociationResponseTimeOut		Unsigned32,

dot11DisassociateReason				INTEGER,

dot11DisassociateStation			MacAddress,

dot11DeauthenticateReason			INTEGER,

dot11DeauthenticateStation			MacAddress,

dot11AuthenticateFailStatus			INTEGER,

dot11AuthenticateFailStation			MacAddress,

dot11MultiDomainCapabilityImplemented	TruthValue,

dot11MultiDomainCapabilityEnabled		TruthValue,

dot11CountryString				OCTET STRING,

dot11SpectrumManagementImplemented		TruthValue,

dot11SpectrumManagementRequired		TruthValue,

dot11RSNAOptionImplemented			TruthValue,

dot11RSNAPreauthenticationImplemented	TruthValue,

dot11RegulatoryClassesImplemented		TruthValue,

dot11RegulatoryClassesRequired		TruthValue,

dot11QosOptionImplemented			TruthValue,

dot11ImmediateBlockAckOptionImplemented	TruthValue,

dot11DelayedBlockAckOptionImplemented	TruthValue,

dot11DirectOptionImplemented			TruthValue,

dot11APSDOptionImplemented			TruthValue,

dot11QackOptionImplemented			TruthValue,

dot11QBSSLoadOptionImplemented		TruthValue,

dot11QueueRequestOptionImplemented		TruthValue,

dot11TXOPRequestOptionImplemented		TruthValue,

dot11MoreDataAckOptionImplemented		TruthValue,

dot11AssociatedinNQBSS				TruthValue,

dot11DLSAllowdinQBSS				TruthValue,

dot11DLSAllowed					TruthValue,

dot11AssociateStation				MacAddress,

dot11AssociateID					INTEGER,

dot11AssociateFailStation			MacAddress,

dot11AssociateFailStatus			INTEGER,

dot11ReassociateStation				MacAddress,

dot11ReassociateID				INTEGER,

dot11ReassociateFailStation			MacAddress,

dot11ReassociateFailStatus			INTEGER,

dot11RadioMeasurementCapable				TruthValue,

dot11RadioMeasurementEnabled				TruthValue,

dot11RadioMeasurementProbeDelay			INTEGER,

dot11MeasurementPilotEnabled				TruthValue,

dot11MeasurementPilotPeriod				INTEGER,

dot11MeasurementPilotTransmitPriority		INTEGER,

dot11MakeBeforeBreakImplemented			TruthValue }

Insert the following at the end of the “dotStationConfig TABLE”, following “dot11RegulatoryClassesRequired”:

dot11MakeBeforeBreakImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This object indicates if the entity is make Before Break

capable."

::= { dot11StationConfigEntry 57 }

TGr Editor: Delete the changes to the “dot11AuthenticationAlgorithms TABLE”

TGr editor: delete the “dot11FastBSSTransitionConfig” section, and replace with the following:

Insert the following after the “dot11RegulatoryClasses TABLE” and before the “MAC Attribute Templates”:

-- ******************************************************************** -- * dot11MakeBeforeBreakConfig TABLE -- ******************************************************************** dot11MakeBeforeBreakConfigTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11MakeBefofreBreakConfigEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION "The table containing Make Before Break configuration objects."

::= { dot11smt 15 }

dot11MakeBeforeBreakConfigEntry OBJECT-TYPE

SYNTAX Dot11MakeBeforeBreakConfigEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11MakeBeforeBreakConfigTable."

INDEX { ifIndex }

::= { dot11MakeBeforeBreakConfigTable 1 }

Dot11MakeBeforeBreakConfigEntry ::=

SEQUENCE {

dot11MakeBeforeBreakEnabled			TruthValue }

dot11MakeBeforeBreakEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"When this object is set to TRUE, this shall indicate that Make Before Break is enabled on this entity. The entity will advertise the Make Before Break related Information Elements in its Beacon and Probe Response frames. This object requires that dot11MakeBeforeBreakImplemented also be set to TRUE."

::= { dot11MakeBeforeBreakConfigEntry 1 }

Insert at the end of the “Groups - units of conformance” as follows:

dot11MBBComplianceGroup OBJECT-GROUP

OBJECTS { dot11MakeBeforeBreakEnabled }

STATUS current

DESCRIPTION

"This object class provides the objects from the IEEE 802.11 MIB required to manage Make Before Break functionality. Note that additional objects for managing this functionality are located in the IEEE 802.11 Make Before Break MIB."

::= { dot11Groups 39 }

dot11SMTbase7 OBJECT-GROUP

OBJECTS { dot11MediumOccupancyLimit,

dot11CFPollable,

dot11CFPPeriod,

dot11CFPMaxDuration,

dot11AuthenticationResponseTimeOut,

dot11PrivacyOptionImplemented,

dot11PowerManagementMode,

dot11DesiredSSID,

dot11DesiredBSSType,

dot11OperationalRateSet,

dot11BeaconPeriod,

dot11DTIMPeriod,

dot11AssociationResponseTimeOut,

dot11DisassociateReason,

dot11DisassociateStation,

dot11DeauthenticateReason,

dot11DeauthenticateStation,

dot11AuthenticateFailStatus,

dot11AuthenticateFailStation,

dot11MultiDomainCapabilityImplemented,

dot11MultiDomainCapabilityEnabled,

dot11CountryString,

dot11SpectrumManagementImplemented,

dot11SpectrumManagementRequired,

dot11RSNAOptionImplemented,

dot11RegulatoryClassesImplemented,

dot11RegulatoryClassesRequired,

dot11QosOptionImplemented,

dot11ImmediateBlockAckOptionImplemented,

dot11DelayedBlockAckOptionImplemented,

dot11DirectOptionImplemented,

dot11APSDOptionImplemented,

dot11QAckOptionImplemented,

dot11QBSSLoadOptionImplemented,

dot11QueueRequestOptionImplemented,

dot11TXOPRequestOptionImplemented,

dot11MoreDataAckOptionImplemented,

dot11AssociateinNQBSS,

dot11DLSAllowedinQBSS,

dot11DLSAllowed,

dot11MakeBeforeBreakImplemented }

STATUS current

DESCRIPTION

"The SMTbase7 object class provides the necessary support at the STA to manage the processes in the STA so that the STA may work cooperatively as a part of an IEEE 802.11 net-work, when the STA is capable of multidomain operation. This object group should be implemented when the multido-main capability option is implemented."

::= { dot11Groups 40 }

References: