IEEE P802.11
Wireless LANs

| Text for resolving comments 1519, 3885, 6943 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-09-18 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Srinivas Kandala | Airgo Networks, Inc. | 900 Arastradero Road Palo Alto CA 95014 | (650) 475-1977 | srini@airgonetworks.com |

Abstract

This document contains the normative text for resolving comments 1519, 3885 and 6943.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Instruct the editor to change subclause 9.20.4 as shown below:

- Protection in 20/40 MHz BSS

An HT STA that receives a frame that requires the transmission of a response control frame shall select the format of the response frame as described in Response frame format in 20/40 MHz BSS.

An HT STA which transmits a response control frame shall send the frame using the same channel as that on which it received the frame that elicited the response transmission.

| - Response frame format in 20/40 MHz BSS |  |  |  |
| --- | --- | --- | --- |
| STA Mode | STA Rx Capability | Rx Event | Tx Response format |
| 20/40 MHz mode | 20/40 capable | non-HT duplicate | non-HT duplicate |
| 20/40 MHz mode | 20/40 capable that cannot distinguish between the reception of a non-HT PPDU and a non-HT duplicate PPDU | non-HT | non-HT (Ed: CID 51) |
| 20 MHz mode | 20/40 capable |  | HT or non-HT |

(Ed: CID 1453)

(Ed: CID 3887) An HT STA that receives a control frame in a 20 MHz PPDU that does not contain any feedback request or feedback (Ed: CID 1458) response transmits the required response frame using a non-HT frame, if such a response frame is required. (Ed: CID 1459)

(Ed: CID 3887) An HT STA that receives a control frame in a 40 MHz PPDU that does not contain any feedback request or feedback (Ed: CID 1458) response transmits the required response frame using a non-HT duplicate frame, if such a response frame is required (Ed: CID 1459).

.

(Ed: CID 3887) TheHT PPDU, if the response contains link adaptation and explicit channel state feedback as follows:

- the frame containing a link adaptation request (the “request frame”) shall be transmitted using the same channel width as the intended transmission width, and

- the frame containing feedback response shall be transmitted using the same channel width as the request frame.

- Following text being moved by approved comment resolution into 9.20.4 (above this note), I had to change the title back to the original “Protection in 20/40 MHz BSS”, as its contents no longer related to response frame generation. I then removed the duplicate heading that occurred at this point in the draft.

If the Operating mode subfield in the HT Information Element in the received beacon or probe response is set to 10 or 11 indicating that there are 20 MHz capable HT STAs and/or non-HT STAs associated in the BSS, then an HT STA transmitting a 40 MHz PPDU shall ensure that this transmission is protected

If such protection is needed, it shall be implemented in the one of the following ways:

- Use RTS/CTS exchang to protect the 40 MHz transmission. The Duration field in the RTS frame shall be set to cover the (first) frame exchange or the entire TXOP.

- Use CTS-to-self to protect the 40 MHz transmission . The  Duration field in the CTS frame shall be set so that it covers the (first) frame exchange or the entire TXOP.

- When 20 MHz non-HT STAs are associated in the BSS, transmit the MPDU at a 20 MHz transmission with the Duration field set to cover the (first) frame exchange or the entire TXOP.

- When only 20 MHz HT STAs are associated in the BSS, transmit the first PPDU as an MPDU or an A-MPDU at a 20 MHz transmission with the Duration field in the MPDU set to cover the (first) frame exchange or the entire TXOP.

- The HT STA that has 40 MHz transmission scheduled shall use the non-HT duplicate frame format for transmitting the RTS and CTS frames. The HT STA that receives an RTS frame using the non-HT duplicate frame format shall respond with a CTS frame using the non-HT duplicate frame format.

- There are certain modes in which 20 MHz wide transmissions need to be protected. For these the above two techniques may be used. Alternatively, the Duration field in the MPDU that is contained in the first PPDU shall cover the entire duration of the frame exchange or entire TXOP, as applicable.

A 40 MHz capable HT STA shall be able to receive non-HT duplicate frames on the primary channel. (Ed: CID 7841)

References: