IEEE P802.11
Wireless LANs

| LB84-CID-1751-Fragmentation-text-proposal |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-09-18 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Sanjiv Nanda | Qualcomm, Inc. | 5775 Morehouse Drive, San Diego, CA 92121 | 858 845 2375 | snanda@qualcomm.com |

Abstract

This document includes proposed text for modification of the TGn 1.04 draft. The text is intended to provide a resolution for CID 1751 on whether fragmentation is permitted for HT STA. In addition the following CIDs are also resolved: 1468. 3891.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

The following comments are addressed. These comments are marked deferred pending resolution of comment 1751.

| 1751 | 85 | 19 | 9.4 | T | Is it allowed for 11n devices to use fragmentation mechanism? Text in the BlockACK implies me to disallow fragmentation. I'm confused. | Clarify |
| --- | --- | --- | --- | --- | --- | --- |
| 1468 | 85 | 24 | 9.4 | T | Fragmentation is strictly forbidden for frames which are covered by compressed BA agreements. It is possible to have a coexisting fragmentation threshold and perform fragmentation on non-BA frames. | Modify the language to allow the possibility that some frames are sent unfragmented, even though they exceed the fragmentation threshold limit. |
| 3891 | 85 |  | 9.4 | DT | It is somewhat counter-intuitive that individual MSDUs, A-MSDUs and MMPDUs are subject to dot11FragmentationThreshold but not A-MPDU. | Indicate in the subclause that fragments shall not be aggregated into an A-MPDU (somewhat implied by the use of mandatory compressed BA) . |

Change paragraphs 1-3 in clause 9.1.5, on page 71 of TGn draft 1.03, as follows:

The process of partitioning an MSDU, an A-MSDU or an MMPDU into smaller MAC level frames, MPDUs, is called fragmentation. Fragmentation creates MPDUs smaller than the original MSDU, A-MSDU or MMPDU length to increase reliability, by increasing the probability of successful transmission of the MSDU, A-MSDU or MMPDU in cases where channel characteristics limit reception reliability for longer frames. QSTAs may use fragmentation to use the medium efficiently in consideration of the duration available in granted TXOPs, as long as the rules in 9.4 are followed. Fragmentation is accomplished at each immediate transmitter. The process of recombining MPDUs into a single MSDU, A-MSDU or MMPDU is defined as defragmentation. Defragmentation is accomplished at each immediate recipient.

Only MPDUs MSDUs, A-MSDUs or MMPDUs with a unicast receiver address may be fragmented. Broadcast/multicast frames shall not be fragmented even if their length exceeds dot11FragmentationThreshold. Unicast frames transmitted by an HT STA to an HT STA using Block Ack acknowledgement shall not be fragmented. HT-immediate or HT-delayed BlockAck agreements negotiated between HT STAs use only the compressed Block Ack format that does not permit the acknowledgment of fragments. An MSDU or A-MSDU transmitted under HT-immediate or HT-delayed BlockAck shall not be fragmented, even if its length exceeds the dot11FragmentationThreshold. If an MSDU, A-MSDU or MMPDU to be transmitted by a HT STA to another HT STA is fragmented, each fragment can only be acknowledged using a Fragment-ACK exchange.

When a directed MSDU is received from the LLC, or a number of MSDUs are aggregated to produce a directed A-MSDU or a directed MMPDU is received from the MLME that would result in a length greater than dot11FragmentationThreshold when after the MAC header and FCS are added, the MSDU, A-MSDU or MMPDU shall be fragmented, unless as noted above if the MSDU or A-MSDU is transmitted under an HT-immediate or HT-delayed BlockAck agreement. To avoid fragmentation following aggregation, implementations should limit the size of A-MSDU aggregates to be smaller than the dot11FragmentationThreshold. The MSDU or MMPDU is divided into MPDUs. Each fragment is a frame no larger than dot11FragmentationThreshold. It is possible that any fragment may be a frame smaller than dot11FragmentationThreshold. An illustration of fragmentation is shown in Figure 155.

Change paragraph 1 in clause 9.4 of REVma, as follows:

The MAC may fragment and reassemble directed MSDUs, A-MSDUs or MMPDUs. The fragmentation and defragmentation mechanisms allow for fragment retransmission. HT-immediate or HT-delayed BlockAck agreements negotiated between HT STAs use only the compressed Block Ack format that does not permit the acknowledgment of fragments. An MSDU or A-MSDU transmitted under HT-immediate or HT-delayed BlockAck shall not be fragmented, even if its length exceeds the dot11FragmentationThreshold. If an MSDU, A-MSDU or MMPDU to be transmitted by a HT STA to another HT STA is fragmented, each fragment can only be acknowledged using a Fragment-ACK exchange.