IEEE P802.11
Wireless LANs

| Normative Text Changes for Network Advice on Preferred Channel |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-NOV-15 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Jari Jokela | Nokia | Visiokatu 1, 33720 Tampere, Finland | +358504860445 | jari.jokela@nokia.com |
| Marian Rudolf | InterDigital Communications | Montreal, QC | 514-9046258 | Marian.Rudolf@InterDigital.com |
| Joe Kwak | InterDigital Communications | 482 Degas Bolingbrook, IL 60440 | 630-739-4159 | joekwak@sbcglobal.net |

Abstract

This document contains normative text for the use of Net Advice transmissions on Preferred Channel for Power Savings.   As explained in the accompanying presentation 06/0956r1, the proposed normative text changes define a new Network Advice frame and a Preferred Channel procedures which insure activity on the designated Preferred Channel for the regulatory class in use.  This ensures that STAs attempting network discovery by scanning need only to scan the appropriate set of Preferred Channels to determine if a BSS is operating in any locale.

[NOTE TO EDITOR:  Change bars are used to show changes and additions from draft 802.11vD0.05]

### 7.4 Action frame format details

### 7.4.6Wireless Network Management action details

Insert and renumber new clause.7.4.6.x after 7.4.6.4 as follows:

### 7.4.6.X Network Advice frame format

The Network Advice frame is used by STAs in a BSS to advertise the presence of the BSS on the Preferred Channel of the Regulatory Class for that BSS.. The Network Advice  frame body format is shown in Figure x1 .

|  | Country String | Regulatory Class | BSS Channel Number | Current TXID | Next TXID | Interval Timing |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: | 3 | 1 | 1 | 1 | 1 | 2 |

Figure x1—Network Advice frame body format

Country String indicates the value contained in the dot11CountryString attribute of the STA transmitting the Network Advice frame.

Regulatory Class indicates the channel set for the advertised BSS.. Regulatory Class and Channel Number together specify the channel frequency for the advertised BSS.   Valid values of Regulatory Class are shown in Annex J.

BSS Channel Number indicates the channel number for the advertised BSS. Channel Number is defined within a Regulatory Class as shown in Annex J.

Current TXID indicates the least significant octet of the MAC Address of the STA which is transmitting Network Advice frames in the current Network Advice Interval.

Next TXID indicates the least significant octet of the MAC Address of the STA which will transmit Network Advice frames in the next Network Advice Interval.

The Interval Timing field contains two subfields as defined in Figure x2.

|  | Network Advice Interval | Next Interval Start Time |
| --- | --- | --- |
| Bits: | 4 | 12 |

Figure x2—Interval Timing field format

Network Advice Interval subfield indicates the duration of the Network Advice Intervals in 10 second time units.

Next Interval Start Time indicates the remaining time until the start of the next Network Advice Interval in 50 msec units.

# 10. Layer management

## 10.3 MLME SAP Interface

Insert the following new clause after Clause 10.3.31 in 802.11v Draft 0.01 as indicated below:

### 10.3.x Network Advice

### 10.3.x Network Advice Request

This set of primitives supports the signaling of Network Advice Request frames between an AP and associated non-AP STAs.

#### 10.3.x.1 MLME-NETADVICE.request

##### 10.3.x.1.1 Function

This primitive requests transmission of a Network Advice frame on a Preferred Channel.

##### 10.3.x.1.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-NETADVICE.request	(
Country String,
Regulatory Class
BSS Channel Number
Current TXID
Next TXID
Interval Timing
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Country String | Octet String, 3 octets | 0-255 | The Country String is the dot11CountryString attribute, a shortened representation of the Country IE. |
| Regulatory Class | Integer | 0 – 255 | For the BSS Channel Number, as defined in AnnexJ. |
| BSS Channel Number | Integer | 0 – 255 | Channel number for the advertised BSS, defined within the Regulatory Class in Annex J. |
| Current TXID | Integer | 0-255 | Current TXID indicates the least significant octet of the MAC Address of the STA which is transmitting Net Advice frames in the current interval. |
| Next TXID | Integer | 0 – 255 | Next TXID indicates the least significant octet of the MAC Address of the STA which will transmit Net Advice frames in the next interval. |
| Interval Timingt | Integer | 0-65535 | Interval Timing consists of 2 timing subfields: 1. Net Advice Interval subfield which indicates the duration of the Network Advice Intervals in 10 second time units, and 2. Next Interval Start Time which indicates the remaining time until the start of the next Network Advice Interval in 50 msec units. |

##### 10.3.x.1.3 When Generated

This primitive is generated by the SME to request that a Network Advice frame be sent on the Preferred Channel to advertise the existence of an operating BSS on the channel indicated in the Network Advice frame. This request is sent regularly during the  Network Advice Interval by the on-duty Network Advice STA. This request may also be sent by a STA to accept on-duty assignment for transmission of Network Advice frames for the following Network Advice Interval.

##### 10.3.x.1.4 Effect of Receipt

On receipt of this primitive, the MLME constructs a Network Advice  frame of action type. The STA then attempts to transmit this frame on the Preferred Channel for the current Regulatory Class.

#### 10.3.x.2 MLME-NETADVICE.confirm

##### 10.3.x.2.1 Function

This primitive reports the result of a request to send a Network Advice frame on the Preferred Channel.

##### 10.3.x.2.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-NETADVICE.confirm	(
ResultCode,
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| ResultCode | Enumeration | SUCCESS, INVALID_ PARAMETERS, TIMEOUT, TRANSMISSION_FAILURE, UNSPECIFIED_ FAILURE | Indicates the result of the corresponding MLME-NETADVICE.request. |

##### 10.3.x.2.3 When Generated

This primitive is generated by the MLME when the request to transmit a Network Advice Request frame completes and indicates the results of the request.

##### 10.3.x.2.4 Effect of Receipt

On receipt of this primitive, the SME evaluates the ResultCode.

#### 10.3.x.3 MLME-NETADVICE.indication

##### 10.3.x.3.1 Function

This primitive indicates that a Network Advice Request frame was received from the AP with which the STA is associated.

##### 10.3.x.3.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-NETADVICE.indication	(
Country String,
Regulatory Class
BSS Channel Number
Current TXID
Next TXID
Interval Timing
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Country String | Octet String, 3 octets | 0-255 | The Cuntry String is the dot11CountryString attribute, a shortened representation of the Country IE. |
| Regulatory Class | Integer | 0 – 255 | For the BSS Channel Number, as defined in AnnexJ. |
| BSS Channel Number | Integer | 0 – 255 | Channel number for the advertised BSS, defined within the Regulatory Class in Annex J. |
| Current TXID | Integer | 0-255 | Current TXID indicates the least significant octet of the MAC Address of the STA which is transmitting Net Advice frames in the current interval. |
| Next TXID | Integer | 0 – 255 | Next TXID indicates the least significant octet of the MAC Address of the STA which will transmit Net Advice frames in the next interval. |
| Interval Timingt | Integer | 0-65535 | Interval Timing consists of 2 timing subfields: 1. Net Advice Interval subfield which indicates the duration of the Network Advice Intervals in 10 second time units, and 2. Next Interval Start Time which indicates the remaining time until the start of the next Network Advice Interval in 50 msec units. |

##### 10.3.x.3.3 When Generated

This primitive is generated by the MLME when a valid Network Advice frame is received.

##### 10.3.x.3.4 Effect of Receipt

On receipt of this primitive the SME shall operate according to the procedure in 11.15.x.

- 11  MAC sublayer management entity

- 11.15  Wireless Network Management Procedures

Insert and renumber new clause 11.15.x after 11.15.3 as follows:

- 11.15.X Peferred Channel Procedures

The Preferred Channel procedures permit a STA to save power while searching for suitable BSSs for WLAN service. Preferred Channel Procedures decrease the number of channels to be scanned while a STA is discovering available networks and BSSs.

Each defined Regulatory Class in Annex J includes a Channel Set which is an ordered list of channels in that regulatory class. The Preferred Channel is selected randomly from the Channel Set for each regulatory class.  The Preferred Channel Procedures guarantee periodic transmissions on the Preferred Channel providing network access information.  In this way, only the Preferred Channel needs to be scanned within any regulatory class.

The Preferred Channel changes daily and is randomly selected from the Channel Set list of channels using the same RC4  PRNG used for WEP encryption.  The Preferred Channel Order is obtained as follows:

Preferred Channel Order = (call RC4(SEED,64))modQ

where a call to RC4() returns a random 64 bit number,

where Q is the number of channels in a defined channel set,

and where  SEED= DATE*10E12 +DATE*10E6 +DATE ,

where 	DATE= YYYY*365 + MM*31 +DD,

and where DDMMYYYY is the current date.

If the Preferred Channel Order is 0, the Preferred Channel for that  regulatory class is the first channel listed in the channel set.  If the Preferred Channel Order is 1, the Preferred Channel is the second channel listed, and so on.

- 11.15.X.1  AP Procedure for Preferred Channel

The AP shall use an initial channel selection algorithm which favors selection of the Preferred Channel for beacon transmission when initiating operation of an infrastructure BSS..

When an AP transmits beacons for an infrastructure BSS on a non-Preferred Channel it shall periodically monitor the Preferred Channel for frame transmissions. If a Network Advice frames is detected on the Preferred Channel, the AP may set dot11NetworkAdviceEnabled to true in STAs in the BSS in order to advertise the BSS on the Preferred Channel.

If no beacons are detected and no network advice frames are detected on the Preferred Channel, the AP shall set dot11NetworkAdviceEnabled to true in one or more STAs in the BSS.  When an AP has set dot11NetworkAdviceEnabled to true at any STA, the AP shall ensure that transmission on the Preferred Channel is permitted by local regulations.  If local regulations indicate that transmission on the Preferred Channel is not allowed, the AP shall set dot11NetworkAdviceEnabled to false in all STAs in the BSS.

- 11.15.X.2  STA Procedure for Preferred Channel

A STA which is initiating an IBSS shall use a channel selection algorithm which disfavors selection of the Preferred Channel for beacon transmission.

STA procedures for Preferred Channel are dependent on the NetworkAdviceDuty state variable.  The NetworkAdviceDuty state variable has three states:  NO_DUTY, ON_DUTY, OFF_DUTY.  The default state for NetworkAdviceDuty is NO_DUTY.

If dot11NetworkAdviceEnabled is true and the STA state is NO_DUTY, the  STA shall periodically monitor the Preferred Channel for frame transmissions. If a Network Advice frame is detected on the Preferred Channel, the STA shall use the Network Advice Timing information in the received frame to update and track Network Advice Interval Timing and shall change state to OFF_DUTY.  If no beacons are detected and no network advice frames are detected on the Preferred Channel, the STA shall change state to ON_DUTY and shall set Network Advice Interval Timing to default values.

If dot11NetworkAdviceEnabled is true and the STA state is ON_DUTY and at the end of the current Network Advice Interval, the STA shall switch to the Preferred Channel and begin transmitting Network Advice frames at the beacon transmission rate using the timing rules which apply for beacon transmissions.  The STA shall schedule Network Advice frame transmissions once at each TBTT.  The Network Advice frame shall include: a Current TXID field indicating the MAC address of the STA, a Next TXID field set to 0, and Interval Timing field set to indicate the beginning of the next Interval.  If the STA receives a Network Advice frame with a non-zero Next TXID field, it shall transmit (echo) the received frame and use the Next TXID value from the received frame in subsequent Network Advice frame transmissions for the duration of this Network Advice Interval.  After transmitting a Network Advice Frame with a non-zero Next TXID field, the STA may switch back to the operating channel.  At the next TBTT and after receiving the beacon on the operating channel, the STA shall switch to the Preferred Channel, shall transmit a Network Advice frame and then may switch back to the operating channel.   At the end of the current Network Advice Interval and if the STA has transmitted a NetworkAdviceFrame with a non-zero TXID during this interval, the STA shall discontinue transmitting Network Advice frames and shall change state to OFF_DUTY.  If dot11NetworkAdviceEnabled is true and if the STA state is ON_DUTY and if the STA is receiving frames on the Preferred Channel, then if the STA receives a beacon frame, the STA shall cease transmitting Network Advice frames, shall change state to NO_DUTY, and may switch to the operating channel.

If NetworkAdviceEnabled is true and the STA state is OFF_DUTY and at the end of the current Network Advice Interval, the STA shall switch to the Preferred Channel and begin frame reception.  If the STA receives a Network Advice frame with Next TXID set to 0, the STA shall randomly select a time before the next TBTT.  If the STA receives a Network Advice frame with a non-zero Next TXID field which does not indicate its own MAC address, the STA shall switch back to its operating channel. At the randomly selected time and if the STA is on the Preferred Channel, the STA shall transmit a Network Advice frame with the Next TXID field set to indicate the STA's own MAC address. If the STA then receives a Network Advice frame with the Next TXID field set to indicate its own MAC address, the STA shall change state to ON_DUTY and switch to the operating channel.  If dot11NetworkAdviceEnabled is true and if the STA state is OFF_DUTY and if the STA is receiving frames on the Preferred Channel, then if the STA receives a beacon frame, the STA shall change state to NO_DUTY, and may switch to the operating channel.

Insert the following new rows in clause A.4.14:

### A. 4.14 Wireless Network Management extensions

| Item | Protocol Capability | References | Status | Support |
| --- | --- | --- | --- | --- |
| RME5 | Preferred Channel Procedures | 11.15.X | CFv:M | Yes, No, N/A |
| RME5.1 | AP Procedure for Preferred Channel | 7.4.6.X, 11.15.X.1 | (CFv AND CF1):M | Yes, No, N/A |
| RME5.2 | STA Procedure for Preferred Channel | 7.4.6.(X), 11.15.X.2 | CFv :M | Yes, No, N/A |