IEEE P802.11
Wireless LANs

| LB84-Clause-11.9.5-Comments-Classification |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-09-18 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Sanjiv Nanda | Qualcomm, Inc. | 5775 Morehouse Drive, San Diego, CA 92121 | 858 845 2375 | snanda@qualcomm.com |

Abstract

This document reviews all the comments related to clause 11.9.5 of D1.0 and identifies the issues that need to be resolved to address the comments. These issues require submissions and review within the Coexistence adhoc.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

This document reviews all the comments related to clause 11.9.5 of D1.0 and identifies the issues that need to be resolved to address the comments. These issues require submissions and review within the Coexistence adhoc.

Coexistence Issue A: CCA on the extension channel

This is already under consideration by the adhoc and a number of contributions have addressed this issue. This addresses CIDs 3614 and 1521.

Coexistence Issue B: Scanning for OBSS

Several commenters propose that rules must be specified for scanning for OBSS at the time that an AP or STA initializes a BSS, and periodically during BSS operation. A submission is required. This would address 8 CIDs.

Coexistence Issue C: Non-HT Duplicate Mode at 2.4 GHz

This is already under consideration by the adhoc and contributions have addressed this issue. This addresses CID 7154.

Editorial Issues with clause 11.9.5

These issues are being addressed in Document 1413r0 (Author: Assaf Kasher).

| 3614 | 149 |  | 11.9.5 | The indicated "co-existence" is not sufficient. There is no guidance on how often the scanning should be done; how the scanning should be performed during large data transfer. All the text appears to say is "apple pie is good, be nice" | Mandate that all stations maintain a extension channel CCA and will combine it with control channel CCA for their backoff counter decrement and transmission decisions. Further mandate that the AP should monitor traffic periodically (by scanning the channel at least once every dot11ScanPeriod) on the extension channel and use DFS based on the amount of load on the extension channel. Of course, I would greatly appreciate if a more thorugh mechanism is put in, but the above is a minimum. | SN - Coexistence Issue B: Scanning for OBSS. Needs submission on specifying minimum requirements for AP and STA to scan the extension channel before starting BSS and during BSS operation. Same as CID 12112. Coexistnece Issue A: CCA on extension channel |
| --- | --- | --- | --- | --- | --- | --- |
| 1061 | 150 | 2 | 11.9.5.1 | do you mean "BSSs" instead of "devices" | change "devices" to "BSSs" | SN - Counter. Change devices to STA |
| 12112 | 150 | 7 | 11.9.5.2 | Manual configuration ends up as "users chooses 40 MHz on their lucky number channel", and thus is not backwards compatible with OBSSs on the extension channel | Manual configuration of 40 MHz must be disallowed without a prior gating scan of the overlapping channels, and subsequent in-service scanning of the overlapping channels, including the requirement to vacate busy channels whose CCA and virtual NAV cannot be respected. | SN - Coexistence Issue B: Scanning of OBSS. Needs submission on specifying minimum requirements for AP and STA to scan the extension channel before starting BSS and during BSS operation. |
| 12246 | 150 | 7 | 11.9.5.2 | Manual configuration ends up as "users chooses 40 MHz on their lucky number channel", and thus is not backwards compatible with OBSSs on the extension channel | See previous | SN - Duplicate of 11212. Coexistence Issue B |
| 1521 | 150 | 9 | 11.9.5.2 | Optional scanning of an overlapped channel when the device cannot respect the CCA or virtual NAV of the overlapped channel is not backwards compatible with OBSSs on the extension channel. | A station should sense CCA (and virtual NAV) on every channel it is about to transmit on. | SN - Coexistnece Issue A: CCA on extension channel |
| 12113 | 150 | 9 | 11.9.5.2 | Optional scanning of an overlapped channel when the device cannot respect the CCA or virtual NAV of the overlapped channel is not backwards compatible with OBSSs on the extension channel. | Prior and in-service scanning must be mandatory. Testable performance requirements for this scanning must be provided. | SN - Coexistence Issue B: Scanning of OBSS. Needs submission on specifying minimum requirements for AP and STA to scan the extension channel before starting BSS and during BSS operation. Duplicate of 12112. |
| 12247 | 150 | 9 | 11.9.5.2 | Optional scanning of an overlapped channel when the device cannot respect the CCA or virtual NAV of the overlapped channel is not backwards compatible with OBSSs on the extension channel. | See previous | SN - Coexistence Issue B: Scanning of OBSS. Needs submission on specifying minimum requirements for AP and STA to scan the extension channel before starting BSS and during BSS operation. Duplicate of 12112. |
| 12114 | 150 | 12 | 11.9.5.2 | "The AP should reselect new parameters if an HT BSS that does not have the same control channel, extension channel offset starts operating on an overlapped channel". This will cause thrashing and does not promote a good frequency pan | We should define preferred 40MHz bands, and preferred control channels. In the absence of radar, the HT AP that does not conform to the preferred channelization must be the AP that must reselect its parameters. | SN - Reject. 40 MHz bands are predefined. If there is a 20 MHz OBSS, then it is preferable for the control channel to overlap with the OBSS, than the extension channel. Therefore, for a 20/40 BSS a preferred control channel is not specified. |
| 12248 | 150 | 12 | 11.9.5.2 | "The AP should reselect new parameters if an HT BSS that does not have the same control channel, extension channel offset starts operating on an overlapped channel". Simple statements like this do not solve the very complex issue of channel management in a hererogenous channel environment | Remove all statements regarding how to solve channel management. Omly leave the requirement to avoid collisions on the extension channel. | SN - Deferred. Pending resolution of 12112. |
| 1062 | 150 | 14 | 11.9.5.2 | I cannot make sense of the phrase: "Given a choice of 40MHz channel selections recommended transmission by the previous rules," | help! | SN - Accept. See document 1413 |
| 6758 | 150 | 14 | 11.9.5.2 | Based upon the current channel assignments, it likely that adjacent BSS's assign overlapping frequencies causing coexistance problems especially for 40 MHz channeliztion. | Definite channelization separately for 20 and 40 MHz which eliminate the possibility for overlapping. | SN - Accept. See Table n78 (D1.03) |
| 6879 | 150 | 14 | 11.9.5.2 | The recommended practice here is contradictory with the mandatory requirement for the 5 GHZ band in the PHY | Replace this paragraph to make it consistent with the PHY | SN - Accept |
| 7154 | 150 | 14 | 11.9.5.2 | The 20/40 channel coexistence--and indeed much else in the draft--works best if channels align on 20 MHz and 40 MHz centers. As the channel selection for 20/40 is written, it is likely that neighboring BSSs will often asssign partially overlapping frequency bands by chosing any of the permitted frquency channels, making multiple neighbor BSS co-existence problematic. | Require that 11n APs (and STAs in an IBSS) operating in 20 MHz mode select their frequency channel only from a set of explicitly specified (in the standard, as a function of regulatory region) set of channels that are seperated by 20 MHz (instead of allowing all channels). Require that 11n APs (and STA in an IBSS) operating in 40 MHz mode select their frequency channel from a set of explicitly specified channels that are seperated by 40 MHz. (The specified set of frequency channels for 40 MHz operations must be consistent with the list for 20 MHz such that the 40 MHz channels consist of two 20 MHz channels.) It may not be deemed expedient to seperate the specified channels by 20 MHz at 2.4 GHz (as this is contrary to current practice) in which case the legacy duplicate mode should be forbidden at 2.4 GHz, but the channelization should still be explicitly specified to prevent partial overlap of neighboring BSSs or IBSSs. (See my comment on section 20.1.3.) | SN - Accept. Except that for Coexistence Issue C: Specification of non-HT duplicate mode at 2.4 GHz |
| 7200 | 150 | 14 | 11.9.5.2 | Text does not match the rule stated in the tables n78 and n79 within PHY sections (20.3.8.1 and 20.3.8.2). | Fix it. | SN - Accept. See document 1413 |
| 1065 | 150 | 20 | 11.9.5.3 | it is not clear that an AP "has" a channel when it powers up, but I think that I can see the intent here - but there is no period of time specified, so the statement is sort of pointless as written | change the sentence beginning with "An HT AP" to "An HT AP shall receive the beacons of other HT BSSs on its desired channel of operation for ProbeDelay time before starting a BSS." | SN - Accept. |
| 2820 | 150 | 20 | 11.9.5.3 | Can't mandate that a STA receive anything. Too many unknowns, like signal strength and interference | Change to "An HT AP shall listen for Beacons of other…" | SN - Counter. CID 1065 |
| 12115 | 150 | 20 | 11.9.5.3 | No performance requirements are mentioned | Prior and in-service scanning must be mandatory. Testable performance requirements for this scanning must be provided. | SN - Coexistence Issue B: Scanning of OBSS |
| 6880 | 150 | 22 | 11.9.5.3 | How can the AP do "background scanning"? This places a requirement for the AP to have multiple radios. | Delete this sentence | SN - Counter. SN - Coexistence Issue B: Scanning of OBSS |
| 12116 | 150 | 22 | 11.9.5.3 | No performance requirements are mentioned | Prior and in-service scanning must be mandatory. Testable performance requirements for this scanning must be provided. | SN - Coexistence Issue B: Scanning of OBSS |
| 1066 | 150 | 25 | 11.9.5.3 | seems like it is not necessary for a STA to dissociate - why is it not possible for the STA to simply also switch to the 20MHz mode? | change "The STA" to "A STA" change "de-associate" to "disassociate" and change the "shall" to "may" | SN - Counter. Removed the sentence. |
| 4533 | 150 | 27 | 11.9.5.3 | This needs to be much more specific | Specify rules that the AP must follow to ensure that power-save STAs are informed about the channel transition | SN - Accept. See document 1413 |
| 6881 | 150 | 27 | 11.9.5.3 | Too vague. | Specify rules that the AP must follow to ensure that power-save STAs are informed about the channel transition | SN - Accept. See document 1413 |
| 7511 | 150 |  | 11.9.5.3 | "An HT AP shall receive the beacons of other HT BSSs on its channel when it first powers up. " This doesn't say how long, or what "first powers up" means. | This is actually a requirement on an SME. It should be described in terms of the MLME primitives. | SN - Counter. CID 2820 |
| 7512 | 150 |  | 11.9.5.3 | "In any transition of channel width, the AP should take in consideration any STAs that asleep. " Considerate APs are nice. But what this means is opaque. Unlike the channel switch announcement, the channel width change takes place immediately. It is not clear how the AP can be considerate. | Remove the sentence. | SN - Counter. See document 1413 |