IEEE P802.11
Wireless LANs

| AES-CMAC MIC Support |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-09-20 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Dorothy Stanley | Aruba Networks | 1322 Crossman Ave Sunnyvale, CA 94089 | +1 630-363-1389 | dstanley@arubanetworks.com |
| TGr Participants |  |  |  |  |

Abstract

This document provides text changes to add the ability to use AES-CMAC as the algorithm for FT key management MIC calculations, and to use the same MIC algorithm for protection of key management data included in EAPOL-Key messages, re-association request and response messages, action frames and authentication frames. Text changes to extend use of AES-CMAC to non-FT RSNAs are also included.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Insert the following Normative reference in Clause 2:

FIPS SP800-38B- Dworkin, M., “Recommendation for Block Cipher

Modes of Operation: The CMAC Mode for Authentication”, May 2005.

    - EAPOL-Key Frames

Insert text in Clause 8.5.2 list item 1, as sub-bullet item iii as shown below:

i) The value 1 shall be used for all EAPOL-Key frames to and from a STA when neither the

group nor pairwise ciphers are CCMP for Key Descriptor 1. This value indicates the

following:

—HMAC-MD5 is the EAPOL-Key MIC.

—ARC4 is the EAPOL-Key encryption algorithm used to protect the Key Data field.

ii) The value 2 shall be used for all EAPOL-Key frames to and from a STA when either the

pairwise or the group cipher is AES-CCMP for Key Descriptor 2. This value indicates the

following:

—HMAC-SHA1-128 is the EAPOL-Key MIC. HMAC is defined in IETF RFC 2104; and

SHA1, by FIPS PUB 180-1-1995. The output of the HMAC-SHA1 shall be truncated

to its 128 MSBs (octets 0–15 of the digest output by HMAC-SHA1), i.e., the

last four octets generated shall be discarded.

—The NIST AES key wrap is the EAPOL-Key encryption algorithm used to protect the

Key Data field. IETF RFC 3394 defines the NIST AES key wrap algorithm.

iii) The value 3 shall be used for all EAPOL-Key frames to and from a STA when

either the pairwise or the group cipher is AES-CCMP and dot11RSNAAuthenticationSuiteSelected is 3 or 4 (part of a Fast BSS Transition in an RSN). This value indicates the following:

—AES-CMAC is the EAPOL-Key MIC. AES-CMAC is defined by FIPS SP800-38B. The output of the AES-CMAC shall be 128 bits.

—The NIST AES key wrap is the EAPOL-Key encryption algorithm used to protect the

Key Data field. IETF RFC 3394 defines the NIST AES key wrap algorithm.

7.3.2.43 Fast BSS transition information element (FTIE)

Change from:

The MIC Control field is two octets, and is defined in Figure 112N. The MIC algorithm is one of the values from Table 43B: The Information Element Count of the MIC Control field contains the number of information elements that are included in the MIC calculation. A value of zero indicates no MIC is present.

To:

The MIC Control field is two octets, and is defined in Figure 112N. The Information Element Count of the MIC Control field contains the number of information elements that are included in the MIC calculation. A value of zero indicates no MIC is present.

Delete table 43B -MIC algorithm negotiation

Change Figure 112N to delete the MIC algorithm text, and indicate that B0-B7 comprise a single reserved field.

8A.5.3 FT authentication sequence: contents of fast BSS transition confirm

Change from:

— The MIC algorithm of the MIC Control field shall be set to one of the values given in Table 43B in 7.3.2.43.

— The MIC shall be calculated using the KCK, by the algorithm selected by the MIC algorithm of the MIC Control field, on the concatenation, in the following order, of:

To

- The MIC shall be calculated using the KCK and the HMAC-MD5 algorithm when neither the pairwise nor the group cipher is AES-CCMP.

- The MIC shall be calculated using the KCK and the AES-CMAC algorithm when either the pairwise or group cipher is AES-CCMP. The output of the AES-CMAC shall be 128 bits.

- The MIC shall be calculated on the concatenation, in the following order, of:

8A.5.4 FT authentication sequence: contents of fast BSS transition acknowledgement

Change from:

- The MIC algorithm of the MIC Control field shall be set to one of the values given in Table 43B in 7.3.2.43.

- When this message of the authentications equence appears in a Reassociation Response, the Optional parameters may include a GTK. If a GTK is included, it shall be encrypted as follows:

- If the MIC algorithm is 1 (HMAC-MD5), then the GTK shall be encrypted using ARC4

- If the MIC algorithm is 2 (HMAC-SHA1-128), then the GTK shall be encrypted using NIST AES Key Wrap algorithm, as defined in RFC 3394.

- The MIC shall be calculated using the KCK, by the algorithm selected by the MIC algorithm of the MIC Control field, on the concatenation, in the following order, of:

To

- When this message of the authentication sequence appears in a Reassociation Response, the Optional parameters may include a GTK. If a GTK is included, it shall be encrypted using ARC4 when neither the pairwise nor the group cipher is AES-CCMP. It shall be encrypted using the NIST AES Key Wrap algorithm when either the pairwise or group cipher is AES-CCMP.

- The MIC shall be calculated using the KCK and the HMAC-MD5 algorithm when neither the pairwise nor the group cipher is AES-CCMP.

- The MIC shall be calculated using the KCK and the AES-CMAC algorithm when either the pairwise or group cipher is AES-CCMP. The output of the AES-CMAC algorithm shall be 128 bits.

- The MIC shall be calculated on the concatenation, in the following order, of:

In Clauses

8.5.3.1, 8.5.3.2, 8.5.3.3, 8.5.3.4, 8.5.4.1, 8.5.4.2, 8.5.8.1.1, 8.5.8.1.2,

8.5.8.1.3, 8.5.8.1.4, 8.5.8.1.5, 8.5.8.3, 8.5.8.4,

Change from

Key Descriptor Version = 1 (ARC4 encryption with HMAC-MD5) or 2 (NIST AES key wrap

with HMAC-SHA1-128)

to

Key Descriptor Version = 1 (ARC4 encryption with HMAC-MD5) or 2 (NIST AES key wrap

with HMAC-SHA1-128) or 3 (NIST AES key wrap with AES-CMAC)

Part 2: Changes to add use of AES-CMAC to non-FT RSNA

- Need a mechanism for the STA/Supplicant to indicate to the AP/Authenticator that AES-CMAC is supported. Options for this mechanism include:

  - Add a MIC AKM Suite selector field to the RSNIE and increment the RSNIE version field.

  - Use a bit from the RSNIE capabilities field to indicate AES-CMAC support; this option is selected and the required text changes are shown below.

  - New information element

At the end of clause 7.3.2.25.3,

Change from

— Bits 6–8 and 10–15: Reserved. The remaining subfields of the RSN Capabilities field are reserved

and shall be set to 0 on transmission and ignored on reception.

To

- Bit 6: AES-CMAC: This bit is set when the STA’s Supplicant supports AES-CMAC as a MIC cipher suite.

— Bits 7–8 and 10–15: Reserved. The remaining subfields of the RSN Capabilities field are reserved

and shall be set to 0 on transmission and ignored on reception.

And change Figure 91 to insert “AES-CMAC” in B6, and show B7-8 as reserved.

- Need changes to the 8.5.2 text to indicate which suites can be used:

Insert the following text in Clause 8.5.2 list item 1 as shown below:

iii) The value 3 shall be used for all EAPOL-Key frames to and from a STA when

either the pairwise or the group cipher is AES-CCMP and dot11RSNAAuthenticationSuiteSelected is 3 or 4 (part of a Fast BSS Transition in an RSN). The value 3 shall be used for all EAPOL-Key frames to and from a STA when either the pairwise or the group cipher is AES-CCMP and the RSNA capabilities field B6 (AES-CMAC) is set to 1. This value indicates the following:

References:

TGr reflector e-mail: Lily Chen – Keep 128bit MIC