IEEE P802.11
Wireless LANs

| TGn LB84 Submission for HT Duplicate CIDs 253 and 692 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-09-20 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Eldad Perahia | Intel Corporation |  |  | eldad.perahia@intel.com |
| Assaf Kasher | Intel Corporation | POB 1659, Haifa 31015, Israel | 97248651547 | assaf.kasher@intel.com |
| Gal Basson | Intel Corporation | POB 1659, Haifa 31015, Israel |  | gal.basson@intel.com |
| Vinko Erceg | Broadcom | 15435 Innovation Dr. San Diego, CA 92128 | 1-858-521-5885 | verceg@broadcom.com |
| Jason Trachewsky | Broadcom |  |  | jat@broadcom.com |

Abstract

This document contains proposed changes to the IEEE P802.11n Draft to address the following LB84 comments:

253 and 692

The changes marked in this document are based on TGn Draft version P802 11n D1 03.pdf.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# Introduction

## Interpretation of a Motion to Adopt

A motion to approve this submission means that the editing instructions and any changed or added material are actioned in the TGn Draft.  This introduction, is not part of the adopted material.

Editing instructions formatted like this are intended to be copied into the TGn Draft (i.e. they are instructions to the 802.11 editor on how to merge the TGn amendment with the baseline documents).

TGn Editor:  Editing instructions preceded by “TGn Editor” are instructions to the TGn editor to modify existing material in the TGn draft.   As a result of adopting the changes, the TGn editor will execute the instructions rather than copy them to the TGn Draft.

Summission Note: Notes to the reader of this submission are not part of the motion to adopt.  These notes are there to clarify or provide context.

# Proposed Resolution

| CID | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- |
| 253 | Pilot sequence for second space-time stream, when MCS = 32 and STBC is used, is not defined. | Use the following pilot sequence for the 2nd space-time stream (MCS=32 and STBC): 1 (-21 pilot location), 1 (-7), -1 (7), and 1 (21). The same pseduo-random cover sequence should be used to modulated the pilots on the 2nd space-time stream. | Counter: disallow the use of STBC with MCS 32 as per 06/YYYY |
| 692 | For the specific MCS 32 (STBC) there is not explicit pilot pattern defined | Clarify and extend pilot defininition | Counter: disallow the use of STBC with MCS 32 as per 06/YYYY |

# CID 253, 692

TGn Editor: Modify Table n79—Constellation mapper output to spatial mapper input  for STBC (Draft 1.03) as follows (remove MCS 32 from second row, second column)

- Table n79—Constellation mapper output to spatial mapper input  for STBC

|  | HT-SIG MCS field (bits 0-6 in HT-SIG1 ) |  | HT-SIG STBC field (bits 4–5 in HT-SIG2 ) |  | Space time stream in OFDM symbol (2n) | Space time stream in OFDM symbol (2n+1) |
| --- | --- | --- | --- | --- | --- | --- |
| 2 | 0-7 | 1 | 1 | 1 |  |  |
|  |  |  |  | 2 |  |  |
| 3 | 8-15, 33-38 | 2 | 1 | 1 |  |  |
|  |  |  |  | 2 |  |  |
|  |  |  |  | 3 |  |  |
| 4 | 8-15 | 2 | 2 | 1 |  |  |
|  |  |  |  | 2 |  |  |
|  |  |  |  | 3 |  |  |
|  |  |  |  | 4 |  |  |
| 4 | 16-23, 39, 41, 43, 46, 48, 50 | 3 | 1 | 1 |  |  |
|  |  |  |  | 2 |  |  |
|  |  |  |  | 3 |  |  |
|  |  |  |  | 4 |  |  |

TGn Editor: Modify text in clause 21.3.10.9 page 196 line 27(Draft 1.03)

In the case of 40MHz transmissions (excluding MCS 32)  pilot signals are inserted in sub-carriers -53, -25, -11, 11, 25, 53; the pilot sequence for the nth symbols and iSTSth space time stream is defined as follows:

TGn Editor: Delete text in clause 21.3.10.9 page 196 line 59-65(Draft 1.03)

“In the non-HT and HT duplicate modes there are 8 pilots in sub-carriers -53, -39, -25, -11, 11, 25, 39, 53.

The pilot values are the same as the values in sub-carriers -21, -7, 7, 21 in a 20 MHz transmission, repeated

in the negative and positive sub-carriers and rotated 90° in the upper sub-carriers relative to the lower subcarriers.”

TGn Editor: Modify the title of Table n81 (Draft 1.03) as follows

Table n81—Pilots values for 40 MHz transmission (excluding MCS 32)