IEEE P802.11
Wireless LANs

| LB84-multirate-support-text-proposal |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-11-11 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Solomon Trainin | Intel | POB 1659, Haifa 31015, Israel | 97248655738 | solomon.trainin@intel.com |
| Sanjiv Nanda | Qualcomm | 5775 Morehouse Drive, San Diego, CA 92121 | +1-858-845-2375 | snanda@qualcomm.com |
| Matthew Fischer | Broadcom | 190 Mathilda Place, Sunnyvalce, CA 94086 | +1 408 543 3370 | mfischer@broadcom.com |
| Menzo Wentink | Conexant | Oudegracht 3a, Utrecht, the Netherlands | +31 65 183 6231 | menzo.wentink@conexant.com |
| Yuichi Morioka | Sony Corporation | 6-7-35 Kitashinagawa, Shinagawa-ku Tokyo, 1410001 Japan | +81-3-6409-3201 | Yuichi.Morioka@jp.sony.com |

Abstract

This document contains proposed changes to the IEEE P802.11n Draft to address the following LB84 comments:

- MAC Comments: CID 54, 1474, 3894, 4642, 6800, 6801, 7901, 8097, 8098, 8099, 9895, 10022, 10080, 10302

- Coex Comments: CID 6783, 6785

regarding subclause 9.6 on multi-rate support.

Also, the following CIDs have been re-opened to be addressed in this document.

- MAC Comments: CID 684, 800, 1473, 6803, 7285, 8095, 10304

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

## Background of proposal in this submission

This submission is aimed to revise subclause 9.6 “multirate support” to incorporate newly defined features in TGn.  These features include;

- Implicit BAR mechanism

- STBC Control Frames

- L-SIG TXOP Protection

- HT PPDU selection

Also, a rework of clause numbering was done to improve readability.

The document provides a COUNTER to the following CIDs;

54, 684, 800, 1473, 1474, 3894, 4642, 6783, 6785, 6800, 6801, 6803, 7285, 7901, 8095, 8098, 8099, 8097, 8099, 9895, 10022, 10080, 10302, 10304

An accompanying document, doc11-06/1710r0 summarized the suggested resolution for the relavent CIDs.

## Proposal

TGn Editor: Replace subclause 9.6 of TGn draft 1.05 as follows.

  - Multirate support

Some PHYs have multiple data transfer rate capabilities that allow implementations to perform dynamic rate switching with the objective of improving performance. The algorithm for performing rate switching is beyond the scope of this standard, but in order to ensure coexistence and interoperability on multirate-capable PHYs, this standard defines a set of rules to be followed by all STAs.

Frames carried in non-HT PPDU shall be transmitted at Supported Rates, Extended Supported Rates or at rates of the BSSBasicRateSet. Frames carried in HT PPDU shall be transmitted at Supported MCSs, BSSBasicMCSset, or at BSSBasicSTBCMCS when the frame is an STBC control frame.

Control fFrames shall be transmitted at rates which are in accordance with this subclause at one of the rates/MCS in the BSSBasicRateSet/BSSBasicMCSset parameter except in the following cases:

— When the transmitting STA’s protection mechanism for non-ERP receivers is enabled and the frame is a protection mechanism frame and it is a frame which initiates an exchange. In this case the frame shall be transmitted at a rate according to the rules for determining the rates of transmission of protection frames in 9.13.1

— When the Dual CTS protection is enabled and the frame is a protection mechanism frame and it is a frame which initiates an exchange. In this case the frame shall be transmitted at a rate according to the rules for determining the rates of transmission of protection frames in 9.2.5.5a

— When the control frame is a BlockAckReq or BlockAck frame

In the former case, the control frame shall be transmitted at a rate according to the separate rules for determining the rates of transmission of protection frames in 9.13.

In the latter case, the control frame shall be transmitted in accordance with this subclause.

TGn Editor: Insert the following subclauses after 9.6 of TGn draft 1.05.

9.6.1a Rate selection for Data and Management frames

9.6.1a.1 Rate selection for data and management frames with multicast and broadcast addresses

All data and management frames with a broadcast or multicast address in the Address 1 field shall be transmitted at one of the rates included in the BSSBasicRateSet parameter if the BSSBasicRateSet is not empty.

All data and management frames with a broadcast or multicast address in the Address 1 field other than Beacons, shall be transmitted at one of the rates included in the BSSBasicMCSSet parameter if the BSSBasicRateSet is empty and the BSSBasicMCSSet is not empty.

Beacons shall be transmitted at one of the mandatory PHY rates if the BSSBasicRateSet parameter is empty and the BSSBasicMCSSet is not empty.

All data and management frames with a broadcast or multicast address in the Address 1 field shall be transmitted at one of the mandatory PHY rates if both the BSSBasicRateSet parameter and the BSSBasicMCSSet parameter are empty. (E.g. a scanning STA that is not yet associated with a BSS)

Except, a STA that has a value of true for the MIB attribute dot11TxSTBCOptionEnabled shall use the BasicSTBCMCS when it transmits secondary STBC Beacon frames and multicast frames.

9.6.1a.2 Rate selection for polling frames

All data frames of subtype (QoS) (+)CF-Poll sent in the CP shall be transmitted at a rate from the BSSBasicRateSet parameter or from the BSSBasicMCSset parameter so that they will be understood by all STAs in the BSS, unless an RTS/CTS protection exchange has already been performed before the transmission of the data frame of subtype (QoS) (+)CF-Poll and the Duration/ID field in the RTS frame covers the entire TXOP in which case the rate shall accord with the rate selection rules in 9.6.1c.

9.6.1a.3 Rate selection for +CF-Ack frames

For frames of type (QoS) Data+CF-Ack, (QoS) Data+CF-Poll+CFAck, and (QoS) CF-Poll+CF-Ack, the rate or MCS chosen to transmit the frame shall be supported by both the addressed recipient STA and the STA to which the ACK frame is intended. The BlockAck control frame shall be sent at the same rate and modulation class as the BlockAckReq frame if it is sent in response to a BlockAckReq frame.

9.6.1a.4 Rate selection for other Data and Mangement frames

All other data and management frames (not mentioned in subclauses 9.6.1a.1, 9.6.1a.2, 9.6.1a.3) shall be sent using any data rate subject to the following constraints:

No STA shall transmit a frame at a rate or MCS that is not supported by the receiver STA(s), as reported in any Supported Rates element, Extended Supported Rates element or Supported MCS field and in the management frames transmitted by the receiver STA(s).

Under no circumstances shall a STA initiate transmission of a frame at a data rate higher than the greatest rate in the OperationalRateSet or the HTOperationalMCSset, which are parameters of the MLME-JOIN.request primitive.

In the case where the supported rate set of the receiving STA(s) is not known, the transmitting STA shall transmit at a rate selected from the BSSBasicRateSet parameter or in the BSSBasicMCSset parameter or a rate or MCS at which the transmitting STA has received a frame from the receiving STA.

These rules also apply to A-MPDUs that aggregate MPDUs of type Data with any other types of MPDU.

9.6.1b Rate selection for control frames

9.6.1b.1 Rate selection for control frames that initialize a TXOP

If control frames that initiate a TXOP are carried in non-HT PPDUs, the transmitting STA shall transmit these frames using one of the rates in the BSSBasicRate set parameter or at a rate from the mandatory rate set of the attached PHY if the BSSBasicRateSet parameter is empty.

If control frames that initiate a TXOP are carried in HT PPDUs, then the transmitting STA shall transmit these frames using an MCS supported by the receiver STA, as reported in the Supported MCS field in the HT capabilities element in management frames transmitted by that STA or a rate or MCS at which the transmitting STA has received a frame from the receiving STA.

Selection between HT PPDU format and non-HT PPDU format for control frame transmission shall be determined by the rules in 9.6.3b.

9.6.1b.2 Rate selection for control frames that are not control response frames

If a control frame that is not a control response frame is not the first frame transmitted within a TXOP and the control frame is carried in a non-HT PPDU, then the transmitting STA shall transmit the control frame at the highest rate in the BSSBasicRateSet parameter that is less than or equal to the rate/non-HT reference rate (9.6.2) of the previously transmitted frame that was directed to the same receiving STA. If no rate in the BSSBasicRateSet parameter meets these conditions, then the control frame shall be transmitted at the highest mandatory rate of the PHY that is less than or equal to the rate/non-HT reference rate (9.6.2) of the previously transmitted frame that was directed to the same receiving STA.

If a control frame that is not a control response frame is not the first frame transmitted within a TXOP and it is carried in an HT PPDU, then the transmitting STA shall transmit these frames using an MCS supported by the receiver STA, as reported in the Supported MCS field in the HT capabilities element in management frames transmitted by that STA or a rate or MCS at which the transmitting STA has received a frame from the receiving STA,

The decision to transmit the control frame as an HT PPDU or as a non-HT PPDU shall be performed in accordance with the rules in 9.6.3b.

9.6.1b.3 Rate selection for control response frames

To allow the transmitting STA to calculate the contents of the Duration/ID field, a STA responding to a received frame shall transmit its Control response frame according to the following rules:

When the Control response frame (CTS, ACK or Immediate BlockAck including response to implicit BlockAckReq) is carried in a non-HT PPDU or in an HT PPDU that has a value of NON_HT_CW40 for the TXVECTOR parameter CW, then it shall be transmitted at the highest rate in the BSSBasicRateSet parameter that is less than or equal to the rate/non-HT reference rate (9.6.2) of the immediately previous frame in the frame exchange sequence (as defined in 9.12). If no rate in the BSSBasicRateSet parameter meets these conditions, then the Control response frame shall be transmitted at the highest mandatory rate of the PHY that is less than or equal to the rate of the received frame, and that is of the same modulation class as the received frame.

If the Control Response frame is carried in an HT PPDU that does not have a value of NON_HT_CW40 for the TXVECTOR parameter CW, then it shall be transmitted at the MCS as determined by the procedure defined in 9.6.3a Response frame MCS computation.

If the received frame is of a modulation class other than HT PHY and the Control Response frame is carried in a non-HT PPDU, then the Control Response frame shall be transmitted using the same modulation class as the received frame. In addition, the Control Response frame shall be sent using the same value for the TXVECTOR parameter PREAMBLE_TYPE as the received frame.

If the received frame is of the modulation class HT PHY and the Control Response frame is carried in a non-HT PPDU, then the Control Response frame shall be transmitted using one of the ERP-OFDM PHY or OFDM PHY modulation classes.

Selection between HT and non-HT PPDU for control response frame transmission shall be performed in accordance with the rules in 9.6.3b.

9.6.1b.3.1 Alternative rates and MCSs for control response frames

For a non-HT PPDU control response frame or a control response frame that is an HT PPDU with TXVECTOR parameter CW equal to NON_HT_CW40, an alternative rate to that determined by following the procedure of 9.6.1f may be used provided that the duration of the control response frame at the alternative rate is the same as the duration of the control response frame at the originally chosen rate and the alternative rate is in either the BSSBasicRateSet parameter or the mandatory rate set of the PHY and the modulation class of the control response frame at the alternative rate is the same class as that of the originally selected rate.

For an HT PPDU control response frame with TXVECTOR parameter CW not equal to NON_HT_CW40, an alternative MCS to that determined by following the procedure of 9.6.1f may be used provided that the duration of the control response frame at the alternative MCS is the same as the duration of the control response frame at the originally chosen MCS and the alternative MCS is in the CandidateMCSSet that was generated according to the procedure of 9.6.1f.

For the Clause 17, Clause 18, and Clause 19, and clause 21 PHYs, the time required to transmit a frame for use in the Duration/ID field is determined using the PLME-TXTIME.request primitive (see 10.4.6) and the PLMETXTIME.confirm primitive (see 10.4.7), both defined in 17.4.3, 18.3.4, 19.8.3.1, 19.8.3.2, or 19.8.3.3, or 21.4.3 depending on the value of the TXVECTOR parameter PREAMBLE_TYPE. In QSTAs, the Duration/ID field may cover multiple frames and may involve using the PLME-TXTIME.request primitive several times.

9.6.3a Control response frame MCS computation

If the control response frame format is to be transmitted as an HT PPDU with TXVECTOR parameter CW not equal to NON_HT_CW40 in accordance with rules in 9.6.3b, then the responding STA shall choose an MCS from a set of MCS called the CandidateMCSSet. The CandidateMCSSet is determined using the following rules:

If the frame eliciting the response was sent using an STBC MCS, then instead of establishing a CandidateMCSSet, the rules for STBC exchanges found in 9.2.5.5a Dual CTS protection are used to determine the MCS of the response transmission.

If any of the following four conditions is true,

- the frame eliciting the response included +HTC with the MRQ bit set to 1

- the frame eliciting the response included +HTC with the TRQ bit set to 1

- the frame eliciting the response had the SOUNDING parameter value of SOUNDING

- the frame eliciting the response had an L-SIG Duration value (9.13.4) and is a TXOP initiating frame

then the CandidateMCSSet is the MCS Set consisting of the logical AND of the Supported MCS Set of the STA that sent the frame that is eliciting the response and the set of MCS that the responding STA is capable of transmitting.

If none of the above conditions is true, then the CandidateMCSSet is the BSSBasicMCSSet. If the BSSBasicMCSSet is empty, then the CandidateMCSSet shall consist of the set of mandatory HT PHY MCS.

The selection of the value for the CW parameter of the TXVECTOR of the response transmission shall obey the rules found in 9.20.4.

MCS values from the CandidateMCSSet which do not satisfy those rules shall be eliminated from the CandidateMCSSet.

The choice of a response MCS is made as follows:

If the frame eliciting the response is a non-HT PPDU or an HT-PPDU with TXVECTOR parameter CW equal to NON_HT_CW40,

- Eliminate from the MCSCandidateSet all MCS that have a Data Rate greater than or equal to the Data Rate of the received PPDU (for applicable Data Rates, see the set of tables found in 21.6 Rate dependent parameters for HT Modulation and Coding Schemes (MCS)).

- Find the highest indexed MCS from the MCSCandidateSet. The index of this MCS is the index of the MCS which shall be used for the response transmission.

If the frame eliciting the response is an HT-PPDU with TXVECTOR parameter CW not equal to NON_HT_CW40,

- Eliminate from the MCSCandidateSet all MCS which have an index which is higher than the index of the MCS of the received frame.

- Determine the highest Nss value of all of the MCS in the MCSCandidateSet that is equal to or less than the Nss value of the MCS of the received frame and eliminate all MCS from the MCSCandidateSet which have an Nss value which is not equal to this Nss value.

- Find the highest-indexed MCS of the MCSCandidateSet for which the modulation value of each stream is less than or equal to the modulation value of each stream of the MCS of the received frame and for which the code ratio value of each stream is less than or equal to the code ratio value of each stream of the MCS from the received frame. The index of this MCS is the index of the MCS which shall be used for the response transmission.

If there is no MCS that meets the condition in step 3), then repeat step 3) with the set of MCS with the next highest Nss values from the MCSCandidateSet and continue likewise until step 3) produces a non-empty result.

9.6.3b HT PPDU/non-HT PPDU selection for control frames

A control frame shall be carried in a non-HT PPDU or an HT PPDU with TXVECTOR parameter CW equal to NON_HT_CW40 except under the conditions listed below.

A control frame shall be carried in an HT PPDU with TXVECTOR parameter CW not equal to NON_HT_CW40 when the control frame meets any of the following conditions:

- the control frame has a SOUNDING parameter value of SOUNDING

- the control frame contains an L-SIG Duration value (see 9.13.4)

- the control frame is sent at an STBC rate

A control frame shall be carried in HT PPDU with TXVECTOR parameter CW not equal to NON_HT_CW40 when the control frame is a response to a frame that meets the following conditions:

- the frame eliciting the response included an +HTC with the TRQ bit set to 1 and the NDP announcement bit was set to 0 and if this responder sets the Implicit TxBF Receiving Capable field to 1

- the frame eliciting the response was an RTS frame carried in HT PPDU

- the frame eliciting the response was sent at STBC rate and the Dual CTS Protection bit was set to 1 (see 9.2.5.4.1)

A control frame may be carried in HT PPDU with TXVECTOR parameter CW not equal to NON_HT_CW40 when the control frame meets any of the following conditions:

- the control frame contains an +HTC with the MRQ bit set to 1

- the control frame contains an +HTC with the TRQ bit set to 1

## References:

[1] IEEE P802.11n-D1.05 (insertions and deletions marked).pdf

[2] IEEE P802.11-REVma-D9.0-redline-changed-pages.pdf

[3] 11-06-1710-00-000n-multirate-support-CIDs.xls