IEEE P802.11
Wireless LANs

| TGn LB84 Submission for PHY Characteristics CIDs 7808, 7546, 10068, 11997, 3401 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-9-28 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Eldad Perahia | Intel Corporation |  |  | eldad.perahia@intel.com |

Abstract

This document contains proposed changes to the IEEE P802.11n Draft to address the following LB84 comments:

7808, 7546, 10068, 11997, 3401

The changes marked in this document are based on TGn Draft version P802 11n D1 04.pdf.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# Introduction

These CIDs add aRIFSTime and aPPDUMaxTime to the PHY characteristics.  In addition, D1.0 deleted aMPDUMaxLength from clause 10.4.3.2, which is used in Clause 17.  The modifications contained here undo the deletion of aMPDUMaxLength from clause 10.4.3.2.

## Interpretation of a Motion to Adopt

A motion to approve this submission means that the editing instructions and any changed or added material are actioned in the TGn Draft.  This introduction, is not part of the adopted material.

Editing instructions formatted like this are intended to be copied into the TGn Draft (i.e. they are instructions to the 802.11 editor on how to merge the TGn amendment with the baseline documents).

TGn Editor:  Editing instructions preceded by “TGn Editor” are instructions to the TGn editor to modify existing material in the TGn draft.   As a result of adopting the changes, the TGn editor will execute the instructions rather than copy them to the TGn Draft.

Summission Note: Notes to the reader of this submission are not part of the motion to adopt.  These notes are there to clarify or provide context.

# Proposed Resolution

| CID | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- |
| 7808 | No reference to PHY characteristics for clause 17 & Clause 19 mode of operation | Add reference to PHY characteristics for clause 17 & Clause 19 mode of operation i.e. modify the table to include PHY characteristics for all modes of operation or add reference to clause 17 & clause 19 PHY characteristics | Counter: accepted in principle per 06/1564 |
| 7546 | Table n83 should include aRIFSTime = 2us. | Add it here, to the dashed list below the table and to the PLME-CHARACTERISTICS primitive. | Counter: accepted in principle: Modification to Table n87 (D1.04) resolved in D1.04. Modification to the PLME-CHARACTERISTICS primitive resolved per 06/1564 |
| 10068 | There is a description of "aLTFOneLength is the length of the First High Throughput Long Training Field." But this may be misleading. In MM, L-LTF would have the length of aLTFOneLength, and the first HT-LTF has aLTFTwoLength. | Remove "High Throughput". As a result, this line would become as; "aLTFOneLength is the length of the First Long Training Field." | Accepted. Resolved in D1.04 |
| 11997 | Define the max time of aggregated frame transmission. This change and the following are used for that. This value allows transmission of max A-MSDU (7935 byte) by lowest MCS Index=0. | Insert this after aPSDUMaxLength in the Table n83: aPPDUMaxTime 10ms | Counter: accepted in principle per 06/1564 |
| 3401 | aLTFOneLength is different in Mixed Mode format | Change in value in 11th row and 2nd column to: 8 micro sec for GF Format and 4 micro sec for MM Format. | Counter: Refer to resolution for CID 10068 |

# CID 7808

TGn Editor: insert the following text at the end of clause 21.4.4 PHY characteristics (D1.04)

For non-HT modes of operation refer to clause 17 and clause 19 for PHY characteristics.

# CID 7546, 11997

TGn Editor: modify the text in clause 10.4.3.2, pg 130, line 24-57  (D1.04) as follows:

PLME-CHARACTERISTICS.confirm(

aSlotTime,

aSIFSTime,

aCCATime,

aPHY-RX-START-Delay,

aRxTxTurnaroundTime,

aTxPLCPDelay,

aRxPLCPDelay,

aRxTxSwitchTime,

aTxRampOnTime,

aTxRampOffTime,

aTxRFDelay,

aRxRFDelay,

aAirPropagationTime,

aMACProcessingDelay,

aPreambleLength,

aSTFOneLength,

aSTFTwoLength,

aLTFOneLength,

aLTFTwoLength,

aPLCPHeaderLength,

aPLCPHeaderTwoLength,

aMPDUDurationFactor,

aMPDUMaxLength,

aPSDUMaxLength,

aPPDUMaxTime

aCWmin,

aCWmax

aRIFSTime

)

TGn Editor: modify the text in clause 10.4.3.2, pg 130, line 63-65  (D1.04) as follows:

The parameters aMPDUDurationFactor, aMPDUMaxLength, aPSDUMaxLength, aSTFOneLength, aSTFTwoLength, aLTFOneLength, aLTFTwoLength, aPLCPHeaderTwoLength, aRIFSTime, aPPDUMaxTime are not used by all PHYs defined within this standard.

TGn Editor: modify the table in clause 10.4.3.2, pg 131, line 6-51  (D1.04) as follows:

| Name | Type | Description |
| --- | --- | --- |
| aSTFOneLength | integer | The Short Training Field One Length (in µs) |
| aSTFTwoLength | integer | The Short Training Field Two Length (in µs) |
| aLTFOneLength | integer | The Long Training Field One Length (in µs) |
| aLTFTwoLength | integer | The Long Training Field Two Length (in µs) |
| aPLCPHeader- Length | integer | The current PHY’s PLCP header length (in µs). If the actual value of the length of the modulated header is not an integral number of µs, the value is rounded up to the next higher value. |
| aPLCPTHeaderTwo- Length | integer | The PLCP header length (in µs) if there is need for more than one PLCP header |
| aMPDUDuration- Factor | integer | The overhead added by the PHY to the MPDU as it is transmitted through the WM expressed as a scaling factor applied to the number of bits in the MPDU. The value of aMPDUDurationFactor is generated by the following equation: Truncate[((PPDUbits/PSDUbits)–1) x 109)]. The total time to transmit a PPDU over the air is generated by the following equation rounded up to the next integer µs: aPreambleLength + aPLCPHeaderLength + (((aMPDUDurationFactor ｴ 8 ｴ PSDUoctets) / 109) + (8 ｴ PSDUoctets)) / data rate where data rate is in Mb/s. The total time (in µs) to the beginning of any octet in a PPDU from the first symbol of the preamble can be calculated using the duration factor in the following equation: Truncate[aPreambleLength + aPLCPHeaderLength + (((aMPDUDurationFactor ｴ 8 ｴ N) / 109) + (8 ｴ N)) / data rate] + 1, where data rate is in Mb/s and where N counts the number of octets in the PPDU prior to the desired octet, but does not count the number of octets in the preamble PLCP header. |
| aMPDUMax- Length | integer | The maximum number of octets in an MPDU that can be conveyed by a PLCP protocol data unit (PPDU). |
| aPSDUMax- Length | integer | The maximum number of octets in a PSDU that can be conveyed by a PLCP protocol data unit (PPDU). |
| aPPDUMaxTime | integer | The maximum amount of time (in milliseconds) of a PPDU. |
| aRIFSTime | integer | The time (in microseconds) by which multiple transmissions from a single transmitter may be separated, when no SIFS-separated response transmission is expected. See 9.2.3.6. |

TGn Editor: modify Table n87-MIMO PHY characteristics (D1.04) as follows:

| - MIMO PHY characteristics |  |
| --- | --- |
| Characteristics | Value |
| aRIFSTime | 2 µs |
| aSlotTime | 9 µs |
| aSIFSTime | 16 µs |
| aCCATime | < 4 µs |
| aPHY-RX-START-Delay | 56 µs |
| aRxTxTurnaroundTime | < 2 µs |
| aTxPLCPDelay | Implementation dependent |
| aRxPLCPDelay | Implementation dependent |
| aRxTxSwitchTime | << 1 µs |
| aTxRampOnTime | Implementation dependent |
| aTxRampOffTime | Implementation dependent |
| aTxRFDelay | Implementation dependent |
| aRxRFDelay | Implementation dependent |
| aAirPropagationTime | << 1 µs |
| aMACProcessingDelay | < 2 µs |
| aPreambleLength | 16 µs |
| aSTFOneLength | 8 µs |
| aSTFTwoLength | 4 µs |
| aLTFOneLength | 8 µs |
| aLTFTwoLength | 4 µs |
| aPLCPHeaderLength | 4 µs |
| aPLCPHeaderTwoLength | 8 µs |
| aPSDUMaxLength | 65535 |
| aPPDUMaxTime | 10ms |
| aCWmin | 15 |
| aCWmax | 1023 |