IEEE P802.11
Wireless LANs

| TGn LB84 Submission for PLCP Transmit and Receive Procedure CIDs |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-10-16 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Eldad Perahia | Intel Corporation |  |  | eldad.perahia@intel.com |

Abstract

This document contains proposed changes to the IEEE P802.11n Draft to address the following LB84 comments:

7784, 7064, 8156, 7785, 12208, 269, 270, 7065, 271, 7529, 7530, 7799, 12209, 12210, 12211, 347, 12212, 7068, 12214, 177, 272, 273, 7069, 12213, 12159, 1071, 10385

The changes marked in this document are based on TGn Draft version P802 11n D1 04.pdf.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# Introduction

## Interpretation of a Motion to Adopt

A motion to approve this submission means that the editing instructions and any changed or added material are actioned in the TGn Draft.  This introduction, is not part of the adopted material.

Editing instructions formatted like this are intended to be copied into the TGn Draft (i.e. they are instructions to the 802.11 editor on how to merge the TGn amendment with the baseline documents).

TGn Editor:  Editing instructions preceded by “TGn Editor” are instructions to the TGn editor to modify existing material in the TGn draft.   As a result of adopting the changes, the TGn editor will execute the instructions rather than copy them to the TGn Draft.

Summission Note: Notes to the reader of this submission are not part of the motion to adopt.  These notes are there to clarify or provide context.

# Proposed Resolution

| CID | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- |
| 7784 | PLCP transmit procedure does not provide option for transmitting clause 19 frames | Include reference to clause 19 PLCP transmit procedure. Also update the associated TXVECTOR and RXVECTOR parameters.Add PLCP transmit state machine to transmit clause 19 frames | Counter: refer to 06/1571 |
| 7064 | Expansion matrix should not be required in spatial spreading or beamforming are not being performed | make PMD_EXPANSION_MAT optional | Counter: refer to 06/1571 |
| 8156 | Encoding method cannot be completely determined by ADVANCED_CODING and MCS parameters of TXVECTOR. Also on page 231, line 3 | Add ",BW" after "ADVANCED_CODING" | Counter: accepted in principle. Refer to 06/1571 |
| 7785 | Incorrect specification in " The scrambled and encoded data shall then be exchanged between the MAC and the PHY ". Scrambling and encoding is a PHY function and not a MAC function. | The data shall then be exchanged between the MAC and the PHY | Counter: accepted in principle. Refer to 06/1571 |
| 12208 | "LENGTH or HT_LENGTH yet p230 line 17 says for legacy devices, follow the procedure of clause 17 | Is this redundant, or is it required for duplicate legacy? If needed for duplicate legacy, then the paragraph at the start of 20.3.16 should be made more explicit | Counter: accepted in principle. Refer to 06/1571 |
| 269 | Figure is inconsistent with advanced coding | Correct figure | Counter: refer to 06/1571 |
| 270 | Figure is inconsistent with advanced coding | Correct figure | Counter: refer to 06/1571 |
| 7065 | If format = N_HT, the TX HT-Preamble block should be skipped | skip TX HT-Preamble block when format = N_HT | Accept: refer to 06/1571 |
| 271 | Figure is inconsistent with advanced coding | Correct figure | Counter: refer to 06/1571 |
| 7529 | Figure n71. The "End of Wait" state has actions copied from RX HT-SIG | I think this is ment to indicate PHY_CCA.ind (idle) | Accept: refer to 06/1571 |
| 7530 | Figure n71. The "Decode Symbol" state indicates descrambling and decoding. But actually they're done the other way round. | Replace action with decode and descramble symbol | Accept: refer to 06/1571 |
| 7799 | No reference to PLCP receive procedure for clause 17, clause 19 frame formats | Include reference to clause 17 & clause 19 PLCP receive procedure | Counter: accept in principle. refer to 06/1571 |
| 12209 | "a significant received signal strength" is only way way to do CCA. Preamble detection is another | Replace by "a busy channel" | Accept: refer to 06/1571 |
| 12210 | This sentence does not make sense. A number of lines seem to have been omitted | Refer to clause 17 for the missing text | Counter: accept in principle. refer to 06/1571 |
| 12211 | "A Viterbi decoder is recommended" is not relevant to LDPCs | Rewrite more generally | Counter: accept in principle. refer to 06/1571 |
| 347 | Statement about Viterbi assumes that BCC is used | Add "if the binary convolutional code is used" after "… is recommended" | Accept: already included in D1.04 |
| 12212 | "and the PLCP format, SIGNAL field or HT-SIG field are completely recognizable and supported" | Perhaps rewrite as "if the PLCP format and either the SIGNAL field or HT-SIG field are completely recognizable and supported". However, I believe it sufficient that just the SIGNAL field is completely recognizable and supported, so a better version is the clause 17 version "and if the SIGNAL field is completely recognizable and supported" | Counter: refer to 06/1571 |
| 7068 | In HT mode, is it necessary to abort the packet if parity check of the L-header fails? | allow issuing a PHY-RXSTART.indicate if parity of L-header is invalid but CRC check of HT-header is valid. | Counter: refer to 06/1571 |
| 12214 | It is well known that the SIGNAL field's parity bit is inadequate for determining if a valid SIGNAL field was received. Therefore a more robust test of a valid SIGNAL field than just " test Parity" in the figure on p237 or "parity check" in the text on p234 line 29 is required | The parity bit only works when there is very likely to be only 0 or 1 bit errors in the SIGNAL field. Therefore redefine a valid SIGNAL field to be when there are very likely to be only 0 or 1 bit errors and the parity bit is correct. The choice of BER estimator is open but it must work well enough to reach the PER requirements of section 20.3.15.1. This change is completely consistent with the "completely recognizable" language at line 9 or p234 | Counter: refer to 06/1571 |
| 177 | Test in this paragraph is inconsistent with advanced coding | Correct text | Counter: refer to 06/1571 |
| 272 | Figure is inconsistent with advanced coding | Correct figure | Counter: refer to 06/1571 |
| 273 | Figure is inconsistent with advanced coding | Correct figure | Counter: refer to 06/1571 |
| 7069 | In HT mode, is it necessary to abort the packet if parity check of the L-header fails? | allow issuing a PHY-RXSTART.indicate if parity of L-header is invalid but CRC check of HT-header is valid in Figure n71 | Counter: accept in principle. refer to 06/1571 |
| 12213 | It is well known that the SIGNAL field's parity bit is inadequate for determining if a valid SIGNAL field was received. Therefore a more robust test of a valid SIGNAL field than just " test Parity" in the figure on p237 or "parity check" in the text on p234 line 29 is required | The parity bit only works when there is very likely to be only 0 or 1 bit errors in the SIGNAL field. Therefore redefine a valid SIGNAL field to be when there are very likely to be only 0 or 1 bit errors and the parity bit is correct. The choice of BER estimator is open but it must work well enough to reach the PER requirements of section 20.3.15.1. This change is completely consistent with the "completely recognizable" language at line 9 or p234 | Counter: refer to 06/1571 |
| 12159 | Behavior of a PHY when receiving the reserved MCS rates 77-127 in an otherwise valid HT-PLCP is undefined. The only error exit line from the "Validate PLCP/Check PLCP fields" requires that the packet's duration be predicted and CCA asserted as busy. | Either define a nominal rate for the reserved MCS rates (i.e. allow them to be used in the future), or change the figure above section 20.4 to abort in this case. | Counter: refer to 06/1571 |
| 1071 | Figure n71 - change the flow to allow a passing HT-SIG CRC frame to be received even if the L-SIG parity failed. The HT-SIG CRC is a more powerful error check, and therefore, should take priority in the mixed mode frame case. See also the text in this section. | Change the flow in figure n71 to allow reception when HT-SIG CRC is good, independent of the result of the mixed mode L-SIG parity check. Add text to reflect this change. | Counter: accept in principle. refer to 06/1571 |
| 10385 | The flow chart doesn't show if a device cannot support capability like beamforming and STBC, how does it behave? | Revise the text on the most side box to " Unsuported rate or MCS or ADVANCED_CODING or STBC capability or beamforming capability, predict duration" . | Counter: refer to 06/1571 |

# CID 7784, 12208, 269

TGn Editor: In D1.04, pg 223 lines 62-65 and  pg 224, line 1-8, modify paragraph as follows

There are three options for transmit PLCP procedure are shown in Figure n67 (PLCP transmit procedure (HT Mixed Mode packet format)), Figure n68 (PLCP transmit procedure (Greenfield packet format)) and Figure n69 (PLCP transmit state machine). The first two options, for which typical transmit procedures are shown in Figure n67 (PLCP transmit procedure (HT Mixed Mode packet format)) and Figure n68 (PLCP transmit procedure (Greenfield packet format)), are selected if the FORMAT field of PHY-TXSTART.request(TXVECTOR) is set to HT_MM or HT_GF respectively.  These transmit procedures do not describe the operation of optional features, such as LDPC.  The third option is to follow the transmit procedure as in clause 15, 17, 18, or 19 if the FORMAT field is set to NOT_HT.  Additionally, if the FORMAT field is set to NOT_HT and CW indicates HT_CW_20DN, follow the transmit procedure as in clause 17.  In all these options, in order to transmit data, PHY-TXSTART.request shall be enabled so that the PHY entity shall be in the transmit state.  Further, the PHY shall be set to operate at the appropriate frequency through station management via the PLME. Other transmit parameters, such as MCS Coding types and TX power, are set via the PHY-SAP with the PHY-TXSTART.request(TXVECTOR), as described in 21.2.2 (TXVECTOR parameters).

Original text:

Three options for transmit PLCP procedure are shown in Figure n67 (PLCP transmit procedure (HT Mixed Mode packet format)), Figure n68 (PLCP transmit procedure (Greenfield packet format)) and Figure n69 (PLCP transmit state machine). The first two options are selected if the FORMAT field of PHY-TXSTART.request(TXVECTOR) is set to HT_MM or HT_GF respectively. The third option is to follow the transmit procedure as in clause 17 if the FORMAT field is set to NOT_HT. In all these options, in order to transmit data, PHY-TXSTART.request shall be enabled so that the PHY entity shall be in the transmit state.  Further, the PHY shall be set to operate at the appropriate frequency through station management via the PLME. Other transmit parameters, such as MCS Coding types and TX power, are set via the PHY-SAP with the PHY-TXSTART.request(TXVECTOR), as described in 21.2.2 (TXVECTOR parameters).

# CID 8156

TGn Editor: In D1.04, pg 244 line 32, modify text as follows

The encoding method shall be based on the LDPC_CODING, CW, and MCS parameter of the TXVECTOR.

# CID 7785, 8156

TGn Editor: In D1.04,  pg 244 line 32, modify paragraph as follows

The PLCP shall then issue a PMD_TXSTART.request, and transmission of the PLCP preamble and PLCP header may start, based on the parameters passed in the PHY-TXSTART.request primitive. The data shall then be exchanged between the MAC and the PHY through a series of PHY-DATA.request(DATA) primitives issued by the MAC, and PHY-DATA.confirm primitives issued by the PHY.  Once PLCP preamble transmission is started, the PHY entity shall immediately initiate data scrambling and data encoding. The encoding method shall be based on the LDPC_CODING, CW, and MCS parameter of the TXVECTOR. The scrambled and encoded data shall then be exchanged between the MAC and the PHY through a series of PHY-DATA.request(DATA) primitives issued by the MAC, and PHY-DATA.confirm primitives issued by the PHY. A modulation rate change, if any, shall be initiated starting with the SERVICE field data of the PLCP header, as described in 21.3.2 (PLCP frame format).

Original text:

The PLCP shall then issue a PMD_TXSTART.request, and transmission of the PLCP preamble and PLCP header may start, based on the parameters passed in the PHY-TXSTART.request primitive. Once PLCP preamble transmission is started, the PHY entity shall immediately initiate data scrambling and data encoding. The encoding method shall be based on the LDPC_CODING and MCS parameter of the TXVECTOR. The scrambled and encoded data shall then be exchanged between the MAC and the PHY through a series of PHY-DATA.request(DATA) primitives issued by the MAC, and PHY-DATA.confirm primitives issued by the PHY. A modulation rate change, if any, shall be initiated starting with the SERVICE field data of the PLCP header, as described in 21.3.2 (PLCP frame format).

# CID 12208

TGn Editor: In D1.04, pg 244, line 47-50, modify text as follows

Normal termination occurs after the transmission of the final bit of the last PSDU octet, according to the number supplied in the L_LENGTH or LENGTH field depending on whether the packet is in non-HT format or HT format.

# CID 270

TGn Editor: In D1.04, pg 224, lines 63-65 and pg 225, line 1 modify text as follows

A typical state machine implementation of the transmit PLCP is provided in Figure n68 (PLCP transmit procedure (Greenfield packet format)) Figure n69. Requests (.req) and confirmations(.confirm) are issued once with designated states.  This state machine does not describe the operation of optional features, such as LDPC or STBC.

# CID 7065

The change to figure n69 includes skipping the “TX HT-Preamble” step with FORMAT = NON_HT.

TGn Editor: In D1.04, replace figure n69 with the following figure

- Figure n69—PLCP transmit state machine

Original figure:

- Figure n69—PLCP transmit state machine

# CID 7064

TGn Editor: In D1.04, pg 223 line 26, add the following bullet item

PMD_EXPANSIONS_MAT_ON

TGn Editor: In D1.04, pg 238 Table n89, add the following primitive with an “X” in the “Request” column

PMD_EXPANSIONS_MAT_ON

# CID 271, 7799, 272

TGn Editor: In D1.04, pg 226, lines 61-65 and pg 227, line 1-4, modify the paragraph as follows

Typcial The PLCP receive procedures are is shown in Figure n70 (PLCP receive procedure for HT Mixed Mode PLCP format) and Figure n71 (PLCP receive procedure for Greenfield format PLCP) and Figure n72 (PLCP receive state machine).  The receive procedures correspond to Non_HT PLCP format, HT Mixed Mode format and HT Greenfield format, respectively.  These receive procedures do not describe the operation of optional features, such as LDPC.  If the detected format indicates Non_HT PLCP format, refer to receive procedure in clause 17 or 19.  In order to receive data, PHY-TXSTART.request shall be disabled so that the PHY entity is in the receive state.  Further, through station management (via the PLME) the PHY is set to the appropriate frequency.  Other receive parameters, such as RSSI and indicated DATARATE, may be accessed via the PHY-SAP.

Original paragraph:

The PLCP receive procedure is show in Figure n70 (PLCP receive procedure for HT Mixed Mode PLCP format), Figure n71 (PLCP receive procedure for Greenfield format PLCP) and Figure n72 (PLCP receive state machine).  The receive procedures correspond to Non_HT PLCP format, HT Mixed Mode format and HT Greenfield format.  In order to receive data, PHY-TXSTART.request shall be disabled so that the PHY entity is in the receive state.  Further, through station management (via the PLME) the PHY is set to the appropriate frequency.  Other receive parameters, such as RSSI and indicated DATARATE, may be accessed via the PHY-SAP.

# CID 12209, 12210, 12211, 12212

TGn Editor: In D1.04, pg 227, line 6-20, modify the paragraph as follows

Upon receiving the transmitted PLCP preamble, PMD_RSSI.indicate shall report a busy channel significant received signal strength level to the PLCP.  This indicates activity to the MAC via PHY-CCA.indicate.  PHY-CCA.indicate(BUSY) shall be issued for reception of a signal prior to correct reception of the PLCP frame.  The PMD primitive PMD_RSSI is issued to update the RSSI and parameter reported to the MAC.

After PHY-CCA.indicate is issued, the PHY entity shall begin receiving the training symbols and searching

for SIGNAL and HT-SIG in order to set the length of the data stream, the demodulation type, code type, and the decoding rate.  Once the HT-SIG is detected, without any errors detected by a CRC, FEC decode shall be initiated and the PLCP SERVICE fields and data shall be received, decoded (a Viterbi decoder is recommended if the binary convolutional code is being used), and checked by ITU-T CRC-32.  If the FCS by the ITU-T CRC-32 check fails, the PHY receiver shall return to the RX IDLE state, as depicted in Figure n70 (PLCP receive procedure for HT Mixed Mode PLCP format).  Should the status of CCA return to the IDLE state during reception prior to completion of the full PLCP processing, the PHY receiver shall return to the RX IDLE state.  If the PLCP header reception is successful (and the PLCP format SIGNAL field or and HT-SIG field are completely recognizable and supported), a PHY-RXSTART.indicate(RXVECTOR) shall be issued.  The RXVECTOR associated with this primitive include the parameters specified in Table n64 (RXVECTOR parameters).  Also, in this case, the HT PHY ensures that the CCA shall indicate a busy medium for the intended duration of the transmitted frame, as indicated by the LENGTH field.

Original paragraph:

Upon receiving the transmitted PLCP preamble, PMD_RSSI.indicate shall report a significant received signal strength level to the PLCP.  This indicates activity to the MAC via PHY-CCA.indicate.  PHY-CCA.indicate(BUSY) shall be issued for reception of a signal prior to correct reception of the PLCP initiated and the PLCP SERVICE fields and data shall be received, decoded (a Viterbi decoder is recommended if the binary convolutional code is being used), and checked by ITU-T CRC-32.  If the FCS by the ITU-T CRC-32 check fails, the PHY receiver shall return to the RX IDLE state, as depicted in Figure n70 (PLCP receive procedure for HT Mixed Mode PLCP format).  Should the status of CCA return to the IDLE state during reception prior to completion of the full PLCP processing, the PHY receiver shall return to the RX IDLE state.  If the PLCP header reception is successful (and the PLCP format, SIGNAL field or HT-SIG field are completely recognizable and supported), a PHY-RXSTART.indicate(RXVECTOR) shall be issued.  The RXVECTOR associated with this primitive include the parameters specified in Table n64 (RXVECTOR parameters).  Also, in this case, the HT PHY ensures that the CCA shall indicate a busy medium for the intended duration of the transmitted frame, as indicated by the LENGTH field.

# CID 7068, 12214, 1071

TGn Editor: In D1.04, pg 227, lines 42-43 modify the text as follows

If the PLCP header is receivable, but the parity check of the L-header or the CRC check of the header is not valid, a PHY-RXSTART.indicate is not issued.

# CID 177

TGn Editor: In D1.04, pg 227, lines 46-48 modify the text as follows

If the binary convolutional code is used, any data received after the indicated data length are considered pad bits (to fill out an OFDM symbol) and should be discarded.

# CID 273

TGn Editor: In D1.04, pg 227, lines 50-52 add the following the sentence

A typical state machine implementation of the receive PLCP is given in Figure n72 (PLCP receive state machine).

This state machine does not describe the operation of optional features, such as LDPC or STBC.

# CID 7530, 7069, 12213, 12159, 1071, 10385, 7529

Changes include (1) in “RX L-SIG” state, remove “parity fail” choice (2) In “Detect HT-SIG” state, add “Not HT-SIG and Parity fail” choice (3) In “Detect HT-SIG” state, modify “Not HT-SIG” to “Not HT-SIG and Parity valid” choice (4) In “Wait End of PLCP Frame” modify action to “If not HT and unsupported RATE, or if HT and unsupported mode indicated by HT-SIG, predict duration” (5) In “RX HT-SIG” state change choice to “CRC Fail or Reserved HT-SIG parameter value” (6) In “End of Wait” state, change action to “PHY_CCA.ind(IDLE)”

TGn Editor: In D1.04, replace figure n72 with the following figure

Detect

SIG

Type

Determine

type

of

SIG

Field

Detect

L

-

PLCP

Receive

L

-

SIG

Field

Detect

HT

-

SIG

Determine

whether

HT

-

SIG

follows

SIG

RX

HT

-

SIG

RX

and

test

CRC

Receive

following

Training

RX

L

-

SIG

RX

and

test

Parity

RX

PLCP

fields

Change

modulation

according

to

HT

-

SIG

or

L

-

SIG

Validate

PLCP

Check

PLCP

fields

Setup

PSDU

RX

Set

Length

PHY

_

RXSTART

.

ind

(

RXVECTOR

)

RX

HT

-

SIG

RX

and

test

CRC

Receive

following

Training

Wait

END

of

PLCP

Frame

If not HT and

unsupported

RATE, or

if HT and

unsupported

mode indicated

by HT-SIG,

predict duration

RX

Symbol

PMD

_

DATA

.

ind

Decode

Symbol

Decode and

Descramble

symbol

Decrement

Length

P

HY

_

DATA

.

ind

(

DATA

)

(

bit

removing

if

needed

)

Decrement

length

count

End

of

PSDU

RX

PHY

_

RXEND

.

ind

(

NoError

)

PHY

_

CCA

.

ind

(

IDLE

)

Signal

Not

Valid

PHY

_

RXEND

.

ind

(

Carrier

Lost

)

Decrement

Time

Wait

for

intended

end

of

PSDU

End

of

Wait

PHY_CCA.ind(IDLE)

RX

IDLE

state

CS

/

CCA

PHY

_

CCA

.

ind

(

BUSY

)

HT

_

SIG

L

-

SIG

Signal

Valid

PHY

_

CCA

.

ind

(

IDLE

)

Signal

Invalid

HT

_

SIG

Not HT-SIG

and Parity

valid

CRC Fail or

Reserved MCS

value

CRC

OK

PHY

_

CCA

.

ind

(

IDLE

)

Predicted

Duration

expires

and

PHY

_

CCA

.

ind

(

IDLE

)

PLCP

Correct

PHY

_

CCA

.

ind

(

IDLE

)

PHY

_

CCA

.

ind

(

BUSY

)

Length

>

0

Length

=

0

Time

=

0

Not HT-SIG

and Parity

fail

Figure n72—PLCP receive state machine

Original figure:

Detect

SIG

Type

Determine

type

of

SIG

Field

Detect

L

-

PLCP

Receive

L

-

SIG

Field

Detect

HT

-

SIG

Determine

whether

HT

-

SIG

follows

SIG

RX

HT

-

SIG

RX

and

test

CRC

Receive

following

Training

RX

L

-

SIG

RX

and

test

Parity

RX

PLCP

fields

Change

modulation

according

to

HT

-

SIG

or

L

-

SIG

Validate

PLCP

Check

PLCP

fields

Setup

PSDU

RX

Set

Length

PHY

_

RXSTART

.

ind

(

RXVECTOR

)

RX

HT

-

SIG

RX

and

test

CRC

Receive

following

Training

Wait

END

of

PLCP

Frame

If

unsupported

RATE

or

MCS

,

predict

duration

RX

Symbol

PMD

_

DATA

.

ind

Decode

Symbol

Descramble

and

decode

symobl

Decrement

Length

P

HY

_

DATA

.

ind

(

DATA

)

(

bit

removing

if

needed

)

Decrement

length

count

End

of

PSDU

RX

PHY

_

RXEND

.

ind

(

NoError

)

PHY

_

CCA

.

ind

(

IDLE

)

Signal

Not

Valid

PHY

_

RXEND

.

ind

(

Carrier

Lost

)

Decrement

Time

Wait

for

intended

end

of

PSDU

End

of

Wait

RX

and

test

CRC

Receive

following

Training

RX

IDLE

state

CS

/

CCA

PHY

_

CCA

.

ind

(

BUSY

)

HT

_

SIG

L

-

SIG

Signal

Valid

PHY

_

CCA

.

ind

(

IDLE

)

Signal

Invalid

Parity

Valid

Parity

Fail

HT

_

SIG

Not

HT

-

SIG

CRC

Fail

CRC

OK

PHY

_

CCA

.

ind

(

IDLE

)

Predicted

Duration

expires

and

PHY

_

CCA

.

ind

(

IDLE

)

PLCP

Correct

PHY

_

CCA

.

ind

(

IDLE

)

PHY

_

CCA

.

ind

(

BUSY

)

Length

>

0

Length

=

0

Time

=

0

Figure n72—PLCP receive state machine