IEEE P802.11
Wireless LANs

| Summary of Protection for HT Transmissions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-10-10 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Srinivas Kandala | Airgo Networks, Inc. | 900 Arastradero Road, Palo Alto CA 95014 | (650) 475-1977 | srini@airgonetworks.com |
| Sanjiv Nanda | Qualcomm, Inc. |  |  | snanda@qualcomm.com |
| Amit Bansal | Wipro |  |  | amit.bansal@wipro.com |
| Ali Raissinia | Airgo Networks, Inc. | 900 Arastradero Road, Palo Alto CA 95014 |  | ali@airgonetworks.com |

Abstract

This document summarizes the required protection for HT Transmissions. This document has been submitted to invoke discussion on the necessary actions that need to be taken to clean up protection in IEEE 802.11n Draft 1.04.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

## 1. Introduction

In this contribution we explore the possible combinations of operation of an HT station and analyze what transmissions need to be protected based on the rules described in [1].

## 2. Possible operating modes of an IEEE 802.11n network

In an 802.11n network, the nature of the transmission activity is sensed by the AP (STAs in an IBSS). The AP then updates the following fields in the beacon: Operating Mode and Non-GF STA Present field in the HT IE and Use Protection Field (along with Barker Sequence and Non-ERP fields) in the ERP IE.

When a STA is scheduled to transmit an HT PPDU, then it shall interpret these fields and consequently ensure that the transmission is adequately protected.

Table 1 and 2 summarize the possible scenarios and the actions, if needed, to be taken by the 802.11n station when it transmits HT PPDUs.

Table 1 – Protection Summary when no non-HT STAs are present

| Operating Modes: 00 and 10: Only HT STAs are associated. Therefore all STAs are RIFS capable, i.e., no RIFS protection is required. |  |  |  |
| --- | --- | --- | --- |
| Operating Modes |  | Non-GF Present Field |  |
|  |  | 0: All STA are GF capable | 1: There are non-GF capable STAs present |
| 00 Either all STA are HT-20 or all STA are HT-40 | 20 MHz transmission: | No GF protection required | GF sequences shall be protected. May use any of the listed mechanisms in Note 2 below. |
|  | 40 MHz transmission: | No GF protection required | GF sequences shall be protected. May use any of the listed mechanisms in Note 2 below |
| 10 HT 20/40 BSS: Both 20 MHz HT and 20/40 MHz HT STA are associated | 20 MHz transmission: | No GF protection required | GF sequences shall be protected. May use any of the listed mechanisms in Note 2 below |
|  | 40 MHz transmission: | No GF protection required. | GF sequences shall be protected.May use any of the listed mechanisms in Note 2 below |

Table 2 – Protection Summary when non-HT STAs are present

| Operating Modes: 11 and 01: Non-HT STAs may be present. |  |  |  |
| --- | --- | --- | --- |
| Operating Mode |  | Use Protection Field in ERP IE |  |
|  |  | 0: All non-HT STA are ERP capable | 1: Non-ERP STA are present |
| 11 One or more non-HT STA are associated HT transmissions including RIFS sequences and GF sequences shall be protected. (See Note 1) | 20 MHz transmission: | HT transmissions shall be protected. May use any of the listed mechanisms in Notes 2a-2d below | HT transmissions shall be protected using the mechanism in Note 2b |
|  | 40 MHz transmission: | HT transmissions shall be protected. May use any of the listed mechanisms in Notes 2a-2d below | HT transmissions shall be protected using the mechanism in Note 2b |
| 01 Non-HT STA may be present on the primary or secondary channel HT transmissions including RIFS sequences and GF sequences may be protected | 20 MHz transmission: | May use any of the listed mechanisms in Notes 2a-2d below | HT transmissions shall be protected using the mechanism in Note 2b |
|  | 40 MHz transmission: | May use any of the listed mechanisms in Notes 2a-2d below | HT transmissions shall be protected using the mechanism in Note 2b |

Notes:

- Mixed Mode preamble may be used without any protection if RIFS separation is not used.

- STA may use one of the following mechanisms to achieve protection:

- Use RTS/CTS or CTS-to-self frame exchange

  - For 20 MHz transmissions,  use the rates defined in Clause 17 or Clause 19

  - For 40 MHz transmissions, use the non-HT duplicate rate format defined in clause 21

- Use 20 MHz RTS/CTS or CTS-to-self frame exchange using the rates defined in Clause 18

- Transmit first PPDU using non-HT preamble using a rate in BSSBasicRateSet to set the Duration/ID to cover the rest of the TXOP. The rest of the TXOP may contain GF and/or RIFS sequences.

- L-SIG TXOP protection

- Transmit first PPDU using Mixed Mode preamble at a rate in BSSBasicMCSSet to set the Duration/ID to cover the rest of the TXOP. This rest of the TXOP may contain GF and/or RIFS sequences.

References:

- IEEE 802.11n Draft 1.04, September 2006.