IEEE P802.11
Wireless LANs

| Proposal selected Pseudo Code in LB88 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-10-13 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| David Cypher | NIST | 100 Bureau Drive, Stop 8920 | 301 975 4855 | david.cypher@nist.gov |

Abstract

This contribution proposes a suggested resolution to the selected pseudo code in Letter Ballot 88;  particularly clauses 8.7.2.3A and 8.7.2.4A.  There are two proposed pseudo code, one for each clause.  The suggested changes were too numerous and too complicated (i.e., font, indenting, reorganize, corrections, etc.) to explain to include as individual text IEEE 802.11 WG LB88 comments.  Instead two comments were placed one for each clause with the recommended change to see this contribution.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

8.7.2.3A Per-MPDU Rx pseudo-code for an MMPDU

if dot11RSNAEnabled = TRUE and Bit 6 of RSNA Capability Field is set then

if the Protected Frame subfield of the Frame Control Field is zero then

if Protection for TA is off for Rx then

Receive the unencrypted MPDU without protections

else

Discard the frame body without indication to LLC

endif

elseif Protection is true for TA then

if ((MPDU has individual RA and Pairwise key exists for the MPDU’s TA) or (MPDU has a broadcast/multicast RA and network type is IBSS and IBSS GTK exists for MPDU’s RA)) then

if key is null then

discard the frame body

elseif entry has an AES-CCM key then

decrypt frame using AES-CCM key

if the integrity check fails then

discard the frame

increment dot11RSNAStatsCCMPDecryptErrors

endif

elseif entry has an AES-128-CMAC key then

check integrity of the frame using AES-128-CMAC key

if the ICV fails then

discard the frame

increment dot11RSNAStatsCMACICVErrors

endif

else

discard the frame body

endif

elseif GTK for the Key ID does not exist then

discard the frame body

elseif GTK for the Key ID is null then

discard the frame body

elseif GTK for the Key ID is a CCM key then

decrypt frame using AES-CCM key

if the integrity check fails then

discard the frame

increment dot11RSNAStatsCCMPDecryptErrors

endif

elseif the IGTK for the Key ID is an AES-128-CMAC key then

integrity check the frame using AES-128-CMAC decryption

if the ICV fails then

discard the frame

increment dot11RSNAStatsCMACICVErrors

endif

endif

else

discard the frame body

endif

endif

8.7.2.4A Per-MDPU Rx pseudo-code

if dot11RSNAEnable = TRUE and Bit 6 of RSNA Capability Field is set then

if the frame was not protected then

Receive the MMPDU unprotected

else //Have a protected MMPDU

if Pairwise key is an AES-CCM key then

if its MPDUs had sequential PNs or it consists of only one MPDU then

Accept the MMPDU

else

discard the MMPDU as a replay attack

increment dot11RSNAStatsCCMPReplays

endif

elseif Pairwise key is an AES-128-CMAC key then

if its MPDUs had sequential PNs or it consists of only one MPDU then

Accept the MMPDU

else

discard the MMPDU as a replay attack

increment dot11RSNAStatsCMACReplays

endif

endif

endif

endif

References:

IEEE 802.11 WG LB88: IEEE P802.11w/1.0, October 2006-10-13

IEEE P802.11-REVma-D7.0-redline.pdf