IEEE P802.11
Wireless LANs

| TGn/TGk Impact and Extension Evaluation General (addressing CID 114, 7026, and 4347) |  |  |  |  |
| --- | --- | --- | --- | --- |
| This document begins the discussion of TGn’s potential impacts on the TGk extensions to the 802.11 specification. Date: 2006-10-17 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Joseph Levy | InterDigital Communications Corporation | 2 Huntington Quadrangle 4th Floor, South Wing Melville, NY 11747 | +1 631 622 4139 | joseph.levy@interdigital.com |

Abstract

This document discusses TGn’s potential impacts on the TGk extensions to the 802.11 specification.  Addressing the following CIDs 114, 7026, and 4347 of from LB84.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# Introduction:

The purpose of this document is to address the any required changes or impacts that TGn should have on the current TGk draft.  TGk has created several request/response measurements:  Beacon, Measurement Pilot, Frame, Channel Load, Noise Histogram,  STA Statistics, Location Configuration Information, Neighbor Report, Link Measurement, and  QoS Metrics; and a request-only mechanism: Measurement Pause.  These measurements and mechanisms allow the reporting of the performance of the AP and STA in a way consistent with RRM (radio resource management) applications.  Since TGn is making changes to both the PHY and MAC of 802.11 changes/extensions to TGk’s measurements and mechanism may be required to insure that a TGn enabled AP or STA can exploit the capabilities provided by TGk.

# Discussion:

Below is a working summary of proposed changes and areas of potential change, this is not intended to be an all inclusive and comprehensive list, it is simply a summary of the proposed changes and suggestions of where other change proposal may need to be made.

## Beacon

The request/response measurements related to the Beacon may need to be modified to add additional HT BSS Beacon information.  In addition request/response measurements for the secondary Beacon may need to be added.

## Channel Load

Additional information fields may need to be added to the Channel Load Request to add the ability to request information related to 20 MHz, 20/40 MHz, and 40 MHz channels and operating modes.  The Channel Load Report may also need to be modified to allow for the reporting of the various modes of channel operation.

## Noise Histogram

Additional information fields may need to be added to the Noise Histogram Request to add the ability to request information related to 20 MHz, 20/40 MHz, and 40 MHz channels and operating modes.  The Noise Histogram Report may also need to be modified to allow for the reporting of the various modes of channel operation.

## STA Statistics

Clause 7.3.2.21.8 [1] STA Statistics Request requires additional Group Identity fields to be added to Table 29c-Group Identity for a STA Statistics Request. Proposed Group Identity fields to be added are listed below [2]:

STA  Coutnters from dot11A-MSDU Group – Group Identity = 4

dot11TransmittedAMSDUCountCounter32,

dot11FailedAMSDUCountCounter32,

dot11RetryAMSDUCountCounter32,

dot11MultipleRetryAMSDUCountCounter32,

dot1TransmittedOctetsInAMSDUCountCounter64,

dot11AMSDUAckFailureCount Counter32,

dot11ReceivedAMSDUCountCounter32,

dot1ReceivedOctetsInAMSDUCountCounter64,

STA Counters from dot11A-MPDU Group – Group Identity = 5

dot11TransmittedAMPDUCountCounter32,

dot11TransmittedMPDUsInAMPDUCountCounter32,

dot11TransmittedOctetsInAMPDUCountCounter64,

dot11AMPDUReceivedCountCounter32,

dot11MPDUInReceivedAMPDUCountCounter32,

dot11ReceivedOctetsInAMPDUCountCounter64,

dot11AMPDUDelimiterCRCErrorCountCounter32,

STA Counters from dot11 BAR, Channel Width, PSMP Group – Group Identity = 6

dot11ImplicitBARFailureCountCounter32,

dot11ExplicitBARFailureCountCounter32,

dot11ChannelWidthSwitchCountCounter32,

dot11TwentyMHzFrameTransmittedCountCounter32,

dot11FortyMHzFrameTransmittedCountCounter32,

dot11TwentyMHzFrameReceivedCountCounter32,

dot11FortyMHzFrameReceivedCountCounter32,

dot11PSMPSuccessCountCounter32,

dot11PSMPFailureCountCounter32,

STA Counters from dot11Protection Group (RDG, DualCTS, LSIG) – Group I

dot11GrantedRDGUsedCountCounter32,

dot11GrantedRDGUnusedCountCounter32,

dot11TransmittedFramesInGrantedRDGCountCounter32,

dot11TransmittedOctetsInGrantedRDGCountCounter64,

dot11DualCTSSuccessCountCounter32,

dot11DualCTSFailureCountCounter32,

dot11RTSLSIGSuccessCountCounter32, (Ed: CID 59)

dot11RTSLSIGFailureCountCounter32 (Ed: CID 59)

dot11 BF, STBC Group

dot11BeamformingFrameCountCounter32,

dot11STBCCTSSuccessCountCounter32,

dot11STBCCTSFailureCountCounter32,

dot11nonSTBCCTSSuccessCountCounter32,

dot11nonSTBCCTSFailureCountCounter32,

## Location Configuration Information

It is not anticipated that any changes need to be made to the Location Configuration Information measurement.

## Neighbor Report

Neighbor report should be modified to define the TGn AP type and operating modes.  The TGk Neighbor Report format was reworked about 6 months ago to allow for new extensions by other TG’s what additional changes if any need to be made.

## Link Measurement

Additional information fields may need to be added to the Link Measurement Request to add the ability to request information related to 20 MHz, 20/40 MHz, and 40 MHz channels and operating modes.  The Link Measurement Report may also need to be modified to allow for the reporting of the various modes of channel operation.

## QoS Metrics

Since all HT STAs are QoS STAs, all HT STAs should be able to support the QoS Metrics Request/Response measurement as it now exits.  However, HT STAs have additional capabilities which should be reported on.  Therefore the HT extensions may impact current metrics or require new metrics to be defined.  This may be done by modification to the existing QoS Metrics Request/Response or by adding a new HT Metrics Request/Response measurement.

## Additional Areas

Lastly, TGk has defined a starting toolbox for WIFI measurements, since TGn is expanding the PHY capabilities it may be necessary to add or modify this toolbox to address the added capabilities: MIMO and beam forming.

# References:

[1] – IEEE P802.11kTM/D6.0, Draft Standard for Information Technology Telecommunications and information exchange between systems; Local and metropolitan area networks Specific requirements; Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) specifications; Amendment: Radio Resource Measurement, October 2006

[2] – IEEE P802.11n/D1.05, Draft Standard for Information Technology Telecommunications and information exchange between systems; Local and metropolitan area networks Specific requirements; Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) specifications; Amendment: Enhancements for Higher Throughput October 2006

# CID Reference:

| CID | Comment | Proposed Change | Resolution |  |
| --- | --- | --- | --- | --- |
| 7026 | Coordinate with TGk to insure that SNR for all Rx channels is measured in a way that is available in the MIB for use by upper layers |  | Gen AdHoc: Sept: Joe has had limited interest expressed, but a list of issues will be published after the Sept. Meeting. This is an ongoing effort, and this comment has been addressed. A Submission is expected to address these issues during the next few weeks. Gen AdHoc: Refer to CID 114 Beamforming AdHoc: Transfer to General ad hoc |  |
| 114 | This reviewer asserts that TGk will be an approved part of .11 prior to TGn. There appears to be significant impact and/or extension needed to the TGk mechanisms to extend them to be cognizant of the changes continued in the TGn draft. Having been involved in TGn for close to 3 years now and not being aware of any discussions of the interaction of TGk and TGn, this needs to be addressed by TGn. | Explicitly work with TGk to determine what the impact of TGn and TGk interactions are; TGn should then jointly specify the TGk extensions necessary as part of the TGn draft. | Gen AdHoc: Sept: Joe has had limited interest expressed, but a list of issues will be published after the Sept. Meeting. This is an ongoing effort, and this comment has been addressed. A Submission is expected to address these issues during the next few weeks. Gen AdHoc: There may or may not be admendments that will be part of the 802.11 spec by the time we get to Sponsor ballot. The Baseline had TGk applied for the (line 10 of page 1) purpose of this LB. Propose a group look at the possible interaction. A Strawpoll to determine TGn's level of support. |  |
| 4347 | You need to add an HT bit into the Neighbor report's BSSID field to indicate the Neighbor is an HT Neighbor. Possibly a 40 Mhz operation bit as well as what the extension channel is in a possible optional IE? Possibly HT capabilities, etc. | Liason with TGk to get this into the Neighbor Report. Carefully consider what can remain in the Beacon as opposed to bloating the Neighbor Lists for a STA to consider which candidates are worth considering going off channel to collect the beacon, and/or can be determined after BSS Transistion Candidate selection. | Gen AdHoc: Sept: Joe has had limited interest expressed, but a list of issues will be published after the Sept. Meeting. This is an ongoing effort, and this comment has been addressed. A Submission is expected to address these issues during the next few weeks. Gen ADHoc: Joe and Dave are going to talk with the TGk and gather input on the interactions. See CID 114 |  |