IEEE P802.11
Wireless LANs

| TGn/TGk Impact and Extension Evaluation General (addressing CID 114, 7026, 4347, and 7316) |  |  |  |  |
| --- | --- | --- | --- | --- |
| This document begins the discussion of TGn’s potential impacts on the TGk extensions to the 802.11 specification. Date: 2007-01-15 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Joseph Levy | InterDigital Communications Corporation | 2 Huntington Quadrangle 4th Floor, South Wing Melville, NY 11747 | +1 631 622 4139 | joseph.levy@interdigital.com |
| Assaf Kasher | Intel Corporation | Matam Industrial Park Haifa 31015, Israel | +972-4-8651547 | Assaf.kasher@intel.com |
| Jon Rosdahl | Samsung | 10871 North 5750 West Highland, Utah, 84003 | +1 801 756 1496 | jrosdahl@ieee.org |
| Joseph Kwak | InterDigital Communications Corporation |  | +1 (630) 739-4159 | joekwak@sbcglobal.net |

Abstract

This document discusses TGn’s potential impacts on the TGk extensions to the 802.11 specification.  Addressing the following CIDs 114, 7026, 4347, and 7316, from LB84.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart@ok-brit.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# Introduction:

The purpose of this document is to discuss and propose changes to the TGn amendment to address the impact and/or extension of the current TGk draft amendment, assuming the current amendment is applied the current baseline 802.11 specification.  TGk has created several request/response measurements:  Beacon, Measurement Pilot, Frame, Channel Load, Noise Histogram,  STA Statistics, Location Configuration Information, Neighbor Report, Link Measurement, and  QoS Metrics; and a request-only mechanism: Measurement Pause.  These measurements and mechanisms allow the reporting of the performance of the AP and STA in a way consistent with RRM (radio resource management) applications.  Since TGn is making changes to both the PHY and MAC of 802.11 changes/extensions to TGk’s measurements and mechanism may be required to insure that a TGn enabled AP or STA can exploit the capabilities provided by TGk.

The structure of this document following this introduction is:

- Text Proposals - these are to the current TGn draft [4]

- Discussion - of the issues and potential issues, and provides reasoning, support, and background for the text proposals.

- References – including document references and CID references

# Text Proposals:

## Text Proposal 1 - Beacon

Insert the following additions/modification to the final paragraph of clause 7.3.2.22.6:

The Reported Frame Body field contains the frame body of the reported Beacon, Measurement Pilot, or Probe Response frame. All fixed fields and information elements are included in the order they appeared in the reported frame. Reported TIM elements shall be truncated such that only the first 4 octets of the element are reported and the element length field is modified to indicate the truncated length of 4. Reported IBSS DFS elements shall be truncated such that only the lowest and highest channel number map are reported and the element length field is modified to indicate the truncated length of 13.  Reported RSN elements shall be truncated such that only the first 4 octets of the element are reported and the element length field is modified to indicate the truncated length of 4.  If the Reported Frame Body would cause the Measurement Report element to exceed the maximum information element size then the Reported Frame Body shall be truncated so that the last information element in the Reported Frame Body field shall be a complete information element.

## Text Proposal 2 - STA Statistics

Replace  Table 29c of clause 7.3.2.21.8 STA Statistics Request with the following Table {this table is based on  the corrected version of the table in [5]}  (the following rows have been added: Group Identity for a STA Statistics Request: STA Counters from dot11A-MSDU Group – Group Identity = 11, STA Counters from dot11A-MPDU Group – Group Identity = 12, STA Counters from dot11 BAR, Channel Width, PSMP Group – Group Identity = 13,  STA Counters from dot11Protection Group – Group Identity = 14, STBC Group – Group Identity = 15 (or the next available un-used Group Identity values) and changing “Reserved” to be 16-255 (or reducing the range by 5) – note the reason for providing this change at the next available Group Identity is that multiple ongoing task groups may define Group Identities  and it is not the intent of this change to reverse or revise any of these changes only to add the HT Group Identities:

7.3.2.21.8 STA Statistics Request

Table 29c—Group Identity for a STA Statistics Request

| Statistics Group Name | Group Identity |
| --- | --- |
| STA Counters from dot11CountersTable | 0 |
| STA Counters from dot11MacStatistics group | 1 |
| QoS STA Counters for UP0 from dot11QosCountersTable | 2 |
| QoS STA Counters for UP1 from dot11QosCountersTable | 3 |
| QoS STA Counters for UP2 from dot11QosCountersTable | 4 |
| QoS STA Counters for UP3 from dot11QosCountersTable | 5 |
| QoS STA Counters for UP4 from dot11QosCountersTable | 6 |
| QoS STA Counters for UP5 from dot11QosCountersTable | 7 |
| QoS STA Counters for UP6 from dot11QosCountersTable | 8 |
| QoS STA Counters for UP7 from dot11QosCountersTable | 9 |
| BSS Average Access Delay as described in 7.3.2.39 | 10 |
| STA Counters from dot11A-MSDU Group | 11 |
| STA Counters from dot11A-MPDU Group | 12 |
| STA Counters from dot11 BAR, Channel Width, PSMP Group | 13 |
| STA Counters from dot11Protection Group | 14 |
| STBC Group | 15 |
| Reserved | 16 – 255 |

Insert  the following rows to  Table 31b of clause 7.3.2.22.8 STA Statistics Request after the penultimate entry and prior to the ultimate entry (None) defining Group Identity Request 11-15: Group Identity for a STA Statistics Request: STA Counters from dot11A-MSDU Group – Group Identity = 11, STA Counters from dot11A-MPDU Group – Group Identity = 12, STA Counters from dot11 BAR, Channel Width, PSMP Group – Group Identity = 13,  STA Counters from dot11Protection Group – Group Identity = 14, STBC Group – Group Identity = 15 (or the next available un-used Group Identity values).  Also change the ultimate entry to read 16-255 (or increase the value of the first integer by 5) – note the reason for providing this change at the next available Group Identity Requested is that multiple ongoing task groups may define Group Identities Requested and it is not the intent of this change to reverse or revise any of these changes only to add the HT Group Identities:

| Group Identity Requested | Statistics Returned |
| --- | --- |
| Note: Currently exiting Group Identity Requested and Statistics Returned are not shown |  |
| 11 | STA Counters from dot11A-MSDU Group: dot11TransmittedAMSDUCountCounter32, dot11FailedAMSDUCountCounter32, dot11RetryAMSDUCountCounter32, dot11MultipleRetryAMSDUCountCounter32, dot1TransmittedOctetsInAMSDUCountCounter64, dot11AMSDUAckFailureCount Counter32, dot11ReceivedAMSDUCountCounter32, dot1ReceivedOctetsInAMSDUCountCounter64 |
| 12 | STA Counters from dot11A-MPDU Group: dot11TransmittedAMPDUCountCounter32, dot11TransmittedMPDUsInAMPDUCountCounter32, dot11TransmittedOctetsInAMPDUCountCounter64, dot11AMPDUReceivedCountCounter32, dot11MPDUInReceivedAMPDUCountCounter32, dot11ReceivedOctetsInAMPDUCountCounter64, dot11AMPDUDelimiterCRCErrorCountCounter32 |
| 13 | STA Counters from dot11 BAR, Channel Width, PSMP Group: dot11ImplicitBARFailureCountCounter32, dot11ExplicitBARFailureCountCounter32, dot11ChannelWidthSwitchCountCounter32, dot11TwentyMHzFrameTransmittedCountCounter32, dot11FortyMHzFrameTransmittedCountCounter32, dot11TwentyMHzFrameReceivedCountCounter32, dot11FortyMHzFrameReceivedCountCounter32, dot11PSMPSuccessCountCounter32, dot11PSMPFailureCountCounter32 |
| 14 | STA Counters from dot11Protection Group (RDG, DualCTS, LSIG): dot11GrantedRDGUsedCountCounter32, dot11GrantedRDGUnusedCountCounter32, dot11TransmittedFramesInGrantedRDGCountCounter32, dot11TransmittedOctetsInGrantedRDGCountCounter64, dot11DualCTSSuccessCountCounter32, dot11DualCTSFailureCountCounter32, dot11RTSLSIGSuccessCountCounter32, dot11RTSLSIGFailureCountCounter32 |
| 15 | dot11 BF, STBC Group: dot11BeamformingFrameCountCounter32, dot11STBCCTSSuccessCountCounter32, dot11STBCCTSFailureCountCounter32, dot11nonSTBCCTSSuccessCountCounter32, dot11nonSTBCCTSFailureCountCounter32 |
| 16-255 | None |

## Text Proposal 3 - Neighbor Report

7.3.2.37 Neighbor Report element

EDITORIAL NOTE - 11ma ended with 7.3.2.35; 11k added 7.3.2.36 through 7.3.2.44. This change is intended to modify the subclause numbered 7.3.2.37 in 11k, entitled "Neighbor Report Element" Edit Figure 112C as shown in the following (change being the addition of bit 10 or the next available reserved bit as High Throughput Domain, and changing “Reserved” to be 11-31 or reducing the range by one) – note the reason for providing this change at the next available reserve bit is that multiple ongoing task groups may be making additions to this field and it is not the intent of this change to reverse or revise any of these changes only to add the High Throughput bit:

High

Throughput

EDITORIAL NOTE - Assignment of bit 10 (or any other bit assigned) for High Throughput needs to be approved by IEEE 802.11 ANA. Insert the following paragraph after the paragraph that starts “The Capabilities Subfield” or which ever bit or subfield precedes the newly assigned High Throughput bit:

The High Throughput bit, when set to one, indicates that the AP represented by this BSSID is a HT AP including the HT Capabilities element in its Beacons, and that the contents of that HT Capabilities information element are identical to the HT Capabilities information element advertised by the AP sending the report.

Change the paragraph that starts “Bits 10-31 are reserved...” as follows (or as appropriate if additional Bits have been taken by other amendments):

Bits 11-31 are reserved and shall be set to 0 on transmission and are ignored on reception.

Add the following rows to  Table 43b:  Sub-Element ID 3 HT Capabilities, Sub-Element ID 4 HT Information Element, Sub-Element ID 5 Secondary Channel Offset (or the next available un-used Sub-Element IDs) and changing “Reserved” to be 6-220 (or reducing the range by 3) – note the reason for providing this change at the next available Sub-Element ID is that multiple ongoing task groups may define Sub-Element IDs and it is not the intent of this change to reverse or revise any of these changes only to add the HT extension elements:

Table 43b—Optional Extensions

| Sub-Element ID | Name |
| --- | --- |
| 0 | Reserved |
| 1 | TSF Information |
| 2 | Measurement Pilot Interval |
| 3 | HT Capabilities (see 7.3.2.49 (HT Capabilities element)) |
| 4 | HT Information Element (see 7.3.2.50 (HT Information element)) |
| 5 | Secondary Channel Offset element (see 7.3.2.20a (Secondary Channel Offset element)) |
| 6-220 | Reserved |
| 221 | Vendor Specific |
| 222-255 | Reserved |

Insert the following paragraphs and figures are placed after the final paragraph in 7.3.2.37:

The HT Capabilities sub-element contains the HT Capabilities subfield as shown in Figure 112g-a.

|  | Sub-Element ID | Length | HT Capabilities Info | A-MPDU Parameters (Ed: CID 3755) | Supported MCS set | HT Extended Capabilities (Ed: CID 3755) | TxBF capabilities | ASEL capabilities |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 2 | 1 | 16 | 2 | 4 | 1 |

Figure 112g-a — HT Capabilities sub-element format

The HT Capabilities subfield is 28 octets long and contains the HT Capabilities as specified in 7.3.2.49.

The HT Information Element sub-element contains the HT subfield as shown in Figure 112g-b.

|  |  |  |  | B0 B1 | B2 | B3 | B4 | B5 B7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Element ID | Length (Ed: CID 633) | Primary Channel | Secondary Channel Offset | STA Channel Width (Ed: CID 1209) | RIFS Mode | PSMP STAs Only | Service Interval Granularity |
| Octets: | 1 | 1 | 1 | 1 |  |  |  |  |

| B0 B1 | B2 | B3 | B4 B15 |
| --- | --- | --- | --- |
| Operating Mode | Non-greenfield STAs Present | Transmit Burst Limit | Reserved |
| 2 |  |  |  |

| B0 B5 | B6 | B7 | B8 | B9 | B10 | B11 | B12 B15 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reserved | Dual Beacon (Ed: CID 701) | Dual CTS Protection | Secondary Beacon | L-SIG TXOP Protection Full Support | PCO Active | PCO Phase | Reserved | Basic MCS Set |
| 2 |  |  |  |  |  |  |  | 16 |
| Figure 112g-b HT Information element format |  |  |  |  |  |  |  |  |

The HT Information Element subfield is 24 octets long and contains the HT Information Element as specified in 7.3.2.50.

The Secondary Channel Offset sub-element contains the Secondary Channel Offset subfield as shown in Figure112g-c.

|  | Sub-Element ID | Length | Secondary Channel Offset |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 |
| Figure 112g-c: Secondary Channel Offset sub-element format |  |  |  |

The Secondary Channel Offset subfield is 3 octets long and contains the Secondary Channel Offset element as specified in 7.3.2.20a.

# Discussion:

Below is a working summary of proposed changes and areas of potential change, this is not intended to be an all inclusive and comprehensive list, it is simply a summary of the proposed changes and suggestions of where other change proposals may need to be made.

## Beacon

The request/response measurements related to the Beacon may need to be modified to add additional HT BSS Beacon information.  In addition request/response measurements for the secondary Beacon may need to be added.

Conclusion:

Beacon Request – no need for change – since only legacy 20MHz beacons are used by TGn.

Beacon Response – no need for fundamental change; the TGk Beacon report includes the entire frame body of the reported beacon subject to size limitation using truncation.  Therefore it must be verified that critical TGn fields in the beacon are not truncated.

From [1] 7.3.2.22.6 final paragraph: The Reported Frame Body field contains the frame body of the reported Beacon, Measurement Pilot, or Probe Response frame. All fixed fields and information elements are included in the order they appeared in the reported frame. Reported TIM elements shall be truncated such that only the first 4 octets of the element are reported and the element length field is modified to indicate the truncated length of 4. If the Reported Frame Body would cause the Measurement Report element to exceed the maximum information element size then the Reported Frame Body shall be truncated so that the last information element in the Reported Frame Body field shall be a complete information element.

The maximum information elements size is 255 octets.

Beacon, Order – Information – Notes from Table 8 – Beacon frame body [3] – Max size taken from various locations in [3], (Note the Max size of the TIM elements is restricted to 4 O in clause 7.3.2.22.6):

| Max Size | Order | Information | Notes |
| --- | --- | --- | --- |
| 8 O | 1 | Timestamp |  |
| 2 O | 2 | Beacon interval |  |
| 2 O | 3 | Capability |  |
| 34 O | 4 | Service Set Identifier (SSID) |  |
| 10 O | 5 | Supported rates |  |
| 7 O | 6 | Frequency-Hopping (FH) Parameter Set | The FH Parameter Set information element is present within Beacon frames generated by STAs using FH PHYs. |
| 3 O | 7 | DS Parameter Set | The DS Parameter Set information element is present within Beacon frames generated by STAs using Clause 15, Clause 18, and Clause 19 PHYs. |
| 8 O | 8 | CF Parameter Set | The CF Parameter Set information element is present only within Beacon frames generated by APs supporting a point coordination function (PCF). |
| 4 O | 9 | IBSS Parameter Set | The IBSS Parameter Set information element is present only within Beacon frames generated by STAs in an IBSS. |
| 4 O | 10 | Traffic indication map (TIM) | The TIM information element is present only within Beacon frames generated by APs. |
| 6 O | 11 | Country | The Country information element shall be present when dot11MultiDomainCapabilityEnabled is true or dot11SpectrumManagementRequired is true. |
| 4 O | 12 | FH Parameters | FH Parameters as specified in 7.3.2.10 may be included if dot11MultiDomainCapabilityEnabled is true. |
|  | 13 | FH Pattern Table | FH Pattern Table information as specified in 7.3.2.11 may be included if dot11MultiDomainCapabilityEnabled is true. |
| 3 O | 14 | Power Constraint | Power Constraint element shall be present if dot11SpectrumManagementRequired is true. |
| 5 O | 15 | Channel Switch Announcement | Channel Switch Announcement element may be present if dot11SpectrumManagementRequired is true. |
|  | 16 | Quiet | Quiet element may be present if dot11SpectrumManagementRequired is true. |
| 9 + 2*n n = number of channels 167 O | 17 | IBSS DFS | IBSS DFS element shall be present if dot11SpectrumManagementRequired is true in an IBSS. |
| 7 O | 18 | TPC Report | TPC Report element shall be present if dot11SpectrumManagementRequired is true. |
| 3 O | 19 | ERP Information | The ERP Information element is present within Beacon frames generated by STAs using extended rate PHYs (ERPs) defined in Clause 19 and is optionally present in other cases. |
| 3-256 O | 20 | Extended Supported Rates | The Extended Supported Rates element is present whenever there are more than eight supported rates, and it is optional otherwise. |
| 255 O | 21 | RSN | The RSN information element shall be present within Beacon frames generated by STAs that have dot11RSNAEnabled set to TRUE. |
| 7 O | 22 | QBSS Load | The QBSS Load information element is only present within Beacon frames generated by QoS APs. The QBSS Load element is present when dot11QosOption-Implemented and dot11QBSSLoadImplemented are both true. |
| 20 O | 23 | EDCA Parameter Set | The EDCA Parameter Set information element is only present within Beacon frames generated by QAPs. The EDCA Parameter Set element is present when dot11QosOptionImplemented is true and the QoS Capability element is not present. |
| 3 O | 24 | QoS Capability | The QoS Capability information element is only present within Beacon frames generated by QAPs. The QoS Capability element is present when dot11Qos-OptionImplemented is true and EDCA Parameter Set element is not present. |
|  | Last | Vendor Specific | One or more vendor-specific information elements may appear in this frame. This information element follows all other information elements. |

TGn Beacon extensions taken from [4] clause 7.2.3.17Beacon frame format:

(Ed: CID 2418) Change Table 8-Beacon frame body as follows:

| Order | Information | Notes |
| --- | --- | --- |
| 8 | CF Parameter Set | The CF Parameter Set information element is present only within Beacon frames generated by APs supporting a point coordination function (PCF). This element shall not be present if dot11HighThroughputOptionImplemented is true and the Dual CTS Protection field of the HT Information element is set to 1 |

(Ed: CID 3733)

Insert the following three additional rows (preserving their order) in Table 8-Beacon frame body just before the Vendor Specific IE and insert contiguous numbering in the “Order” column:

| Size taken from various location in the draft. | Order | Information | Notes |
| --- | --- | --- | --- |
| 27 O |  | HT Capabilities | The HT Capabilities element is present when dot11HighThroughputOptionImplemented attribute is true |
| 18 O |  | HT Information | The HT Information element is included by an AP when dot11HighThroughputOptionImplemented attribute is true |
| 3 O |  | Secondary Channel Offset | Secondary Channel Offset element may be present if dot11SpectrumManagementRequired is true and dot11FortyMHzOperationImplemented is true (Ed: CID 1742) |

Proposed resolution:

To insure that the critical HT Beacon IEs are included in the Beacon response it is proposed that following IEs should be truncated in a similar way that the TIM IE is currently truncated in [1]:

IBSS DFS should be truncated to 13 O by including only the minimum and maximum Channel number, Channel map fields.  Therefore the IE will contain: the Element IE, Length, DFS Owner, DFS Recovery Interval, Channel Map for the minimum channel number, and Channel Map for the maximum channel number.

Text to be added to: Reported IBSS DFS elements shall be truncated such that only the lowest and highest channel number map are reported and the element length field is modified to indicate the truncated length of 13.

RSN should be truncated to 4 O.

Text: Reported RSN elements shall be truncated such that only the first 4 octets of the element are reported and the element length field is modified to indicate the truncated length of 4

Truncating these elements will yield a maximum Beacon Report Reported Frame Body of 208 O.  Yielding a maximum Beacon Report of 234 O, which is within the 255 O limit; assuring that TGn fields in the beacon are not truncated.

Text proposal is provided in Text Proposal 1 – Beacon, above.

## Channel Load

Additional information fields may need to be added to the Channel Load Request to add the ability to request information related to 20 MHz, 20/40 MHz, and 40 MHz channels and operating modes.  The Channel Load Report may also need to be modified to allow for the reporting of the various modes of channel operation.

Conclusion:

The current TGk measurement is defined by regulatory class/channel number, which sets the bandwidth of the measurement to the specified channel bandwidth.  Therefore there is no need to change the current TGk draft.

## Noise Histogram

Additional information fields may need to be added to the Noise Histogram Request to add the ability to request information related to 20 MHz, 20/40 MHz, and 40 MHz channels and operating modes.  The Noise Histogram Report may also need to be modified to allow for the reporting of the various modes of channel operation.

Conclusion:

The current TGk measurement is defined by regulatory class/channel number, which sets the bandwidth of the measurement to the specified channel bandwidth.  Therefore there is no need to change the current TGk draft.

## Frame Report Request

Need to consider what will happen with the Frame request report with respect to 20, 20/40, 40 MHz channel bands.  It is not clear as to what should be reported in response to the Frame Report Request.  Note this is optional TGk feature; it uses a “promiscuous” mode configuration of MAC and PHY in order to receive every detected frame (sniffer mode).

The current TGk measurement is defined by regulatory class/channel number, which sets the bandwidth of the measurement to the specified channel bandwidth.  Therefore for channels of 20 MHz or less there are no open issues or need to change the Frame Report Request.  For 40 MHz channels: if the Frame Report is limited to 40 MHz frames and 20 MHz frames on the primary 20 MHz channel then there should be no need to modify the current Frame Report Request. It is strongly recommended that this be acceptable.

If it is required that the secondary 20 MHz channel should also be considered in the Frame Report then changes to TGn would be required, since this would require the ability to monitor two 20 MHz channels simultaneously.

Conclusion:

There is no modification necessary to the Frame Report Request.  The Frame Report for a 40 MHz channel will only contain frame information from the 40 MHz frames and the 20 MHz frames in the primary channel.  Frames from the secondary 20 MHz channel will not be received nor reported on.

## STA Statistics

Clause 7.3.2.21.8 [1] STA Statistics Request provides Group Identity fields for non-TGn STAs.  In order for this section to be complete the following additional Group Identity fields are proposed to be added to Table 29c-Group Identity for a STA Statistics Request of [1].  The counters which are proposed to be added are taken from [2] and are organized into Proposed Group Identity fields as listed below:

STA  Coutnters from dot11A-MSDU Group – Group Identity = 4

dot11TransmittedAMSDUCountCounter32,

dot11FailedAMSDUCountCounter32,

dot11RetryAMSDUCountCounter32,

dot11MultipleRetryAMSDUCountCounter32,

dot1TransmittedOctetsInAMSDUCountCounter64,

dot11AMSDUAckFailureCount Counter32,

dot11ReceivedAMSDUCountCounter32,

dot1ReceivedOctetsInAMSDUCountCounter64,

STA Counters from dot11A-MPDU Group – Group Identity = 5

dot11TransmittedAMPDUCountCounter32,

dot11TransmittedMPDUsInAMPDUCountCounter32,

dot11TransmittedOctetsInAMPDUCountCounter64,

dot11AMPDUReceivedCountCounter32,

dot11MPDUInReceivedAMPDUCountCounter32,

dot11ReceivedOctetsInAMPDUCountCounter64,

dot11AMPDUDelimiterCRCErrorCountCounter32,

STA Counters from dot11 BAR, Channel Width, PSMP Group – Group Identity = 6

dot11ImplicitBARFailureCountCounter32,

dot11ExplicitBARFailureCountCounter32,

dot11ChannelWidthSwitchCountCounter32,

dot11TwentyMHzFrameTransmittedCountCounter32,

dot11FortyMHzFrameTransmittedCountCounter32,

dot11TwentyMHzFrameReceivedCountCounter32,

dot11FortyMHzFrameReceivedCountCounter32,

dot11PSMPSuccessCountCounter32,

dot11PSMPFailureCountCounter32,

STA Counters from dot11Protection Group (RDG, DualCTS, LSIG) – Group Identity = 7

dot11GrantedRDGUsedCountCounter32,

dot11GrantedRDGUnusedCountCounter32,

dot11TransmittedFramesInGrantedRDGCountCounter32,

dot11TransmittedOctetsInGrantedRDGCountCounter64,

dot11DualCTSSuccessCountCounter32,

dot11DualCTSFailureCountCounter32,

dot11RTSLSIGSuccessCountCounter32, (Ed: CID 59)

dot11RTSLSIGFailureCountCounter32 (Ed: CID 59)

dot11 BF, STBC Group – Group Identity = 8

dot11BeamformingFrameCountCounter32,

dot11STBCCTSSuccessCountCounter32,

dot11STBCCTSFailureCountCounter32,

dot11nonSTBCCTSSuccessCountCounter32,

dot11nonSTBCCTSFailureCountCounter32,

Comments on the suitability of the grouping of these STA statistics are solicited.

Note: There is a Group Identity numbering conflict in the current TGk draft [1] – there are different Group Identity numbers in the STA Statistics Report (7.3.2.22.8) section and the STA Statistics Request section (7.3.2.21.8).   This was corrected in the latest version of the TGk draft 6.1 [5] (which is currently not approved by TGk) by correcting Table 29c to be inline with Table 31b.  The Text proposal is based on 6.1 draft [5].

Text proposal is provided in Text Proposal 2 - STA Statistics, above.

## Location Configuration Information

It is not anticipated that any changes need to be made to the Location Configuration Information measurement.

## Neighbor Report

Neighbor report should be modified to define the TGn AP type and operating modes.  The TGk Neighbor Report format was reworked about 6 months ago to allow for new extensions by other TG’s what additional changes if any need to be made.

Optional extension field – this is for gray area things (not on/off features).

Potential additions to the capabilities sub-field or extension field:

HT

LDPC

20/40 Support

SM Power Save

Greenfield

Short GI

STBC

HT Delayed Block Ack

Maximum ASDU Length

DSSS/CCK Mode in 40 MHz

PSMP

L-SIG TXOP Protection

PCO

MCS Feedback

+HTC Support

RD Responder

Beam Forming (1 bit + 18 Fields – 27 bits)

Antenna Selection Capability (7 Fields in 7 bits)

Proposed solution:

Define 1 capabilities sub-field: High Throughput (HT)

Place the remaining capability fields in extension fields.

The Text proposal is provided in Text Proposal 3 - Neighbor Report, above.

## Link Measurement

Since this measurement is basically an RF ping – the Link Measurement Response should be of the same type as the Link Measurement Request.  Therefore there should be no ambiguity in this measurement. No changes are required.

## QoS Metrics

Since all HT STAs are QoS STAs, all HT STAs should be able to support the QoS Metrics Request/Response measurement as it now exits.  However, HT STAs have additional capabilities which should be reported on.  Therefore the HT extensions may impact current metrics or require new metrics to be defined.  This may be done by modification to the existing QoS Metrics Request/Response or by adding a new HT Metrics Request/Response measurement.  No additional capability metrics are proposed at this time.

## RCPI - issues

Need to define RCPI for TGn PHY.

Open issue: the antenna ID for MIMO is not currently defined.

Conclusion: TGn has only defined MIMO RCPI averaged over the MIMO antennas. Therefore the current Antenna ID is adequate to describe the MIMO antenna case. No text changes are required.

The current text from [1] follows:

7.3.2.40 Antenna Information element

The Antenna Information element contains the Antenna ID field as shown in Figure 112j.

Figure 112j—Antenna Information element format

The Length field in octets is set to 1.

The Antenna ID field contains the identifying number for the relevant antenna(s). When included in a Beacon or Probe Response, the Antenna ID identifies the antenna(s) used to transmit the Beacon or Probe Response frame. When included in a measurement report, the Antenna ID identifies the antenna(s) used for the reported measurement. The valid range for the Antenna ID is 1 through 254. The value 0 indicates that the antenna identifier is unknown. The value 255 indicates that this measurement was made with multiple antennas, i.e. antennas were switched during the measurement. The value 1 is the only value used for a STA with only one antenna. STAs with more than one antenna shall assign Antenna IDs to each antenna and each antenna configuration as consecutive, positive integers starting with 1. Each Antenna ID number represents a unique antenna or unique configuration of multiple antennas characterized by a fixed relative position, a fixed relative direction and a fixed peak gain for that position and direction.

## PCO – issues

Basically there are no PCO issues.  Regarding CID 7316 – the counter is: the PCO operational parameters are available as part of the beacon response measurement.

## Additional Areas

Lastly, TGk has defined a starting toolbox for RF measurements, since TGn is expanding the PHY capabilities of 802.11 it may be necessary to add to or modify this toolbox to address the added capabilities of MIMO and beam forming.  The TGn PHY Ad Hoc has been asked think about the need for additional measurements to address MIMO and beamforming; and to provide comments and proposals as they see fit. No additional RF measurements are proposed at this time.

# References:

[1] – IEEE P802.11kTM/D6.0, Draft Standard for Information Technology Telecommunications and information exchange between systems; Local and metropolitan area networks Specific requirements; Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) specifications; Amendment: Radio Resource Measurement, October 2006

[2] – IEEE P802.11n/D1.05, Draft Standard for Information Technology Telecommunications and information exchange between systems; Local and metropolitan area networks Specific requirements; Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) specifications; Amendment: Enhancements for Higher Throughput October 2006

[3] – IEEE P802.11-REVma/D8.0 Draft Standard for Information Technology Telecommunications and information exchange between systems Local and metropolitan area networks Specific requirements

Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) specifications

[4] - IEEE P802.11n/D1.08, Draft Standard for Information Technology Telecommunications and information exchange between systems; Local and metropolitan area networks Specific requirements; Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) specifications; Amendment: Enhancements for Higher Throughput December 2006

[5] – IEEE P802.11kTM/D6.1, Draft Standard for Information Technology Telecommunications and information exchange between systems; Local and metropolitan area networks Specific requirements; Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) specifications; Amendment: Radio Resource Measurement, December 2006

# CID Reference:

| CID | Comment | Proposed Change |
| --- | --- | --- |
| 7026 | Coordinate with TGk to insure that SNR for all Rx channels is measured in a way that is available in the MIB for use by upper layers |  |
| 114 | This reviewer asserts that TGk will be an approved part of .11 prior to TGn. There appears to be significant impact and/or extension needed to the TGk mechanisms to extend them to be cognizant of the changes continued in the TGn draft. Having been involved in TGn for close to 3 years now and not being aware of any discussions of the interaction of TGk and TGn, this needs to be addressed by TGn. | Explicitly work with TGk to determine what the impact of TGn and TGk interactions are; TGn should then jointly specify the TGk extensions necessary as part of the TGn draft. |
| 4347 | You need to add an HT bit into the Neighbor report's BSSID field to indicate the Neighbor is an HT Neighbor. Possibly a 40 Mhz operation bit as well as what the extension channel is in a possible optional IE? Possibly HT capabilities, etc. | Liason with TGk to get this into the Neighbor Report. Carefully consider what can remain in the Beacon as opposed to bloating the Neighbor Lists for a STA to consider which candidates are worth considering going off channel to collect the beacon, and/or can be determined after BSS Transistion Candidate selection. |
| 7316 | Multiple BSSs operating uncoordinated PCOs will clash with each other. | Recommend some way of sharing PCO operational parameters for coordination, perhaps via extensions to 11k. |