IEEE P802.11
Wireless LANs

| TGn LB84 Submission to Resolve CIDs 1570 and 7165: Examples of the Low-Density Parity Check (LDPC) Encoding Process |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-04-05 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| George Vlantis | STMicroelectronics, Inc. | 1060 East Brokaw Road, San Jose CA 95131 | +1.408.451.8109 (o) +1.408.893.9357 (m) | George.Vlantis@st.com |
| Devis Gatti | STMicroelectronics, N.V. | Centro Direzionale Colleoni; Palazzo "La Dialettica"; 20041 Agrate Brianza, Milano; Italy | +39.039.603.7236 | Devis.Gatti@st.com |
| Raffaele Riva | STMicroelectronics, N.V. | Centro Direzionale Colleoni; Palazzo "La Dialettica"; 20041 Agrate Brianza, Milano; Italy | +39.039.603.6255 | Raffaele.Riva@st.com |

Abstract

This document contains proposed changes to the IEEE P802.11n Draft to address the following LB84 comments:

- CID 1570 and 7165

The changes marked in this document are based on TGn Draft version P802.11n-D1.06 and modifications to Annex G as specified in the document entitled: IEEE Std. 802.11-2003, "IEEE Wireless LAN Edition, A compilation based on IEEE Std. 802.11TM-1999 (R2003) and its amendments".

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# CID 1570 and 7165

## Background

The proposed actionable modifications to the TGn Draft version P802.11n-D1.06, resolving the CIDs #1570 and 7165, are contained in this document.  I include the disposition of the CIDs at this writing:

| CID | Name of Commenter(Ed) | Clause Number(C) | Page(C) | Line(C) | Type of Comment T/E(C) | Part of No Vote(Y/N) | Page(Ed) | Line(Ed) | Clause Title(Ed) | Clause(Ed) | Type E/HE/T/ST/DT(Ed) | Duplicate of CID | Resn Status | Assignee | Submission | TGn approval |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1570 | Hedberg, David | 20.3.4.3.3.4 | 201-203 | 11-24 1-19 1-14 | T | N | 201 | 11 | LDPCC PPDU encoding process | 20.3.4.3.3.4 | T |  | D | George Vlantis |  |  |
| 7165 | Petranovich, James | 20.3.4.3.3.4 | 201 | 13 | E | N | 201 | 13 | LDPCC PPDU encoding process | 20.3.4.3.3.4 | T |  | D | George Vlantis |  |  |

| CID | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- |
| 1570 | The LDPCC PPDU encoding process is very complex and not very intuitive. It would greatly add to clarity if a numerical example or two were added in an Annex -- to illustrate how this formatting process works. | Add an Annex with numerical examples to illustrate the LDPCC PPDU encoding process. |  |
| 7165 | LDPCC PPDU encoding process is complicated. It is difficult to determine whether the text unambiguously specified the process. | Add an informative annex to the standard with several examples of a PPDU encoding process, showing the result of each step of the process. |  |

Based on the Comments and Proposed Changes in CIDs #1570 and 7165, the authors decided to craft two examples of the LDPC encoding process for the informative Annex G to achieve the following objectives:

(1) LDPC Example #1 should be as similar as possible to the existing BCC example in Annex G.

(2) LDPC Example #2 should exercise as many alternative branches of the LDPC encoding process not exercised in the first example.

(3) Diversity in the values of the TXVECTOR parameters that affect the LDPC encoding process should be used, to the extent that the size of the example would not grow to a cumbersome length.

The messages being encoded consist of the first 72 characters or the first 112 characters of the well-known "Ode to Joy" by F. Schiller. (Note that the ‘p’ in “Fire-inspired” is omitted, which is consistent with all versions of Annex G):

Joy, bright spark of divinity,

Daughter of Elysium,

Fire-insired we tread

Thy sanctuary.

Thy magic power re-unites

All that custom has divided,

All men become brothers

Under the say of thy gentle wings.

## Key to Editorial Instructions

All editorial instructions are given in bold red  italic font below.  Strike-through font is used to illustrate text that should be deleted from the existing standard referenced above.  Blue text denotes text that should be added.

Bold text has no significance other than to draw the reader’s attention to those sections.  Typically, the explanatory text above the tables refers to these sections within the various tables, or these sections are differentiated because of changes with respect to the previous table.  I would suggest leaving the bold text, unless there is a comment to remove it—the bold font is not required.

With regard to the BCC encoding example changes, because line numbers are not available in published standards, it was most convenient to replicate the text from the IEEE Std. 802.11-2003, and then use the editorial instructions denoted above to illustrate the changes.  Editorial instructions that may appear to be “self-referencing”, e.g. “replace old Table G.13 with new Table G.13”, should be interpreted to mean that the Table G.13 that currently exists in the IEEE Std. 802.11-2003 standard should be replaced with the Table G.13 in this document.

For general guidance, the following BCC encoding example tables change:

Table G.1 is corrected.

Table G.14 is corrected and merged with Table G.13 into a single Table G.13.

Table G.17 is corrected and merged with Table G.16 into a single Table G.16

Table G.24 is broken up into tables G.24 to G.32 for clarity and delineation of symbol boundaries.  Table G.32 contains the data that changed.

## Existing BCC Encoding Example:

This example is identical to the one found in Annex G, with the following bugs fixed:

(1) CRC bug in table G.1 (which propagates to tables G.13, G.14, G.16, G.17, and G.18); and

(2) bit ordering of octets reversed in tables G.13 and G.14

Input TXVECTOR parameters for the BCC Example:

R1-R4 bits in SIGNAL field = 0x0B (Data Rate=36MB/s, QAM 16; Code Rate = 3/4)

Code Rate R = 3/4

TXVectorLength = 100 Octets       (with 16-bit SERVICE field, 6 tail bits,

and 42 bits padding becomes 108 octets =

864 bits to scramble and encode)

Editor: Replace the old title for Sub-clause G.2 with the new title for Sub-clause G.2.

G.2 The message

G.2 The message for the BCC Example

The message being encoded consists of the first 72 characters of the well-known "Ode to Joy" by F. Schiller:

Editor:  Change to bold text the first 72 characters of the message—i.e. up to the letter ‘a’ in “tread” to be consistent with the LDPC examples.  (Note if bold font is removed from the LDPC examples, then ignore this instruction.)

Joy, bright spark of divinity,

Daughter of Elysium,

Fire-insired we tread

Thy sanctuary.

Thy magic power re-unites

All that custom has divided,

All men become brothers

Under the sway of thy gentle wings.

The message is converted to ASCII; then it is prepended with an appropriate MAC header and a CRC32 is added.  The resulting 100 octets PSDU is shown in Table G.1.

Editor: Replace old caption for Table G.1 with the new caption for Table G.1.

Table G.1—The message

Table G.1—The message for the BCC Example

Editor: Replace the old Table G.1 with the new Table G.1 below.  Note that there are three changes: (1) The header for the  left column has changed; (2) seven entries in the left column have changed;  and finally (3) the last 4 entries of the bottom row have  the corrected CRC32 value. It is not required to preserve the bold font.

| Octet ## | Val | Val | Val | Val | Val |
| --- | --- | --- | --- | --- | --- |
| 1...5 | 04 | 02 | 00 | 2E | 00 |
| 6...10 | 60 | 08 | CD | 37 | A6 |
| 11...15 | 00 | 20 | D6 | 01 | 3C |
| 16...20 | F1 | 00 | 60 | 08 | AD |
| 21...25 | 3B | AF | 00 | 00 | 4A |
| 26...30 | 6F | 79 | 2C | 20 | 62 |
| 31...35 | 72 | 69 | 67 | 68 | 74 |
| 36...40 | 20 | 73 | 70 | 61 | 72 |
| 41...45 | 6B | 20 | 6F | 66 | 20 |
| 46...50 | 64 | 69 | 76 | 69 | 6E |
| 51...55 | 69 | 74 | 79 | 2C | 0A |
| 56...60 | 44 | 61 | 75 | 67 | 68 |
| 61...65 | 74 | 65 | 72 | 20 | 6F |
| 66...70 | 66 | 20 | 45 | 6C | 79 |
| 71...75 | 73 | 69 | 75 | 6D | 2C |
| 76...80 | 0A | 46 | 69 | 72 | 65 |
| 81...85 | 2D | 69 | 6E | 73 | 69 |
| 86...90 | 72 | 65 | 64 | 20 | 77 |
| 91...95 | 65 | 20 | 74 | 72 | 65 |
| 96...100 | 61 | 67 | 33 | 21 | B6 |

Editor: Replace the old title for Sub-clause G.5 with the new title for Sub-clause G.5.

G.5 Generating the DATA bits

G.5 Generating the DATA bits for the BCC Example

G.5.1 Delineating, SERVICE field prepending, and zero padding

The transmitted message shown in Table G.1 contains 100 octets or, equivalently, 800 bits.  The bits are prepended by the 16 SERVICE field bits and are appended by 6 tail bits.  The resulting 822 bits are appended by zero bits to yield an integer number of OFDM symbols.  For the 36 Mbit/s mode, there are 144 data bits per OFDM symbol; the overall number of bits is ceiling (822/144) × 144 = 864.  Hence, 864 – 822 = 42 zero bits are appended.

Editor: Replace the 2nd paragraph of Sub-clause G.5.1 with the paragraph below.

The data bits are shown in Table G.13 and Table G.14.  For clarity, only the first and last 144 bits are shown.

All the DATA bits are shown in Table G.13.   Note that the bit-ordering of the octets is most significant bit first.

Editor: Replace old caption for Table G.13 with the new caption for Table G.13.

Table G.13—First 144 DATA bits

Table G.13—The DATA bits before scrambling

Editor: Replace the old Table G.13 with the new Table G.13 below.  Delete Table G.14 and the caption for Table G.14.  (The bold font here denotes the zero bits that are prepended and appended, pluse the corrected CRC values in bits 784-815.   It is not required to preserve the bold font.)

| Bit ## | Binary Val | Binary Val | Binary Val | Hex Val | Hex Val | Hex Val |
| --- | --- | --- | --- | --- | --- | --- |
| 000-023 | 00000000 | 00000000 | 00000100 | 0x00 | 0x00 | 0x04 |
| 024-047 | 00000010 | 00000000 | 00101110 | 0x02 | 0x00 | 0x2E |
| 048-071 | 00000000 | 01100000 | 00001000 | 0x00 | 0x60 | 0x08 |
| 072-095 | 11001101 | 00110111 | 10100110 | 0xCD | 0x37 | 0xA6 |
| 096-119 | 00000000 | 00100000 | 11010110 | 0x00 | 0x20 | 0xD6 |
| 120-143 | 00000001 | 00111100 | 11110001 | 0x01 | 0x3C | 0xF1 |
| 144-167 | 00000000 | 01100000 | 00001000 | 0x00 | 0x60 | 0x08 |
| 168-191 | 10101101 | 00111011 | 10101111 | 0xAD | 0x3B | 0xAF |
| 192-215 | 00000000 | 00000000 | 01001010 | 0x00 | 0x00 | 0x4A |
| 216-239 | 01101111 | 01111001 | 00101100 | 0x6F | 0x79 | 0x2C |
| 240-263 | 00100000 | 01100010 | 01110010 | 0x20 | 0x62 | 0x72 |
| 264-287 | 01101001 | 01100111 | 01101000 | 0x69 | 0x67 | 0x68 |
| 288-311 | 01110100 | 00100000 | 01110011 | 0x74 | 0x20 | 0x73 |
| 312-335 | 01110000 | 01100001 | 01110010 | 0x70 | 0x61 | 0x72 |
| 336-359 | 01101011 | 00100000 | 01101111 | 0x6B | 0x20 | 0x6F |
| 360-383 | 01100110 | 00100000 | 01100100 | 0x66 | 0x20 | 0x64 |
| 384-407 | 01101001 | 01110110 | 01101001 | 0x69 | 0x76 | 0x69 |
| 408-431 | 01101110 | 01101001 | 01110100 | 0x6E | 0x69 | 0x74 |
| 432-455 | 01111001 | 00101100 | 00001010 | 0x79 | 0x2C | 0x0A |
| 456-479 | 01000100 | 01100001 | 01110101 | 0x44 | 0x61 | 0x75 |
| 480-503 | 01100111 | 01101000 | 01110100 | 0x67 | 0x68 | 0x74 |
| 504-527 | 01100101 | 01110010 | 00100000 | 0x65 | 0x72 | 0x20 |
| 528-551 | 01101111 | 01100110 | 00100000 | 0x6F | 0x66 | 0x20 |
| 552-575 | 01000101 | 01101100 | 01111001 | 0x45 | 0x6C | 0x79 |
| 576-599 | 01110011 | 01101001 | 01110101 | 0x73 | 0x69 | 0x75 |
| 600-623 | 01101101 | 00101100 | 00001010 | 0x6D | 0x2C | 0x0A |
| 624-647 | 01000110 | 01101001 | 01110010 | 0x46 | 0x69 | 0x72 |
| 648-671 | 01100101 | 00101101 | 01101001 | 0x65 | 0x2D | 0x69 |
| 672-695 | 01101110 | 01110011 | 01101001 | 0x6E | 0x73 | 0x69 |
| 696-719 | 01110010 | 01100101 | 01100100 | 0x72 | 0x65 | 0x64 |
| 720-743 | 00100000 | 01110111 | 01100101 | 0x20 | 0x77 | 0x65 |
| 744-767 | 00100000 | 01110100 | 01110010 | 0x20 | 0x74 | 0x72 |
| 768-791 | 01100101 | 01100001 | 01100111 | 0x65 | 0x61 | 0x67 |
| 792-815 | 00110011 | 00100001 | 10110110 | 0x33 | 0x21 | 0xB6 |
| 816-839 | 00000000 | 00000000 | 00000000 | 0x00 | 0x00 | 0x00 |
| 840-863 | 00000000 | 00000000 | 00000000 | 0x00 | 0x00 | 0x00 |

Editor: Replace the old title for Sub-clause G.5.2 with the new title for Sub-clause G.5.2.

G.5.2 Scrambling

G.5.2 Scrambling the BCC Example

The 864 bits are scrambled by the scrambler of Figure 117 (in 17.3.5.4).  The initial state of the scrambler is the state 1011101.  The generated scrambling sequence is given in Table G.15.

After scrambling, the 6 bits in location 816 (i.e. the 817th bit) to 821 (the 822nd bit) are zeroed.  The first and last 144 scrambled bits are show in Table G.16 and G.17, respectively.

Editor: Replace the last sentence of the second  paragraph of Sub-clause G.5.2 above  with the sentence  below.

All the scrambled DATA bits are shown in Table G.16, with the bit-ordering being most significant bit first.

Editor: Replace old caption for Table G.16 with the new caption for Table G.16.

Table G.16—First 144 bits after scrambling

Table G.16—The DATA bits after scrambling

Editor: Replace the old Table G.16 with the new Table G.16 below.  Delete Table G.17 and the caption for Table G.17 (The bold font here denote the bits that changed due to the corrected CRC values in bits 784-815, plus the 6 bits that are cleared after scrambling immediate after.  It is not required to preserve the bold font.)

| Bit ## | Binary Val | Binary Val | Binary Val | Hex Val | Hex Val | Hex Val |
| --- | --- | --- | --- | --- | --- | --- |
| 000-023 | 01101100 | 00011001 | 10001001 | 0x6C | 0x19 | 0x89 |
| 024-047 | 10001111 | 01101000 | 00100001 | 0x8F | 0x68 | 0x21 |
| 048-071 | 11110100 | 10100101 | 01100001 | 0xF4 | 0xA5 | 0x61 |
| 072-095 | 01001111 | 11010111 | 10101110 | 0x4F | 0xD7 | 0xAE |
| 096-119 | 00100100 | 00001100 | 11110011 | 0x24 | 0x0C | 0xF3 |
| 120-143 | 00111010 | 11100100 | 10111100 | 0x3A | 0xE4 | 0xBC |
| 144-167 | 01010011 | 10011000 | 11000000 | 0x53 | 0x98 | 0xC0 |
| 168-191 | 00011110 | 00110101 | 10110011 | 0x1E | 0x35 | 0xB3 |
| 192-215 | 11100011 | 11111000 | 00100101 | 0xE3 | 0xF8 | 0x25 |
| 216-239 | 01100000 | 11010110 | 00100101 | 0x60 | 0xD6 | 0x25 |
| 240-263 | 00110101 | 00110011 | 11111110 | 0x35 | 0x33 | 0xFE |
| 264-287 | 11110000 | 01000001 | 00101011 | 0xF0 | 0x41 | 0x2B |
| 288-311 | 10001111 | 01010011 | 00011100 | 0x8F | 0x53 | 0x1C |
| 312-335 | 10000011 | 01000001 | 10111110 | 0x83 | 0x41 | 0xBE |
| 336-359 | 00111001 | 00101000 | 01100110 | 0x39 | 0x28 | 0x66 |
| 360-383 | 01000100 | 01100110 | 11001101 | 0x44 | 0x66 | 0xCD |
| 384-407 | 11110110 | 10100011 | 11011000 | 0xF6 | 0xA3 | 0xD8 |
| 408-431 | 00001101 | 11010100 | 10000001 | 0x0D | 0xD4 | 0x81 |
| 432-455 | 00111011 | 00101111 | 11011111 | 0x3B | 0x2F | 0xDF |
| 456-479 | 11000011 | 01011000 | 11110111 | 0xC3 | 0x58 | 0xF7 |
| 480-503 | 11000110 | 01010010 | 11101011 | 0xC6 | 0x52 | 0xEB |
| 504-527 | 01110000 | 10001111 | 10011110 | 0x70 | 0x8F | 0x9E |
| 528-551 | 01101010 | 10010000 | 10000001 | 0x6A | 0x90 | 0x81 |
| 552-575 | 11111101 | 01111100 | 10101001 | 0xFD | 0x7C | 0xA9 |
| 576-599 | 11010001 | 01010101 | 00010010 | 0xD1 | 0x55 | 0x12 |
| 600-623 | 00000100 | 01110100 | 11011001 | 0x04 | 0x74 | 0xD9 |
| 624-647 | 11101001 | 00111011 | 11001101 | 0xE9 | 0x3B | 0xCD |
| 648-671 | 10010011 | 10001101 | 01111011 | 0x93 | 0x8D | 0x7B |
| 672-695 | 01111100 | 01110000 | 00000010 | 0x7C | 0x70 | 0x02 |
| 696-719 | 00100000 | 10011001 | 10100001 | 0x20 | 0x99 | 0xA1 |
| 720-743 | 01111101 | 10001010 | 00100111 | 0x7D | 0x8A | 0x27 |
| 744-767 | 00010111 | 00111001 | 00010101 | 0x17 | 0x39 | 0x15 |
| 768-791 | 10100000 | 11101100 | 10010101 | 0xA0 | 0xEC | 0x95 |
| 792-815 | 00010110 | 10010001 | 00010000 | 0x16 | 0x91 | 0x10 |
| 816-839 | 00000000 | 11011100 | 01111111 | 0x00 | 0xDC | 0x7F |
| 840-863 | 00001110 | 11110010 | 11001001 | 0x0E | 0xF2 | 0xC9 |

Editor: Replace the old title for Sub-clause G.6 with the new title for Sub-clause G.6.

G.6 Generating the first DATA symbol

G.6 Generating the first DATA symbol for the BCC Example

G.6.1 Coding the DATA bits

Editor: Replace the only paragraph of Sub-clause G.6.1 with the new paragraph for Sub-clause G.6.1 below.  (Note that the omega symbol has been replace with “3 / 4” in the first sentence, and the second sentenced has been rewritten.)

The scrambled bits are coded with a rate Ω convolutional code.  The first 144 scrambled bits of Table G.16 are mapped into the 192 bits of G.18.

The scrambled bits are coded with a rate 3/4 convolutional code.  All the DATA encoded bits are shown in Table G.18, with the bit-ordering being most significant bit first.

Editor: Replace old caption for Table G.18 with the new caption for Table G.18.

Table G.18—Coded bits of  first DATA symbol

Table G.18--The BCC encoded DATA bits

Editor: Replace the old Table G.18 with the new Table G.18 below.

| Bit ## | Binary Val | Binary Val | Binary Val | Binary Val | Hex Val | Hex Val | Hex Val | Hex Val |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0000-0031 | 00101011 | 00001000 | 10100001 | 11110000 | 0x2B | 0x08 | 0xA1 | 0xF0 |
| 0032-0063 | 10011101 | 10110101 | 10011010 | 00011101 | 0x9D | 0xB5 | 0x9A | 0x1D |
| 0064-0095 | 01001010 | 11111011 | 11101000 | 11000010 | 0x4A | 0xFB | 0xE8 | 0xC2 |
| 0096-0127 | 10001111 | 11000000 | 11001000 | 01110011 | 0x8F | 0xC0 | 0xC8 | 0x73 |
| 0128-0159 | 11000000 | 01000011 | 11100000 | 00011001 | 0xC0 | 0x43 | 0xE0 | 0x19 |
| 0160-0191 | 11100000 | 11010011 | 11101011 | 10110010 | 0xE0 | 0xD3 | 0xEB | 0xB2 |
| 0192-0223 | 10101111 | 10011000 | 11111101 | 01011001 | 0xAF | 0x98 | 0xFD | 0x59 |
| 0224-0255 | 00001111 | 10001011 | 01101001 | 01100110 | 0x0F | 0x8B | 0x69 | 0x66 |
| 0256-0287 | 00001100 | 10101010 | 11011001 | 00010000 | 0x0C | 0xAA | 0xD9 | 0x10 |
| 0288-0319 | 01010110 | 10001011 | 10100110 | 01000000 | 0x56 | 0x8B | 0xA6 | 0x40 |
| 0320-0351 | 01100100 | 10110011 | 00100001 | 10011110 | 0x64 | 0xB3 | 0x21 | 0x9E |
| 0352-0383 | 10001110 | 10010001 | 11000001 | 00000101 | 0x8E | 0x91 | 0xC1 | 0x05 |
| 0384-0415 | 10110111 | 10110111 | 11000101 | 11011000 | 0xB7 | 0xB7 | 0xC5 | 0xD8 |
| 0416-0447 | 10000000 | 00101111 | 10100010 | 11011101 | 0x80 | 0x2F | 0xA2 | 0xDD |
| 0448-0479 | 01101111 | 00101011 | 10010111 | 01100001 | 0x6F | 0x2B | 0x97 | 0x61 |
| 0480-0511 | 11011001 | 11011101 | 00001101 | 00010010 | 0xD9 | 0xDD | 0x0D | 0x12 |
| 0512-0543 | 01110110 | 00100111 | 00000010 | 01001100 | 0x76 | 0x27 | 0x02 | 0x4C |
| 0544-0575 | 10010010 | 10111100 | 00010010 | 01001011 | 0x92 | 0xBC | 0x12 | 0x4B |
| 0576-0607 | 01101010 | 11110111 | 01110000 | 00100011 | 0x6A | 0xF7 | 0x70 | 0x23 |
| 0608-0639 | 00100111 | 10001110 | 00000001 | 10110100 | 0x27 | 0x8E | 0x01 | 0xB4 |
| 0640-0671 | 11010110 | 11000011 | 01101010 | 01100000 | 0xD6 | 0xC3 | 0x6A | 0x60 |
| 0672-0703 | 01001101 | 01001011 | 11001011 | 01010001 | 0x4D | 0x4B | 0xCB | 0x51 |
| 0704-0735 | 10011100 | 10110000 | 10000000 | 11101011 | 0x9C | 0xB0 | 0x80 | 0xEB |
| 0736-0767 | 10001001 | 00110100 | 00010100 | 01000000 | 0x89 | 0x34 | 0x14 | 0x40 |
| 0768-0799 | 01101100 | 10011110 | 00101100 | 01010001 | 0x6C | 0x9E | 0x2C | 0x51 |
| 0800-0831 | 01001011 | 01111100 | 01101001 | 00010001 | 0x4B | 0x7C | 0x69 | 0x11 |
| 0832-0863 | 00010101 | 10000110 | 11111101 | 10111110 | 0x15 | 0x86 | 0xFD | 0xBE |
| 0864-0895 | 01011110 | 11111001 | 10111110 | 00101000 | 0x5E | 0xF9 | 0xBE | 0x28 |
| 0896-0927 | 11101111 | 11001010 | 01010101 | 00000011 | 0xEF | 0xCA | 0x55 | 0x03 |
| 0928-0959 | 11111101 | 00100110 | 10010001 | 00111011 | 0xFD | 0x26 | 0x91 | 0x3B |
| 0960-0991 | 10010101 | 11101100 | 01011011 | 00100011 | 0x95 | 0xEC | 0x5B | 0x23 |
| 0992-1023 | 10011001 | 01011111 | 00101000 | 00111110 | 0x99 | 0x5F | 0x28 | 0x3E |
| 1024-1055 | 11010100 | 11101001 | 11110111 | 10111000 | 0xD4 | 0xE9 | 0xF7 | 0xB8 |
| 1056-1087 | 00010011 | 01110101 | 10001110 | 11110010 | 0x13 | 0x75 | 0x8E | 0xF2 |
| 1088-1119 | 10100000 | 00011011 | 01101100 | 11101001 | 0xA0 | 0x1B | 0x6C | 0xE9 |
| 1120-1151 | 00000111 | 01011101 | 10110000 | 10111111 | 0x07 | 0x5D | 0xB0 | 0xBF |

Editor: Replace the old title for Sub-clause G.8 with the new title for Sub-clause G.8.

G.6 The entire packet

G.6 The entire packet for the BCC Example

Editor: Replace the only paragraph of Sub-clause G.6.8 with the new paragraph for Sub-clause G.6.1 below

The packet in its entirety is shown in Table G.24.  The short sequences section, the long sequences section, the SIGNAL field, and the DATA symbols are separated by double lines.

The packet in its entirety is shown in Tables G.24 through G.32.  These tables illustrate the short training sequence section (Table G.24), the long training sequence section (Table G.25), the SIGNAL field (Table G.26), and the six DATA symbols (Table G.27 through G.32).

Editor: Replace the old caption for Table  G.24 with the new caption for Table G.24 below .

Table G.24—The entire packet

Table G.24--Time Domain representation of the short training sequence

Editor: Replace the old Table  G.24 with the new Table G.24 below.  (Note that that the sixth and final DATA symbol (Table G.32) reflects the changes due to the corrected CRC32 value.)

| ## | Real | Imag | ## | Real | Imag | ## | Real | Imag | ## | Real | Imag |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0.023 | 0.023 | 1 | -0.132 | 0.002 | 2 | -0.013 | -0.079 | 3 | 0.143 | -0.013 |
| 4 | 0.092 | 0.000 | 5 | 0.143 | -0.013 | 6 | -0.013 | -0.079 | 7 | -0.132 | 0.002 |
| 8 | 0.046 | 0.046 | 9 | 0.002 | -0.132 | 10 | -0.079 | -0.013 | 11 | -0.013 | 0.143 |
| 12 | 0.000 | 0.092 | 13 | -0.013 | 0.143 | 14 | -0.079 | -0.013 | 15 | 0.002 | -0.132 |
| 16 | 0.046 | 0.046 | 17 | -0.132 | 0.002 | 18 | -0.013 | -0.079 | 19 | 0.143 | -0.013 |
| 20 | 0.092 | 0.000 | 21 | 0.143 | -0.013 | 22 | -0.013 | -0.079 | 23 | -0.132 | 0.002 |
| 24 | 0.046 | 0.046 | 25 | 0.002 | -0.132 | 26 | -0.079 | -0.013 | 27 | -0.013 | 0.143 |
| 28 | 0.000 | 0.092 | 29 | -0.013 | 0.143 | 30 | -0.079 | -0.013 | 31 | 0.002 | -0.132 |
| 32 | 0.046 | 0.046 | 33 | -0.132 | 0.002 | 34 | -0.013 | -0.079 | 35 | 0.143 | -0.013 |
| 36 | 0.092 | 0.000 | 37 | 0.143 | -0.013 | 38 | -0.013 | -0.079 | 39 | -0.132 | 0.002 |
| 40 | 0.046 | 0.046 | 41 | 0.002 | -0.132 | 42 | -0.079 | -0.013 | 43 | -0.013 | 0.143 |
| 44 | 0.000 | 0.092 | 45 | -0.013 | 0.143 | 46 | -0.079 | -0.013 | 47 | 0.002 | -0.132 |
| 48 | 0.046 | 0.046 | 49 | -0.132 | 0.002 | 50 | -0.013 | -0.079 | 51 | 0.143 | -0.013 |
| 52 | 0.092 | 0.000 | 53 | 0.143 | -0.013 | 54 | -0.013 | -0.079 | 55 | -0.132 | 0.002 |
| 56 | 0.046 | 0.046 | 57 | 0.002 | -0.132 | 58 | -0.079 | -0.013 | 59 | -0.013 | 0.143 |
| 60 | 0.000 | 0.092 | 61 | -0.013 | 0.143 | 62 | -0.079 | -0.013 | 63 | 0.002 | -0.132 |
| 64 | 0.046 | 0.046 | 65 | -0.132 | 0.002 | 66 | -0.013 | -0.079 | 67 | 0.143 | -0.013 |
| 68 | 0.092 | 0.000 | 69 | 0.143 | -0.013 | 70 | -0.013 | -0.079 | 71 | -0.132 | 0.002 |
| 72 | 0.046 | 0.046 | 73 | 0.002 | -0.132 | 74 | -0.079 | -0.013 | 75 | -0.013 | 0.143 |
| 76 | 0.000 | 0.092 | 77 | -0.013 | 0.143 | 78 | -0.079 | -0.013 | 79 | 0.002 | -0.132 |
| 80 | 0.046 | 0.046 | 81 | -0.132 | 0.002 | 82 | -0.013 | -0.079 | 83 | 0.143 | -0.013 |
| 84 | 0.092 | 0.000 | 85 | 0.143 | -0.013 | 86 | -0.013 | -0.079 | 87 | -0.132 | 0.002 |
| 88 | 0.046 | 0.046 | 89 | 0.002 | -0.132 | 90 | -0.079 | -0.013 | 91 | -0.013 | 0.143 |
| 92 | 0.000 | 0.092 | 93 | -0.013 | 0.143 | 94 | -0.079 | -0.013 | 95 | 0.002 | -0.132 |
| 96 | 0.046 | 0.046 | 97 | -0.132 | 0.002 | 98 | -0.013 | -0.079 | 99 | 0.143 | -0.013 |
| 100 | 0.092 | 0.000 | 101 | 0.143 | -0.013 | 102 | -0.013 | -0.079 | 103 | -0.132 | 0.002 |
| 104 | 0.046 | 0.046 | 105 | 0.002 | -0.132 | 106 | -0.079 | -0.013 | 107 | -0.013 | 0.143 |
| 108 | 0.000 | 0.092 | 109 | -0.013 | 0.143 | 110 | -0.079 | -0.013 | 111 | 0.002 | -0.132 |
| 112 | 0.046 | 0.046 | 113 | -0.132 | 0.002 | 114 | -0.013 | -0.079 | 115 | 0.143 | -0.013 |
| 116 | 0.092 | 0.000 | 117 | 0.143 | -0.013 | 118 | -0.013 | -0.079 | 119 | -0.132 | 0.002 |
| 120 | 0.046 | 0.046 | 121 | 0.002 | -0.132 | 122 | -0.079 | -0.013 | 123 | -0.013 | 0.143 |
| 124 | 0.000 | 0.092 | 125 | -0.013 | 0.143 | 126 | -0.079 | -0.013 | 127 | 0.002 | -0.132 |
| 128 | 0.046 | 0.046 | 129 | -0.132 | 0.002 | 130 | -0.013 | -0.079 | 131 | 0.143 | -0.013 |
| 132 | 0.092 | 0.000 | 133 | 0.143 | -0.013 | 134 | -0.013 | -0.079 | 135 | -0.132 | 0.002 |
| 136 | 0.046 | 0.046 | 137 | 0.002 | -0.132 | 138 | -0.079 | -0.013 | 139 | -0.013 | 0.143 |
| 140 | 0.000 | 0.092 | 141 | -0.013 | 0.143 | 142 | -0.079 | -0.013 | 143 | 0.002 | -0.132 |
| 144 | 0.046 | 0.046 | 145 | -0.132 | 0.002 | 146 | -0.013 | -0.079 | 147 | 0.143 | -0.013 |
| 148 | 0.092 | 0.000 | 149 | 0.143 | -0.013 | 150 | -0.013 | -0.079 | 151 | -0.132 | 0.002 |
| 152 | 0.046 | 0.046 | 153 | 0.002 | -0.132 | 154 | -0.079 | -0.013 | 155 | -0.013 | 0.143 |
| 156 | 0.000 | 0.092 | 157 | -0.013 | 0.143 | 158 | -0.079 | -0.013 | 159 | 0.002 | -0.132 |

Editor: Append after Table G.24 the new Tables G.25 through G.32 and their associated captions below.

Table G.25--Time Domain representation of the long training sequence

| ## | Real | Imag | ## | Real | Imag | ## | Real | Imag | ## | Real | Imag |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 160 | -0.055 | 0.023 | 161 | 0.012 | -0.098 | 162 | 0.092 | -0.106 | 163 | -0.092 | -0.115 |
| 164 | -0.003 | -0.054 | 165 | 0.075 | 0.074 | 166 | -0.127 | 0.021 | 167 | -0.122 | 0.017 |
| 168 | -0.035 | 0.151 | 169 | -0.056 | 0.022 | 170 | -0.060 | -0.081 | 171 | 0.070 | -0.014 |
| 172 | 0.082 | -0.092 | 173 | -0.131 | -0.065 | 174 | -0.057 | -0.039 | 175 | 0.037 | -0.098 |
| 176 | 0.062 | 0.062 | 177 | 0.119 | 0.004 | 178 | -0.022 | -0.161 | 179 | 0.059 | 0.015 |
| 180 | 0.024 | 0.059 | 181 | -0.137 | 0.047 | 182 | 0.001 | 0.115 | 183 | 0.053 | -0.004 |
| 184 | 0.098 | 0.026 | 185 | -0.038 | 0.106 | 186 | -0.115 | 0.055 | 187 | 0.060 | 0.088 |
| 188 | 0.021 | -0.028 | 189 | 0.097 | -0.083 | 190 | 0.040 | 0.111 | 191 | -0.005 | 0.120 |
| 192 | 0.156 | 0.000 | 193 | -0.005 | -0.120 | 194 | 0.040 | -0.111 | 195 | 0.097 | 0.083 |
| 196 | 0.021 | 0.028 | 197 | 0.060 | -0.088 | 198 | -0.115 | -0.055 | 199 | -0.038 | -0.106 |
| 200 | 0.098 | -0.026 | 201 | 0.053 | 0.004 | 202 | 0.001 | -0.115 | 203 | -0.137 | -0.047 |
| 204 | 0.024 | -0.059 | 205 | 0.059 | -0.015 | 206 | -0.022 | 0.161 | 207 | 0.119 | -0.004 |
| 208 | 0.062 | -0.062 | 209 | 0.037 | 0.098 | 210 | -0.057 | 0.039 | 211 | -0.131 | 0.065 |
| 212 | 0.082 | 0.092 | 213 | 0.070 | 0.014 | 214 | -0.060 | 0.081 | 215 | -0.056 | -0.022 |
| 216 | -0.035 | -0.151 | 217 | -0.122 | -0.017 | 218 | -0.127 | -0.021 | 219 | 0.075 | -0.074 |
| 220 | -0.003 | 0.054 | 221 | -0.092 | 0.115 | 222 | 0.092 | 0.106 | 223 | 0.012 | 0.098 |
| 224 | -0.156 | 0.000 | 225 | 0.012 | -0.098 | 226 | 0.092 | -0.106 | 227 | -0.092 | -0.115 |
| 228 | -0.003 | -0.054 | 229 | 0.075 | 0.074 | 230 | -0.127 | 0.021 | 231 | -0.122 | 0.017 |
| 232 | -0.035 | 0.151 | 233 | -0.056 | 0.022 | 234 | -0.060 | -0.081 | 235 | 0.070 | -0.014 |
| 236 | 0.082 | -0.092 | 237 | -0.131 | -0.065 | 238 | -0.057 | -0.039 | 239 | 0.037 | -0.098 |
| 240 | 0.062 | 0.062 | 241 | 0.119 | 0.004 | 242 | -0.022 | -0.161 | 243 | 0.059 | 0.015 |
| 244 | 0.024 | 0.059 | 245 | -0.137 | 0.047 | 246 | 0.001 | 0.115 | 247 | 0.053 | -0.004 |
| 248 | 0.098 | 0.026 | 249 | -0.038 | 0.106 | 250 | -0.115 | 0.055 | 251 | 0.060 | 0.088 |
| 252 | 0.021 | -0.028 | 253 | 0.097 | -0.083 | 254 | 0.040 | 0.111 | 255 | -0.005 | 0.120 |
| 256 | 0.156 | 0.000 | 257 | -0.005 | -0.120 | 258 | 0.040 | -0.111 | 259 | 0.097 | 0.083 |
| 260 | 0.021 | 0.028 | 261 | 0.060 | -0.088 | 262 | -0.115 | -0.055 | 263 | -0.038 | -0.106 |
| 264 | 0.098 | -0.026 | 265 | 0.053 | 0.004 | 266 | 0.001 | -0.115 | 267 | -0.137 | -0.047 |
| 268 | 0.024 | -0.059 | 269 | 0.059 | -0.015 | 270 | -0.022 | 0.161 | 271 | 0.119 | -0.004 |
| 272 | 0.062 | -0.062 | 273 | 0.037 | 0.098 | 274 | -0.057 | 0.039 | 275 | -0.131 | 0.065 |
| 276 | 0.082 | 0.092 | 277 | 0.070 | 0.014 | 278 | -0.060 | 0.081 | 279 | -0.056 | -0.022 |
| 280 | -0.035 | -0.151 | 281 | -0.122 | -0.017 | 282 | -0.127 | -0.021 | 283 | 0.075 | -0.074 |
| 284 | -0.003 | 0.054 | 285 | -0.092 | 0.115 | 286 | 0.092 | 0.106 | 287 | 0.012 | 0.098 |
| 288 | -0.156 | 0.000 | 289 | 0.012 | -0.098 | 290 | 0.092 | -0.106 | 291 | -0.092 | -0.115 |
| 292 | -0.003 | -0.054 | 293 | 0.075 | 0.074 | 294 | -0.127 | 0.021 | 295 | -0.122 | 0.017 |
| 296 | -0.035 | 0.151 | 297 | -0.056 | 0.022 | 298 | -0.060 | -0.081 | 299 | 0.070 | -0.014 |
| 300 | 0.082 | -0.092 | 301 | -0.131 | -0.065 | 302 | -0.057 | -0.039 | 303 | 0.037 | -0.098 |
| 304 | 0.062 | 0.062 | 305 | 0.119 | 0.004 | 306 | -0.022 | -0.161 | 307 | 0.059 | 0.015 |
| 308 | 0.024 | 0.059 | 309 | -0.137 | 0.047 | 310 | 0.001 | 0.115 | 311 | 0.053 | -0.004 |
| 312 | 0.098 | 0.026 | 313 | -0.038 | 0.106 | 314 | -0.115 | 0.055 | 315 | 0.060 | 0.088 |
| 316 | 0.021 | -0.028 | 317 | 0.097 | -0.083 | 318 | 0.040 | 0.111 | 319 | -0.005 | 0.120 |

Table G.26--Time Domain representation of the SIGNAL field (1 symbol)

| ## | Real | Imag | ## | Real | Imag | ## | Real | Imag | ## | Real | Imag |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 320 | 0.109 | 0.000 | 321 | 0.033 | -0.044 | 322 | -0.002 | -0.038 | 323 | -0.081 | 0.084 |
| 324 | 0.007 | -0.100 | 325 | -0.001 | -0.113 | 326 | -0.021 | -0.005 | 327 | 0.136 | -0.105 |
| 328 | 0.098 | -0.044 | 329 | 0.011 | -0.002 | 330 | -0.033 | 0.044 | 331 | -0.060 | 0.124 |
| 332 | 0.010 | 0.097 | 333 | 0.000 | -0.008 | 334 | 0.018 | -0.083 | 335 | -0.069 | 0.027 |
| 336 | -0.219 | 0.000 | 337 | -0.069 | -0.027 | 338 | 0.018 | 0.083 | 339 | 0.000 | 0.008 |
| 340 | 0.010 | -0.097 | 341 | -0.060 | -0.124 | 342 | -0.033 | -0.044 | 343 | 0.011 | 0.002 |
| 344 | 0.098 | 0.044 | 345 | 0.136 | 0.105 | 346 | -0.021 | 0.005 | 347 | -0.001 | 0.113 |
| 348 | 0.007 | 0.100 | 349 | -0.081 | -0.084 | 350 | -0.002 | 0.038 | 351 | 0.033 | 0.044 |
| 352 | 0.062 | 0.000 | 353 | 0.057 | 0.052 | 354 | 0.016 | 0.174 | 355 | 0.035 | 0.116 |
| 356 | -0.051 | -0.202 | 357 | 0.011 | 0.036 | 358 | 0.089 | 0.209 | 359 | -0.049 | -0.008 |
| 360 | -0.035 | 0.044 | 361 | 0.017 | -0.059 | 362 | 0.053 | -0.017 | 363 | 0.099 | 0.100 |
| 364 | 0.034 | -0.148 | 365 | -0.003 | -0.094 | 366 | -0.120 | 0.042 | 367 | -0.136 | -0.070 |
| 368 | -0.031 | 0.000 | 369 | -0.136 | 0.070 | 370 | -0.120 | -0.042 | 371 | -0.003 | 0.094 |
| 372 | 0.034 | 0.148 | 373 | 0.099 | -0.100 | 374 | 0.053 | 0.017 | 375 | 0.017 | 0.059 |
| 376 | -0.035 | -0.044 | 377 | -0.049 | 0.008 | 378 | 0.089 | -0.209 | 379 | 0.011 | -0.036 |
| 380 | -0.051 | 0.202 | 381 | 0.035 | -0.116 | 382 | 0.016 | -0.174 | 383 | 0.057 | -0.052 |
| 384 | 0.062 | 0.000 | 385 | 0.033 | -0.044 | 386 | -0.002 | -0.038 | 387 | -0.081 | 0.084 |
| 388 | 0.007 | -0.100 | 389 | -0.001 | -0.113 | 390 | -0.021 | -0.005 | 391 | 0.136 | -0.105 |
| 392 | 0.098 | -0.044 | 393 | 0.011 | -0.002 | 394 | -0.033 | 0.044 | 395 | -0.060 | 0.124 |
| 396 | 0.010 | 0.097 | 397 | 0.000 | -0.008 | 398 | 0.018 | -0.083 | 399 | -0.069 | 0.027 |

Table G.27--Time Domain representation of the DATA field: 1st of 6 symbols

| ## | Real | Imag | ## | Real | Imag | ## | Real | Imag | ## | Real | Imag |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 400 | -0.139 | 0.050 | 401 | 0.004 | 0.014 | 402 | 0.011 | -0.100 | 403 | -0.097 | -0.020 |
| 404 | 0.062 | 0.081 | 405 | 0.124 | 0.139 | 406 | 0.104 | -0.015 | 407 | 0.173 | -0.140 |
| 408 | -0.040 | 0.006 | 409 | -0.133 | 0.009 | 410 | -0.002 | -0.043 | 411 | -0.047 | 0.092 |
| 412 | -0.109 | 0.082 | 413 | -0.024 | 0.010 | 414 | 0.096 | 0.019 | 415 | 0.019 | -0.023 |
| 416 | -0.087 | -0.049 | 417 | 0.002 | 0.058 | 418 | -0.021 | 0.228 | 419 | -0.103 | 0.023 |
| 420 | -0.019 | -0.175 | 421 | 0.018 | 0.132 | 422 | -0.071 | 0.160 | 423 | -0.153 | -0.062 |
| 424 | -0.107 | 0.028 | 425 | 0.055 | 0.140 | 426 | 0.070 | 0.103 | 427 | -0.056 | 0.025 |
| 428 | -0.043 | 0.002 | 429 | 0.016 | -0.118 | 430 | 0.026 | -0.071 | 431 | 0.033 | 0.177 |
| 432 | 0.020 | -0.021 | 433 | 0.035 | -0.088 | 434 | -0.008 | 0.101 | 435 | -0.035 | -0.010 |
| 436 | 0.065 | 0.030 | 437 | 0.092 | -0.034 | 438 | 0.032 | -0.123 | 439 | -0.018 | 0.092 |
| 440 | 0.000 | -0.006 | 441 | -0.006 | -0.056 | 442 | -0.019 | 0.040 | 443 | 0.053 | -0.131 |
| 444 | 0.022 | -0.133 | 445 | 0.104 | -0.032 | 446 | 0.163 | -0.045 | 447 | -0.105 | -0.030 |
| 448 | -0.110 | -0.069 | 449 | -0.008 | -0.092 | 450 | -0.049 | -0.043 | 451 | 0.085 | -0.017 |
| 452 | 0.090 | 0.063 | 453 | 0.015 | 0.153 | 454 | 0.049 | 0.094 | 455 | 0.011 | 0.034 |
| 456 | -0.012 | 0.012 | 457 | -0.015 | -0.017 | 458 | -0.061 | 0.031 | 459 | -0.070 | -0.040 |
| 460 | 0.011 | -0.109 | 461 | 0.037 | -0.060 | 462 | -0.003 | -0.178 | 463 | -0.007 | -0.128 |
| 464 | -0.059 | 0.100 | 465 | 0.004 | 0.014 | 466 | 0.011 | -0.100 | 467 | -0.097 | -0.020 |
| 468 | 0.062 | 0.081 | 469 | 0.124 | 0.139 | 470 | 0.104 | -0.015 | 471 | 0.173 | -0.140 |
| 472 | -0.040 | 0.006 | 473 | -0.133 | 0.009 | 474 | -0.002 | -0.043 | 475 | -0.047 | 0.092 |
| 476 | -0.109 | 0.082 | 477 | -0.024 | 0.010 | 478 | 0.096 | 0.019 | 479 | 0.019 | -0.023 |

Table G.28--Time Domain representation of the DATA field: 2nd of 6 symbols

| ## | Real | Imag | ## | Real | Imag | ## | Real | Imag | ## | Real | Imag |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 480 | -0.058 | 0.016 | 481 | -0.096 | -0.045 | 482 | -0.110 | 0.003 | 483 | -0.070 | 0.216 |
| 484 | -0.040 | 0.059 | 485 | 0.010 | -0.056 | 486 | 0.034 | 0.065 | 487 | 0.117 | 0.033 |
| 488 | 0.078 | -0.133 | 489 | -0.043 | -0.146 | 490 | 0.158 | -0.071 | 491 | 0.254 | -0.021 |
| 492 | 0.068 | 0.117 | 493 | -0.044 | 0.114 | 494 | -0.035 | 0.041 | 495 | 0.085 | 0.070 |
| 496 | 0.120 | 0.010 | 497 | 0.057 | 0.055 | 498 | 0.063 | 0.188 | 499 | 0.091 | 0.149 |
| 500 | -0.017 | -0.039 | 501 | -0.078 | -0.075 | 502 | 0.049 | 0.079 | 503 | -0.014 | -0.007 |
| 504 | 0.030 | -0.027 | 505 | 0.080 | 0.054 | 506 | -0.186 | -0.067 | 507 | -0.039 | -0.027 |
| 508 | 0.043 | -0.072 | 509 | -0.092 | -0.089 | 510 | 0.029 | 0.105 | 511 | -0.144 | 0.003 |
| 512 | -0.069 | -0.041 | 513 | 0.132 | 0.057 | 514 | -0.126 | 0.070 | 515 | -0.031 | 0.109 |
| 516 | 0.161 | -0.009 | 517 | 0.056 | -0.046 | 518 | -0.004 | 0.028 | 519 | -0.049 | 0.000 |
| 520 | -0.078 | -0.005 | 521 | 0.015 | -0.087 | 522 | 0.149 | -0.104 | 523 | -0.021 | -0.051 |
| 524 | -0.154 | -0.106 | 525 | 0.024 | 0.030 | 526 | 0.046 | 0.123 | 527 | -0.004 | -0.098 |
| 528 | -0.061 | -0.128 | 529 | -0.024 | -0.038 | 530 | 0.066 | -0.048 | 531 | -0.067 | 0.027 |
| 532 | 0.054 | -0.050 | 533 | 0.171 | -0.049 | 534 | -0.108 | 0.132 | 535 | -0.161 | -0.019 |
| 536 | -0.070 | -0.072 | 537 | -0.177 | 0.049 | 538 | -0.172 | -0.050 | 539 | 0.051 | -0.075 |
| 540 | 0.122 | -0.057 | 541 | 0.009 | -0.044 | 542 | -0.012 | -0.021 | 543 | 0.004 | 0.009 |
| 544 | -0.030 | 0.081 | 545 | -0.096 | -0.045 | 546 | -0.110 | 0.003 | 547 | -0.070 | 0.216 |
| 548 | -0.040 | 0.059 | 549 | 0.010 | -0.056 | 550 | 0.034 | 0.065 | 551 | 0.117 | 0.033 |
| 552 | 0.078 | -0.133 | 553 | -0.043 | -0.146 | 554 | 0.158 | -0.071 | 555 | 0.254 | -0.021 |
| 556 | 0.068 | 0.117 | 557 | -0.044 | 0.114 | 558 | -0.035 | 0.041 | 559 | 0.085 | 0.070 |

Table G.29--Time Domain representation of the DATA field: 3rd of 6 symbols

| ## | Real | Imag | ## | Real | Imag | ## | Real | Imag | ## | Real | Imag |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 560 | 0.001 | 0.011 | 561 | -0.099 | -0.048 | 562 | 0.054 | -0.196 | 563 | 0.124 | 0.035 |
| 564 | 0.092 | 0.045 | 565 | -0.037 | -0.066 | 566 | -0.021 | -0.004 | 567 | 0.042 | -0.065 |
| 568 | 0.061 | 0.048 | 569 | 0.046 | 0.004 | 570 | -0.063 | -0.045 | 571 | -0.102 | 0.152 |
| 572 | -0.039 | -0.019 | 573 | -0.005 | -0.106 | 574 | 0.083 | 0.031 | 575 | 0.226 | 0.028 |
| 576 | 0.140 | -0.010 | 577 | -0.132 | -0.033 | 578 | -0.116 | 0.088 | 579 | 0.023 | 0.052 |
| 580 | -0.171 | -0.080 | 581 | -0.246 | -0.025 | 582 | -0.062 | -0.038 | 583 | -0.055 | -0.062 |
| 584 | -0.004 | -0.060 | 585 | 0.034 | 0.000 | 586 | -0.030 | 0.021 | 587 | 0.075 | -0.122 |
| 588 | 0.043 | -0.080 | 589 | -0.022 | 0.041 | 590 | 0.026 | 0.013 | 591 | -0.031 | -0.018 |
| 592 | 0.059 | 0.008 | 593 | 0.109 | 0.078 | 594 | 0.002 | 0.101 | 595 | -0.016 | 0.054 |
| 596 | -0.059 | 0.070 | 597 | 0.017 | 0.114 | 598 | 0.104 | -0.034 | 599 | -0.024 | -0.059 |
| 600 | -0.081 | 0.051 | 601 | -0.040 | -0.069 | 602 | -0.069 | 0.058 | 603 | -0.067 | 0.117 |
| 604 | 0.007 | -0.131 | 605 | 0.009 | 0.028 | 606 | 0.075 | 0.117 | 607 | 0.118 | 0.030 |
| 608 | -0.041 | 0.148 | 609 | 0.005 | 0.098 | 610 | 0.026 | 0.002 | 611 | -0.116 | 0.045 |
| 612 | -0.020 | 0.084 | 613 | 0.101 | 0.006 | 614 | 0.205 | -0.064 | 615 | 0.073 | -0.063 |
| 616 | -0.174 | -0.118 | 617 | -0.024 | 0.026 | 618 | -0.041 | 0.129 | 619 | -0.042 | -0.053 |
| 620 | 0.148 | -0.126 | 621 | -0.030 | -0.049 | 622 | -0.015 | -0.021 | 623 | 0.089 | -0.069 |
| 624 | -0.119 | 0.011 | 625 | -0.099 | -0.048 | 626 | 0.054 | -0.196 | 627 | 0.124 | 0.035 |
| 628 | 0.092 | 0.045 | 629 | -0.037 | -0.066 | 630 | -0.021 | -0.004 | 631 | 0.042 | -0.065 |
| 632 | 0.061 | 0.048 | 633 | 0.046 | 0.004 | 634 | -0.063 | -0.045 | 635 | -0.102 | 0.152 |
| 636 | -0.039 | -0.019 | 637 | -0.005 | -0.106 | 638 | 0.083 | 0.031 | 639 | 0.226 | 0.028 |

Table G.30--Time Domain representation of the DATA field: 4th of 6 symbols

| ## | Real | Imag | ## | Real | Imag | ## | Real | Imag | ## | Real | Imag |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 640 | 0.085 | -0.065 | 641 | 0.034 | -0.142 | 642 | 0.004 | -0.012 | 643 | 0.126 | -0.043 |
| 644 | 0.055 | 0.068 | 645 | -0.020 | 0.077 | 646 | 0.008 | -0.056 | 647 | -0.034 | 0.046 |
| 648 | -0.040 | -0.134 | 649 | -0.056 | -0.131 | 650 | 0.014 | 0.097 | 651 | 0.045 | -0.009 |
| 652 | -0.113 | -0.170 | 653 | -0.065 | -0.230 | 654 | 0.065 | -0.011 | 655 | 0.011 | 0.048 |
| 656 | -0.091 | -0.059 | 657 | -0.110 | 0.024 | 658 | 0.074 | -0.034 | 659 | 0.124 | 0.022 |
| 660 | -0.037 | 0.071 | 661 | 0.015 | 0.002 | 662 | 0.028 | 0.099 | 663 | -0.062 | 0.068 |
| 664 | 0.064 | 0.016 | 665 | 0.078 | 0.156 | 666 | 0.009 | 0.219 | 667 | 0.147 | 0.024 |
| 668 | 0.106 | 0.030 | 669 | -0.080 | 0.143 | 670 | -0.049 | -0.100 | 671 | -0.036 | -0.082 |
| 672 | -0.089 | 0.021 | 673 | -0.070 | -0.029 | 674 | -0.086 | 0.048 | 675 | -0.066 | -0.015 |
| 676 | -0.024 | 0.002 | 677 | -0.030 | -0.023 | 678 | -0.032 | 0.020 | 679 | -0.002 | 0.212 |
| 680 | 0.158 | -0.024 | 681 | 0.141 | -0.119 | 682 | -0.146 | 0.058 | 683 | -0.155 | 0.083 |
| 684 | -0.002 | -0.030 | 685 | 0.018 | -0.129 | 686 | 0.012 | -0.018 | 687 | -0.008 | -0.037 |
| 688 | 0.031 | 0.040 | 689 | 0.023 | 0.097 | 690 | 0.014 | -0.039 | 691 | 0.050 | 0.019 |
| 692 | -0.072 | -0.141 | 693 | -0.023 | -0.051 | 694 | 0.024 | 0.099 | 695 | -0.127 | -0.116 |
| 696 | 0.094 | 0.102 | 697 | 0.183 | 0.098 | 698 | -0.040 | -0.020 | 699 | 0.065 | 0.077 |
| 700 | 0.088 | -0.147 | 701 | -0.039 | -0.059 | 702 | -0.057 | 0.124 | 703 | -0.077 | 0.020 |
| 704 | 0.030 | -0.120 | 705 | 0.034 | -0.142 | 706 | 0.004 | -0.012 | 707 | 0.126 | -0.043 |
| 708 | 0.055 | 0.068 | 709 | -0.020 | 0.077 | 710 | 0.008 | -0.056 | 711 | -0.034 | 0.046 |
| 712 | -0.040 | -0.134 | 713 | -0.056 | -0.131 | 714 | 0.014 | 0.097 | 715 | 0.045 | -0.009 |
| 716 | -0.113 | -0.170 | 717 | -0.065 | -0.230 | 718 | 0.065 | -0.011 | 719 | 0.011 | 0.048 |
| 720 | -0.026 | -0.021 | 721 | -0.002 | 0.041 | 722 | 0.001 | 0.071 | 723 | -0.037 | -0.117 |

Table G.31--Time Domain representation of the DATA field: 5th of 6 symbols

| ## | Real | Imag | ## | Real | Imag | ## | Real | Imag | ## | Real | Imag |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 724 | -0.106 | -0.062 | 725 | 0.002 | 0.057 | 726 | -0.008 | -0.011 | 727 | 0.019 | 0.072 |
| 728 | 0.016 | 0.059 | 729 | -0.065 | -0.077 | 730 | 0.142 | -0.062 | 731 | 0.087 | 0.025 |
| 732 | -0.003 | -0.103 | 733 | 0.107 | -0.152 | 734 | -0.054 | 0.036 | 735 | -0.030 | -0.003 |
| 736 | 0.058 | -0.020 | 737 | -0.028 | 0.007 | 738 | -0.027 | -0.099 | 739 | 0.049 | -0.075 |
| 740 | 0.174 | 0.031 | 741 | 0.134 | 0.156 | 742 | 0.060 | 0.077 | 743 | -0.010 | -0.022 |
| 744 | -0.084 | 0.040 | 745 | -0.074 | 0.011 | 746 | -0.163 | 0.054 | 747 | -0.052 | -0.008 |
| 748 | 0.076 | -0.042 | 749 | 0.043 | 0.101 | 750 | 0.058 | -0.018 | 751 | 0.003 | -0.090 |
| 752 | 0.059 | -0.018 | 753 | 0.023 | -0.031 | 754 | 0.007 | -0.017 | 755 | 0.066 | -0.017 |
| 756 | -0.135 | -0.098 | 757 | -0.056 | -0.081 | 758 | 0.089 | 0.154 | 759 | 0.120 | 0.122 |
| 760 | 0.102 | 0.001 | 761 | -0.141 | 0.102 | 762 | 0.006 | -0.011 | 763 | 0.057 | -0.039 |
| 764 | -0.059 | 0.066 | 765 | 0.132 | 0.111 | 766 | 0.012 | 0.114 | 767 | 0.047 | -0.106 |
| 768 | 0.160 | -0.099 | 769 | -0.076 | 0.084 | 770 | -0.049 | 0.073 | 771 | 0.005 | -0.086 |
| 772 | -0.052 | -0.108 | 773 | -0.073 | 0.129 | 774 | -0.129 | -0.034 | 775 | -0.153 | -0.111 |
| 776 | -0.193 | 0.098 | 777 | -0.107 | -0.068 | 778 | 0.004 | -0.009 | 779 | -0.039 | 0.024 |
| 780 | -0.054 | -0.079 | 781 | 0.024 | 0.084 | 782 | 0.052 | -0.002 | 783 | 0.028 | -0.044 |
| 784 | 0.040 | 0.018 | 785 | -0.002 | 0.041 | 786 | 0.001 | 0.071 | 787 | -0.037 | -0.117 |
| 788 | -0.106 | -0.062 | 789 | 0.002 | 0.057 | 790 | -0.008 | -0.011 | 791 | 0.019 | 0.072 |
| 792 | 0.016 | 0.059 | 793 | -0.065 | -0.077 | 794 | 0.142 | -0.062 | 795 | 0.087 | 0.025 |
| 796 | -0.003 | -0.103 | 797 | 0.107 | -0.152 | 798 | -0.054 | 0.036 | 799 | -0.030 | -0.003 |

Table G.32--Time Domain representation of the DATA field: 6th of 6 symbols

| ## | Real | Imag | ## | Real | Imag | ## | Real | Imag | ## | Real | Imag |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 800 | 0.029 | -0.026 | 801 | -0.047 | 0.077 | 802 | -0.007 | -0.002 | 803 | 0.050 | -0.021 |
| 804 | 0.046 | -0.040 | 805 | -0.061 | -0.099 | 806 | -0.121 | 0.008 | 807 | 0.014 | 0.050 |
| 808 | 0.145 | 0.034 | 809 | 0.001 | -0.046 | 810 | -0.058 | -0.121 | 811 | 0.040 | 0.001 |
| 812 | -0.029 | 0.041 | 813 | 0.002 | -0.066 | 814 | 0.015 | -0.054 | 815 | 0.010 | -0.029 |
| 816 | 0.008 | -0.119 | 817 | -0.134 | 0.002 | 818 | 0.064 | 0.079 | 819 | 0.095 | -0.102 |
| 820 | -0.069 | -0.014 | 821 | 0.156 | 0.037 | 822 | 0.047 | -0.008 | 823 | -0.076 | 0.025 |
| 824 | 0.117 | -0.143 | 825 | 0.056 | -0.042 | 826 | 0.002 | 0.075 | 827 | -0.039 | -0.058 |
| 828 | -0.092 | 0.014 | 829 | -0.041 | 0.047 | 830 | -0.058 | 0.092 | 831 | 0.012 | 0.154 |
| 832 | 0.079 | 0.091 | 833 | -0.067 | 0.017 | 834 | -0.102 | -0.032 | 835 | 0.039 | 0.084 |
| 836 | -0.036 | 0.014 | 837 | -0.001 | -0.046 | 838 | 0.195 | 0.131 | 839 | 0.039 | 0.067 |
| 840 | -0.007 | 0.045 | 841 | 0.051 | 0.008 | 842 | -0.074 | -0.109 | 843 | -0.033 | 0.070 |
| 844 | -0.028 | 0.176 | 845 | -0.041 | 0.045 | 846 | 0.014 | -0.084 | 847 | 0.054 | -0.040 |
| 848 | 0.110 | -0.020 | 849 | 0.014 | -0.021 | 850 | 0.006 | 0.139 | 851 | 0.008 | 0.011 |
| 852 | -0.060 | -0.040 | 853 | 0.008 | 0.179 | 854 | 0.008 | 0.020 | 855 | 0.044 | -0.114 |
| 856 | 0.021 | -0.015 | 857 | -0.008 | -0.052 | 858 | 0.091 | -0.109 | 859 | -0.025 | -0.040 |
| 860 | -0.049 | 0.006 | 861 | -0.043 | -0.041 | 862 | -0.178 | -0.026 | 863 | -0.073 | -0.057 |
| 864 | 0.000 | -0.031 | 865 | -0.047 | 0.077 | 866 | -0.007 | -0.002 | 867 | 0.050 | -0.021 |
| 868 | 0.046 | -0.040 | 869 | -0.061 | -0.099 | 870 | -0.121 | 0.008 | 871 | 0.014 | 0.050 |
| 872 | 0.145 | 0.034 | 873 | 0.001 | -0.046 | 874 | -0.058 | -0.121 | 875 | 0.040 | 0.001 |
| 876 | -0.029 | 0.041 | 877 | 0.002 | -0.066 | 878 | 0.015 | -0.054 | 879 | 0.010 | -0.029 |
| 880 | 0.004 | -0.059 |  |  |  |  |  |  |  |  |  |

Editor: Append the entire Clauses G.9 (LDPC Example #1) and G.10 (LDPC Example #2), along with all their subclauses and corresponding figures after Clause G.8 and its last (new) Table G.32.  (Note that the bold font in the tables here indicates values that change from table to table. It is not required to preserve the bold font.)

## G.9 Generating Encoded DATA bits -- LDPC Example #1:

LDPC Example #1 is as similar to the BCC Example.  This example illustrates LDPC shortening, encoding, and puncturing of a single codeword.

Input TXVECTOR parameters for LDPC Encoding #1:

TxVectorCW = 0			(TXVECTOR ChannelWidth = 0 => 20MHz)

TxVectorMCS = 4			(MCS = 4; QAM 16; Code Rate = 3/4)

Code Rate R = 3/4

TXVectorLength = 100 octets	(with 16-bit SERVICE field becomes 102 Octets =

816 bits to scramble and encode)

m_STBC = 1				(STBC = OFF)

G.9.1 The message for LDPC Example #1:

The message being encoded consists of the first 72 characters of the well-known "Ode to Joy" by F. Schiller:

Joy, bright spark of divinity,

Daughter of Elysium,

Fire-insired we tread

Thy sanctuary.

Thy magic power re-unites

All that custom has divided,

All men become brothers

Under the sway of thy gentle wings.

The message is converted to ASCII; then it is prepended with an appropriate MAC header and a CRC32 is added.  The resulting 100 octets PSDU is shown in Table G.33

Note that the Message for LDPC Example #1 is identical to the corrected message for the BCC Example, meaning that the FCS field (octets 97-100) has the correct CRC 32 value.

Note that the DurationID field (i.e., octets 3 and 4) remains 0x02E = 46us.

Table G.33—The message for LDPC Example #1

| Octet ## | Val | Val | Val | Val | Val |
| --- | --- | --- | --- | --- | --- |
| 1...5 | 04 | 02 | 00 | 2E | 00 |
| 6...10 | 60 | 08 | CD | 37 | A6 |
| 11...15 | 00 | 20 | D6 | 01 | 3C |
| 16...20 | F1 | 00 | 60 | 08 | AD |
| 21...25 | 3B | AF | 00 | 00 | 4A |
| 26...30 | 6F | 79 | 2C | 20 | 62 |
| 31...35 | 72 | 69 | 67 | 68 | 74 |
| 36...40 | 20 | 73 | 70 | 61 | 72 |
| 41...45 | 6B | 20 | 6F | 66 | 20 |
| 46...50 | 64 | 69 | 76 | 69 | 6E |
| 51...55 | 69 | 74 | 79 | 2C | 0A |
| 56...60 | 44 | 61 | 75 | 67 | 68 |
| 61...65 | 74 | 65 | 72 | 20 | 6F |
| 66...70 | 66 | 20 | 45 | 6C | 79 |
| 71...75 | 73 | 69 | 75 | 6D | 2C |
| 76...80 | 0A | 46 | 69 | 72 | 65 |
| 81...85 | 2D | 69 | 6E | 73 | 69 |
| 86...90 | 72 | 65 | 64 | 20 | 77 |
| 91...95 | 65 | 20 | 74 | 72 | 65 |
| 96...100 | 61 | 67 | 33 | 21 | B6 |

G.9.2 Prepending the SERVICE field for LDPC Example #1

The transmitted message shown in Table G.33 contains 100 octets, or equivalently, 800 bits.  The bits are prepended by the 16 SERVICE field bits (bits 0-15 in Table G.34) ), as per sub-clause 21.3.10.1 “The SERVICE field”, but tail bits and padding bits are not appended as in the BCC Example.  The resulting 816 bits are shown in Table G.34.

Note that the CRC32 value is correct in bits 784-815.  Bit-ordering of octets is most significant bit first.

Table G.34—The DATA bits for LDPC Example #1 before scrambling.

| Bit ## | Binary Val | Binary Val | Binary Val | Hex Val | Hex Val | Hex Val |
| --- | --- | --- | --- | --- | --- | --- |
| 000-023 | 00000000 | 00000000 | 00000100 | 0x00 | 0x00 | 0x04 |
| 024-047 | 00000010 | 00000000 | 00101110 | 0x02 | 0x00 | 0x2E |
| 048-071 | 00000000 | 01100000 | 00001000 | 0x00 | 0x60 | 0x08 |
| 072-095 | 11001101 | 00110111 | 10100110 | 0xCD | 0x37 | 0xA6 |
| 096-119 | 00000000 | 00100000 | 11010110 | 0x00 | 0x20 | 0xD6 |
| 120-143 | 00000001 | 00111100 | 11110001 | 0x01 | 0x3C | 0xF1 |
| 144-167 | 00000000 | 01100000 | 00001000 | 0x00 | 0x60 | 0x08 |
| 168-191 | 10101101 | 00111011 | 10101111 | 0xAD | 0x3B | 0xAF |
| 192-215 | 00000000 | 00000000 | 01001010 | 0x00 | 0x00 | 0x4A |
| 216-239 | 01101111 | 01111001 | 00101100 | 0x6F | 0x79 | 0x2C |
| 240-263 | 00100000 | 01100010 | 01110010 | 0x20 | 0x62 | 0x72 |
| 264-287 | 01101001 | 01100111 | 01101000 | 0x69 | 0x67 | 0x68 |
| 288-311 | 01110100 | 00100000 | 01110011 | 0x74 | 0x20 | 0x73 |
| 312-335 | 01110000 | 01100001 | 01110010 | 0x70 | 0x61 | 0x72 |
| 336-359 | 01101011 | 00100000 | 01101111 | 0x6B | 0x20 | 0x6F |
| 360-383 | 01100110 | 00100000 | 01100100 | 0x66 | 0x20 | 0x64 |
| 384-407 | 01101001 | 01110110 | 01101001 | 0x69 | 0x76 | 0x69 |
| 408-431 | 01101110 | 01101001 | 01110100 | 0x6E | 0x69 | 0x74 |
| 432-455 | 01111001 | 00101100 | 00001010 | 0x79 | 0x2C | 0x0A |
| 456-479 | 01000100 | 01100001 | 01110101 | 0x44 | 0x61 | 0x75 |
| 480-503 | 01100111 | 01101000 | 01110100 | 0x67 | 0x68 | 0x74 |
| 504-527 | 01100101 | 01110010 | 00100000 | 0x65 | 0x72 | 0x20 |
| 528-551 | 01101111 | 01100110 | 00100000 | 0x6F | 0x66 | 0x20 |
| 552-575 | 01000101 | 01101100 | 01111001 | 0x45 | 0x6C | 0x79 |
| 576-599 | 01110011 | 01101001 | 01110101 | 0x73 | 0x69 | 0x75 |
| 600-623 | 01101101 | 00101100 | 00001010 | 0x6D | 0x2C | 0x0A |
| 624-647 | 01000110 | 01101001 | 01110010 | 0x46 | 0x69 | 0x72 |
| 648-671 | 01100101 | 00101101 | 01101001 | 0x65 | 0x2D | 0x69 |
| 672-695 | 01101110 | 01110011 | 01101001 | 0x6E | 0x73 | 0x69 |
| 696-719 | 01110010 | 01100101 | 01100100 | 0x72 | 0x65 | 0x64 |
| 720-743 | 00100000 | 01110111 | 01100101 | 0x20 | 0x77 | 0x65 |
| 744-767 | 00100000 | 01110100 | 01110010 | 0x20 | 0x74 | 0x72 |
| 768-791 | 01100101 | 01100001 | 01100111 | 0x65 | 0x61 | 0x67 |
| 792-815 | 00110011 | 00100001 | 10110110 | 0x33 | 0x21 | 0xB6 |

G.9.3 Scrambling LDPC Example #1

The 816 bits are scrambled by the scrambler of Figure 117 (in sub-clause 17.3.5.4).  The initial state of the scrambler is the state 1011101 binary (0x5D hexadecimal).  The scrambled sequence is given in Table G.35, with the bit-ordering being most significant bit first.  Note that the scrambled entries for the correct CRC32 value are given in bits 784-815.

Table G.35—The DATA bits for LDPC Example #1 after scrambling.

| Bit ## | Binary Val | Binary Val | Binary Val | Hex Val | Hex Val | Hex Val |
| --- | --- | --- | --- | --- | --- | --- |
| 000-023 | 01101100 | 00011001 | 10001001 | 0x6C | 0x19 | 0x89 |
| 024-047 | 10001111 | 01101000 | 00100001 | 0x8F | 0x68 | 0x21 |
| 048-071 | 11110100 | 10100101 | 01100001 | 0xF4 | 0xA5 | 0x61 |
| 072-095 | 01001111 | 11010111 | 10101110 | 0x4F | 0xD7 | 0xAE |
| 096-119 | 00100100 | 00001100 | 11110011 | 0x24 | 0x0C | 0xF3 |
| 120-143 | 00111010 | 11100100 | 10111100 | 0x3A | 0xE4 | 0xBC |
| 144-167 | 01010011 | 10011000 | 11000000 | 0x53 | 0x98 | 0xC0 |
| 168-191 | 00011110 | 00110101 | 10110011 | 0x1E | 0x35 | 0xB3 |
| 192-215 | 11100011 | 11111000 | 00100101 | 0xE3 | 0xF8 | 0x25 |
| 216-239 | 01100000 | 11010110 | 00100101 | 0x60 | 0xD6 | 0x25 |
| 240-263 | 00110101 | 00110011 | 11111110 | 0x35 | 0x33 | 0xFE |
| 264-287 | 11110000 | 01000001 | 00101011 | 0xF0 | 0x41 | 0x2B |
| 288-311 | 10001111 | 01010011 | 00011100 | 0x8F | 0x53 | 0x1C |
| 312-335 | 10000011 | 01000001 | 10111110 | 0x83 | 0x41 | 0xBE |
| 336-359 | 00111001 | 00101000 | 01100110 | 0x39 | 0x28 | 0x66 |
| 360-383 | 01000100 | 01100110 | 11001101 | 0x44 | 0x66 | 0xCD |
| 384-407 | 11110110 | 10100011 | 11011000 | 0xF6 | 0xA3 | 0xD8 |
| 408-431 | 00001101 | 11010100 | 10000001 | 0x0D | 0xD4 | 0x81 |
| 432-455 | 00111011 | 00101111 | 11011111 | 0x3B | 0x2F | 0xDF |
| 456-479 | 11000011 | 01011000 | 11110111 | 0xC3 | 0x58 | 0xF7 |
| 480-503 | 11000110 | 01010010 | 11101011 | 0xC6 | 0x52 | 0xEB |
| 504-527 | 01110000 | 10001111 | 10011110 | 0x70 | 0x8F | 0x9E |
| 528-551 | 01101010 | 10010000 | 10000001 | 0x6A | 0x90 | 0x81 |
| 552-575 | 11111101 | 01111100 | 10101001 | 0xFD | 0x7C | 0xA9 |
| 576-599 | 11010001 | 01010101 | 00010010 | 0xD1 | 0x55 | 0x12 |
| 600-623 | 00000100 | 01110100 | 11011001 | 0x04 | 0x74 | 0xD9 |
| 624-647 | 11101001 | 00111011 | 11001101 | 0xE9 | 0x3B | 0xCD |
| 648-671 | 10010011 | 10001101 | 01111011 | 0x93 | 0x8D | 0x7B |
| 672-695 | 01111100 | 01110000 | 00000010 | 0x7C | 0x70 | 0x02 |
| 696-719 | 00100000 | 10011001 | 10100001 | 0x20 | 0x99 | 0xA1 |
| 720-743 | 01111101 | 10001010 | 00100111 | 0x7D | 0x8A | 0x27 |
| 744-767 | 00010111 | 00111001 | 00010101 | 0x17 | 0x39 | 0x15 |
| 768-791 | 10100000 | 11101100 | 10010101 | 0xA0 | 0xEC | 0x95 |
| 792-815 | 00010110 | 10010001 | 00010000 | 0x16 | 0x91 | 0x10 |

G.9.4 Inserting the shortening bits for LDPC Example #1

The equations of sub-clause 21.3.10.6.5 “LDPC PPDU encoding process” are solved to calculate the following derived parameters for LDPC Example #1 from the input TXVECTOR parameters:

NCW = 1			(number of codewords)

LLDPC = 1944			(size of codeword)

NCBPS = 208			(number of coded bits per symbol)

Navbits = 1248			(number of available bits)

Nshrt = 642			(number of bits to be shortened)

Npunc = 54			(number of bits to be punctured)

NSYM = 6			(number of OFDM symbols)

Nrep = 0			(number of bits to be repeated)

The results of applying shortening bits, as prescribed in paragraph (c) of sub-clause 21.3.10.6.5 “LDPC PPDU encoding process” is given in Table G.36, with the bit-ordering being most significant bit first.  Note that the Nshrt = 642 shortening bits have been inserted as zeroes in bits 816-1457.

Table G.36—The DATA bits for LDPC Example #1 after insertion of shortening bits.

| Bit ## | Binary Val | Binary Val | Binary Val | Hex Val | Hex Val | Hex Val |
| --- | --- | --- | --- | --- | --- | --- |
| 0000-0023 | 01101100 | 00011001 | 10001001 | 0x6C | 0x19 | 0x89 |
| 0024-0047 | 10001111 | 01101000 | 00100001 | 0x8F | 0x68 | 0x21 |
| 0048-0071 | 11110100 | 10100101 | 01100001 | 0xF4 | 0xA5 | 0x61 |
| 0072-0095 | 01001111 | 11010111 | 10101110 | 0x4F | 0xD7 | 0xAE |
| 0096-0119 | 00100100 | 00001100 | 11110011 | 0x24 | 0x0C | 0xF3 |
| 0120-0143 | 00111010 | 11100100 | 10111100 | 0x3A | 0xE4 | 0xBC |
| 0144-0167 | 01010011 | 10011000 | 11000000 | 0x53 | 0x98 | 0xC0 |
| 0168-0191 | 00011110 | 00110101 | 10110011 | 0x1E | 0x35 | 0xB3 |
| 0192-0215 | 11100011 | 11111000 | 00100101 | 0xE3 | 0xF8 | 0x25 |
| 0216-0239 | 01100000 | 11010110 | 00100101 | 0x60 | 0xD6 | 0x25 |
| 0240-0263 | 00110101 | 00110011 | 11111110 | 0x35 | 0x33 | 0xFE |
| 0264-0287 | 11110000 | 01000001 | 00101011 | 0xF0 | 0x41 | 0x2B |
| 0288-0311 | 10001111 | 01010011 | 00011100 | 0x8F | 0x53 | 0x1C |
| 0312-0335 | 10000011 | 01000001 | 10111110 | 0x83 | 0x41 | 0xBE |
| 0336-0359 | 00111001 | 00101000 | 01100110 | 0x39 | 0x28 | 0x66 |
| 0360-0383 | 01000100 | 01100110 | 11001101 | 0x44 | 0x66 | 0xCD |
| 0384-0407 | 11110110 | 10100011 | 11011000 | 0xF6 | 0xA3 | 0xD8 |
| 0408-0431 | 00001101 | 11010100 | 10000001 | 0x0D | 0xD4 | 0x81 |
| 0432-0455 | 00111011 | 00101111 | 11011111 | 0x3B | 0x2F | 0xDF |
| 0456-0479 | 11000011 | 01011000 | 11110111 | 0xC3 | 0x58 | 0xF7 |
| 0480-0503 | 11000110 | 01010010 | 11101011 | 0xC6 | 0x52 | 0xEB |
| 0504-0527 | 01110000 | 10001111 | 10011110 | 0x70 | 0x8F | 0x9E |
| 0528-0551 | 01101010 | 10010000 | 10000001 | 0x6A | 0x90 | 0x81 |
| 0552-0575 | 11111101 | 01111100 | 10101001 | 0xFD | 0x7C | 0xA9 |
| 0576-0599 | 11010001 | 01010101 | 00010010 | 0xD1 | 0x55 | 0x12 |
| 0600-0623 | 00000100 | 01110100 | 11011001 | 0x04 | 0x74 | 0xD9 |
| 0624-0647 | 11101001 | 00111011 | 11001101 | 0xE9 | 0x3B | 0xCD |
| 0648-0671 | 10010011 | 10001101 | 01111011 | 0x93 | 0x8D | 0x7B |
| 0672-0695 | 01111100 | 01110000 | 00000010 | 0x7C | 0x70 | 0x02 |
| 0696-0719 | 00100000 | 10011001 | 10100001 | 0x20 | 0x99 | 0xA1 |
| 0720-0743 | 01111101 | 10001010 | 00100111 | 0x7D | 0x8A | 0x27 |
| 0744-0767 | 00010111 | 00111001 | 00010101 | 0x17 | 0x39 | 0x15 |
| 0768-0791 | 10100000 | 11101100 | 10010101 | 0xA0 | 0xEC | 0x95 |
| 0792-0815 | 00010110 | 10010001 | 00010000 | 0x16 | 0x91 | 0x10 |
| 0816-0839 | 00000000 | 00000000 | 00000000 | 0x00 | 0x00 | 0x00 |
| 0840-0863 | 00000000 | 00000000 | 00000000 | 0x00 | 0x00 | 0x00 |
| 0864-0887 | 00000000 | 00000000 | 00000000 | 0x00 | 0x00 | 0x00 |
| 0888-0911 | 00000000 | 00000000 | 00000000 | 0x00 | 0x00 | 0x00 |
| 0912-0935 | 00000000 | 00000000 | 00000000 | 0x00 | 0x00 | 0x00 |
| 0936-0959 | 00000000 | 00000000 | 00000000 | 0x00 | 0x00 | 0x00 |
| 0960-0983 | 00000000 | 00000000 | 00000000 | 0x00 | 0x00 | 0x00 |
| 0984-1007 | 00000000 | 00000000 | 00000000 | 0x00 | 0x00 | 0x00 |
| 1008-1031 | 00000000 | 00000000 | 00000000 | 0x00 | 0x00 | 0x00 |
| 1032-1055 | 00000000 | 00000000 | 00000000 | 0x00 | 0x00 | 0x00 |
| 1056-1079 | 00000000 | 00000000 | 00000000 | 0x00 | 0x00 | 0x00 |
| 1080-1103 | 00000000 | 00000000 | 00000000 | 0x00 | 0x00 | 0x00 |
| 1104-1127 | 00000000 | 00000000 | 00000000 | 0x00 | 0x00 | 0x00 |
| 1128-1151 | 00000000 | 00000000 | 00000000 | 0x00 | 0x00 | 0x00 |
| 1152-1175 | 00000000 | 00000000 | 00000000 | 0x00 | 0x00 | 0x00 |
| 1176-1199 | 00000000 | 00000000 | 00000000 | 0x00 | 0x00 | 0x00 |
| 1200-1223 | 00000000 | 00000000 | 00000000 | 0x00 | 0x00 | 0x00 |
| 1224-1247 | 00000000 | 00000000 | 00000000 | 0x00 | 0x00 | 0x00 |
| 1248-1271 | 00000000 | 00000000 | 00000000 | 0x00 | 0x00 | 0x00 |
| 1272-1295 | 00000000 | 00000000 | 00000000 | 0x00 | 0x00 | 0x00 |
| 1296-1319 | 00000000 | 00000000 | 00000000 | 0x00 | 0x00 | 0x00 |
| 1320-1343 | 00000000 | 00000000 | 00000000 | 0x00 | 0x00 | 0x00 |
| 1344-1367 | 00000000 | 00000000 | 00000000 | 0x00 | 0x00 | 0x00 |
| 1368-1391 | 00000000 | 00000000 | 00000000 | 0x00 | 0x00 | 0x00 |
| 1392-1415 | 00000000 | 00000000 | 00000000 | 0x00 | 0x00 | 0x00 |
| 1416-1439 | 00000000 | 00000000 | 00000000 | 0x00 | 0x00 | 0x00 |
| 1440-1457 | 00000000 | 00000000 | 00 - - - - - - | 0x00 | 0x00 | 0x0- |

G.9.5 Encoding the data for LDPC Example #1

The DATA with shortening bits are LDPC encoded as a single (NCW = 1) codeword (LLDPC = 1944; R = 3/4) as prescribed by paragraph (c) of subclause 21.3.10.6.4 “PPDU encoding process”.  The results are given in Table G.37, with the bit-ordering being most significant bit first.  Note that the LDPC encoder appends 486 bits, bits 1458-1943, after the shortening bits.

Table G.37—The DATA bits for LDPC Example #1 after LDPC encoding.

| Bit ## | Binary Val | Binary Val | Binary Val | Hex Val | Hex Val | Hex Val |
| --- | --- | --- | --- | --- | --- | --- |
| 0000-0023 | 01101100 | 00011001 | 10001001 | 0x6C | 0x19 | 0x89 |
| 0024-0047 | 10001111 | 01101000 | 00100001 | 0x8F | 0x68 | 0x21 |
| 0048-0071 | 11110100 | 10100101 | 01100001 | 0xF4 | 0xA5 | 0x61 |
| 0072-0095 | 01001111 | 11010111 | 10101110 | 0x4F | 0xD7 | 0xAE |
| 0096-0119 | 00100100 | 00001100 | 11110011 | 0x24 | 0x0C | 0xF3 |
| 0120-0143 | 00111010 | 11100100 | 10111100 | 0x3A | 0xE4 | 0xBC |
| 0144-0167 | 01010011 | 10011000 | 11000000 | 0x53 | 0x98 | 0xC0 |
| 0168-0191 | 00011110 | 00110101 | 10110011 | 0x1E | 0x35 | 0xB3 |
| 0192-0215 | 11100011 | 11111000 | 00100101 | 0xE3 | 0xF8 | 0x25 |
| 0216-0239 | 01100000 | 11010110 | 00100101 | 0x60 | 0xD6 | 0x25 |
| 0240-0263 | 00110101 | 00110011 | 11111110 | 0x35 | 0x33 | 0xFE |
| 0264-0287 | 11110000 | 01000001 | 00101011 | 0xF0 | 0x41 | 0x2B |
| 0288-0311 | 10001111 | 01010011 | 00011100 | 0x8F | 0x53 | 0x1C |
| 0312-0335 | 10000011 | 01000001 | 10111110 | 0x83 | 0x41 | 0xBE |
| 0336-0359 | 00111001 | 00101000 | 01100110 | 0x39 | 0x28 | 0x66 |
| 0360-0383 | 01000100 | 01100110 | 11001101 | 0x44 | 0x66 | 0xCD |
| 0384-0407 | 11110110 | 10100011 | 11011000 | 0xF6 | 0xA3 | 0xD8 |
| 0408-0431 | 00001101 | 11010100 | 10000001 | 0x0D | 0xD4 | 0x81 |
| 0432-0455 | 00111011 | 00101111 | 11011111 | 0x3B | 0x2F | 0xDF |
| 0456-0479 | 11000011 | 01011000 | 11110111 | 0xC3 | 0x58 | 0xF7 |
| 0480-0503 | 11000110 | 01010010 | 11101011 | 0xC6 | 0x52 | 0xEB |
| 0504-0527 | 01110000 | 10001111 | 10011110 | 0x70 | 0x8F | 0x9E |
| 0528-0551 | 01101010 | 10010000 | 10000001 | 0x6A | 0x90 | 0x81 |
| 0552-0575 | 11111101 | 01111100 | 10101001 | 0xFD | 0x7C | 0xA9 |
| 0576-0599 | 11010001 | 01010101 | 00010010 | 0xD1 | 0x55 | 0x12 |
| 0600-0623 | 00000100 | 01110100 | 11011001 | 0x04 | 0x74 | 0xD9 |
| 0624-0647 | 11101001 | 00111011 | 11001101 | 0xE9 | 0x3B | 0xCD |
| 0648-0671 | 10010011 | 10001101 | 01111011 | 0x93 | 0x8D | 0x7B |
| 0672-0695 | 01111100 | 01110000 | 00000010 | 0x7C | 0x70 | 0x02 |
| 0696-0719 | 00100000 | 10011001 | 10100001 | 0x20 | 0x99 | 0xA1 |
| 0720-0743 | 01111101 | 10001010 | 00100111 | 0x7D | 0x8A | 0x27 |
| 0744-0767 | 00010111 | 00111001 | 00010101 | 0x17 | 0x39 | 0x15 |
| 0768-0791 | 10100000 | 11101100 | 10010101 | 0xA0 | 0xEC | 0x95 |
| 0792-0815 | 00010110 | 10010001 | 00010000 | 0x16 | 0x91 | 0x10 |
| 0816-0839 | 00000000 | 00000000 | 00000000 | 0x00 | 0x00 | 0x00 |
| 0840-0863 | 00000000 | 00000000 | 00000000 | 0x00 | 0x00 | 0x00 |
| 0864-0887 | 00000000 | 00000000 | 00000000 | 0x00 | 0x00 | 0x00 |
| 0888-0911 | 00000000 | 00000000 | 00000000 | 0x00 | 0x00 | 0x00 |
| 0912-0935 | 00000000 | 00000000 | 00000000 | 0x00 | 0x00 | 0x00 |
| 0936-0959 | 00000000 | 00000000 | 00000000 | 0x00 | 0x00 | 0x00 |
| 0960-0983 | 00000000 | 00000000 | 00000000 | 0x00 | 0x00 | 0x00 |
| 0984-1007 | 00000000 | 00000000 | 00000000 | 0x00 | 0x00 | 0x00 |
| 1008-1031 | 00000000 | 00000000 | 00000000 | 0x00 | 0x00 | 0x00 |
| 1032-1055 | 00000000 | 00000000 | 00000000 | 0x00 | 0x00 | 0x00 |
| 1056-1079 | 00000000 | 00000000 | 00000000 | 0x00 | 0x00 | 0x00 |
| 1080-1103 | 00000000 | 00000000 | 00000000 | 0x00 | 0x00 | 0x00 |
| 1104-1127 | 00000000 | 00000000 | 00000000 | 0x00 | 0x00 | 0x00 |
| 1128-1151 | 00000000 | 00000000 | 00000000 | 0x00 | 0x00 | 0x00 |
| 1152-1175 | 00000000 | 00000000 | 00000000 | 0x00 | 0x00 | 0x00 |
| 1176-1199 | 00000000 | 00000000 | 00000000 | 0x00 | 0x00 | 0x00 |
| 1200-1223 | 00000000 | 00000000 | 00000000 | 0x00 | 0x00 | 0x00 |
| 1224-1247 | 00000000 | 00000000 | 00000000 | 0x00 | 0x00 | 0x00 |
| 1248-1271 | 00000000 | 00000000 | 00000000 | 0x00 | 0x00 | 0x00 |
| 1272-1295 | 00000000 | 00000000 | 00000000 | 0x00 | 0x00 | 0x00 |
| 1296-1319 | 00000000 | 00000000 | 00000000 | 0x00 | 0x00 | 0x00 |
| 1320-1343 | 00000000 | 00000000 | 00000000 | 0x00 | 0x00 | 0x00 |
| 1344-1367 | 00000000 | 00000000 | 00000000 | 0x00 | 0x00 | 0x00 |
| 1368-1391 | 00000000 | 00000000 | 00000000 | 0x00 | 0x00 | 0x00 |
| 1392-1415 | 00000000 | 00000000 | 00000000 | 0x00 | 0x00 | 0x00 |
| 1416-1439 | 00000000 | 00000000 | 00000000 | 0x00 | 0x00 | 0x00 |
| 1440-1463 | 00000000 | 00000000 | 00100110 | 0x00 | 0x00 | 0x26 |
| 1464-1487 | 00111101 | 10101001 | 10011100 | 0x3D | 0xA9 | 0x9C |
| 1488-1511 | 01000000 | 11010111 | 10110010 | 0x40 | 0xD7 | 0xB2 |
| 1512-1535 | 10000110 | 11100011 | 10111111 | 0x86 | 0xE3 | 0xBF |
| 1536-1559 | 01000011 | 10100101 | 11011001 | 0x43 | 0xA5 | 0xD9 |
| 1560-1583 | 00001101 | 00000110 | 11010110 | 0x0D | 0x06 | 0xD6 |
| 1584-1607 | 01100000 | 11110100 | 00011111 | 0x60 | 0xF4 | 0x1F |
| 1608-1631 | 00110001 | 00001100 | 00010011 | 0x31 | 0x0C | 0x13 |
| 1632-1655 | 01110110 | 00001111 | 10011111 | 0x76 | 0x0F | 0x9F |
| 1656-1679 | 11011010 | 10011111 | 10101001 | 0xDA | 0x9F | 0xA9 |
| 1680-1703 | 01110100 | 01011001 | 11011100 | 0x74 | 0x59 | 0xDC |
| 1704-1727 | 10001001 | 11110010 | 11100010 | 0x89 | 0xF2 | 0xE2 |
| 1728-1751 | 11011000 | 01101000 | 10100001 | 0xD8 | 0x68 | 0xA1 |
| 1752-1775 | 01100011 | 00011101 | 10100101 | 0x63 | 0x1D | 0xA5 |
| 1776-1799 | 10100110 | 10000000 | 11010001 | 0xA6 | 0x80 | 0xD1 |
| 1800-1823 | 10001001 | 01010111 | 11011100 | 0x89 | 0x57 | 0xDC |
| 1824-1847 | 10110011 | 01011101 | 00110011 | 0xB3 | 0x5D | 0x33 |
| 1848-1871 | 01110000 | 11011100 | 10110010 | 0x70 | 0xDC | 0xB2 |
| 1872-1895 | 11110110 | 00111001 | 00111101 | 0xF6 | 0x39 | 0x3D |
| 1896-1919 | 00100011 | 10011011 | 00110110 | 0x23 | 0x9B | 0x36 |
| 1920-1943 | 00111110 | 00010101 | 00010001 | 0x3E | 0x15 | 0x11 |

G.9.6 Removing the shortening bits and puncturing for LDPC Example #1

The shortening bits, applied before LDPC encoding, are now removed as prescribed in paragraph (c) of subclause 21.3.10.6.5 “PPDU encoding process”.  Finally, either puncturing is applied as described in paragraph (d) of the same subclause or the copying of repeated bits are applied as described in paragraph (e) of the same subclause. In LDPC Encoding Example #1, because Npunc = 54 is non-zero and Nrep = 0, puncturing is prescribed, completing the LDPC encoding process.

The results are given in Table G.38, with the bit-ordering being most significant bit first.   Note that the Nshrt = 642 shortening bits have been removed and that the Npunc = 54 bits have been punctured from Table G.37 to produce bits 816-1247 of Table G.38.

Table G.38—The DATA bits for LDPC Example #1 after removal of shortening bits and application of puncturing

| Bit ## | Binary Val | Binary Val | Binary Val | Hex Val | Hex Val | Hex Val |
| --- | --- | --- | --- | --- | --- | --- |
| 0000-0023 | 01101100 | 00011001 | 10001001 | 0x6C | 0x19 | 0x89 |
| 0024-0047 | 10001111 | 01101000 | 00100001 | 0x8F | 0x68 | 0x21 |
| 0048-0071 | 11110100 | 10100101 | 01100001 | 0xF4 | 0xA5 | 0x61 |
| 0072-0095 | 01001111 | 11010111 | 10101110 | 0x4F | 0xD7 | 0xAE |
| 0096-0119 | 00100100 | 00001100 | 11110011 | 0x24 | 0x0C | 0xF3 |
| 0120-0143 | 00111010 | 11100100 | 10111100 | 0x3A | 0xE4 | 0xBC |
| 0144-0167 | 01010011 | 10011000 | 11000000 | 0x53 | 0x98 | 0xC0 |
| 0168-0191 | 00011110 | 00110101 | 10110011 | 0x1E | 0x35 | 0xB3 |
| 0192-0215 | 11100011 | 11111000 | 00100101 | 0xE3 | 0xF8 | 0x25 |
| 0216-0239 | 01100000 | 11010110 | 00100101 | 0x60 | 0xD6 | 0x25 |
| 0240-0263 | 00110101 | 00110011 | 11111110 | 0x35 | 0x33 | 0xFE |
| 0264-0287 | 11110000 | 01000001 | 00101011 | 0xF0 | 0x41 | 0x2B |
| 0288-0311 | 10001111 | 01010011 | 00011100 | 0x8F | 0x53 | 0x1C |
| 0312-0335 | 10000011 | 01000001 | 10111110 | 0x83 | 0x41 | 0xBE |
| 0336-0359 | 00111001 | 00101000 | 01100110 | 0x39 | 0x28 | 0x66 |
| 0360-0383 | 01000100 | 01100110 | 11001101 | 0x44 | 0x66 | 0xCD |
| 0384-0407 | 11110110 | 10100011 | 11011000 | 0xF6 | 0xA3 | 0xD8 |
| 0408-0431 | 00001101 | 11010100 | 10000001 | 0x0D | 0xD4 | 0x81 |
| 0432-0455 | 00111011 | 00101111 | 11011111 | 0x3B | 0x2F | 0xDF |
| 0456-0479 | 11000011 | 01011000 | 11110111 | 0xC3 | 0x58 | 0xF7 |
| 0480-0503 | 11000110 | 01010010 | 11101011 | 0xC6 | 0x52 | 0xEB |
| 0504-0527 | 01110000 | 10001111 | 10011110 | 0x70 | 0x8F | 0x9E |
| 0528-0551 | 01101010 | 10010000 | 10000001 | 0x6A | 0x90 | 0x81 |
| 0552-0575 | 11111101 | 01111100 | 10101001 | 0xFD | 0x7C | 0xA9 |
| 0576-0599 | 11010001 | 01010101 | 00010010 | 0xD1 | 0x55 | 0x12 |
| 0600-0623 | 00000100 | 01110100 | 11011001 | 0x04 | 0x74 | 0xD9 |
| 0624-0647 | 11101001 | 00111011 | 11001101 | 0xE9 | 0x3B | 0xCD |
| 0648-0671 | 10010011 | 10001101 | 01111011 | 0x93 | 0x8D | 0x7B |
| 0672-0695 | 01111100 | 01110000 | 00000010 | 0x7C | 0x70 | 0x02 |
| 0696-0719 | 00100000 | 10011001 | 10100001 | 0x20 | 0x99 | 0xA1 |
| 0720-0743 | 01111101 | 10001010 | 00100111 | 0x7D | 0x8A | 0x27 |
| 0744-0767 | 00010111 | 00111001 | 00010101 | 0x17 | 0x39 | 0x15 |
| 0768-0791 | 10100000 | 11101100 | 10010101 | 0xA0 | 0xEC | 0x95 |
| 0792-0815 | 00010110 | 10010001 | 00010000 | 0x16 | 0x91 | 0x10 |
| 0816-0839 | 10011000 | 11110110 | 10100110 | 0x98 | 0xF6 | 0xA6 |
| 0840-0863 | 01110001 | 00000011 | 01011110 | 0x71 | 0x03 | 0x5E |
| 0864-0887 | 11001010 | 00011011 | 10001110 | 0xCA | 0x1B | 0x8E |
| 0888-0911 | 11111101 | 00001110 | 10010111 | 0xFD | 0x0E | 0x97 |
| 0912-0935 | 01100100 | 00110100 | 00011011 | 0x64 | 0x34 | 0x1B |
| 0936-0959 | 01011001 | 10000011 | 11010000 | 0x59 | 0x83 | 0xD0 |
| 0960-0983 | 01111100 | 11000100 | 00110000 | 0x7C | 0xC4 | 0x30 |
| 0984-1007 | 01001101 | 11011000 | 00111110 | 0x4D | 0xD8 | 0x3E |
| 1008-1031 | 01111111 | 01101010 | 01111110 | 0x7F | 0x6A | 0x7E |
| 1032-1055 | 10100101 | 11010001 | 01100111 | 0xA5 | 0xD1 | 0x67 |
| 1056-1079 | 01110010 | 00100111 | 11001011 | 0x72 | 0x27 | 0xCB |
| 1080-1103 | 10001011 | 01100001 | 10100010 | 0x8B | 0x61 | 0xA2 |
| 1104-1127 | 10000101 | 10001100 | 01110110 | 0x85 | 0x8C | 0x76 |
| 1128-1151 | 10010110 | 10011010 | 00000011 | 0x96 | 0x9A | 0x03 |
| 1152-1175 | 01000110 | 00100101 | 01011111 | 0x46 | 0x25 | 0x5F |
| 1176-1199 | 01110010 | 11001101 | 01110100 | 0x72 | 0xCD | 0x74 |
| 1200-1223 | 11001101 | 11000011 | 01110010 | 0xCD | 0xC3 | 0x72 |
| 1224-1247 | 11001011 | 11011000 | 11100100 | 0xCB | 0xD8 | 0xE4 |

## G.10 Generating Encoded DATA bits -- LDPC Example #2:

LDPC Example #2 exercises the alternative branches of the LDPC encoding procedure not exercised in LDPC Example #1, employs multiple codewords, and diversifies the TXVECTOR parameters, without making the length of this example cumbersome.

This example exhibits LDPC shortening, encoding, and padding by repetition.   The length of the text of the message was increased by 40 octets from 72 characters to 144 characters, in order to illustrate padding (rather than puncturing) and encoding of multiple codewords.

Input TXVECTOR parameters for LDPC Example #2:

TxVectorCW = 1			(TXVECTOR ChannelWidth = 1 => 40MHz)

TxVectorMCS = 1			(MCS = 1; QPSK; Code Rate = 1/2)

Code Rate R = 1/2

TXVectorLength = 140 Octets	(with 16-bit SERVICE field becomes 142 Octets =

1136 bits to scramble and encode)

m_STBC = 2				(STBC = ON)

G.10.1 The message for LDPC Example #2:

The message being encoded consists of the first 112 characters of the well-known "Ode to Joy" by F. Schiller:

Joy, bright spark of divinity,

Daughter of Elysium,

Fire-insired we tread

Thy sanctuary.

Thy magic power re-unites

All that custom has divided,

All men become brothers

Under the sway of thy gentle wings.

The message is converted to ASCII; then it is prepended with an appropriate MAC header and a CRC32 is added.  The resulting 100 octets PSDU is shown in Table G.39.

Because of the additional 40 characters, note that the Message for LDPC Example #2 has a different FCS field (octets 137-140) than the previous examples, and that the DurationID field (i.e., octets 3 and 4) changes to 0x036 = 54us.

Table G.39—The message for LDPC Example #2

| Octet ## | Val | Val | Val | Val | Val |
| --- | --- | --- | --- | --- | --- |
| 1...5 | 04 | 02 | 00 | 36 | 00 |
| 6...10 | 60 | 08 | CD | 37 | A6 |
| 11...15 | 00 | 20 | D6 | 01 | 3C |
| 16...20 | F1 | 00 | 60 | 08 | AD |
| 21...25 | 3B | AF | 00 | 00 | 4A |
| 26...30 | 6F | 79 | 2C | 20 | 62 |
| 31...35 | 72 | 69 | 67 | 68 | 74 |
| 36...40 | 20 | 73 | 70 | 61 | 72 |
| 41...45 | 6B | 20 | 6F | 66 | 20 |
| 46...50 | 64 | 69 | 76 | 69 | 6E |
| 51...55 | 69 | 74 | 79 | 2C | 0A |
| 56...60 | 44 | 61 | 75 | 67 | 68 |
| 61...65 | 74 | 65 | 72 | 20 | 6F |
| 66...70 | 66 | 20 | 45 | 6C | 79 |
| 71...75 | 73 | 69 | 75 | 6D | 2C |
| 76...80 | 0A | 46 | 69 | 72 | 65 |
| 81...85 | 2D | 69 | 6E | 73 | 69 |
| 86...90 | 72 | 65 | 64 | 20 | 77 |
| 91...95 | 65 | 20 | 74 | 72 | 65 |
| 96...100 | 61 | 64 | 0A | 54 | 68 |
| 101...105 | 79 | 20 | 73 | 61 | 6E |
| 106...110 | 63 | 74 | 75 | 61 | 72 |
| 111...115 | 79 | 2E | 0A | 54 | 68 |
| 116...120 | 79 | 20 | 6D | 61 | 67 |
| 121...125 | 69 | 63 | 20 | 70 | 6F |
| 126...130 | 77 | 65 | 72 | 20 | 72 |
| 131...135 | 65 | 2D | 75 | 6E | 69 |
| 136...140 | 74 | 3B | DB | B5 | 22 |

G.10.2 Prepending the SERVICE field for LDPC Example #2

The transmitted message shown in Table G.39 contains 140 octets, or equivalently, 1120 bits.  The bits are prepended by the 16 SERVICE field bits (bits 0-15 in Table G.40) ), as per sub-clause 21.3.10.1 “The SERVICE field”, but tail bits and padding bits are not appended as in the BCC Example.  The resulting 1136 bits are shown in Table G.40.  Bit-ordering is most significant bit first.

Table G.40—The DATA bits for LDPC Example #2 before Scrambling.

| Bit ## | Binary Val | Binary Val | Binary Val | Hex Val | Hex Val | Hex Val |
| --- | --- | --- | --- | --- | --- | --- |
| 0000-0023 | 00000000 | 00000000 | 00000100 | 0x00 | 0x00 | 0x04 |
| 0024-0047 | 00000010 | 00000000 | 00110110 | 0x02 | 0x00 | 0x36 |
| 0048-0071 | 00000000 | 01100000 | 00001000 | 0x00 | 0x60 | 0x08 |
| 0072-0095 | 11001101 | 00110111 | 10100110 | 0xCD | 0x37 | 0xA6 |
| 0096-0119 | 00000000 | 00100000 | 11010110 | 0x00 | 0x20 | 0xD6 |
| 0120-0143 | 00000001 | 00111100 | 11110001 | 0x01 | 0x3C | 0xF1 |
| 0144-0167 | 00000000 | 01100000 | 00001000 | 0x00 | 0x60 | 0x08 |
| 0168-0191 | 10101101 | 00111011 | 10101111 | 0xAD | 0x3B | 0xAF |
| 0192-0215 | 00000000 | 00000000 | 01001010 | 0x00 | 0x00 | 0x4A |
| 0216-0239 | 01101111 | 01111001 | 00101100 | 0x6F | 0x79 | 0x2C |
| 0240-0263 | 00100000 | 01100010 | 01110010 | 0x20 | 0x62 | 0x72 |
| 0264-0287 | 01101001 | 01100111 | 01101000 | 0x69 | 0x67 | 0x68 |
| 0288-0311 | 01110100 | 00100000 | 01110011 | 0x74 | 0x20 | 0x73 |
| 0312-0335 | 01110000 | 01100001 | 01110010 | 0x70 | 0x61 | 0x72 |
| 0336-0359 | 01101011 | 00100000 | 01101111 | 0x6B | 0x20 | 0x6F |
| 0360-0383 | 01100110 | 00100000 | 01100100 | 0x66 | 0x20 | 0x64 |
| 0384-0407 | 01101001 | 01110110 | 01101001 | 0x69 | 0x76 | 0x69 |
| 0408-0431 | 01101110 | 01101001 | 01110100 | 0x6E | 0x69 | 0x74 |
| 0432-0455 | 01111001 | 00101100 | 00001010 | 0x79 | 0x2C | 0x0A |
| 0456-0479 | 01000100 | 01100001 | 01110101 | 0x44 | 0x61 | 0x75 |
| 0480-0503 | 01100111 | 01101000 | 01110100 | 0x67 | 0x68 | 0x74 |
| 0504-0527 | 01100101 | 01110010 | 00100000 | 0x65 | 0x72 | 0x20 |
| 0528-0551 | 01101111 | 01100110 | 00100000 | 0x6F | 0x66 | 0x20 |
| 0552-0575 | 01000101 | 01101100 | 01111001 | 0x45 | 0x6C | 0x79 |
| 0576-0599 | 01110011 | 01101001 | 01110101 | 0x73 | 0x69 | 0x75 |
| 0600-0623 | 01101101 | 00101100 | 00001010 | 0x6D | 0x2C | 0x0A |
| 0624-0647 | 01000110 | 01101001 | 01110010 | 0x46 | 0x69 | 0x72 |
| 0648-0671 | 01100101 | 00101101 | 01101001 | 0x65 | 0x2D | 0x69 |
| 0672-0695 | 01101110 | 01110011 | 01101001 | 0x6E | 0x73 | 0x69 |
| 0696-0719 | 01110010 | 01100101 | 01100100 | 0x72 | 0x65 | 0x64 |
| 0720-0743 | 00100000 | 01110111 | 01100101 | 0x20 | 0x77 | 0x65 |
| 0744-0767 | 00100000 | 01110100 | 01110010 | 0x20 | 0x74 | 0x72 |
| 0768-0791 | 01100101 | 01100001 | 01100100 | 0x65 | 0x61 | 0x64 |
| 0792-0815 | 00001010 | 01010100 | 01101000 | 0x0A | 0x54 | 0x68 |
| 0816-0839 | 01111001 | 00100000 | 01110011 | 0x79 | 0x20 | 0x73 |
| 0840-0863 | 01100001 | 01101110 | 01100011 | 0x61 | 0x6E | 0x63 |
| 0864-0887 | 01110100 | 01110101 | 01100001 | 0x74 | 0x75 | 0x61 |
| 0888-0911 | 01110010 | 01111001 | 00101110 | 0x72 | 0x79 | 0x2E |
| 0912-0935 | 00001010 | 01010100 | 01101000 | 0x0A | 0x54 | 0x68 |
| 0936-0959 | 01111001 | 00100000 | 01101101 | 0x79 | 0x20 | 0x6D |
| 0960-0983 | 01100001 | 01100111 | 01101001 | 0x61 | 0x67 | 0x69 |
| 0984-1007 | 01100011 | 00100000 | 01110000 | 0x63 | 0x20 | 0x70 |
| 1008-1031 | 01101111 | 01110111 | 01100101 | 0x6F | 0x77 | 0x65 |
| 1032-1055 | 01110010 | 00100000 | 01110010 | 0x72 | 0x20 | 0x72 |
| 1056-1079 | 01100101 | 00101101 | 01110101 | 0x65 | 0x2D | 0x75 |
| 1080-1103 | 01101110 | 01101001 | 01110100 | 0x6E | 0x69 | 0x74 |
| 1104-1127 | 00111011 | 11011011 | 10110101 | 0x3B | 0xDB | 0xB5 |
| 1128-1135 | 00100010 | -------- | -------- | 0x22 | ---- | ---- |

G.10.3 Scrambling LDPC Example #2

The 1136 bits are scrambled by the scrambler of Figure 117 (in sub-clause 17.3.5.4).  The initial state of the scrambler is the state 1011101 binary (0x5D hexadecimal).  The scrambled sequence is given in Table G.41, with the bit-ordering being most significant bit first

Table G.41—The DATA bits for LDPC Example #2 after scrambling.

| Bit ## | Binary Val | Binary Val | Binary Val | Hex Val | Hex Val | Hex Val |
| --- | --- | --- | --- | --- | --- | --- |
| 0000-0023 | 01101100 | 00011001 | 10001001 | 0x6C | 0x19 | 0x89 |
| 0024-0047 | 10001111 | 01101000 | 00111001 | 0x8F | 0x68 | 0x39 |
| 0048-0071 | 11110100 | 10100101 | 01100001 | 0xF4 | 0xA5 | 0x61 |
| 0072-0095 | 01001111 | 11010111 | 10101110 | 0x4F | 0xD7 | 0xAE |
| 0096-0119 | 00100100 | 00001100 | 11110011 | 0x24 | 0x0C | 0xF3 |
| 0120-0143 | 00111010 | 11100100 | 10111100 | 0x3A | 0xE4 | 0xBC |
| 0144-0167 | 01010011 | 10011000 | 11000000 | 0x53 | 0x98 | 0xC0 |
| 0168-0191 | 00011110 | 00110101 | 10110011 | 0x1E | 0x35 | 0xB3 |
| 0192-0215 | 11100011 | 11111000 | 00100101 | 0xE3 | 0xF8 | 0x25 |
| 0216-0239 | 01100000 | 11010110 | 00100101 | 0x60 | 0xD6 | 0x25 |
| 0240-0263 | 00110101 | 00110011 | 11111110 | 0x35 | 0x33 | 0xFE |
| 0264-0287 | 11110000 | 01000001 | 00101011 | 0xF0 | 0x41 | 0x2B |
| 0288-0311 | 10001111 | 01010011 | 00011100 | 0x8F | 0x53 | 0x1C |
| 0312-0335 | 10000011 | 01000001 | 10111110 | 0x83 | 0x41 | 0xBE |
| 0336-0359 | 00111001 | 00101000 | 01100110 | 0x39 | 0x28 | 0x66 |
| 0360-0383 | 01000100 | 01100110 | 11001101 | 0x44 | 0x66 | 0xCD |
| 0384-0407 | 11110110 | 10100011 | 11011000 | 0xF6 | 0xA3 | 0xD8 |
| 0408-0431 | 00001101 | 11010100 | 10000001 | 0x0D | 0xD4 | 0x81 |
| 0432-0455 | 00111011 | 00101111 | 11011111 | 0x3B | 0x2F | 0xDF |
| 0456-0479 | 11000011 | 01011000 | 11110111 | 0xC3 | 0x58 | 0xF7 |
| 0480-0503 | 11000110 | 01010010 | 11101011 | 0xC6 | 0x52 | 0xEB |
| 0504-0527 | 01110000 | 10001111 | 10011110 | 0x70 | 0x8F | 0x9E |
| 0528-0551 | 01101010 | 10010000 | 10000001 | 0x6A | 0x90 | 0x81 |
| 0552-0575 | 11111101 | 01111100 | 10101001 | 0xFD | 0x7C | 0xA9 |
| 0576-0599 | 11010001 | 01010101 | 00010010 | 0xD1 | 0x55 | 0x12 |
| 0600-0623 | 00000100 | 01110100 | 11011001 | 0x04 | 0x74 | 0xD9 |
| 0624-0647 | 11101001 | 00111011 | 11001101 | 0xE9 | 0x3B | 0xCD |
| 0648-0671 | 10010011 | 10001101 | 01111011 | 0x93 | 0x8D | 0x7B |
| 0672-0695 | 01111100 | 01110000 | 00000010 | 0x7C | 0x70 | 0x02 |
| 0696-0719 | 00100000 | 10011001 | 10100001 | 0x20 | 0x99 | 0xA1 |
| 0720-0743 | 01111101 | 10001010 | 00100111 | 0x7D | 0x8A | 0x27 |
| 0744-0767 | 00010111 | 00111001 | 00010101 | 0x17 | 0x39 | 0x15 |
| 0768-0791 | 10100000 | 11101100 | 01010101 | 0xA0 | 0xEC | 0x55 |
| 0792-0815 | 10001010 | 00111111 | 01101011 | 0x8A | 0x3F | 0x6B |
| 0816-0839 | 10110110 | 11011000 | 10110001 | 0xB6 | 0xD8 | 0xB1 |
| 0840-0863 | 10001000 | 10000100 | 00001111 | 0x88 | 0x84 | 0x0F |
| 0864-0887 | 00101100 | 10001000 | 10101000 | 0x2C | 0x88 | 0xA8 |
| 0888-0911 | 11111000 | 10010010 | 10100000 | 0xF8 | 0x92 | 0xA0 |
| 0912-0935 | 10110111 | 10011110 | 00111100 | 0xB7 | 0x9E | 0x3C |
| 0936-0959 | 01100100 | 01010101 | 00001110 | 0x64 | 0x55 | 0x0E |
| 0960-0983 | 01111000 | 11111011 | 01110011 | 0x78 | 0xFB | 0x73 |
| 0984-1007 | 01010100 | 00000000 | 01000010 | 0x54 | 0x00 | 0x42 |
| 1008-1031 | 10101011 | 10000010 | 10111111 | 0xAB | 0x82 | 0xBF |
| 1032-1055 | 11100111 | 11001011 | 00100110 | 0xE7 | 0xCB | 0x26 |
| 1056-1079 | 11110011 | 01000000 | 00001101 | 0xF3 | 0x40 | 0x0D |
| 1080-1103 | 00000111 | 01101010 | 00010101 | 0x07 | 0x6A | 0x15 |
| 1104-1127 | 00010111 | 11111111 | 10100101 | 0x17 | 0xFF | 0xA5 |
| 1128-1135 | 11011100 | -------- | -------- | 0xDC | ---- | ---- |

G.10.4 Inserting the shortening bits for LDPC Example #2

The equations of sub-clause 21.3.10.6.5 “LDPC PPDU encoding process” are solved to calculate the following derived parameters for LDPC Example #2 from the input TXVECTOR parameters:

NCW = 2			(number of codewords)

LLDPC = 1296			(size of codeword)

NCBPS = 216			(number of coded bits per symbol)

Navbits = 2592			(number of available bits)

Nshrt = 160			(number of bits to be shortened)

Npunc = 0			(number of bits to be punctured)

NSYM = 12			(number of OFDM symbols)

Nrep = 160			(number of bits to be repeated)

The results of applying shortening bits, as prescribed in paragraph (c) of sub-clause 21.3.10.6.5 “LDPC PPDU encoding process” is given in Table G.42, with the bit-ordering being most significant bit first.  Note that the Nshrt = 160 shortening bits have been inserted as zeroes—80 zeroes at bits 568-647 and 80 zeroes at bits 1216-1295—which equally distributes the shortening bits across the NCW = 2 codewords.

Table G.42—The DATA bits for LDPC Example #2 after insertion of shortening bits.

| Bit ## | Binary Val | Binary Val | Binary Val | Hex Val | Hex Val | Hex Val |
| --- | --- | --- | --- | --- | --- | --- |
| 0000-0023 | 01101100 | 00011001 | 10001001 | 0x6C | 0x19 | 0x89 |
| 0024-0047 | 10001111 | 01101000 | 00111001 | 0x8F | 0x68 | 0x39 |
| 0048-0071 | 11110100 | 10100101 | 01100001 | 0xF4 | 0xA5 | 0x61 |
| 0072-0095 | 01001111 | 11010111 | 10101110 | 0x4F | 0xD7 | 0xAE |
| 0096-0119 | 00100100 | 00001100 | 11110011 | 0x24 | 0x0C | 0xF3 |
| 0120-0143 | 00111010 | 11100100 | 10111100 | 0x3A | 0xE4 | 0xBC |
| 0144-0167 | 01010011 | 10011000 | 11000000 | 0x53 | 0x98 | 0xC0 |
| 0168-0191 | 00011110 | 00110101 | 10110011 | 0x1E | 0x35 | 0xB3 |
| 0192-0215 | 11100011 | 11111000 | 00100101 | 0xE3 | 0xF8 | 0x25 |
| 0216-0239 | 01100000 | 11010110 | 00100101 | 0x60 | 0xD6 | 0x25 |
| 0240-0263 | 00110101 | 00110011 | 11111110 | 0x35 | 0x33 | 0xFE |
| 0264-0287 | 11110000 | 01000001 | 00101011 | 0xF0 | 0x41 | 0x2B |
| 0288-0311 | 10001111 | 01010011 | 00011100 | 0x8F | 0x53 | 0x1C |
| 0312-0335 | 10000011 | 01000001 | 10111110 | 0x83 | 0x41 | 0xBE |
| 0336-0359 | 00111001 | 00101000 | 01100110 | 0x39 | 0x28 | 0x66 |
| 0360-0383 | 01000100 | 01100110 | 11001101 | 0x44 | 0x66 | 0xCD |
| 0384-0407 | 11110110 | 10100011 | 11011000 | 0xF6 | 0xA3 | 0xD8 |
| 0408-0431 | 00001101 | 11010100 | 10000001 | 0x0D | 0xD4 | 0x81 |
| 0432-0455 | 00111011 | 00101111 | 11011111 | 0x3B | 0x2F | 0xDF |
| 0456-0479 | 11000011 | 01011000 | 11110111 | 0xC3 | 0x58 | 0xF7 |
| 0480-0503 | 11000110 | 01010010 | 11101011 | 0xC6 | 0x52 | 0xEB |
| 0504-0527 | 01110000 | 10001111 | 10011110 | 0x70 | 0x8F | 0x9E |
| 0528-0551 | 01101010 | 10010000 | 10000001 | 0x6A | 0x90 | 0x81 |
| 0552-0575 | 11111101 | 01111100 | 00000000 | 0xFD | 0x7C | 0x00 |
| 0576-0599 | 00000000 | 00000000 | 00000000 | 0x00 | 0x00 | 0x00 |
| 0600-0623 | 00000000 | 00000000 | 00000000 | 0x00 | 0x00 | 0x00 |
| 0624-0647 | 00000000 | 00000000 | 00000000 | 0x00 | 0x00 | 0x00 |
| 0648-0671 | 10101001 | 11010001 | 01010101 | 0xA9 | 0xD1 | 0x55 |
| 0672-0695 | 00010010 | 00000100 | 01110100 | 0x12 | 0x04 | 0x74 |
| 0696-0719 | 11011001 | 11101001 | 00111011 | 0xD9 | 0xE9 | 0x3B |
| 0720-0743 | 11001101 | 10010011 | 10001101 | 0xCD | 0x93 | 0x8D |
| 0744-0767 | 01111011 | 01111100 | 01110000 | 0x7B | 0x7C | 0x70 |
| 0768-0791 | 00000010 | 00100000 | 10011001 | 0x02 | 0x20 | 0x99 |
| 0792-0815 | 10100001 | 01111101 | 10001010 | 0xA1 | 0x7D | 0x8A |
| 0816-0839 | 00100111 | 00010111 | 00111001 | 0x27 | 0x17 | 0x39 |
| 0840-0863 | 00010101 | 10100000 | 11101100 | 0x15 | 0xA0 | 0xEC |
| 0864-0887 | 01010101 | 10001010 | 00111111 | 0x55 | 0x8A | 0x3F |
| 0888-0911 | 01101011 | 10110110 | 11011000 | 0x6B | 0xB6 | 0xD8 |
| 0912-0935 | 10110001 | 10001000 | 10000100 | 0xB1 | 0x88 | 0x84 |
| 0936-0959 | 00001111 | 00101100 | 10001000 | 0x0F | 0x2C | 0x88 |
| 0960-0983 | 10101000 | 11111000 | 10010010 | 0xA8 | 0xF8 | 0x92 |
| 0984-1007 | 10100000 | 10110111 | 10011110 | 0xA0 | 0xB7 | 0x9E |
| 1008-1031 | 00111100 | 01100100 | 01010101 | 0x3C | 0x64 | 0x55 |
| 1032-1055 | 00001110 | 01111000 | 11111011 | 0x0E | 0x78 | 0xFB |
| 1056-1079 | 01110011 | 01010100 | 00000000 | 0x73 | 0x54 | 0x00 |
| 1080-1103 | 01000010 | 10101011 | 10000010 | 0x42 | 0xAB | 0x82 |
| 1104-1127 | 10111111 | 11100111 | 11001011 | 0xBF | 0xE7 | 0xCB |
| 1128-1151 | 00100110 | 11110011 | 01000000 | 0x26 | 0xF3 | 0x40 |
| 1152-1175 | 00001101 | 00000111 | 01101010 | 0x0D | 0x07 | 0x6A |
| 1176-1199 | 00010101 | 00010111 | 11111111 | 0x15 | 0x17 | 0xFF |
| 1200-1223 | 10100101 | 11011100 | 00000000 | 0xA5 | 0xDC | 0x00 |
| 1224-1247 | 00000000 | 00000000 | 00000000 | 0x00 | 0x00 | 0x00 |
| 1248-1271 | 00000000 | 00000000 | 00000000 | 0x00 | 0x00 | 0x00 |
| 1272-1295 | 00000000 | 00000000 | 00000000 | 0x00 | 0x00 | 0x00 |

G.10.5 Encoding the data for LDPC Example #2

The DATA with shortening bits are LDPC encoded as two (NCW = 2) codewords (LLDPC = 1296; R = 1/2) as prescribed by paragraph (c) of subclause 21.3.10.6.4 “PPDU encoding process”.  The results are given in Table G.43, with the bit-ordering being most significant bit first.  Note that the LDPC encoder appends 648 bits, bits 648-1295, after the first shortened codeword and another 648 bits, bits 1944-2591, after the second shortened codeword.

Table G.43—The DATA bits for LDPC Example #2 after LDPC encoding.

| Bit ## | Binary Val | Binary Val | Binary Val | Hex Val | Hex Val | Hex Val |
| --- | --- | --- | --- | --- | --- | --- |
| 0000-0023 | 01101100 | 00011001 | 10001001 | 0x6C | 0x19 | 0x89 |
| 0024-0047 | 10001111 | 01101000 | 00111001 | 0x8F | 0x68 | 0x39 |
| 0048-0071 | 11110100 | 10100101 | 01100001 | 0xF4 | 0xA5 | 0x61 |
| 0072-0095 | 01001111 | 11010111 | 10101110 | 0x4F | 0xD7 | 0xAE |
| 0096-0119 | 00100100 | 00001100 | 11110011 | 0x24 | 0x0C | 0xF3 |
| 0120-0143 | 00111010 | 11100100 | 10111100 | 0x3A | 0xE4 | 0xBC |
| 0144-0167 | 01010011 | 10011000 | 11000000 | 0x53 | 0x98 | 0xC0 |
| 0168-0191 | 00011110 | 00110101 | 10110011 | 0x1E | 0x35 | 0xB3 |
| 0192-0215 | 11100011 | 11111000 | 00100101 | 0xE3 | 0xF8 | 0x25 |
| 0216-0239 | 01100000 | 11010110 | 00100101 | 0x60 | 0xD6 | 0x25 |
| 0240-0263 | 00110101 | 00110011 | 11111110 | 0x35 | 0x33 | 0xFE |
| 0264-0287 | 11110000 | 01000001 | 00101011 | 0xF0 | 0x41 | 0x2B |
| 0288-0311 | 10001111 | 01010011 | 00011100 | 0x8F | 0x53 | 0x1C |
| 0312-0335 | 10000011 | 01000001 | 10111110 | 0x83 | 0x41 | 0xBE |
| 0336-0359 | 00111001 | 00101000 | 01100110 | 0x39 | 0x28 | 0x66 |
| 0360-0383 | 01000100 | 01100110 | 11001101 | 0x44 | 0x66 | 0xCD |
| 0384-0407 | 11110110 | 10100011 | 11011000 | 0xF6 | 0xA3 | 0xD8 |
| 0408-0431 | 00001101 | 11010100 | 10000001 | 0x0D | 0xD4 | 0x81 |
| 0432-0455 | 00111011 | 00101111 | 11011111 | 0x3B | 0x2F | 0xDF |
| 0456-0479 | 11000011 | 01011000 | 11110111 | 0xC3 | 0x58 | 0xF7 |
| 0480-0503 | 11000110 | 01010010 | 11101011 | 0xC6 | 0x52 | 0xEB |
| 0504-0527 | 01110000 | 10001111 | 10011110 | 0x70 | 0x8F | 0x9E |
| 0528-0551 | 01101010 | 10010000 | 10000001 | 0x6A | 0x90 | 0x81 |
| 0552-0575 | 11111101 | 01111100 | 00000000 | 0xFD | 0x7C | 0x00 |
| 0576-0599 | 00000000 | 00000000 | 00000000 | 0x00 | 0x00 | 0x00 |
| 0600-0623 | 00000000 | 00000000 | 00000000 | 0x00 | 0x00 | 0x00 |
| 0624-0647 | 00000000 | 00000000 | 00000000 | 0x00 | 0x00 | 0x00 |
| 0648-0671 | 00001001 | 11000001 | 11111011 | 0x09 | 0xC1 | 0xFB |
| 0672-0695 | 01101000 | 11001101 | 00000101 | 0x68 | 0xCD | 0x05 |
| 0696-0719 | 10110110 | 11000111 | 01100101 | 0xB6 | 0xC7 | 0x65 |
| 0720-0743 | 10100101 | 10011001 | 11100000 | 0xA5 | 0x99 | 0xE0 |
| 0744-0767 | 01110011 | 01110000 | 01101101 | 0x73 | 0x70 | 0x6D |
| 0768-0791 | 01011110 | 01111001 | 11100011 | 0x5E | 0x79 | 0xE3 |
| 0792-0815 | 01100111 | 00100111 | 01011110 | 0x67 | 0x27 | 0x5E |
| 0816-0839 | 10010101 | 10101000 | 11110110 | 0x95 | 0xA8 | 0xF6 |
| 0840-0863 | 00110101 | 01001000 | 10100111 | 0x35 | 0x48 | 0xA7 |
| 0864-0887 | 00100110 | 00101001 | 00110001 | 0x26 | 0x29 | 0x31 |
| 0888-0911 | 00101110 | 00011001 | 11110100 | 0x2E | 0x19 | 0xF4 |
| 0912-0935 | 00110100 | 01101111 | 01010000 | 0x34 | 0x6F | 0x50 |
| 0936-0959 | 01010000 | 11101001 | 11000100 | 0x50 | 0xE9 | 0xC4 |
| 0960-0983 | 00000110 | 11011001 | 11101110 | 0x06 | 0xD9 | 0xEE |
| 0984-1007 | 11111000 | 00011011 | 11011001 | 0xF8 | 0x1B | 0xD9 |
| 1008-1031 | 01101100 | 10000110 | 11010011 | 0x6C | 0x86 | 0xD3 |
| 1032-1055 | 11101001 | 01100100 | 11001000 | 0xE9 | 0x64 | 0xC8 |
| 1056-1079 | 11110001 | 10100001 | 00001011 | 0xF1 | 0xA1 | 0x0B |
| 1080-1103 | 11000010 | 01000100 | 01010100 | 0xC2 | 0x44 | 0x54 |
| 1104-1127 | 10100000 | 10001100 | 10111011 | 0xA0 | 0x8C | 0xBB |
| 1128-1151 | 10100011 | 11100100 | 10101001 | 0xA3 | 0xE4 | 0xA9 |
| 1152-1175 | 10101011 | 01010000 | 11100010 | 0xAB | 0x50 | 0xE2 |
| 1176-1199 | 01110000 | 00101000 | 00110110 | 0x70 | 0x28 | 0x36 |
| 1200-1223 | 11111100 | 00110000 | 00110100 | 0xFC | 0x30 | 0x34 |
| 1224-1247 | 01101010 | 01001001 | 00100010 | 0x6A | 0x49 | 0x22 |
| 1248-1271 | 11010101 | 00000111 | 11001111 | 0xD5 | 0x07 | 0xCF |
| 1272-1295 | 00110101 | 00111010 | 10001110 | 0x35 | 0x3A | 0x8E |
| 1296-1319 | 10101001 | 11010001 | 01010101 | 0xA9 | 0xD1 | 0x55 |
| 1320-1343 | 00010010 | 00000100 | 01110100 | 0x12 | 0x04 | 0x74 |
| 1344-1367 | 11011001 | 11101001 | 00111011 | 0xD9 | 0xE9 | 0x3B |
| 1368-1391 | 11001101 | 10010011 | 10001101 | 0xCD | 0x93 | 0x8D |
| 1392-1415 | 01111011 | 01111100 | 01110000 | 0x7B | 0x7C | 0x70 |
| 1416-1439 | 00000010 | 00100000 | 10011001 | 0x02 | 0x20 | 0x99 |
| 1440-1463 | 10100001 | 01111101 | 10001010 | 0xA1 | 0x7D | 0x8A |
| 1464-1487 | 00100111 | 00010111 | 00111001 | 0x27 | 0x17 | 0x39 |
| 1488-1511 | 00010101 | 10100000 | 11101100 | 0x15 | 0xA0 | 0xEC |
| 1512-1535 | 01010101 | 10001010 | 00111111 | 0x55 | 0x8A | 0x3F |
| 1536-1559 | 01101011 | 10110110 | 11011000 | 0x6B | 0xB6 | 0xD8 |
| 1560-1583 | 10110001 | 10001000 | 10000100 | 0xB1 | 0x88 | 0x84 |
| 1584-1607 | 00001111 | 00101100 | 10001000 | 0x0F | 0x2C | 0x88 |
| 1608-1631 | 10101000 | 11111000 | 10010010 | 0xA8 | 0xF8 | 0x92 |
| 1632-1655 | 10100000 | 10110111 | 10011110 | 0xA0 | 0xB7 | 0x9E |
| 1656-1679 | 00111100 | 01100100 | 01010101 | 0x3C | 0x64 | 0x55 |
| 1680-1703 | 00001110 | 01111000 | 11111011 | 0x0E | 0x78 | 0xFB |
| 1704-1727 | 01110011 | 01010100 | 00000000 | 0x73 | 0x54 | 0x00 |
| 1728-1751 | 01000010 | 10101011 | 10000010 | 0x42 | 0xAB | 0x82 |
| 1752-1775 | 10111111 | 11100111 | 11001011 | 0xBF | 0xE7 | 0xCB |
| 1776-1799 | 00100110 | 11110011 | 01000000 | 0x26 | 0xF3 | 0x40 |
| 1800-1823 | 00001101 | 00000111 | 01101010 | 0x0D | 0x07 | 0x6A |
| 1824-1847 | 00010101 | 00010111 | 11111111 | 0x15 | 0x17 | 0xFF |
| 1848-1871 | 10100101 | 11011100 | 00000000 | 0xA5 | 0xDC | 0x00 |
| 1872-1895 | 00000000 | 00000000 | 00000000 | 0x00 | 0x00 | 0x00 |
| 1896-1919 | 00000000 | 00000000 | 00000000 | 0x00 | 0x00 | 0x00 |
| 1920-1943 | 00000000 | 00000000 | 00000000 | 0x00 | 0x00 | 0x00 |
| 1944-1967 | 01100100 | 10110110 | 01010100 | 0x64 | 0xB6 | 0x54 |
| 1968-1991 | 00110001 | 00000001 | 01100001 | 0x31 | 0x01 | 0x61 |
| 1992-2015 | 00101001 | 00010011 | 01110000 | 0x29 | 0x13 | 0x70 |
| 2016-2039 | 01010000 | 10000000 | 11001110 | 0x50 | 0x80 | 0xCE |
| 2040-2063 | 01000101 | 11000000 | 10101000 | 0x45 | 0xC0 | 0xA8 |
| 2064-2087 | 11001101 | 11111000 | 01111100 | 0xCD | 0xF8 | 0x7C |
| 2088-2111 | 01010011 | 01010001 | 01001110 | 0x53 | 0x51 | 0x4E |
| 2112-2135 | 11010011 | 10101110 | 00010011 | 0xD3 | 0xAE | 0x13 |
| 2136-2159 | 11110000 | 11101101 | 10111111 | 0xF0 | 0xED | 0xBF |
| 2160-2183 | 10001110 | 10010100 | 00110100 | 0x8E | 0x94 | 0x34 |
| 2184-2207 | 11111011 | 00010000 | 11011001 | 0xFB | 0x10 | 0xD9 |
| 2208-2231 | 10111110 | 00110001 | 10011111 | 0xBE | 0x31 | 0x9F |
| 2232-2255 | 01100000 | 00011100 | 10100110 | 0x60 | 0x1C | 0xA6 |
| 2256-2279 | 01010101 | 11111001 | 10100110 | 0x55 | 0xF9 | 0xA6 |
| 2280-2303 | 10101010 | 00111000 | 01110001 | 0xAA | 0x38 | 0x71 |
| 2304-2327 | 01111010 | 10101100 | 10110010 | 0x7A | 0xAC | 0xB2 |
| 2328-2351 | 11110101 | 11010001 | 10000001 | 0xF5 | 0xD1 | 0x81 |
| 2352-2375 | 01010000 | 11110001 | 00001011 | 0x50 | 0xF1 | 0x0B |
| 2376-2399 | 10111101 | 10010011 | 10001011 | 0xBD | 0x93 | 0x8B |
| 2400-2423 | 10100010 | 10010110 | 00100101 | 0xA2 | 0x96 | 0x25 |
| 2424-2447 | 11100011 | 01101100 | 11000111 | 0xE3 | 0x6C | 0xC7 |
| 2448-2471 | 00000101 | 00011000 | 00101000 | 0x05 | 0x18 | 0x28 |
| 2472-2495 | 11110011 | 00111001 | 11011000 | 0xF3 | 0x39 | 0xD8 |
| 2496-2519 | 00010001 | 01110101 | 00010111 | 0x11 | 0x75 | 0x17 |
| 2520-2543 | 11011101 | 11111011 | 11010010 | 0xDD | 0xFB | 0xD2 |
| 2544-2567 | 10101010 | 11101011 | 10100110 | 0xAA | 0xEB | 0xA6 |
| 2568-2591 | 10000101 | 10110011 | 01011000 | 0x85 | 0xB3 | 0x58 |

G.10.6 Removing the shortening bits and repetition for LDPC Example #2

The shortening bits, applied before LDPC encoding, are now removed as prescribed in paragraph (c) of subclause 21.3.10.6.5 “PPDU encoding process”.  Finally, either puncturing is applied as described in paragraph (d) of the same subclause or the copying of repeated bits are applied as described in paragraph (e) of the same subclause. In LDPC Encoding Example #1, because Npunc = 0 and Nrep = 160 is non-zero, repetition is prescribed, completing the LDPC encoding process.

The results are given in Table G.44, with the bit-ordering being most significant bit first.

Note that the first 80 shortening bits (bits 568-647 from Table G.43) have been removed from the first codeword between bit 567 and 568 of Table G.44, and that the second 80 shortening bits (bits 1864-1943 of Table G.43) have been removed between bits 1215 and 1216 of Table G.44.  Also, note that 80 bits have been repeated from the beginning of the first codeword (bits 0-79) to the end of the first codeword (bits 1216-1295), and that 80 bits have been repeated from the beginning of the second codeword (bits 1296-1375) to end of the second codeword (bits 2512-2591) in Table G.44.

Table G.44—The DATA bits for LDPC Example #2 after removal of shortening bits and copying of repetition bits.

| Bit ## | Binary Val | Binary Val | Binary Val | Hex Val | Hex Val | Hex Val |
| --- | --- | --- | --- | --- | --- | --- |
| 0000-0023 | 01101100 | 00011001 | 10001001 | 0x6C | 0x19 | 0x89 |
| 0024-0047 | 10001111 | 01101000 | 00111001 | 0x8F | 0x68 | 0x39 |
| 0048-0071 | 11110100 | 10100101 | 01100001 | 0xF4 | 0xA5 | 0x61 |
| 0072-0095 | 01001111 | 11010111 | 10101110 | 0x4F | 0xD7 | 0xAE |
| 0096-0119 | 00100100 | 00001100 | 11110011 | 0x24 | 0x0C | 0xF3 |
| 0120-0143 | 00111010 | 11100100 | 10111100 | 0x3A | 0xE4 | 0xBC |
| 0144-0167 | 01010011 | 10011000 | 11000000 | 0x53 | 0x98 | 0xC0 |
| 0168-0191 | 00011110 | 00110101 | 10110011 | 0x1E | 0x35 | 0xB3 |
| 0192-0215 | 11100011 | 11111000 | 00100101 | 0xE3 | 0xF8 | 0x25 |
| 0216-0239 | 01100000 | 11010110 | 00100101 | 0x60 | 0xD6 | 0x25 |
| 0240-0263 | 00110101 | 00110011 | 11111110 | 0x35 | 0x33 | 0xFE |
| 0264-0287 | 11110000 | 01000001 | 00101011 | 0xF0 | 0x41 | 0x2B |
| 0288-0311 | 10001111 | 01010011 | 00011100 | 0x8F | 0x53 | 0x1C |
| 0312-0335 | 10000011 | 01000001 | 10111110 | 0x83 | 0x41 | 0xBE |
| 0336-0359 | 00111001 | 00101000 | 01100110 | 0x39 | 0x28 | 0x66 |
| 0360-0383 | 01000100 | 01100110 | 11001101 | 0x44 | 0x66 | 0xCD |
| 0384-0407 | 11110110 | 10100011 | 11011000 | 0xF6 | 0xA3 | 0xD8 |
| 0408-0431 | 00001101 | 11010100 | 10000001 | 0x0D | 0xD4 | 0x81 |
| 0432-0455 | 00111011 | 00101111 | 11011111 | 0x3B | 0x2F | 0xDF |
| 0456-0479 | 11000011 | 01011000 | 11110111 | 0xC3 | 0x58 | 0xF7 |
| 0480-0503 | 11000110 | 01010010 | 11101011 | 0xC6 | 0x52 | 0xEB |
| 0504-0527 | 01110000 | 10001111 | 10011110 | 0x70 | 0x8F | 0x9E |
| 0528-0551 | 01101010 | 10010000 | 10000001 | 0x6A | 0x90 | 0x81 |
| 0552-0575 | 11111101 | 01111100 | 00001001 | 0xFD | 0x7C | 0x09 |
| 0576-0599 | 11000001 | 11111011 | 01101000 | 0xC1 | 0xFB | 0x68 |
| 0600-0623 | 11001101 | 00000101 | 10110110 | 0xCD | 0x05 | 0xB6 |
| 0624-0647 | 11000111 | 01100101 | 10100101 | 0xC7 | 0x65 | 0xA5 |
| 0648-0671 | 10011001 | 11100000 | 01110011 | 0x99 | 0xE0 | 0x73 |
| 0672-0695 | 01110000 | 01101101 | 01011110 | 0x70 | 0x6D | 0x5E |
| 0696-0719 | 01111001 | 11100011 | 01100111 | 0x79 | 0xE3 | 0x67 |
| 0720-0743 | 00100111 | 01011110 | 10010101 | 0x27 | 0x5E | 0x95 |
| 0744-0767 | 10101000 | 11110110 | 00110101 | 0xA8 | 0xF6 | 0x35 |
| 0768-0791 | 01001000 | 10100111 | 00100110 | 0x48 | 0xA7 | 0x26 |
| 0792-0815 | 00101001 | 00110001 | 00101110 | 0x29 | 0x31 | 0x2E |
| 0816-0839 | 00011001 | 11110100 | 00110100 | 0x19 | 0xF4 | 0x34 |
| 0840-0863 | 01101111 | 01010000 | 01010000 | 0x6F | 0x50 | 0x50 |
| 0864-0887 | 11101001 | 11000100 | 00000110 | 0xE9 | 0xC4 | 0x06 |
| 0888-0911 | 11011001 | 11101110 | 11111000 | 0xD9 | 0xEE | 0xF8 |
| 0912-0935 | 00011011 | 11011001 | 01101100 | 0x1B | 0xD9 | 0x6C |
| 0936-0959 | 10000110 | 11010011 | 11101001 | 0x86 | 0xD3 | 0xE9 |
| 0960-0983 | 01100100 | 11001000 | 11110001 | 0x64 | 0xC8 | 0xF1 |
| 0984-1007 | 10100001 | 00001011 | 11000010 | 0xA1 | 0x0B | 0xC2 |
| 1008-1031 | 01000100 | 01010100 | 10100000 | 0x44 | 0x54 | 0xA0 |
| 1032-1055 | 10001100 | 10111011 | 10100011 | 0x8C | 0xBB | 0xA3 |
| 1056-1079 | 11100100 | 10101001 | 10101011 | 0xE4 | 0xA9 | 0xAB |
| 1080-1103 | 01010000 | 11100010 | 01110000 | 0x50 | 0xE2 | 0x70 |
| 1104-1127 | 00101000 | 00110110 | 11111100 | 0x28 | 0x36 | 0xFC |
| 1128-1151 | 00110000 | 00110100 | 01101010 | 0x30 | 0x34 | 0x6A |
| 1152-1175 | 01001001 | 00100010 | 11010101 | 0x49 | 0x22 | 0xD5 |
| 1176-1199 | 00000111 | 11001111 | 00110101 | 0x07 | 0xCF | 0x35 |
| 1200-1223 | 00111010 | 10001110 | 01101100 | 0x3A | 0x8E | 0x6C |
| 1224-1247 | 00011001 | 10001001 | 10001111 | 0x19 | 0x89 | 0x8F |
| 1248-1271 | 01101000 | 00111001 | 11110100 | 0x68 | 0x39 | 0xF4 |
| 1272-1295 | 10100101 | 01100001 | 01001111 | 0xA5 | 0x61 | 0x4F |
| 1296-1319 | 10101001 | 11010001 | 01010101 | 0xA9 | 0xD1 | 0x55 |
| 1320-1343 | 00010010 | 00000100 | 01110100 | 0x12 | 0x04 | 0x74 |
| 1344-1367 | 11011001 | 11101001 | 00111011 | 0xD9 | 0xE9 | 0x3B |
| 1368-1391 | 11001101 | 10010011 | 10001101 | 0xCD | 0x93 | 0x8D |
| 1392-1415 | 01111011 | 01111100 | 01110000 | 0x7B | 0x7C | 0x70 |
| 1416-1439 | 00000010 | 00100000 | 10011001 | 0x02 | 0x20 | 0x99 |
| 1440-1463 | 10100001 | 01111101 | 10001010 | 0xA1 | 0x7D | 0x8A |
| 1464-1487 | 00100111 | 00010111 | 00111001 | 0x27 | 0x17 | 0x39 |
| 1488-1511 | 00010101 | 10100000 | 11101100 | 0x15 | 0xA0 | 0xEC |
| 1512-1535 | 01010101 | 10001010 | 00111111 | 0x55 | 0x8A | 0x3F |
| 1536-1559 | 01101011 | 10110110 | 11011000 | 0x6B | 0xB6 | 0xD8 |
| 1560-1583 | 10110001 | 10001000 | 10000100 | 0xB1 | 0x88 | 0x84 |
| 1584-1607 | 00001111 | 00101100 | 10001000 | 0x0F | 0x2C | 0x88 |
| 1608-1631 | 10101000 | 11111000 | 10010010 | 0xA8 | 0xF8 | 0x92 |
| 1632-1655 | 10100000 | 10110111 | 10011110 | 0xA0 | 0xB7 | 0x9E |
| 1656-1679 | 00111100 | 01100100 | 01010101 | 0x3C | 0x64 | 0x55 |
| 1680-1703 | 00001110 | 01111000 | 11111011 | 0x0E | 0x78 | 0xFB |
| 1704-1727 | 01110011 | 01010100 | 00000000 | 0x73 | 0x54 | 0x00 |
| 1728-1751 | 01000010 | 10101011 | 10000010 | 0x42 | 0xAB | 0x82 |
| 1752-1775 | 10111111 | 11100111 | 11001011 | 0xBF | 0xE7 | 0xCB |
| 1776-1799 | 00100110 | 11110011 | 01000000 | 0x26 | 0xF3 | 0x40 |
| 1800-1823 | 00001101 | 00000111 | 01101010 | 0x0D | 0x07 | 0x6A |
| 1824-1847 | 00010101 | 00010111 | 11111111 | 0x15 | 0x17 | 0xFF |
| 1848-1871 | 10100101 | 11011100 | 01100100 | 0xA5 | 0xDC | 0x64 |
| 1872-1895 | 10110110 | 01010100 | 00110001 | 0xB6 | 0x54 | 0x31 |
| 1896-1919 | 00000001 | 01100001 | 00101001 | 0x01 | 0x61 | 0x29 |
| 1920-1943 | 00010011 | 01110000 | 01010000 | 0x13 | 0x70 | 0x50 |
| 1944-1967 | 10000000 | 11001110 | 01000101 | 0x80 | 0xCE | 0x45 |
| 1968-1991 | 11000000 | 10101000 | 11001101 | 0xC0 | 0xA8 | 0xCD |
| 1992-2015 | 11111000 | 01111100 | 01010011 | 0xF8 | 0x7C | 0x53 |
| 2016-2039 | 01010001 | 01001110 | 11010011 | 0x51 | 0x4E | 0xD3 |
| 2040-2063 | 10101110 | 00010011 | 11110000 | 0xAE | 0x13 | 0xF0 |
| 2064-2087 | 11101101 | 10111111 | 10001110 | 0xED | 0xBF | 0x8E |
| 2088-2111 | 10010100 | 00110100 | 11111011 | 0x94 | 0x34 | 0xFB |
| 2112-2135 | 00010000 | 11011001 | 10111110 | 0x10 | 0xD9 | 0xBE |
| 2136-2159 | 00110001 | 10011111 | 01100000 | 0x31 | 0x9F | 0x60 |
| 2160-2183 | 00011100 | 10100110 | 01010101 | 0x1C | 0xA6 | 0x55 |
| 2184-2207 | 11111001 | 10100110 | 10101010 | 0xF9 | 0xA6 | 0xAA |
| 2208-2231 | 00111000 | 01110001 | 01111010 | 0x38 | 0x71 | 0x7A |
| 2232-2255 | 10101100 | 10110010 | 11110101 | 0xAC | 0xB2 | 0xF5 |
| 2256-2279 | 11010001 | 10000001 | 01010000 | 0xD1 | 0x81 | 0x50 |
| 2280-2303 | 11110001 | 00001011 | 10111101 | 0xF1 | 0x0B | 0xBD |
| 2304-2327 | 10010011 | 10001011 | 10100010 | 0x93 | 0x8B | 0xA2 |
| 2328-2351 | 10010110 | 00100101 | 11100011 | 0x96 | 0x25 | 0xE3 |
| 2352-2375 | 01101100 | 11000111 | 00000101 | 0x6C | 0xC7 | 0x05 |
| 2376-2399 | 00011000 | 00101000 | 11110011 | 0x18 | 0x28 | 0xF3 |
| 2400-2423 | 00111001 | 11011000 | 00010001 | 0x39 | 0xD8 | 0x11 |
| 2424-2447 | 01110101 | 00010111 | 11011101 | 0x75 | 0x17 | 0xDD |
| 2448-2471 | 11111011 | 11010010 | 10101010 | 0xFB | 0xD2 | 0xAA |
| 2472-2495 | 11101011 | 10100110 | 10000101 | 0xEB | 0xA6 | 0x85 |
| 2496-2519 | 10110011 | 01011000 | 10101001 | 0xB3 | 0x58 | 0xA9 |
| 2520-2543 | 11010001 | 01010101 | 00010010 | 0xD1 | 0x55 | 0x12 |
| 2544-2567 | 00000100 | 01110100 | 11011001 | 0x04 | 0x74 | 0xD9 |
| 2568-2591 | 11101001 | 00111011 | 11001101 | 0xE9 | 0x3B | 0xCD |