IEEE P802.11
Wireless LANs

| LB84-Clause-11.16-Submission |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-11-08 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Shravan Surineni | Qualcomm | 9 Damon Mill Square, Suite 2A Concord,MA 01742 | +1-781-276-0926 | shravans@qualcomm.com |
| Sanjiv Nanda | Qualcommm | 5775 Morehouse Drive, San Diego, CA 92121 | +1-858-845-2375 | snanda@qualcomm.com |
| Bjorn Bjerke | Qualcomm | 9 Damon Mill Square, Suite 2A Concord,MA 01742 | +1-781-276-0912 | bbjerke@qualcomm.com |
| Yuichi Morioka | Sony |  |  | morioka@wcs.sony.co.jp |
| Srinivas Kandala | Airgo |  |  | srini@airgonetworks.com |
| Amit Bansal | Wipro |  |  | amit.bansal@wipro.com |
| Tomoko Adachi | Toshiba |  |  | tomo.adachi@toshiba.co.jp |

Abstract

This document contains proposed changes to the 802.11 TGn Draft D1.05 to address comments related to section 11.16.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

## Interpretation of a Motion to Adopt

A motion to adopt the changes defined in this submission means that the editing instructions and any changed or added material are actioned in the TGn Draft.  This introduction and headings are not part of the adopted material.

Editing instructions formatted like this are intended to be copied into the TGn Draft (i.e. they are instructions to the 802.11 editor on how to merge the TGn amendment with the baseline documents).

TGn Editor:  Editing instructions preceded by “TGn Editor” are instructions to the TGn editor to modify existing material in the TGn draft.   As a result of adopting the changes, the TGn editor will execute the instructions rather than copy them to the TGn Draft.

Submission Note: Notes to the reader of this submission are not part of the motion to adopt.  These notes are there to clarify or set in context the response to the omission.

TGn Editor: Change section 11.16 as described below:

Insert the following subclause heading:

EDITORIAL NOTE—In the referenced baseline documents, TGk stops at 11.14, there is then a gap, and TGp has 11.16.  The following subclause numbering assumes that TGp will be modified to use 11.15.

- 20/40 MHz Operation

Insert the following new subclause:

- Basic functionality in BSS 20/40 MHz mode

The following terms are used in this subclause to describe transmitted PPDU formats:

- Non-HT OFDM is a clause 21 transmission using  FORMAT=NON_HT and CW=20 MHz

- 40 MHz HT is a clause 21 transmission using FORMAT=HT_MM or HT_GF and CW=40 MHz

- 20 MHz HT is a clause 21 transmission using FORMAT=HT_MM or HT_GF and CW=20 MHz

- Non-HT Duplicate OFDM is a clause 21 transmission using FORMAT=NON_HT and CW=HT_CW_20DN

- DSSS/CCK is a clause 15 or clause 18 transmission

Submission Note: Definitions of Non-HT OFDM PPDU and Non-HT Duplicate OFDM PPDU are included in 3.x, and should be deleted from here. The other definitions can stay.

An HT AP declares its capabilities BSS operating bandwidth (20 MHz only or 20/40 MHz) in the Supported Channel Width Set in the HT capability Capabilities element contained in each beacon. Protection requirements for HT frames are part of the HT HT Information Element transmitted by the AP.

Submission Note: All protection related text is removed from this clause, except for the following paragraph that indicates how an AP may set certain fields in the HT information element and the ERP information element to manage protection within the BSS:

The HT AP indicates protection requirements in the BSS through the Operating Mode field and Non-Greenfield STAs Present field of the HT information element as well as the Use Protection field of the ERP information element. Protection requirements for different settings of these fields are defined in 9.13.

Long NAV protection may be used to protect TXOP of several frames.

Any 20/40 MHz capable HT STA, 20 MHz capable HT STA and or non-HT STA may be associated in the a 20/40 MHz capable BSS.

An HT A non-AP HT STA declares its operating bandwidth (20 MHz only or 20/40 MHz) in capabilities by the Supported Channel Width Set in the HT capability Capabilities element issued with its Association Request.

An HT STA may switch between 20/40 MHz capable and 20 MHz capable operation by disassociation and (re-)association.

A 20/40 MHz capable HT STA transmits shall (Ed: CID 1008) transmit and receives receive HT frames in use either the 40 MHz channel or the 20 MHz control primary channel to transmit and receives receive 20 MHz HT frames.  The Recommended Notify Channel Width action frame may be used by a non-AP HT STA to choose an actual channel width. Transmission of this frame by a non-AP STA to another STA requests that the other STA AP send frames in the indicated width. This frame shall be sent in the control primary channel using non-HT or non-HT duplicate OFDM PPDUmode. The STA may use Non-HT OFDM frames with rates selected from the BSSBasicRateSet for protection of 20 MHz HT frames in the control primary channel unless the Use Protection field of the ERP information element is set.  If setset to 1 (Ed: CID 626), DSSS/CCK frames shall be used for protection.

A 20 MHz capable HT STA shall transmit and receive HT frames in use the 20 MHz control primary channel to transmit and receive HT frames. and may use Non-HT OFDM frames with rates selected from the BSSBasicRateSet for protection unless the Use Protection field of the ERP information element is setset to 1 (Ed: CID 626).  If setset to 1 (Ed: CID 626), DSSS/CCK frames shall be used for protection. .

The 20/40 MHz capable AP controls the type of protection used in order to avoid collision between 20 MHz capable STAs and other 20/40 MHz capable STAs operating in 40 MHz channel, as described in 9.20.1 (Rules for operation in 20/40 MHz BSS).

NOTE—These STAs can collide because the 20 MHz capable STAs cannot respect the virtual carrier of the 40 MHz capable STAs.

Submission Note: PPDU type selection rules per Operating Mode  are defined  in 9.13, so the following sentence is  deleted.

In an HT Mixed Mode, all broadcast frames and control frame not included in an A-MPDU shall be sent so that they can be received by a non-HT STA operating on the primary channel. (Ed: CID 1200)

Submission Note: Table n60 only needs to define PPDU format to be used  for Beacon transmission. Verify whether this is addressed in document 1557 to determine if Table n60 is redundant.

The different HT AP Modes PPDU Formats for Beacon transmission are defined in Table n60 (PPDU Formats for Beacon Transmission).

| - AP Mode definitions PPDU Formats for Beacon transmission |  |  |  |
| --- | --- | --- | --- |
| AP Mode Band and BSS Operating Bandwidth | PPDU Format for Beacon transmission | non-AP STA operation | Protection required at non-AP STA |
| 5 GHz band BSS 20/40 MHz | Non-HT OFDM | 40 MHz | Non-HT Duplicate OFDM |
| 5 GHz band BSS 20/40 MHz | Non-HT OFDM | 20 MHz HT | Non-HT OFDM |
| 5 GHz band BSS 20 MHz | Non-HT OFDM | 20 MHz HT | Non-HT OFDM |
| 2.4 GHz band BSS 20/40 MHz | DSSS/CCK or non-HT OFDM. See NOTE 1. | 40 MHz | Non-HT Duplicate OFDM |
| 2.4 GHz band BSS 20/40 MHz | DSSS/CCK or non-HT OFDM. See NOTE 1. | 20 MHz HT | DSSS/CCK or non-HT OFDM. See NOTE 2. |
| 2.4 GHz band BSS 20 MHz | DSSS/CCK or non-HT OFDM. See NOTE 1. | 20 MHz HT | DSSS/CCK or non-HT OFDM. See NOTE 2. |
| NOTE 1— As determined by the BSSBasicRateSet NOTE 2— If the Use_Protection field of the ERP information element is setset to 1 (Ed: CID 626), DSSS/CCK frames are used for protection, otherwise OFDM frames are used as determined by the BSSBasicRateSet. |  |  |  |

Submission Note: All PCO related discussion is removed from this clause, except for the following two paragraphs:

- Phased Coexistence Operation (PCO) is an optional BSS mode with alternating 20 MHz and 40 MHz phases controlled by a PCO capable AP. A PCO capable AP advertises its BSS as 20/40 MHz capable and PCO capable. A non-PCO capable STA regards the BSS as a 20/40 MHz BSS and may associate with the BSS without regard to this field.

A STA may associate with a PCO BSS as a PCO capable STA by setting the PCO field in the HT Extended Capabilities element to 1. If the PCO transition time of the STA does not meet the time indicated by the PCO AP in the PCO Transition Time field, then the PCO capable STA regards the BSS as a 20/40 MHz BSS and may associate with the BSS as a 20/40 MHz STA.

Editor: Remove Annex S. The information is redundant. (Ed: CID 7487 and 3604)

Insert the following new subclause

    - Support of DSSS/CCK in 40 MHz

(Ed: CID 1107, moved from 7.3.2.46.2. Introductory paragraph an editorial conjecture. Rewording a conjecture based on intent of original text.)

Transmission and reception of PPDUs using DSSS/CCK is by HT STA operating in 20/40MHz Mode is managed using the DSSS/CCK Mode in 40 MHz (Ed: CID 635) subfield of the HT Capabilities Info field (see 7.3.2.46.3 (A-MPDU Parameters   7.3.2.46.2 (HT Capabilities Info (Ed: CID 3755) field)).

An HT AP determines if it permits DSSS/CCK transmissions in 20/40 MHz mode. If the DSSS/CCK Mode in 40 MHz (Ed: CID 635) subfield is set to 1 in Beacon and Probe Response frames, an associated HT STA may generate DSSS/CCK transitions transmissions. If the subfield is set to 0:

- Associated HT STAs shall not generate DSSS/CCK transmissions

- The AP shall not include an ERP information element in its Beacon and Probe Response transmissions

- The AP shall not include DSSS/CCK rates in the Supported Rates information element

- The AP shall refuse association requests from non-HT STA any STA that includes DSSS/CCK rates in its Supported Rates information element.

- The AP shall refuse association requests from an HT STA that have has the DSSS/CCK Mode in 40 MHz (Ed: CID 635) subfield set to 1.

- The Beacon shall be transmitted using an OFDM basic rate

Submission Note: Editor’s comment below regarding “intent” needs discussion.

An HT STA declares its intention to use DSSS/CCK rates through the DSSS/CCK Mode in 40 MHz (Ed: CID 635) subfield of its Association and Reassociation Request transmissions. This subfield is set to 0 if the STA does not intend to use DSSS/CCK rates. It is set to 1 if the STA intends to use DSSS/CCK rates.

(Ed: the rewording preserves the semantics of the referenced subclause. However, those semantics are probably wrong. The bit should represent the STA’s capability when transmitted by the non-AP STA, rather than the intent (“uses”) in D1.02).

- References:

CIDs addressed by this submission:

| 1049 | 151 | 22 | 11.15.1 | The HT aslo sets the BSS parameters, not just the operable parameters for itself. | add a sentence (or add to an existing one) that the HT AP also advertises the allowed behavior of the STA associated with the BSS within the additional HT information element, and that associated STA shall obey that advertised allowed behavior. | Counter: This information is now included. Protection behaviour is in 9.13. |
| --- | --- | --- | --- | --- | --- | --- |
| 1050 | 151 | 24 | 11.15.1 | personally, I always think that it is good to include the condition assumed in effect by virtue of the section heading within the sentence stating a restriction | add "within a BSS operating in the 40/20 mode." to the sentence regarding longNAV operation | Reject: LongNAV is not limited to 20/40 HT BSS. Protection behaviour is in 9.13. |
| 1051 | 151 | 25 | 11.15.1 | Wrong article. | change "associated in the" to "associated in a" | Accept. |
| 9887 | 151 | 25 | 11.15.1 | 20 MHz capable STA? well, these STAs could be HT and non-HT STAs. Lines 25-26 needs to be re-written. | Change it to: "40/20 MHz capable STAs, 20 MHz only capable HT STAs and non-HT STAs may be associated in the 40/20 MHz capable BSS." | Accept. Already changed in D1.03 Ed: Classified as T as we don't have a T/E classification |
| 7483 | 151 |  | 11.15.1 | "Long NAV protection may be used to protect TxOP of several frames." - this is not adding anything | Remove the sentence. | Accept – remove the lines |
| 7484 | 151 |  | 11.15.1 | Although IBSS operation has not been described, we should assume that it is supported. Also, the management of DLS links needs to be included. | Replace: "to choose a" with "to recommend a". Replace: "Transmission of this frame by a non-AP STA requests that the AP send frames in indicated width." with: "Transmission of this frame by a non-AP STA requests that its peer send frames addressed to it using the indicated channel width." | Counter: Replaced with “Transmission of this frame by a non-AP STA to another STA requests that the other STA AP send frames in the indicated width.” |
| 7485 | 151 |  | 11.15.1 | "These STAs can collide because the 20MHz capable STAs cannot respect virtual carrier of the 40MHz capable STAs. " - it looks like this is a problem that hasn't been fixed, which is not true. | Remove the note. | Accept. |
| 7486 | 151 |  | 11.15.1 | What are these AP modes? Are they internal to the AP? What additional value does table n57 bring? | Remove Table n57 or relate it to normative behaviour. Or move it into 11.15.2 and merge with table n58. | Counter. The only normative behavior remaining in the first Table is regarding beacon transmission PPDU format. Second Table has been removed. |
| 6866 | 152 | 5 | 11.15.1 | Does not state that duplicate frames must be used for protection even though Table n57 rows 1 and 4 state this | Must use duplicate BSSBasicRateSet OFDM non-HT frames for protection | Counter. Protection related behavior is now in 9.13. |
| 6867 | 152 | 13 | 11.15.1 | Note is incorrect since the protection by the 40 MHz STA is done using 20 MHz control frames | Delete this sentence | Accept. Protection related behavior is now in 9.13. |
| 439 | 152 | 15 | 11.15.1 | "2.4MHz" | "2.4GHz" | Accept. D1.03 already includes this change. |
| 823 | 152 | 15 | 11.15.1 | AP Mode column refers to band and channel width, but should not use 5.2GHz as shorthand for the 5 GHz band | Change first three entries in table n57 to say 5 GHz band BSS | Accept. D1.03 addressed this by making the change required here. |
| 12118 | 152 | 15 | 11.15.1 | 2.4MHz band does not presently contain a recognized 802.11 PHY | Replace by 2.4GHz | Accept. This change has already been made in D1.03 |
| 6868 | 152 | 16 | 11.15.2 | Operating Modes defined here are inconsistent with operating modes in Table n19, p 53. | Rationalize the two sets of operating modes. | Counter. AP modes is has been replaced with “Operating Bandwidth”. |
| 6869 | 152 | 16 | 11.15.2 | Operating Modes section is informative but contains rules that are normative: e.g., "Beacons are sent in non-HT mode in the control channel" is defined only in this Table. | Beacon and protection rules to be used when operating as 40/20 are normative. | Accept. Protection rules are moved to 9.13. |
| 9889 | 152 | 19 | 11.15.2 | This line states that AP and non-AP STAs use the same protection mechanism : RTS/CTS and CF-End. This sentencec is odd, since it implies that only these mechanisms are used for the AP and non-AP STAs. | Need to clarify. | Accept. Reference to specific frame types for protection has been removed. |
| 1052 | 152 |  | 11.15.1 | no reference and no normative description - there is a table n57 with no text referencing it. | add text which references the table n57 and which includes normative language with respect to the itmes in the table, for example, "when the BSS mode is as described in the BSS mode column of table n57, then the AP shall transmit beacons as per the beacon transmission column in table n57 with the additional HT information element containing the mode bit setting xx" etc. etc. | Counter. Normative behavior for beacon transmission has been included in the Table. |
| 7770 | 153 | 3 | 11.15 | It is not clear what is meant by "Other (overlapping) BSS may sit in the extension channel". This implies restriction on channel selection that are not defined in standard. | Overlapping BSS may present in control channel or extension channel or both control and extension channel | Counter. This sentence is redundant so has been removed. |
| 440 | 153 | 3 | 11.15.2 | I was led to believe that 40MHz operation was optional. If that's true then the primary operational mode cannot be "40/20MHz" | Change the primary operational mode to "20MHz", especially in the 2.4GHz band | Counter. There is no “primary operation mode.” |
| 824 | 153 | 3 | 11.15.2 | Band column should not use 5.2 GHz as shorthand for the 5 GHz band | Change first entry in table n58 to say 5 GHz, like 20.1 does | Accept. This change has already been done in D1.03 |
| 8042 | 153 | 3 | 11.15.2 | How can 40/20MHz BSS capability be the primary operational HT mode in both 2.4GHz & 5.2GHz? 40MHz operation is optional. | Change "primary operational HT mode" to 20MHz for both 2.4GHz & 5.2GHz | Counter. There is no “primary operation mode.” |
| 9890 | 153 | 3 | 11.15.2 | In table n58 it states that "This is the primary operational HT mode". Who decides that? It is unclear. Further down, "OR" should be "or", "self CTS" should be "CTS-to-self". | Need to clarify, otherwise remove the first sentence and fix the editorial. | Counter. There is no “primary operation mode.” Reference to specific frame types for protection has been removed. |
| 9891 | 153 | 3 | 11.15.2 | Why 5.2GHz band? |  | Accept. This change has already been done in D1.03 |
| 4579 | 153 |  | 11.15.2 | Two modes called “the primary operational HT mode”. Should one of these be the primary mode or are these both primary modes (a primary mode). | Replace “the primary operational HT mode” with “a primary operational HT mode” in 5.2 and 2.4 GHz 40/20MHz definitions. | Accept. There is no “primary operation mode.” Ed AdHoc: Transfer to Gen AdHoc Gen AdHoc: Similar issue to cid 7038, CoExistence is addressing this issue. Transfer to Coexistence. Res: 20MHz operation is the primary mode of operation and 40MHz is optional. So the text has to reflect this. |
| 7772 | 154 | 3 | 11.15 | It is not clear what is meant by "Other (overlapping) BSS may sit in the extension channel". This implies restriction on channel selection that are not defined in standard. | Overlapping BSS may present in control channel or extension channel or both control and extension channel | Duplicate of 7770. |
| 7038 | 154 | 3 | 11.15.2 | In Table n58, row 2.4GHz band, 40/20MHz capability, the definition of mode starts with "This is the primary operational HT mode." I would suspect that in the 2.4GHz band with so few channels, and the channel spacing issues that this is not the primary mode of operation. | remove statement | Accept. There is no “primary operation mode.” Ed AdHoc: Transfer to Gen AdHoc Gen AdHoc: There are two issues here, 1 - the Primary as what is most used, and 2 what is the primary mode specified. The Coexistence group will be discussing this and so we should let that group resolve this comment. Res: 20MHz operation is the primary mode of operation and 40MHz is optional. So the text has to reflect this. |
| 2853 | 155 | 6 | 11.15.3 | an additional line would be useful for 20MHz BSS and a 40/20MHz capable STA | Make change indicated in comment | Reject. In this case STA will associate with 20 MHz BSS as a 20 MHz STA. |
| 7773 | 155 | 6 | 11.15.3 | From table n59, it can be seen that, a 40 Mhz non-PCO STA can transmit and receive 40 Mhz frames during both 20 Mhz and 40 MHz PCO phases. Where as a 40 Mhz PCO STA can trasnmit and receive 40 Mhz frames during 40 Mhz PCO phase only. For a 40/20 MHz STA that can effectively do CCA on both primary and extension channel what is the additional benefit of PCO? | Justify PCO operation for a STA. For a 40/20 MHz STA that can effectively do CCA on both primary and extension channel what is the additional benefit of PCO? | Withdrawn. Commenter withdraws comment |
| 10600 | 155 | 6 | 11.15.3 | an additional line would be useful for 20MHz BSS and a 40/20MHz capable STA | as in comment | Reject. In this case STA will associate with 20 MHz BSS as a 20 MHz STA. |
| 9892 | 155 |  | 11.15.3 | 20/40 MHz capable STAs should be able to receive non-HT duplicate frames in both, control and extension channel. | Change the intersection of Rows 2,5, and 6 with Column 6 to "cntrl and extension chann". | Reject. This Table has been removed. 20/40 MHz capable STA are required to receive non-HT duplicate frames in the primary channel, but may combine duplicate transmissions on the primary and secondary channels. |

The following CIDs need discussion in the Adhoc:

| 4649 | 152 | 15 | 11.15.1 | Table n57 refers to specific band allocations. How are the allocations for Japan and those assigned to the WAVE technology (aka TGp) being dealt with. Comment also applies to Table n58 | Clarification of how all band allocations will be dealt with is required. | Needs discussion. Should we add a comment to specify that the use of these channels is subject to regulatory? Or just point to regulatory documents? |
| --- | --- | --- | --- | --- | --- | --- |

CIDs related to Protection Mechanisms:

| 10296 | 151 |  | 11.15.1 | The operation of an IBSS has not been described. | Is the additional HT information elements element present in beacons transmitted in an IBSS? If so, what is the requirement for an IBSS STA to set the various protection fields? Can it use the recommended transmission width field, and if so, does it replace use of the action frame for notifying its neighbors? What are the IBSS STA rules for starting an managing channel widths? | Defer. Is it addressed by Srini or Matthew submission on 9.13? |
| --- | --- | --- | --- | --- | --- | --- |

CIDs addressed by submission 1413:

| 12117 | 152 | 15 | 11.15.1 | Table n57 does not address devices on overlapping channels such as the extension channel. | Prior and in-service scanning must be mandatory. Testable performance requirements for this scanning must be provided. | Counter. Scanning for selection of extension channel is useful, but not mandatory. Submission 1413. |
| --- | --- | --- | --- | --- | --- | --- |

- CIDs related to 40 MHz operation in 2.4 GHz band:

| 1728 | 151 |  | 11.15.1 | 40MHz operation in 2.4GHz is not appropriate considering that there are so many devices only capable of DSSS/CCK are operating. | One idea is to separate the control channel and extension channel by 25MHz in 2.4GHz band. | Defer. This is addressed by separate submission from van Nee. |
| --- | --- | --- | --- | --- | --- | --- |
| 1561 | 152 | 3 | 11.15.2 | Table n58 discusses other overlapping BSSs that sit in the extension channel in 2.4G band; yet channelization in 2.4G band is conventionally on 25 MHz centers. The entire issue of operation with respect to (partially) overlapping BSSs that are centered 25 MHz away from the control channel should be clarified in the spec. e.g. should there be an adjacent channel interference spec specifically for an interferer 25 MHz away? | Clarify the 40/20 mode operation for the 2.4G band with respect to (partially) overlapping BSSs that may be centered 25 MHz away from the control channel. | Duplicate of 1728 |
| 286 | 152 | 15 | 11.15.1 | The forth row in tablen57 does not allow the use of DSSS/CCK MAC protection frame since 40MHz transmission of two DSSS/CCK waveform overlaps one another. Not supporting any MAC protection frame during the 40MHz transmission causes interoperability issue with the legacy 11.b devices and therefore needs to be addressed in the draft in order to fully comply with the PAR. With proper modification of the spec a possible solution whereby the duplicated DSSS/CCK MAC protection frames can be transmitted in 25MHz channel separation which is in fact the "most deployed" channel setting for the 2.4GHz band. | Change the spec for 2.4GHz band in order to allow the duplicated DSSS/CCK MAC protection frame to be transmitted in 25MHz separation between control and extension channels. | Defer. Requires submission on duplicate DSSS/CCK for 2.4 GHz |
| 430 | 152 | 15 | 11.15.1 | The fourth row in table 57 does not allow the use of DSSS/CCK MAC protection frame since 40MHz transmission of two DSSS/CCK waveform overlaps one another. Not supporting any MAC protection frame during the 40MHz transmission causes interoperability issue with the legacy 11.b devices and therefore needs to be addressed in the draft in order to fully comply with the PAR. With proper modification of the spec a possible solution whereby the duplicated DSSS/CCK MAC protection frames can be transmitted in 25MHz channel separation which is in fact the "most deployed" channel setting for the 2.4GHz band. | Change the spec for 2.4GHz band in order to allow the duplicated DSSS/CCK MAC protection frame to be transmitted in 25MHz separation between control and extension channels. | Duplicate of 286 |
| 689 | 152 | 15 | 11.15.1 | In the 2.4GHz band the channel spacing between the primary and extension channel for 40MHz operation is set to 20MHz which is not consistent with the usual 25MHz spacing (channel 1/6/11). This channel spacing prevents preamble based CCA due to the 5MHz offset mismatch. | Use a 25MHz spacing in the 2.4GHz band between control and extension channel. | Duplicate of 1728 |
| 705 | 152 | 15 | 11.15.1 | Table n57, row 4 does not allow the use of DSSS/CCK MAC protection frame since 40MHz transmission of two DSSS/CCK waveform overlaps one another. Not supporting any MAC protection frame during the 40MHz transmission causes interoperability issue with the legacy 11.b devices. | Change the spec for 2.4GHz band in order to allow the duplicated DSSS/CCK MAC protection frame to be transmitted in 25MHz separation between control and extension channels. | Duplicate of 286 |
| 2848 | 152 | 15 | 11.15.1 | table 57 row 4 calls for DSSS/CCK beacons in 40 MHz mode for 2.4GHz band. But DSSS transmission frequency specification not compatible with 40MHz. Single transmission not wide enough to cover 40 MHz, double transmission waveform overlaping with each other. | Specify how to transmit DSSS/CCK in 40 MHz for 2.4G band, or redefine 40 MHz operation using 25 MHz channel spacing for 2.4 GHz band. | Duplicate of 286 and 1728 |
| 3602 | 152 | 15 | 11.15.1 | The forth row in tablen57 does not allow the use of DSSS/CCK MAC protection frame since 40MHz transmission of two DSSS/CCK waveform overlaps one another. Not supporting any MAC protection frame during the 40MHz transmission causes interoperability issue with the legacy 11.b devices and therefore needs to be addressed | Change the spec for 2.4GHz band in order to allow the duplicated DSSS/CCK MAC protection frame to be transmitted in 25MHz separation between control and extension channels. Also, allow legacy part of the MM preamble (up to and including HT-SIG) to be transmitted in 25MHz separation. | Duplicate of 286 and 1728 |
| 4570 | 152 | 15 | 11.15.1 | Row 4 of Table n 57 does not permit DSS/CCK protection (as is allowed in rows 5 and 6). Not supporting a MAC protection frame during 40MHz transmission will cause interoperability issues with 11b. | Allow the duplicated DSSS/CCK MAC protection frame to be transmitted in 25MHz separation between control and extension channels in 2.4GHz. | Duplicate of CID 286 |
| 7195 | 152 | 15 | 11.15.1 | The forth row in tablen57 does not allow the use of DSSS/CCK MAC protection frame since 40MHz transmission of two DSSS/CCK waveform overlaps one another. Not supporting any MAC protection frame during the 40MHz transmission causes interoperability issue with the legacy 11.b devices and therefore needs to be addressed in the draft in order to fully comply with the PAR. With proper modification of the spec a possible solution whereby the duplicated DSSS/CCK MAC protection frames can be transmitted in 25MHz channel separation which is in fact the "most deployed" channel setting for the 2.4GHz band. Also, a possible solution for duplicated non-HT OFDM frame is to allow for legacy part of the MM preamble (up to and including HT-SIG) to be transmitted in 25MHz separation while operating in 2.4GHz band. | Change the spec for 2.4GHz band in order to allow the duplicated DSSS/CCK MAC protection frame to be transmitted in 25MHz separation between control and extension channels. Also, allow legacy part of the MM preamble (up to and including HT-SIG) to be transmitted in 25MHz separation. | Duplicate of 286 and 1728 |
| 7871 | 152 | 15 | 11.15.1 | MAC protection mechanizm does not work efficiently because most of legacy devices use ch1, ch6 and ch11 (25 MHz separation) to minimize the interference each other. | To increase the efficiency of the MAC protection mechanizm, change the spec from 20 MHz to 25 MHz for channel separation between control and extension channels. | Duplicate of 286 and 1728 |
| 9886 | 152 |  | 11.15.1 | Table n57: using 40 MHz in 2.4 GHz is going to downgrade performance of legacy BSSs. 40 MHz should be moved from 2.4 GHz. | Remove rows 5 and 6 from Table n57. | Defer pending resolution of 286 and 1728 |
| 423 | 153 | 3 | 11.15.2 | I disagree with the use of 40MHz channels in the 2.4 space, given the backward compatibility issues it creates | 40MHz operation should be restricted to the 5 GHz band. | Defer pending resolution of 286 and 1728 |
| 7182 | 154 |  | 11.15.2 | Calls out 40MHz Channel operation in the 2.4GHz band. This interfers with legacy 20MHz .11b/g channels. Its not an efficient use of the 2.4GHz spectrum. | Restrict 20MHz channel operation in the 2.4GHz band and allow 20/40MHz operating channels in the 5GHz band. | Duplicate of 423 |
| 9888 | 154 |  | 11.15.2 | using 40 MHz in 2.4 GHz is going to downgrade performance of legacy BSSs. 40 MHz should be moved from 2.4 GHz. | Remove 40/20 MHz and PCO operation for 2.4 GHz. | Duplicate of 423 |
| 12119 | 153 | 3 | 11.15.2 | This table does not acknowledge that at 2.4 GHz there may be overlapping BSSs in channels other than the extension channel, particularly a channel 25 MHz offset | Identify all OBSS cases, or disallow 40MHz operation at 2.4 GHz | Duplicate of 423 |
| 12250 | 153 | 3 | 11.15.2 | This table does not acknowledge that at 2.4 GHz there may be overlapping BSSs in channels other than the extension channel, particularly a channel 25 MHz offset | 40MHz operation should be limited to the 5 GHz band. | Duplicate of 423 |
| 3373 | 153 | 3 | 11.15.2 | Due to the limited number of non-overlapping channels in the 2.4 GHz band (ie 3 channels) I am concernded about the potential interference caused to legacy 802.11b/g devices operating in either the expanded channel or in the adjacent channel to a HT STA operating in 40 MHz mode. | Restrict 40 MHz operation to the 5 GHz channel only. | Duplicate of 423 |