IEEE P802.11
Wireless LANs

| Antenna selection feedback and joint TX/RX antenna selection |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-10-18 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Hongyuan Zhang | Marvell | 5488 Marvell Ln Santa Clara, CA 95054 | 408 222 1837 | hongyuan@marvell.com |
| Carlos Aldana Joonsuk Kim | Broadcom | 190 Mathilda Place, Sunnyvale, CA 94086 | 408 543 3370 | caldana@broadcom.com |
| Andy Molisch Zhifeng (Jeff) Tao Quan Zhou | Mitsubishi | 201 Broadway Cambridge, MA 02139 | 6176217558 | molisch@merl.com tao@merl.com |

Abstract

This document contains 802.11n D1.05 submission to solve the following comments: CID 2348 and 5119

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

## The Comment

| CID | Clause (Ed) | Comment |
| --- | --- | --- |
| 2348 | General | AS Indices Feedback Bit Ordering is ambiguous when arbitrary TX AS training sequence (overlapping antenna subsets) is applied |

## Solution (Informative)

(1) In the case that the receiver feeds back the indices of the selected antennas, the receiver has to perform the selection of the best antennas. In that case, the receiver has to know which antenna subsets were excited by the transmitter.

(2) If the transmitter uses disjoint subsets during the transmission, the receiver can easily determine and feed back the indices of the optimum antennas. However, if the transmitter uses overlapping subsets, the receiver does not know which antennas were used multiple times, and thus cannot determine the optimum antennas.

(3) If the transmitter uses overlapping subsets during the transmission, the receiver can feedback explicit CSI so that the transmitter can determine the optimum antennas.

(4) For AS sounding, the spatial mapping matrix has to be known at RX, and has to stay fixed for all packets at TX; it therefore should be specified in the standard.

## Proposal

TGn Editor:  Replace Table n5 with the following table

| - The ASEL Command and ASEL Data parts of the ASELC subfieldinput.doc |  |  |  |  |
| --- | --- | --- | --- | --- |
| ASEL Command (B9 to B11) | Interpretation of ASEL Command |  | ASEL Data (B12 to B15) |  |
| 000 | Transmit Antenna Selection Sounding Indication (TXASSI) without preference for the type of feedback |  | Number of remaining sounding PPDUs to be transmitted 0 to 15 |  |
| 001 | Transmit Antenna Selection Sounding Request (TXASSR) |  | Reserved |  |
| 010 | Receive Antenna Selection Sounding Indication (RXASSI) |  | Number of remaining sounding PPDUs to be received 0 to 15 |  |
| 011 | Receive Antenna Selection Sounding Request (RXASSR) |  | Total number sounding PPDUs required 0 to 15 |  |
| 100 | Sounding Label |  | Sequence number of the sounding PPDU corresponding to a CSI Matrices frame in ASEL feedback 0 to 15 |  |
| 101 |  | No feedback, ASEL training failure |  | Reserved |
| 110 |  | Transmit Antenna Selection Sounding Indication (TXASSI) requesting feedback of explicit CSI |  | Number of remaining sounding PPDUs to be transmitted 0 to 15 |
| 111 | Reserved |  | Reserved |  |

TGn Editor:  Replace item b) in Sec. 9.18.2 by the following text

The transmitter sends out consecutive sounding PPDUs separated by SIFS in a TXOP using burst transmission with no ACK over different antenna sets, with setting TX ASEL sounding indication, and sounding frame format. A single sounding PPDU is transmitted over one antenna set. If the transmitter allows antenna indices feedback (by setting the ASEL command field to 000), then the antenna sets from which the sounding PPDUs are transmitted shall be disjoint. If the transmitter uses NDP sounding PPDUs for the AS sounding, then the spatial mapping matrix shall be set equal to the identity matrix starting with the +HTC frame. If the transmitter uses non-NDP sounding PPDUs for the AS sounding, then the spatial mapping matrix shall be an FFT matrix.

NOTE— For example, in the case of sounding over all disjointed antenna sets, the number of consecutive

sounding PPDUs equals the smallest integer that is greater than or equal to the number of antennas divided by the number of RF chains. These sounding PPDUs should be sent within a single TXOP.

TGn Editor:  Replace the first paragraph of  item d) in Sec. 9.18.2 by the following text

If the ASEL Command field in the sounding frames is set to 110, then after receiving all the sounding PPDUs, the receiver shall explicitly feedback the full size channel state   information.  If the ASEL Command field in the sounding frames is set to 000, then after receiving all the sounding PPDUs, the receiver may either explicitly feedback the full size channel state   information, or conduct antenna selection computation and feedback the selected antenna indices in a subsequent TXOP. The feedback format is defined in 7.4.8.11 (Antenna Selection Indices Feedback).

## The Comment

| CID | Clause (Ed) | Comment |
| --- | --- | --- |
| CID 5119 | General | Current spec does not define joint TX/RX AS training when both devices have AS capability. |

## Solution (Informative)

(1) Performing antenna selection at one link end only (fixing the antennas at the other link end) does not exploit the diversity available when both stations can do AS.

(2) Even AS that first selects antennas at one link end and then at the other cannot fully explore the gains achieved by joint TX/RX AS: after one side antenna switching, the previous AS result on the other side may not be optimal, because the antennas (and therefore the “effective” channel) has changed. However, the gains of joint AS compared to subsequent AS at the link ends are minor: under typical circumstances, only 1dB gain can be achieved. On the other hand, joint AS requires significantly higher overhead.

(3) For this reason, joint AS should not be enabled by the standard, but rather alternating selection at TX and RX should be used if both TX and RX have AS capability

Proposal

Add the following sentence at the end of Sec. 9.18.2

When both transmitter and receiver have antenna selection capabilities, the peer station shall use fixed antenna subset to receive or transmit the consecutive sounding PPDUs in the training processes defined in Figures n46 and n47; and TX and RX antenna selection trainings shall be done alternatingly.