IEEE P802.11
Wireless LANs

| Key distribution – resolution of 23 “general” comments |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-10-25 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Bill Marshall | TGr Editor | 180 Park Ave, Florham Park, NJ 07932 | 973-360-8718 | wtm@research.att.com |

Abstract

This document contains the text changes needed to implement the proposed resolution to twenty three of the LB87 comments categorized as “issue #33: Key Distribution – Go back to D2.2 scheme”, specifically #865, 946, 947, 952, 954, 970, 977, 979, 980, 981, 983, 998, 1011, 1035, 1043, 1085, 1120, 1122, 1339, 1471, 1491, 2031, and 2032.

Many other comments are resolved as side-affects of these; a total of 330 LB87 comments would be resolved by acceptance of this submission.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Make the following changes to D3.0:

7.2.3.1:

Delete row 31 of Table 8, NAS-ID.

7.2.3.9:

Delete row 30 of Table 15, NAS-ID.

7.2.3.10:

Delete rows 10 and 11 of Table 16, R1 Distribution Token, and R1 Distribution Valid.

Change heading of column 4 of Table 17 from “4-11” to “4-9”

Delete mention of R1 distribution token and R1 distribution valid from Table 17.

7.3.2:

Delete Element ID 58 and 59 from Table 26, R1 Distribution Token and R1 Distribution Valid.

Change “Reserved” row from “60-126” to “58-126”

7.3.2.49:

Delete this clause.

7.3.2.50:

Delete this clause.

7.3.2.51:

Delete this clause.

7.4.7.1:

Delete row 4 of Table 57C, R1DISTIE.

7.4.7.2:

Delete rows 4 and 5 of Table 57D, R1DISTIE and R1DISTVIE.

8.5A.1:

Change fifth paragraph as follows:

The FT key hierarchy derives its keys using a Key Derivation Function (KDF) as defined in 8.5A.3 with separate labels to further distinguish derivations. Each level of the hierarchy derives keys for the level below it. The distribution of keys from the PMK-R0 key holder to the PMK-R1 key holders is outside the scope of this specification. However, to ensure security, this key hierarchy requires that keys only be distributed down the key hierarchy, and by the use of a secure transport protocol.

8.5A.4:

Delete key derivation for “mk”.

Delete dash list items for KDF-256, mk, and “Distribution of PMK-R1”.

8.5A.6:

Change this clause as follows:

8.5A.6 PMK-R1 distribution within a mobility domain

The following procedures are used to distribute PMK-R1s.

The STA first derives PMK-R0 keys for use in fast BSS transitions (via the FT Initial Association procedures defined in clause 8A.2) utilizing information from the target AP as defined in 8.5A.4. When the IEEE 802.1X AKM is used to establish keys, the PMK-R0 Key Holder derives the PMK-R0 key utilizing the MSK acquired through EAP based authentication.

The PMK-R0 shall be stored in a component called the PMK-R0 Key Holder. Each PMK-R0 Key Holder is responsible for deriving a PMK-R1 key for key for each PMK-R1 Key Holder in the mobility domain. The PMK-R1 shall be stored in a component called the PMK-R1 Key Holder. PMK-R1 keys are distributed to PMK-R1 Key Holders by the PMK-R0 Key Holder using a secure three party protocol described herein.

The Mobility Domain Controller (MDC) shall mutually authenticate and authorize all members of the mobility domain that can be Key Holders. Using the MDC all Key Holders can establish security associations with each other. The identities they use during the authentication process shall be their NAS-Id which is identical to the information included in the NASID IE in all beacons and probe responses.

The Key Distribution Protocol has the following properties

- Authentication of entities involved in key distribution

- Confidentiality of the key distribution

- Authorization of the status of a PMK-R1 holder

- Validation of the authorization

- Receipt of the key is acknowledged

- Correctness of the authorization attributes assigned to the STA

The Key Distribution Protocol is initiated by the STA by including an R1DIST Information element along with other Fast BSS Transition Information Elements in either Authentication or Action frames. The token of this element shall be the AES-KeyWrap of a sixteen (16) octet random number chosen by the STA, called Ns, concatenated with the identity of the target AP as determined by the NASID Information Element in its beacons and probe responses. The key used to perform this wrapping shall be "mk" described in section 8.5A.4.

The target AP shall extract the token from the R1DIST Information Element and send it along with the MAC address of the STA to the R0KH identified in the FTIE under the protection of the security association the target AP (a Key Holder) shares with the PMK-R0 Key Holder. If the target AP does not recognize the R0KH in the FTIE or does not have a security association with it authentication fails with status 38 ("the request has not been successful as one or more parameters have invalid values").

Upon receipt of this message the R0KH looks up the PMK-R0 security association it shares with the STA based upon the MAC address in the message. It then unwraps the token using "mk" describe in section 8.5A.4. If the unwrapping fails or the R0KH does not have a PMK-R0 security association for that STA it returns a failure message to the target AP which then fails the authentication attempt with status 38. The R0KH compares the NAS-Id from the unwrapped token with the identity the target AP used during when establishing its shared security association. If the two do not match the R0KH returns a failure message to the target AP which then fails the authentication attempt with status 38. The R0KH then chooses a sixteen (16) octet random number, called Na, and constructs a message back to the target AP consisting of the STA's MAC address, Ns, Na, the PMK-R1, the PMK-R1 Name, all authorization attributes associated with PMK-R0 (e.g. session lifetime), and a token representing an AES-KeyWrapping of the NAS-Id of the target AP from the shared security association and Na. The wrapping shall use "mk" described in section 8.5A.4.

Upon receipt of this message the target AP shall instantiate a PMK-R1 security association and construct either an Authentication response frame or response action frame depending on the type of request received originally. This frame will include, along with other Fast BSS Transition IEs, an RT1DIST IE which shall encapsulate the token it received from the PMK-R0 Key Holder, and an R1DISTV IE which shall consist of the digest of SHA-256(Ns|Na).

Upon receipt of this frame the STA shall perform the following additional validation: it shall unwrap the token in the R1DIST IE and compare the enclosed NAS-Id with the one it received from the target AP in a beacon or probe response. If they do not match the STA must treat this as an authentication failure. If they match the STA computes the digest of SHA-256(Ns|Na) using Na from the unwrapped token and compares it to the value in the R1DISTV IE. If they do not match the STA must treat this as an authentication failure. If they match the STA instantiates its own version of a PMK-R1 security association to share with the target AP and continues with Fast BSS Transition.

8.5A.6 Fast BSS Transition key holders

8.5A.6.1 Key Holder requirements

The R0 and R1 key holders are responsible for the derivation of keys in the FT key hierarchy. For Fast BSS Transition, the functions of the IEEE 802.1X Authenticator are distributed among the R0KH, and R1KH. Each key holder shall have a name that is communicated to the STA and other key holders which is bound into the key derivation. Each key holder name shall be mapped to a physical entity in the DS where it resides.

The R0KeyHolder shall meet the following requirements:

— The R0KeyHolder shall be co-resident with the NAS Client functionality of the IEEE 802.1X Authenticator.

— The R0KeyHolder identifier (R0KH-ID) shall be set to the NAS ID as defined in RFC 2865.

— When the PMK-R0 lifetime expires, the R0KeyHolder shall delete the PMK-R0 SA and should revoke all PMK-R1s derived from the PMK-R0.

The R1KeyHolder shall meet the following requirements:

— The R1KeyHolder identity (R1KH-ID) shall be set to the MAC address of the physical entity that stores the PMK-R1 and uses it to generate the PTK. That same MAC address shall be used to advertise the PMK-R1 identity to the STA and the R0KeyHolder.

— The R1KeyHolder shall provide the IEEE 802.1X Authenticator function to derive and distribute the GTK to all connected STA's.

— When the PMK-R1 lifetime expires, the R1KeyHolder shall delete the PMK-R1 SA and shall revoke all PTKs derived from the PMK-R1 using the MLME-DELETEKEYS primitive.

8.5A.6.2 PMK-R1 distribution within a Mobility Domain Assumptions

The STA first derives PMK-R0 keys for use in fast BSS transitions (via the FT Initial Mobility Domain Association procedures defined in 8A.2) utilizing information from the target AP as defined in 8.5A.4. When the AKM negotiated is 00-0F-AC:3, the PMK-R0 Key Holder derives the PMK-R0 key utilizing the MSK acquired through IEEE 802.1X authentication; when the AKM negotiated is 00-0F-AC:4, the PMKR0 key is derived using the PSK.

Each PMK-R0 Key Holder is responsible for deriving a PMK-R1 key for each PMK-R1 Key Holder within the Mobility Domain. The PMK-R0 Key Holder is responsible for transmitting the derived PMK-R1 keys securely to those key holders, along with the PMKR1Name value and KeyLifetime associated with that PMK-R1.

Each PMK-R1 Key Holder shall derive the PTK mutually with the STA.

The PMK-R0 shall be stored in a component called the PMK-R0 Key Holder. The PMK-R1 shall be stored in a component called the PMK-R1 Key Holder. The PMK-R1 can be pushed from the PMK-R0 Key Holder to the PMK-R1 Key Holder, or pulled by the PMK-R1 Key Holder from the PMK-R0 Key Holder.

The PMK-R0 Key Holder and the PMK-R1 Key Holder have a trustworthy channel between them that can be used to exchange cryptographic keys without exposure to any intermediate parties. This standard assumes that the key transfer includes the PMK-R1, the PMK-R1 context and the associated key authorizations.

Each Key Holder is assumed to be addressable as a distinct entity. Each Key Holder is assumed to be expressed as a unique identifier within the Mobility Domain. The identities of the Key Holders are assumed to be the same as those advertised to the STA.

8A.1.1:

Delete “The NAS-Id of the authenticator associated with the AP advertising Fast BSS Transition capability shall be the value of dot11FTNASID.”

8A.3.1 and 8A.3.2:

Delete R1DIST and R1DISTV from Figures and from message exchanges, overriding changes in 11-06-1611.

Change “If the STA has no record of authorizing the target AP to be a PMK-R1 holder it shall include an R1DIST IE in its Authentication-Request (see clause 8.5A.6). If an R1DIST IE was present in the Authentication-Request than an R1DIST IE and an R1DISTV IE must be in the accompanying Authentication-Response (and must be correctly processed according to clause 8.5A.6) or authentication shall be deemed to have failed.”

To “If the target AP does not have the key identified by PMKR1Name, it may retrieve that key from the R0KH identified by the STA. See clause 8.5A.6.”

8A.4.1 and 8A.4.2:

Delete R1DIST and R1DISTV from Figures, overriding changes in 11-06-1611.

8A.5:

Change “In an RSN the first two messages in the sequence, ‘Fast BSS Transition Request’ and ‘Fast BSS Transition Response,’ are used to authorize the target AP to become a valid PMK-R1 holder and to allow the STA and target AP to provide their random contributions, SNonce and ANonce, respectively.”

To “In an RSN the first two messages in the sequence, ‘Fast BSS Transition Request’ and ‘Fast BSS Transition Response,’ are used to allow the STA and target AP to provide their random contributions, SNonce and ANonce, respectively.”

8A.5:

Delete rows “R1 Distribution” and “R1 Distribution Valid” from Table 64A.

8A.5.1:

Change “If the STA has no record of previously authorizing the NAS identified by the target AP’s NAS-Id it includes an R1DIST IE. The target AP uses the R0KH-ID, the STA MAC, and the token conveyed in the R1DIST IE to obtain the appropriate PMK-R1.”

To “The target AP can use the PMKR0Name to derive the PMKR1Name, and if the target AP does not have the key identified by PMKR1Name, it may attempt to retrieve that key from the R0KH identified by R0KH-ID. See 8.5A.6.”

8A.5.1:

Delete paragraph starting “The R1DIST IE shall be present…”

8A.5.2:

Delete paragraph starting “The R1DIST IE shall be present…” and paragraph starting “The R1DISTV IE shall be present…”

Annex D:

Delete dot11FastBSSTransitionConfigEntry dot11FTNASId.

Delete dot11FTNASID.

Resolve LB87 comments as follows:

| 14 | Counter. NAS-ID deleted. |
| --- | --- |
| 72 | Counter. NAS-ID deleted. |
| 73 | Counter. NAS-ID deleted. |
| 88 | Counter. NAS-ID deleted. |
| 92 | Accepted |
| 102 | Accepted |
| 106 | Counter. NAS-ID deleted. |
| 107 | Counter. NAS-ID deleted. |
| 289 | Counter. NAS-ID deleted. |
| 292 | Counter. NAS-ID deleted. |
| 293 | Counter. NAS-ID deleted. |
| 294 | Counter. NAS-ID deleted. |
| 295 | Counter. NAS-ID deleted. |
| 296 | Accepted |
| 297 | Counter. NAS-ID deleted. |
| 298 | Counter. NAS-ID deleted. |
| 299 | Counter. NAS-ID deleted. |
| 300 | Accepted |
| 381 | Counter. NAS-ID deleted. |
| 382 | Accepted |
| 383 | Counter. NAS-ID deleted. |
| 385 | Accepted |
| 394 | Counter. Row deleted from table |
| 395 | Counter. Row deleted from table |
| 396 | Counter. Row deleted from table |
| 397 | Counter. Row deleted from table |
| 398 | Counter. Row deleted from table |
| 403 | Counter. R1 distribution token deleted |
| 404 | Counter. R1 distribution token deleted |
| 405 | Counter. R1 distribution token deleted |
| 406 | Counter. R1 distribution token deleted |
| 407 | Counter. R1 distribution token deleted |
| 408 | Counter. R1 distribution token deleted |
| 409 | Counter. R1 distribution token deleted |
| 410 | Counter. R1 distribution token deleted |
| 411 | Counter. R1 distribution token deleted |
| 412 | Counter. R1 distribution token deleted |
| 413 | Counter. R1 distribution token deleted |
| 414 | Counter. R1 distribution token deleted |
| 415 | Counter. R1 distribution token deleted |
| 416 | Counter. R1 distribution token and R1 distribution valid deleted |
| 417 | Counter. R1 distribution token and R1 distribution valid deleted |
| 418 | Counter. R1 distribution token and R1 distribution valid deleted |
| 419 | Counter. R1 distribution token and R1 distribution valid deleted |
| 420 | Counter. R1 distribution token and R1 distribution valid deleted |
| 421 | Counter. R1 distribution token and R1 distribution valid deleted |
| 422 | Counter. R1 distribution token and R1 distribution valid deleted |
| 423 | Counter. R1 distribution token and R1 distribution valid deleted |
| 424 | Counter. R1 distribution token and R1 distribution valid deleted |
| 427 | Counter. Clause 8.5A.6 deleted |
| 429 | Counter. Clause 8.5A.6 deleted |
| 433 | Counter. R1 distribution token and R1 distribution valid deleted |
| 436 | Counter. R1 distribution token and R1 distribution valid deleted |
| 441 | Counter. NAS-ID deleted. |
| 442 | Counter. NAS-ID deleted. |
| 443 | Counter. NAS-ID deleted. |
| 485 | Counter. Clause 8.5A.6 deleted. |
| 487 | Counter. Clause 8.5A.6 deleted. |
| 545 | Counter. NAS-ID deleted. |
| 551 | Accepted. Text added to 8.5A.6.1. |
| 552 | Counter. NAS-ID deleted. |
| 604 | Accepted |
| 605 | Counter. NAS-ID deleted. |
| 606 | Counter. Clause 7.3.2.49 deleted. |
| 607 | Accepted |
| 608 | Counter. NAS-ID deleted. |
| 610 | Counter. NAS-ID deleted. |
| 611 | Accepted. Clause deleted. |
| 612 | Accepted |
| 613 | Counter. Clause 7.3.2.50 deleted. |
| 615 | Counter. Clause 7.3.2.50 deleted. |
| 616 | Counter. Clause 7.3.2.50 deleted. |
| 617 | Counter. Clause 7.3.2.50 deleted. |
| 618 | Counter. Clause 7.3.2.50 deleted. |
| 620 | Accepted |
| 621 | Counter. Clause 7.3.2.50 deleted. |
| 622 | Accepted |
| 623 | Counter. Clause 7.3.2.51 deleted. |
| 624 | Counter. Clause 7.3.2.51 deleted. |
| 625 | Counter. Clause 7.3.2.51 deleted. |
| 626 | Counter. Clause 7.3.2.51 deleted. |
| 627 | Counter. Clause 7.3.2.51 deleted. |
| 628 | Counter. Clause 7.3.2.51 deleted. |
| 629 | Accepted |
| 630 | Counter. Clause 7.3.2.51 deleted |
| 631 | Counter. Clause 7.3.2.51 deleted. |
| 632 | Counter. Clause 7.3.2.51 deleted |
| 635 | Counter. Clause 7.3.2.51 deleted |
| 636 | Counter. Clause 7.3.2.51 deleted |
| 637 | Counter. Clause 7.3.2.51 deleted |
| 650 | Counter. R1DISTIE deleted |
| 651 | Counter. R1DISTIE deleted |
| 652 | Counter. R1DISTIE deleted |
| 653 | Counter. R1DISTIE deleted |
| 654 | Counter. R1DISTIE deleted |
| 655 | Counter. R1DISTIE deleted |
| 656 | Counter. R1DISTIE deleted |
| 657 | Counter. R1DISTIE deleted |
| 658 | Counter. R1DISTIE deleted |
| 663 | Counter. R1DISTIE deleted |
| 664 | Counter. R1DISTIE deleted |
| 665 | Counter. R1DISTIE deleted |
| 666 | Counter. R1DISTIE deleted |
| 667 | Counter. R1DISTIE deleted |
| 668 | Counter. R1DISTIE deleted |
| 669 | Counter. R1DISTIE deleted |
| 670 | Counter. R1DISTIE deleted |
| 671 | Counter. R1DISTIE deleted |
| 672 | Counter. R1DISTIE deleted |
| 673 | Counter. R1DISTVIE deleted |
| 674 | Counter. R1DISTVIE deleted |
| 675 | Counter. R1DISTVIE deleted |
| 676 | Counter. R1DISTVIE deleted |
| 784 | Accepted |
| 785 | Accepted |
| 789 | Counter. Clause 8.5A.6 restored from D2.2 |
| 820 | Accepted |
| 855 | Counter. Mk deleted. |
| 865 | Accepted |
| 866 | Accepted |
| 868 | Counter. Mk deleted. |
| 869 | Counter. Mk deleted. |
| 872 | Counter. Mk deleted. |
| 873 | Counter. Clause 8.5A.6 deleted |
| 874 | Counter. Mk deleted. |
| 876 | Counter. Mk deleted. |
| 887 | Counter. Mk deleted. |
| 888 | Counter. Mk deleted. |
| 889 | Counter. Mk deleted. |
| 896 | Counter. Mk deleted |
| 890 | Accepted |
| 944 | Counter. Clause 8.5A.6 deleted |
| 946 | Accepted |
| 947 | Accepted |
| 948 | Accepted |
| 949 | Accepted |
| 950 | Accepted |
| 951 | Accepted |
| 952 | Accepted |
| 953 | Accepted |
| 954 | Accepted |
| 955 | Counter. Clause 8.5A.6 deleted |
| 956 | Counter. Clause 8.5A.6 deleted |
| 957 | Accepted |
| 958 | Counter. Clause 8.5A.6 deleted |
| 959 | Counter. Clause 8.5A.6 deleted |
| 966 | Accepted |
| 968 | Accepted |
| 969 | Accepted |
| 970 | Accepted |
| 972 | Accepted |
| 974 | Accepted. Statement added to 8.5A.1. |
| 975 | Counter. Clause 8.5A.6 deleted |
| 976 | Counter. Clause 8.5A.6 deleted |
| 977 | Accepted |
| 979 | Accepted |
| 980 | Accepted |
| 981 | Accepted |
| 982 | Accepted |
| 983 | Accepted |
| 984 | Accepted |
| 985 | Accepted |
| 986 | Counter. Clause 8.5A.6 deleted |
| 987 | Counter. Clause 8.5A.6 restored from D2.2 |
| 996 | Accepted. Statement added to 8.5A.1 |
| 998 | Accepted |
| 999 | Accepted |
| 1000 | Counter. Clause 8.5A.6 restored from D2.2 |
| 1001 | Counter. Clause 8.5A.6 deleted |
| 1002 | Counter. Clause 8.5A.6 restored from D2.2 |
| 1003 | Counter. Clause 8.5A.6 restored from D2.2 |
| 1004 | Counter. Clause 8.5A.6 deleted |
| 1005 | Counter. Clause 8.5A.6 deleted |
| 1006 | Counter. Clause 8.5A.6 restored from D2.2 |
| 1007 | Counter. Clause 8.5A.6 deleted |
| 1008 | Accepted. Clause 8.5A.6 deleted |
| 1009 | Counter. Clause 8.5A.6 deleted |
| 1011 | Accepted |
| 1012 | Accepted |
| 1035 | Accepted |
| 1036 | Counter. Clause 8.5A.6 deleted |
| 1037 | Accepted |
| 1038 | Accepted |
| 1039 | Counter. Clause 8.5A.6 deleted |
| 1040 | Counter. Clause 8.5A.6 deleted |
| 1041 | Counter. Clause 8.5A.6 deleted |
| 1042 | Counter. Clause 8.5A.6 deleted. Other uses are in IEEE mandated boilerplate. |
| 1043 | Accepted |
| 1044 | Accepted. Clause 8.5A.6 deleted. |
| 1045 | Counter. Clause 8.5A.6 deleted. |
| 1046 | Counter. Clause 8.5A.6 deleted. |
| 1049 | Accepted |
| 1050 | Counter. Clause 8.5A.6 deleted |
| 1051 | Accepted |
| 1052 | Accepted |
| 1053 | Counter. Clause 8.5A.6 restored from D2.2 |
| 1056 | Counter. Clause 8.5A.6 deleted. |
| 1057 | Accepted |
| 1058 | Counter. Clause 8.5A.6 deleted |
| 1059 | Accepted. Clause 8.5A.6 deleted |
| 1060 | Counter. Clause 8.5A.6 deleted |
| 1061 | Counter. Clause 8.5A.6 deleted |
| 1062 | Counter. Clause 8.5A.6 deleted |
| 1064 | Accepted |
| 1065 | Counter. Clause 8.5A.6 deleted |
| 1066 | Counter. Clause 8.5A.6 deleted |
| 1067 | Counter. Clause 8.5A.6 deleted |
| 1068 | Counter. Clause 8.5A.6 deleted |
| 1069 | Accepted |
| 1070 | Accepted |
| 1071 | Counter. Clause 8.5A.6 deleted |
| 1072 | Counter. Clause 8.5A.6 deleted |
| 1075 | Accepted |
| 1076 | Counter. Clause 8.5A.6 deleted |
| 1077 | Counter. Clause 8.5A.6 deleted |
| 1078 | Counter. Clause 8.5A.6 deleted |
| 1079 | Counter. Clause 8.5A.6 deleted |
| 1080 | Accepted |
| 1081 | Counter. NAS-ID deleted. |
| 1082 | Accepted |
| 1084 | Counter. NAS-ID deleted. |
| 1085 | Accepted |
| 1086 | Counter. Clause 8.5A.6 deleted |
| 1087 | Counter. Clause 8.5A.6 deleted |
| 1088 | Counter. Clause 8.5A.6 deleted |
| 1090 | Counter. Clause 8.5A.6 deleted |
| 1091 | Counter. Clause 8.5A.6 deleted |
| 1092 | Counter. Clause 8.5A.6 deleted |
| 1094 | Counter. Clause 8.5A.6 deleted |
| 1095 | Counter. Clause 8.5A.6 deleted |
| 1096 | Counter. Clause 8.5A.6 deleted |
| 1097 | Counter. Clause 8.5A.6 deleted |
| 1098 | Counter. Clause 8.5A.6 deleted |
| 1099 | Accepted |
| 1101 | Counter. Clause 8.5A.6 deleted |
| 1102 | Counter. Clause 8.5A.6 deleted |
| 1103 | Counter. Clause 8.5A.6 deleted |
| 1104 | Counter. Clause 8.5A.6 deleted |
| 1105 | Counter. Clause 8.5A.6 deleted |
| 1106 | Accepted |
| 1107 | Counter. Clause 8.5A.6 deleted |
| 1109 | Counter. Clause 8.5A.6 deleted |
| 1110 | Counter. Clause 8.5A.6 deleted |
| 1111 | Counter. Clause 8.5A.6 deleted |
| 1112 | Counter. Clause 8.5A.6 deleted |
| 1113 | Counter. Clause 8.5A.6 deleted |
| 1114 | Counter. Clause 8.5A.6 deleted |
| 1115 | Counter. Clause 8.5A.6 deleted |
| 1116 | Counter. Clause 8.5A.6 deleted |
| 1117 | Counter. Clause 8.5A.6 deleted |
| 1118 | Counter. Clause 8.5A.6 deleted |
| 1119 | Counter. Clause 8.5A.6 deleted |
| 1120 | Accepted |
| 1122 | Accepted |
| 1123 | Counter. Clause 8.5A.6 deleted |
| 1124 | Counter. Clause 8.5A.6 deleted |
| 1125 | Counter. Clause 8.5A.6 deleted |
| 1126 | Counter. Clause 8.5A.6 deleted |
| 1127 | Counter. Clause 8.5A.6 deleted |
| 1128 | Counter. Clause 8.5A.6 deleted |
| 1129 | Counter. Clause 8.5A.6 deleted |
| 1130 | Counter. Clause 8.5A.6 deleted |
| 1131 | Counter. Clause 8.5A.6 deleted |
| 1132 | Counter. Clause 8.5A.6 deleted |
| 1133 | Counter. Clause 8.5A.6 deleted |
| 1134 | Counter. Clause 8.5A.6 deleted |
| 1196 | Counter. Clause 8.5A.6 deleted |
| 1197 | Accepted |
| 1316 | Accepted. R0KH is the NAS-ID of the authenticator for initial association. Text restored to 8.5A.6.1. |
| 1339 | Accepted |
| 1342 | Counter. Indicated text deleted. |
| 1344 | Counter. Indicated text deleted. |
| 1345 | Counter. Indicated text deleted. |
| 1346 | Counter. Indicated text deleted. |
| 1347 | Counter. Indicated text deleted. |
| 1348 | Counter. Indicated text deleted. |
| 1349 | Counter. Indicated text deleted. |
| 1350 | Counter. Indicated text deleted. |
| 1351 | Counter. Indicated text deleted. |
| 1352 | Counter. Indicated text deleted. |
| 1353 | Accepted |
| 1354 | Accepted |
| 1355 | Counter. Indicated text deleted. |
| 1356 | Counter. Indicated text deleted. |
| 1364 | Counter. Clause 8.5A.6 deleted |
| 1369 | Counter. Indicated text deleted. |
| 1370 | Counter. Indicated text deleted. |
| 1371 | Counter. Indicated text deleted. |
| 1372 | Counter. Indicated text deleted. |
| 1373 | Counter. Indicated text deleted. |
| 1374 | Counter. Indicated text deleted. |
| 1375 | Counter. Indicated text deleted. |
| 1376 | Counter. Indicated text deleted. |
| 1377 | Counter. Indicated text deleted. |
| 1378 | Counter. Indicated text deleted. |
| 1379 | Accepted |
| 1380 | Accepted |
| 1381 | Counter. Indicated text deleted. |
| 1382 | Counter. Indicated text deleted. |
| 1453 | Accepted |
| 1454 | Counter. R1 distribution row deleted |
| 1456 | Counter. R1 distribution row deleted |
| 1459 | Counter. Clause 8.5A.6 deleted |
| 1460 | Counter. Indicated text deleted. |
| 1461 | Counter. Indicated text deleted. |
| 1462 | Counter. Indicated text deleted. |
| 1471 | Accepted |
| 1472 | Counter. Indicated text deleted. |
| 1474 | Counter. Clause 8.5A.6 deleted. |
| 1475 | Counter. Indicated text deleted. |
| 1491 | Accepted |
| 1492 | Counter. Indicated text deleted. |
| 1494 | Counter. Clause 8.5A.6 deleted |
| 1495 | Counter. Indicated text deleted. |
| 1496 | Counter. Indicated text deleted. |
| 1497 | Counter. Clause 8.5A.6 deleted. |
| 1559 | Counter. Clause 8.5A.6 deleted. |
| 1567 | Counter. Clause 8.5A.6 deleted. |
| 1572 | Counter. Clause 8.5A.6 deleted |
| 1580 | Counter. Clause 8.5A.6 deleted |
| 1657 | Counter. Clause 8.5A.6 deleted |
| 2031 | Accepted |
| 2032 | Accepted |
| 2034 | Counter. Information elements 58, 59, and NASID deleted |
| 2037 | Counter. Clause 8.5A.6 deleted |
| 2041 | Accepted |
| 2042 | Accepted |
| 2046 | Accepted. Text included in 8.5A.6.1 |
| 2049 | Accepted. Text included in 8.5A.6.1 |
| 2051 | Counter. NAS-ID deleted |
| 2052 | Accepted |
| 2053 | Accepted. NAS-ID deleted |