IEEE P802.11
Wireless LANs

| Key distribution with optional R1KH authentication |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-10-26 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Bill Marshall | TGr Editor | 180 Park Ave, Florham Park, NJ 07932 | 973-360-8718 | wtm@research.att.com |

Abstract

Several comments in LB87 requested that the 3-party-protocol in D3.0 for PMK-R1 key distribution be made optional.

This submission specifies the exact changes being requested to D3.0, and the resolution text for the comments that requested this.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

This submission assumes that neither 11-06-1612 nor 11-06-1613 were accepted, but that 11-06-1596 was accepted. It is based on D3.0 as modified by 11-06-1596.

A “push” capability is added to the D3.0 three-party-protocol by making the cryptographic authentication of the R1KH optional.  Thus, the R0KH is allowed to send the PMK-R1 to the R1KH based on administrative configuration, without explicit authorization from a STA.

The STA can request authentication of the R1KH through use of the R1DIST optional parameter in the FTIE. If the R1DIST is present in the FT Request, the R1KH shall contact the R0KH for the proper response and pass it on to the STA in the FT Response.

If the STA does not include such R1DIST optional parameter, then the R1KH may use the PMK-R1 obtained via an earlier exchange with the R0KH (either by a push or a pull).

Support for the R1KH authentication is indicated by a policy bit in the MDIE.

Changes to D3.0 (as modified by 11-06-1596):

7.3.2.45:

Change Figure 112T to define bit 4 as “R1KH Authentication”, leaving bits 5-7 as “reserved”

Add a new paragraph at end of 7.3.2.45:

R1KH-authentication is set to true if the AP supports the key distribution and authentication protocol described in 8.5A.7.

8.5A.7:

Insert near/at the end of the first paragraph “The PMK-R1 can be transferred from the PMK-R0 Key Holder to the PMK-R1 Key Holder at any time (a “push” operation), or transferred on request from the PMK-R1 Key Holder (a “pull” operation). However, if the STA requires key holder authentication of the R1KH, only the latter is possible.”

Change first phrase of second paragraph from “The Key distribution Protocol is intiated by the STA by…” to “Key holder authentication is an optional procedure that may be requested by a STA. Availability of this procedure is advertised by the AP in the FT resource and resource policy value of the MDIE, and shall not be requested of an AP that does not advertise its support. This procedure is initiated by…”

8A.1.1:

Add to second paragraph: “R1KH-authentication shall be set according to the value of the MIB variable dot11FTR1KHAuthenticationEnabled.”

Add at end of subclause

NOTE – It is assumed by this standard the the value of dot11FTR1KHAuthenticationEnabled will be administered consistently across the Mobility Domain.

8A.3.1 and 8A.3.2:

Change “If the STA has no record of authorizing the target AP…” to “If the STA is utilizing the key holder authentication procedure, and has no record of authorizing the target AP…”

Change “If an R1DIST was present in the Authentication-Request then…” to “If an R1DIST was present in the Authentication-Request and the AP supports R1KH-Authentication, then…”

8A.5.1:

Change “If the STA has no record of previously authorizing the R1KH…” to “If the STA is utilizing the key hold authentication procedure, and has no record of authorizing the R1KH…” (once in initial paragraph, and again in FTIE contents definition)

8A.5.2:

Change “If the STA included an R1DIST optional parameter in the fast BSS transition request, then…” to “If the STA included an R1DIST optional parameter in the fast BSS transition request, and the AP supports R1KH-Authentication, then…”

A:

Add a PICS entry: PC35.13 “R1KH Authentication procedures” referencing 8.5A.7, Status PC35:O, with support Y/N/N/A.

D:

Add to dot11FastBSSTransitionConfigEntry, dot11FTR1KHAuthenticationEnabled, TruthValue.

Add after dot11ReassociationDeadline,

dot11FTR1KHAuenticationEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“When this object is set to TRUE, this shall indicate that the R1KH Authentication protocol described in 8.5A.7 is enabled on this AP entity.”

Resolve LB87 comments as follows:

| 485 | Accepted. Text changes to MDIE definition in 11-06-XXXX. |
| --- | --- |
| 784 | Counter. Push model added to key distribution protocol |
| 785 | Counter. Push model added to key distribution protocol |
| 820 | Counter. Push model added to key distribution protocol |
| 873 | Accepted. Key authentication protocol optional |
| 949 | Counter. Push model added to key distribution protocol |
| 950 | Counter. Push model added to key distribution protocol |
| 951 | Counter. Push model added to key distribution protocol |
| 957 | Accepted. Push model added to key distribution protocol |
| 966 | Accepted. Text changes in 11-06-XXXX |
| 968 | Accepted. Text changes in 11-06-XXXX |
| 969 | Accepted. Text changes in 11-06-XXXX |
| 972 | Accepted. Text changes in 11-06-XXXX |
| 984 | Accepted. Text changes in 11-06-XXXX |
| 1001 | Accepted. Text changes in 11-06-XXXX |
| 1002 | Accepted. Text changes in 11-06-XXXX |
| 1003 | Accepted. Text changes in 11-06-XXXX |
| 1004 | Accepted. Text changes in 11-06-XXXX |
| 1005 | Accepted. Text changes in 11-06-XXXX |
| 1006 | Accepted. Text changes in 11-06-XXXX |
| 1007 | Accepted. Text changes in 11-06-XXXX |
| 1008 | Accepted. Text changes in 11-06-XXXX |
| 1009 | Accepted. Text changes in 11-06-XXXX |
| 1012 | Accepted. Text changes in 11-06-XXXX |
| 1572 | Counter. Push model added to key distribution protocol |
| 2037 | Accepted. Text changes in 11-06-XXXX |
| 2042 | Accepted. Text changes in 11-06-1596 and 11-06-XXXX |

#485 revised.

Option #3:

#551 revised