IEEE P802.11
Wireless LANs

| Proposal to change the text for SNR fields in Explicit Feedback Report |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-11-08 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Joonsuk Kim | Broadcom | 190 Mathilda place Sunnyvale, CA 94086 | 1-408-543-3455 | joonsuk@broadcom.com |
| Gal Basson | Intel | POB 1659, Haifa 31015, Israel | 972547885404 | gal.basson@intel.com |
| Assaf Kasher | Intel | POB 1659, Haifa 31015, Israel | +972-4-8651547 | assaf.kasher@intel.com |

Abstract

This document contains proposed changes to the IEEE P802.11n Draft to address the following LB84 comments:

- CID 984, CID 985

The changes marked in this document are based on TGn Draft version D1.05

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

## Introduction

This document provides explicit changes for a couple of comments to unify the format over CSI, Non-compressed and Compressed explicit feedback options from 7.3.1.33 to 7.3.1.35, which are accepted in principle. It also suggests resolutions on a couple of other comments which was already resolved in D1.05.

## CIDs to cover

### CID 984, 985

These comments were accepted unanimously, but text to instruct the editor is missing. This document provides the resolution. Basically, MIMO CSI feedback and Uncompressed V feedback has a report for either SNR per Rx antenna or per stream, but compressed V feedback has one averaged SNR report for all streams. We need to unify this format for all.

### CID 4752

We propose to resolve this as Counter since D1.05 does not have averaged value of SNR over streams.

m=28−2Ng

n=58−3Ng

## Proposal to change

In 7.3.1.30

TGn Editor:  Remove the subclause 7.3.1.30 in page 42 and Figure n23 in page 44.

### In 7.3.1.33

TGn Editor:  Change the following text at the line 32 in page 48 as follows

Grouping is a method to group adjacent subcarriers to reduce the network overhead. With grouping, the size of the CSI Matrices Report is Nr×8+Ns×(3+2×Nb×Nc×Nr) bits, where the number of subcarriers sent Ns is a function of Ng and whether matrices for 40 MHz or 20 MHz are sent. The value of Ns and the specific carriers for which matrices are sent is shown in Table n18 (Number of matrices and carrier grouping).

### In 7.3.1.34

TGn Editor:  Change the following text at the line 54 in page 49 as follows,

Grouping is a method to group adjacent subcarriers to reduce the network overhead. With grouping, the size of the Non-compressed Beamforming Matrix Report is Nc×8+Ns×(2×Nb×Nc×Nr) bits, where the number of subcarriers sent Ns is a function of Ng and whether matrices for 40 MHz or 20 MHz are sent. The value of Ns and the specific carriers for which matrices are sent is shown in Table n18 (Number of matrices and carrier grouping).

TGn Editor:  Change the Table n19 and Table n20 in page 50. (Basically, replace “Steering Matrix” with “Beamforming matrix V” for both of Tables.)

Table n19 – MIMO Non-compressed Beamforming Matrices Report fixed field (20 MHz)

| Field | Size | Meaning |
| --- | --- | --- |
| SNR in stream 1 | 8 bits | Average Signal to Noise Ratio in the STA sending the report for stream 1 |
|  |  |  |
| SNR in stream Nc | 8bits | Average Signal to Noise Ratio in the STA sending the report for stream Nc. |
| Beamforming Matrix V for Carrier -28 | 2× Nb×Nc×Nr bits | Beamforming Matrix V |
| Beamforming Matrix V for Carrier -28+Ng | 2× Nb×Nc×Nr bits | Beamforming Matrix V |
| … |  |  |
| Beamforming Matrix V for Carrier -1 | 2× Nb×Nc×Nr bits | Beamforming Matrix V |
| Beamforming Matrix V for Carrier 1 | 2× Nb×Nc×Nr bits | Beamforming Matrix V |
| Beamforming Matrix V for Carrier 1+ Ng | 2× Nb×Nc×Nr bits | Beamforming Matrix V |
| … |  |  |
| Beamforming Matrix V for Carrier 28 | 2× Nb×Nc×Nr bits | Beamforming Matrix V |

m=26Ng

Table n43 – MIMO Non-compressed Beamforming Matrices Report fixed field (40 MHz)

| Field | Size | Meaning |
| --- | --- | --- |
| SNR in stream 1 | 8 bits | Average Signal to Noise Ratio in the STA sending the report for stream 1 |
|  |  |  |
| SNR in stream Nc | 8bits | Average Signal to Noise Ratio in the STA sending the report for stream Nc. |
| Beamforming Matrix V for Carrier -58 | 2× Nb×Nc×Nr bits | Beamforming Matrix V |
| Beamforming Matrix V for Carrier -58+Ng | 2× Nb×Nc×Nr bits | Beamforming Matrix V |
| … |  |  |
| Beamforming Matrix V for carrier -2 | 2× Nb×Nc×Nr bits | Beamforming Matrix V |
| Beamforming Matrix V for carrier 2 | 2× Nb×Nc×Nr bits | Beamforming Matrix V |
| Beamforming Matrix V for Carrier 2+ Ng | 2× Nb×Nc×Nr bits | Beamforming Matrix V |
| … |  |  |
| Beamforming Matrix V for carrier 58 | 2× Nb×Nc×Nr bits | Beamforming Matrix V |

n=55Ng

### In 7.3.1.35

TGn Editor:  Change the following text at the second column of Table n21 in page 51.

Replace “Number of angles” with “Number of angles (Na)”.

TGn Editor:  Add the following text from the line 27 in page 52.

The Compressed beamforming matrices report for 20MHz has the structure defined in Table n<xx> (Compressed Beamforming Matrices Report fixed field (20 Mhz)).

The Compressed beamforming matrices report for 40MHz has the structure defined in Table n<yy> (Compressed Beamforming Matrices Report fixed field (40 Mhz)).

Table <xx> – Compressed Beamforming Matrices Report fixed field (20 MHz)

| Field | Size | Meaning |
| --- | --- | --- |
| SNR in stream 1 | 8 bits | Average Signal to Noise Ratio in the STA sending the report for stream 1 |
|  |  |  |
| SNR in stream Nc | 8bits | Average Signal to Noise Ratio in the STA sending the report for stream Nc. |
| Beamforming Matrix V for Carrier -28 | Na×(b+b )/2 bits | Beamforming Matrix V |
| Beamforming Matrix V for Carrier -28+Ng | Na×(b+b )/2 bits | Beamforming Matrix V |
| … |  |  |
| Beamforming Matrix V for Carrier -1 | Na×(b+b )/2 bits | Beamforming Matrix V |
| Beamforming Matrix V for Carrier 1 | Na×(b+b )/2 bits | Beamforming Matrix V |
| Beamforming Matrix V for Carrier 1+Ng | Na×(b+b )/2 bits | Beamforming Matrix V |
| … |  |  |
| Beamforming Matrix V for Carrier 28 | Na×(b+b )/2 bits | Beamforming Matrix V |

where m=26NgNa is the number of angles used for beamforming matrix V (see Table n21).

Table <yy> – Compressed Beamforming Matrices Report fixed field (40 MHz)

| Field | Size | Meaning |
| --- | --- | --- |
| SNR in stream 1 | 8 bits | Average Signal to Noise Ratio in the STA sending the report for stream 1 |
|  |  |  |
| SNR in stream Nc | 8bits | Average Signal to Noise Ratio in the STA sending the report for stream Nc. |
| Beamforming Matrix V for Carrier -58 | Na×(b+b )/2 bits | Beamforming Matrix V |
| Beamforming Matrix V for Carrier -58+Ng | Na×(b+b )/2 bits | Beamforming Matrix V |
| … |  |  |
| Beamforming Matrix V for carrier -2 | Na×(b+b )/2 bits | Beamforming Matrix V |
| Beamforming Matrix V for carrier 2 | Na×(b+b )/2 bits | Beamforming Matrix V |
| Beamforming Matrix V for Carrier 2+Ng | Na×(b+b )/2 bits | Beamforming Matrix V |
| … |  |  |
| Beamforming Matrix V for carrier 58 | Na×(b+b )/2 bits | Beamforming Matrix V |

where n=55NgNa is the number of angles used for beamforming matrix V (see Table n21).

TGn Editor:  Change the following text in the line 28 in page 52 as follows,

Grouping is a method to group adjacent subcarriers to reduce the network overhead. With grouping, the size of the Compressed Beamforming Matrix Report is Nc×8+Ns×(Na×(b+b )/2) bits, where, the number of subcarriers sent Ns is a function of Ng and whether matrices for 40 MHz or 20 MHz are sent. Ns and the specific carriers for which matrices are sent is defined in Table n18 (Number of matrices and carrier grouping).

Note that <xx> and <yy> are new numbers assigned by the editor.

In 7.4.8.10

TGn Editor:  Remove the 4th field (Receiver SNR) in Table n4 in page 75.