IEEE P802.11
Wireless LANs

| TGn LB84 PCO Comments |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-10-31 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Tomoko Adachi | Toshiba Corporation | 1, Komukai Toshiba-cho, Saiwai-ku, Kawasaki, 212-8582 Japan |  | tomo.adachi@toshiba.co.jp |
| Peter Loc | Marvell |  |  | ploc@marvell.com |
| Yuichi Morioka | Sony Corporation |  |  | morioka@wcs.sony.co.jp |
| Matt Smith | Atheros |  |  | msmith@atheros.com |
| Luke Qian | Cisco Systems, Inc. | 4125 Highlander Parkway, Richfield, OH 44286, USA |  | lchia@cisco.com |

Abstract

This document contains proposed resolutions for the following LB84 comments:

- 7374, 11377, 1232, 1516, 9888, 1556, 4277, 3605, 7197

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# CID 7374 (R)

| 7374 | 0 |  | General | 1. PCO doesn't resolve the issue of overlapping 11g/b BSS: 11b/g devices do not understand duplicate packets (CTS-to-self in duplicated non-HT mode is useless). 2. How the AP detects that the PCO is not providing a performance benefit in order to deactivate it? 3. PCO introduces a jitter (due to "Operation transition time" and dot11PCO40MaxDuration) that is not tolerable for real time applications. 4. Non-HT stations in PS mode could wake-up during 40MHz phase. | Remove PCO from the spec. |
| --- | --- | --- | --- | --- | --- |

## Proposed Resolution:

Reject

1.

40 MHz channelization problem is equally applicable to PCO or non PCO 20/40 operation.

2.

There are various information that the AP can use:

- Information of the number of each frame type (20 MHz PPDU or 40 MHz PPDU) during 20 MHz and 40 MHz phases

- Counter information of FCS errors, retry, or fail during 20 MHz and 40 MHz phases

The algorithm of whether to use the function or not is beyond the scope of this standard.

3.

The maximum and minimum duration values of 20 MHz and 40 MHz phases are defined as MIB attributes in order to cope with jitter. These MIB attributes can be set within the range to satisfy the requirement. When the AP decides that the support of jitter sensitive application is difficult, then the AP can deactivate PCO.

Ex. The AP can know the requirement of the STA by TSPEC.

4.

The AP is enabled to transmit a non-HT duplicate CTS-to-self and 40 MHz CF-End to reserve the primary and secondary channels again for the 40 MHz phase.

This is resolved in 06/1632r2.

# CID 11377 (A)

| 11377 | 59 | 8 | 7.4.7.3 | Size of "PCO Phase" is not specified. 1-bit, 8-bit, 16-bit, 32-bit? | As in comment |
| --- | --- | --- | --- | --- | --- |

## Proposed Resolution:

Accept

This comment is already resolved in D1.05. The size is specified.

# CID 1232 (A)

| 1232 | 59 |  | 7.4.7.3 | PCO Phase field needs to be defined | Define the PCO Phase field as a new subclause within 7.3.1 (i.e. as 7.3.1.x) |
| --- | --- | --- | --- | --- | --- |

## Proposed Resolution:

Accept

This comment is already resolved in D1.05. The field is defined in a new subclase within 7.3.1.26.

# CID 1516 (R)

| 1516 | 129 | 23 | 9.23.1 | Sufficient rules and modes for 40/20MHz operation are described in 9.32.2. The PCO mode adds unnecessary complexity to the document for minimal benefit | Remove PCO here, in 11.16, and throughout the document |
| --- | --- | --- | --- | --- | --- |

## Proposed Resolution:

Reject

40 MHz transmission uses two 20 MHz channels.

PCO is a mode that an AP reserves the two channels for STAs of the BSS including itself to transmit in 40 MHz.

So it is clear that the contention between 40 MHz access and 20 MHz access is reduced and the overall network performance will improve.

Also, it is possible at the receiver to reduce the implementation load of interference tolerance to secondary channel.

Straw Poll result: 11/1/7

# CID 9888 (R)

| 9888 | 154 |  | 11.15.2 | using 40 MHz in 2.4 GHz is going to downgrade performance of legacy BSSs. 40 MHz should be moved from 2.4 GHz. | Remove 40/20 MHz and PCO operation for 2.4 GHz. |
| --- | --- | --- | --- | --- | --- |

## Proposed Resolution:

Defer

40 MHz channelization problem is equally applicable to PCO or non PCO 20/40 operation.

This will be resolved the same with other comments saying to remove 40 MHz in 2.4 GHz.

# CID 1556 (R)

| 1556 | 156 | 3 | 11.16 | Current protection mechanisms do not do enough to protect legacy devices that happen to be running on an extension channel of a 40MHz HT system | Making PCO Mandatory will mitigate this situation somewhat |
| --- | --- | --- | --- | --- | --- |

## Proposed Resolution:

Reject

PCO is optional because it may not be benifitial in all conditions.

PCO is not a protection mechanism.

# CID 4277 (R)

| 4277 | 156 | 3 | 11.16 | PCO mode adds unnecessary complexity to the standard. There are sufficient rules for handling 40/20 operation. | Remove PCO mode |
| --- | --- | --- | --- | --- | --- |

## Proposed Resolution:

Reject

There are benefits over 20/40 MHz operation and it can also reduce the implementation load of interference tolerance at the receiver in some scenarios. See resolution for CID 1516.

# CID 3605 (R)

| 3605 | 156 |  | 11.16 | PCO is way too complex mechanism to manage co-existence when addition of a secondary CCA, some judicious scanning is sufficient. Let us not have another "me too" mechanism which requires extremely complex management and will never be implemented. | Delete all references to PCO |
| --- | --- | --- | --- | --- | --- |

## Proposed Resolution:

Reject

An AP can do the CCA in 40 MHz when it transmits the non-HT duplicate CTS-to-self for reserving the secondary channel to start a 40 MHz phase. As the primary channel is already reserved, the 40 MHz CCA will have the same effect of doing the CCA in the secondary channel.

See resolution for CID 1516.

13/1/7

# CID 7197 (C)

| 7197 | 157 | 10 | 11.16.1 | The paragraph allows non-AP PCO STAs to be associated in PCO BSS. This scenario allows non-AP PCO STA to send CF-End frames in duplicated mode and thus impact the integrity of 40MHz transmission time as directed by AP PCO STA. | Add an HT capability for AP so that it could reject associating non-AP STAs that are not capable of PCO operation. |
| --- | --- | --- | --- | --- | --- |

## Proposed Resolution:

Straw Poll: Add a bit in the capability field so that the AP can inform STAs that this BSS will only accept PCO capable STAs

2/12/8

Reject

The PCO AP knows that a STA is PCO capable by the PCO field set to 1 in the HT Extended Capabilities field in the HT Capabilities element of an Association Request.

The intent of the group is not to add a new bit in the capability field.

## References:

[1] 11-06-1578-00-000n-tgn-Draft P802.11-D1.05-redline-insertions only.pdf