IEEE P802.11
Wireless LANs

| MDIE Policy bit Definitions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Date: 2006-11-03 |  |  |  |  |  |
| Author(s): |  |  |  |  |  |
| Name | Company | Address | Phone |  | email |
| Michael Montemurro | RIM | 5090 Commerce Blvd, Mississauga, ON. L4W 5W4 | +1-905-629-4746 Ext. 4999 |  | mmontemurro@rim.com |
| Bill Marshall | TGr Editor | 180 Park Ave, Florham Park, NJ 07932 |  | 973-360-8718 | wtm@research.att.com |

Abstract

This submission is an update to the definition of the policy bits in the Mobility Domain information element, and all the corresponding places in D3.0 that need to stay in sync.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Change Figure 112T a follows:

“Reservation over air” to “Fast Transition over-the-air”

“Reservation over DS” to “Fast Transition over-the-DS”

“Reserve Option” to “Resource request supported”

Reserved to be only bits 5-6-7

Define bit 4 as “Reservation prior to reassociation required”

Change second paragraph under Figure 112T as follows:

The policy bits in the MDIE control the behavior of STAs performing a resource request as part of Fast BSS Transitions.

If the Fast BSS Transition capability bit is set to 0, then bits 1-4 will also be set to 0.

The AP sets FastTransition-over-air or FastTransition-over-DS to one according to the policy for Fast BSS Transition for the Mobility Domain. At least one of FastTransition-over-air or FastTransition -over-DS is advertised by the AP.

If Resource-request-supported is set to one then the STA may perform the Fast BSS Transition resource request procedures of 8A.8.

Reservation-prior-to-reassociation-required is only valid if Resource-request-supported is set to one, otherwise its use is reserved. If Reservation-prior-to-reassociation-required is set to one then the STA will not include a RIC in its Reassociation Request without first performing the resource request protocol of 8A.4.

If FastTransition-over-air is set to zero, then the STA will not initiate an over-the-air Fast BSS Transition. If FastTransition-over-DS is set to zero, then the STA will not initiate an over-the-DS Fast BSS Transition.

[Note to TGr editor – move this final paragraph to the end of the paragraph describing over-the-air and over-the-DS. Moving it in this submission would obscure the changes being shown by MSWord.]

Change second paragraph of 8A.1.1 as follows:

The Mobility Domain Identifier shall be the value of dot11FTMobilityDomainID. The resource policy bits in the MDIE, Fast Transition over air, Fast Transition over DS, Resource request supported, and Reservation prior to reassociation required, shall be set according to the values of the MIB variables dot11FTOverAirEnabled, dot11FTOverDSEnabled, dot11FTResourceRequestSupported, and dot11FTReservationPriorToReassocaitionRequired, respectively. If Fast BSS-Transition is supported, at least one of Fast Transition over air or Fast Transition over DS shall be set to one. The NAS-ID of the authenticator associated with the AP advertising Fast BSS Transition capability shall be the value of dot11FTNASID.

Insert a new paragraph at start of 8A.3.1:

A STA shall not initiate a Fast BSS Transition over-the-air to a target AP whose MDIE contained the Fast Transition over air bit set to zero.

Insert a new paragraph at start of 8A.3.2:

A STA shall not initiate a Fast BSS Transition over-the-DS to a target AP whose MDIE contained the Fast Transition over DS bit set to zero.

Change third paragraphof 8A.3.3 as follows:

The Information Elements in Fast BSS Transition Confirm and Fast BSS Transition Acknowledgement, and their required contents, shall be as given in 8A.5.3 and 8A.5.4. If the MDIE advertised by the target AP contained the Resource-Request-Supported bit set to zero, the STA shall not include a RIC in the Reassociation Request. If the MDIE advertised by the target AP contained the Reservation-Prior-to-Reassociation-Required bit set to one, then the STA shall not include a RIC in its Reassociation Request without first performing a resource request as in 8A.4.

Change third paragraph of 8A.4 as follows:

Availability of the reservation mechanism is advertised by the target AP in the MDIE. If Resource-Request-Supported is set to zero, then the STA shall not send a reservation request to the AP. An AP that receives a reservation request from a STA, and does not support the reservation mechanism, shall respond with a status code 38 ("The request has not been successful as one or more parameters have invalid values").

Change fifth paragraph of 8A.5.1 as follows:

The MDIE shall contain the Mobility Domain Identifier, and the Fast BSS Transition Capability and policy settings obtained from the target AP. The target AP advertises the MDIE in beacons and probe responses. The Mobility Domain Identifier shall be identical to that obtained during the FT Initial Mobility Domain Association exchange.

Change MIB entries:

dot11FTReservationOverAirEnabled to dot11FTOverAirEnabled

dot11FTReservationOverDSEnabled to dot11FTOverDSEnabled

dot11FTReservationRequired to dot11FTReservationPriortoReassociationRequired

Inset a new entry in dot11FastBSSTransitionConfigEntry as follows:

dot11FTResourceRequestSupported		TruthValue,

Insert a new MID definition as follows:

dot11FTResourceRequestSupported OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS currnet

DESCRIPTION

“When this object is set to TRUE, this shall indicate that the Fast BSS Transition resource request procedures of 8A.8 are supported onthis AP entity.”

::= {dot11FastBSSTransitionConfigEntry 13}