IEEE P802.11
Wireless LANs

| TGn LB84 Submission for Miscellaneous PHY CIDs |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-11-09 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Eldad Perahia | Intel Corporation |  |  | eldad.perahia@intel.com |

Abstract

This document contains proposed changes to the IEEE P802.11n Draft to address the following LB84 comments:

3999, 444, 3429, 3444, 178, 4840, 2907, 10532, 196

The changes marked in this document are based on TGn Draft version P802 11n D1.05.pdf.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# Introduction

## Interpretation of a Motion to Adopt

A motion to approve this submission means that the editing instructions and any changed or added material are actioned in the TGn Draft.  This introduction, is not part of the adopted material.

Editing instructions formatted like this are intended to be copied into the TGn Draft (i.e. they are instructions to the 802.11 editor on how to merge the TGn amendment with the baseline documents).

TGn Editor:  Editing instructions preceded by “TGn Editor” are instructions to the TGn editor to modify existing material in the TGn draft.   As a result of adopting the changes, the TGn editor will execute the instructions rather than copy them to the TGn Draft.

Summission Note: Notes to the reader of this submission are not part of the motion to adopt.  These notes are there to clarify or provide context.

# Proposed Resolution

| CID | Page(Ed) | Line(Ed) | Clause(Ed) | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- | --- |
| 3999 | 159 | 14 | 20.1 | while it says "compliant with 802.11a/b/g standards", P.160, L.32 says non-HT modes are for Clause 17 or 19. How about clause 14, 15 and 18? | a little clarification might help readers understand how "legacy" is positioned against the rest of the 802.11 standard? | Counter: Changes already in D1.05 replace references to 11a/b/g with clause 17, 18, and 19. Refer to 11-06-1315r1 for further explanation of relationship between clause 21,19,18,17,15, which have already been incorporporated in D1.05. |
| 444 | 160 | 32 | 20.1.3 | "in this mode packets are transmitted according to clause 17 (OFDM) or 19 (ERP) specification." Really? What about "b" devices, which use DSSS? |  | Counter: clause 19 includes ERP-DSSS which includes capabilities of clause 15. Refer to 11-06-1315r1 for further explanation of relationship between clause 21,19,18,17,15, which have already been incorporporated in D1.05. |
| 3429 | 160 |  | 20.1.3 | Non-HT doesn't include clause 18 PHY modes? | Include in the Non-HT Mode clause 15 and clause 18 modes. | Counter: clause 19 includes ERP-DSSS and ERP-CCK which includes capabilities of clause 15 and clause 18, respectively. Refer to 11-06-1315r1 for further explanation of relationship between clause 21,19,18,17,15, which have already been incorporporated in D1.05. |
| 3444 | 166 | 22 | 20.2.3.5 | 802.11b modes need to be accomodated in non-HT. | Add Clause 18. | Counter: Accept in principle. Changes already in D1.05 - RXVECTOR NON_HT_MODULATION field includes ERP-DSSS and ERP-CCK. Refer to 11-06-1315r1 for further explanation of relationship between clause 21,19,18,17,15, which have already been incorporporated in D1.05. |
| 178 | 168 | 32 | 20.3.2 | There is no mention of HT duplicate format | Mention the HT duplicate format in the text | Counter: Accept in principle, refer to 06/1658 |
| 4840 | 169 | 2 | 20.3.2 | The term "PLCP packet" has not been defined. | Replace "PLCP packet" with "PPDU" or if you must, "PLCP frame". However, if using "PLCP frame", use this term in 20.2.2.5 and 20.2.3.5. | Counter: Accept in principle, refer to 06/1658 |
| 2907 | 169 | 4 | 17 | If the clause 20 PHY is really a superset of the clause 17 PHY, then clause 17 should be deleted, and all references to clause 17 replaced by a reference to clause 20 with the specific subset of the clause 20 parameters. | Make change indicated in comment | Reject: Don't agree that clause 21 is a complete superset of clause 17. Refer to 11-06-1315r1 for further explanation of relationship between clause 21,19,18,17,15. Similar to CID2910 |
| 10532 | 169 | 4 | 17 | If the clause 20 PHY is really a superset of the clause 17 PHY, then clause 17 should be deleted, and all references to clause 17 replaced by a reference to clause 20 with the specific subset of the clause 20 parameters. | as in comment. Definition of 11a/b/g to be done through the PICS, selecting which of the features of the clause 20 PHY that shall be implemented for each. | Reject: Don't agree that clause 21 is a complete superset of clause 17. Refer to 11-06-1315r1 for further explanation of relationship between clause 21,19,18,17,15. Similar to CID2910 |
| 196 | 172 | 8 | 20.3.2.2 | The number of bits appended to information bit stream is known once the user know whether one or two BCCs are used. If 1 BCC is used then 6 bits are appended. It is an exact number. Additional pad bits may be needed in order to ensure that coded bit stream lines up on OFDM symbol boundaries. Do not combine the two techniques into one separate them. | Separate the addition of tail bits from pad bits. Remove the words "at least" and add sentence that talks about adding pad bits in order to ensure coded bit stream lines on OFDM symbol boundaries. | Counter: Accept in principle, refer to 06/1568 |

# CID 178

TGn Editor: In D1.05, pg 198, modify the first paragraph of clause 21.3.2  as follows

Two formats are defined for the PLCP (PHY Layer Convergence Protocol): HT Mixed Mode and Greenfield.

These two formats are called HT formats. Figure n50 (PLCP packet format) shows the non-HT format and

the HT formats. There is also an HT duplicate format (specified in 21.3.10.10.4 (Transmission in HT duplicate mode)) used for MCS 32 that provides the lowest rate in a 40 MHz channel.  In addition to the HT formats, there is a non-HT duplicate format (specified in 21.3.10.11 (Non-HT duplicate transmission)) that duplicates the 20 MHz non-HT packet in two 20 MHz halves of a 40 MHz channel.

# CID 4840

TGn Editor: In D1.05, pg 199, line 25 modify the title of Figure n50  as follows

Figure n50—PLCP packet PPDU frame format

# CID 196

TGn Editor: In D1.05, pg 203, line 24-31 modify text  as follows

If BCC encoding is to be used, as indicated by the LDPC_CODING field of the TXVECTOR, tail bits are appended to the PSDU. extend the resulting bit string with zero bits so that the resulting length is a multiple of NDBPS, as described in 21.3.10 (The Data field). If a single BCC encoder is used (the value of  is 1), the bit string is be extended by at least 6 bits. If two BCC encoders are used (the value of  is 2), the bit string is be extended by at least 12 bits.  If necessary, the bit string is further extended with zero bits so that the resulting length is a multiple of NDBPS, as described in 21.3.10 (The Data field).