IEEE P802.11
Wireless LANs

| TGn LB84 Submission for Channel Allocation CIDs |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-11-07 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Eldad Perahia | Intel Corporation |  |  | eldad.perahia@intel.com |

Abstract

This document contains proposed changes to the IEEE P802.11n Draft to address the following LB84 comments:

279, 4706, 7117, 7118, 12037, 12055, 280, 281, 12195, 7119

The changes marked in this document are based on TGn Draft version P802 11n D1.05.pdf.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# Introduction

In order to move all the regulatory specific information out of clause 21 and into Annex J, we modify the channel number scheme for 40MHz and add 40MHz channels to tables in Annex I and Annex J.  Only tables in Annex J for USA and Europe were modified to include 40MHz.  The author does not believe that 40MHz is currently permitted in Japan.

Note: TGkD5.0 has modified Annex I and Annex J.  The changes in this submission builds upon those modifications.

## Interpretation of a Motion to Adopt

A motion to approve this submission means that the editing instructions and any changed or added material are actioned in the TGn Draft.  This introduction, is not part of the adopted material.

Editing instructions formatted like this are intended to be copied into the TGn Draft (i.e. they are instructions to the 802.11 editor on how to merge the TGn amendment with the baseline documents).

TGn Editor:  Editing instructions preceded by “TGn Editor” are instructions to the TGn editor to modify existing material in the TGn draft.   As a result of adopting the changes, the TGn editor will execute the instructions rather than copy them to the TGn Draft.

Summission Note: Notes to the reader of this submission are not part of the motion to adopt.  These notes are there to clarify or provide context.

# Proposed Resolution

| CID | Page(Ed) | Line(Ed) | Clause(Ed) | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- | --- |
| 279 | 222 | 12 | 20.3.8.1 | The Japanese regulatory domain is not listed | Please update the table to include Japan | Counter: Refer to 06/1659 |
| 4706 | 222 | 12 | 20.3.8.1 | Table n78 should perhaps be in the informative annex I, for the reasons mentioned in comment 112 | Move Table n78 into Annex I | Counter: Refer to 06/1659 |
| 7117 | 222 | 12 | 20.3.8.1 | Is there something restricting the use of 40 MHz operation in the United States for the 5.5 to 5.7 band? | If no restriction add United States to that portion of the table. | Counter: Refer to 06/1659 |
| 7118 | 222 | 12 | 20.3.8.1 | Is there something restricting the use of 40 MHz operation in Europe for the 5.15 to 5.25 band, in addition to the 5.25-5.35 and 5.725-5.825 bands | If no restriction add Europe to those portions of the table. | Counter: Refer to 06/1659 |
| 12037 | 222 | 12 | 20.3.8.1 | Table n78 provides very few frequency combinations for 40 MHz operation | We should add the folllowing center frequencies in the United States: 5210, 5250, 5290, 5775. We should add the following center frequencies in Europe: 5530, 5570, 5610, 5650. It gives the STA a much better chance to find a clean 40 MHz. It would be fine to make the frequencies defined in the table preferred over the ones suggested above, but in the event that a clean 40 MHz cannont be found with the center frequencies in the table, the STA should have the opportunity to find a clean 40 MHz with the carrier frequencies defined above. | Counter: Refer to 06/1659 |
| 12055 | 222 | 12 | 20.3.8.1 | Table n78 provides very few frequency combinations for 40 MHz operation | We should add the folllowing center frequencies in the United States: 5210, 5250, 5290, 5775. We should add the following center frequencies in Europe: 5530, 5570, 5610, 5650. It gives the STA a much better chance to find a clean 40 MHz. It would be fine to make the frequencies defined in the table preferred over the ones suggested above, but in the event that a clean 40 MHz cannont be found with the center frequencies in the table, the STA should have the opportunity to find a clean 40 MHz with the carrier frequencies defined above. | Counter: Refer to 06/1659 |
| 280 | 223 | 9 | 20.3.8.2 | The Japanese regulatory domain is not listed | Please update the table to include Japan | Counter: Refer to 06/1659 |
| 281 | 223 | 9 | 20.3.8.2 | 40 MHz allocation should extend over all channels defined in Europe. For example, why does table not continue with the following pairs under the columns: Extension = 1 and Extension = -1 (8, 12), (9,13) | Update the tables to include (8,12) and (9,13) for Europe | Counter: Refer to 06/1659 |
| 12195 | 223 | 9 | 20.3.8.2 | Last I heard European channels extedned to channel 13. | Define 40MHz channels up to channel 13 | Counter: Refer to 06/1659 |
| 7119 | 221 | 16 | 20.3.8.2 | The definition for channel center frequency does not match 18.4.6.2 Table 112. Are we restricting HT operation to this current subset? | Use definition from 18.4.6.2 Table 112 | Counter: Refer to 06/1659 |

TGn Editor: In D1.05, modify subclause 21.3.14 as follows

21.3.14 Channel numbering and channelization

The STA may operate in the 5GHz band and/or 2.4GHz band (Ed: CID 429). When using 20 MHz channels

it uses channels defined in 17.3.8.3 (5 GHz band) or 18.4.6 (2.4 GHz band). When using 40 MHz channels,

it can operate in the channels defined in 21.3.14.1 (Channel allocation in the 2.4 GHz Band) and 21.3.14.2

(Channel allocation in the 5 GHz band).

Insert the following new subclause:

21.3.14.1 Channel allocation in the 2.4 GHz Band

Channel center frequencies are defined at every integral multiple of 5 MHz in the 2.4 GHz band. The relationship

between center frequency and channel number is given by the following equation:

Channel center frequency = 2407 Channel starting frequency + 5 × nch (MHz) 			(21-72)

where

nch = 1,2,…141

The 40 MHz channels in 2.4 GHz are specified in the same way as in 5 GHz: (Nprimary_ch, Secondary). The

first field represents the channel number of the primary channel, and the second one indicates whether the

secondary channel is above or below the primary channel (1 -> above, -1 -> below). For example, a 40 MHz

channel consisting of channel 2 and channel 6 where channel 6 is the primary channel shall be specified as

(6, -1).Table n83 (40 MHz channel allocation in the 2.4 GHz band) lists the valid settings of these two fields

in the 2.4 GHz band.

Insert the following new subclause:

21.3.14.2 Channel allocation in the 5 GHz band

Channel center frequencies are defined at every integral multiple of 5 MHz above 5 GHz (for 20 MHz channels).

The relationship between center frequency and channel number is given as follows:

Channel center frequency = 5000 Channel starting frequency + 5 × nch (MHz) 		(21-73)

where

nch = 0,1,…200

Table n83—40 MHz channel allocation in the 2.4 GHz band

| Regulatory domain | Ncontrol_chNprimary_ch |  | Center Frequency (MHz) |
| --- | --- | --- | --- |
|  | Extension Secondary = 1 | Extension Secondary = -1 |  |
| United States Canada Europe | 1 2 3 4 5 6 7 | 5 6 7 8 9 10 11 | 2422 2427 2432 2437 2442 2447 2452 |

The 40 MHz channels in 5 GHz are specified by two fields: (Nprimary_ch, Secondary). The first field represents

the channel number of the primary channel, and the second one indicates whether the secondary channel is

above or below the primary channel (1 -> above, -1 -> below). For example, a 40 MHz channel consisting of

channel 36 and 40 where channel 36 is the primary channel shall be specified as (36,1). Table n84 (40 MHz

channel allocation in the 5 GHz band ) lists the valid settings of these two fields in the 5 GHz band.

Table n84—40 MHz channel allocation in the 5 GHz band

| Regulatory domain | Band (GHZ) | Ncontrol_chNprimary_ch |  | Center frequency (MHz) |
| --- | --- | --- | --- | --- |
|  |  | Extension Secondary = 1 | Extension Secondary = -1 |  |
| United States | U-NII lower band (5.15-5.25) | 36 44 | 40 48 | 5190 5230 |
| United States | U-NII middle band (5.25-5.35) | 52 60 | 56 64 | 5270 5310 |
| Europe | ETSI (5.5-5.7) | 100 108 116 124 132 | 104 112 120 128 136 | 5510 5550 5590 5630 5670 |
| United States | U-NII upper band (5.725-5.825) | 149 157 | 153 161 | 5755 5795 |

Insert the following new subclause:

21.3.14.3 40 MHz channelization

The set of valid operating channel numbers by regulatory domain is defined in Annex J.

The 40 MHz channels are specified by two fields: (Nprimary_ch, Secondary).  The first field represents

the channel number of the primary channel, and the second one indicates whether the secondary channel is above or below the primary channel (1 -> above, -1 -> below). The secondary channel number shall be Nprimary_ch + Secondary*4.

For example, a 40 MHz channel consisting of 40MHz channel number 36 and Secondary 1 specifies the primary channel is 36 and the secondary channel is 40.

TGn Editor: In D1.05, modify Annex I(TGma8.0 and TGk5.0) as follows

Insert a new row after the tenth Behavior limits set row, and change the last row of Table I.3 as shown:

| Behavior limits set | USA | Europe | Japan |
| --- | --- | --- | --- |
| 11 40MHz Primary channel with Secondary = 1 | Reserved | Reserved | Reserved |
| 12 40MHz Primary channel with Secondary = -1 | Reserved | Reserved | Reserved |
| 13 11-255 | Reserved | Reserved | Reserved |

TGn Editor: In D1.05, modify Annex J(TGma8.0 and TGk5.0) as follows

Insert new penultimate rows and change the last row of Table J.1 as shown:

| Regulatory Class | Channel Starting Frequency (GHz) | Channel Spacing (MHz) | Channel set | Transmit Power limit (mW) | Emissions Limits set | Behavior Limits set |
| --- | --- | --- | --- | --- | --- | --- |
| 13 | 5 | 40 | 36, 44 | 40 | 1 | 1,2,11 |
| 14 | 5 | 40 | 52, 60 | 200 | 1 | 1,11 |
| 15 | 5 | 40 | 100, 108, 116, 124, 132 | 200 | 1 | 1,11 |
| 16 | 5 | 40 | 149, 157 | 800 | 1 | 1,11 |
| 17 | 5 | 40 | 149, 157 | 1000 | 4 | 10,11 |
| 18 | 5 | 40 | 40, 48 | 40 | 1 | 1,2,12 |
| 19 | 5 | 40 | 56, 64 | 200 | 1 | 1,12 |
| 20 | 5 | 40 | 104, 112, 120, 128, 136 | 200 | 1 | 1,12 |
| 21 | 5 | 40 | 153, 161 | 800 | 1 | 1,12 |
| 22 | 5 | 40 | 153, 161 | 1000 | 4 | 10,12 |
| 23 | 2.407 | 40 | 1-7 | 1000 | 4 | 10,11 |
| 24 | 2.407 | 40 | 5-11 | 1000 | 4 | 10,12 |
| 1225-255 | Reserved | Reserved | Reserved | Reserved | Reserved | Reserved |

Insert new penultimate rows and change the last row of Table J.2 as shown:

| Regulatory Class | Channel Starting Frequency (GHz) | Channel Spacing (MHz) | Channel set | Transmit Power limit (mW) | Emissions Limits set | Behavior Limits set |
| --- | --- | --- | --- | --- | --- | --- |
| 5 | 5 | 40 | 36, 44 | 200 | 1 | 2,3,11 |
| 6 | 5 | 40 | 52, 60 | 200 | 1 | 1,3,4,11 |
| 7 | 5 | 40 | 100, 108, 116, 124, 132 | 1000 | 1 | 1,3,4,11 |
| 8 | 5 | 40 | 40, 48 | 200 | 1 | 2,3,12 |
| 9 | 5 | 40 | 56, 64 | 200 | 1 | 1,3,4,12 |
| 10 | 5 | 40 | 104, 112, 120, 128, 136 | 1000 | 1 | 1,3,4,12 |
| 11 | 2.407 | 40 | 1-9 | 100 | 4 | 10,11 |
| 12 | 2.407 | 40 | 5-13 | 100 | 4 | 10,12 |
| 413-255 | Reserved | Reserved | Reserved | Reserved | Reserved | Reserved |