IEEE P802.11
Wireless LANs

| TGn LB84 Submission Related to MAC comments |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-11-09 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Adrian Stephens | Intel Corporation | 15 JJ Thompson Avenue, Cambridge, CB3 0FD, United Kingdom |  | adrian.p.stephens@intel.com |

Abstract

This document contains proposed changes to the IEEE P802.11n Draft to address the following LB84 comments:

- 4529, 1803, 1808, 7607, 2517, 2518, 1296, 2519, 5146, 9991, 2521, 2523, 2525, 2526, 2528, 2530, 2532, 2533, 34, 7886, 2534, 7887, 2535, 2537, 2538, 2541, 2543, 2545, 2546, 2547, 2548, 4558, 4559, 9993, 9895, 3613, 1053, 2855, 10519, 7489,

The changes marked in this document are based on TGn Draft version D1.06.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# Introduction

## Interpretation of a Motion to Adopt

A motion to approve this submission means that the editing instructions and any changed or added material are actioned in the TGn Draft.  This introduction, is not part of the adopted material.

Editing instructions formatted like this are intended to be copied into the TGn Draft (i.e. they are instructions to the 802.11 editor on how to merge the TGn amendment with the baseline documents).

TGn Editor:  Editing instructions preceded by “TGn Editor” are instructions to the TGn editor to modify existing material in the TGn draft.   As a result of adopting the changes, the TGn editor will execute the instructions rather than copy them to the TGn Draft.

Summission Note: Notes to the reader of this submission are not part of the motion to adopt.  These notes are there to clarify or provide context.

# Proposed Resolutions

Note, there are a large number of ER (editorial recycled) comments related to section 9.12.   They arise from a cut and paste error that misaligned comments and their resolutions.   The proposed resolution to these comments is copied from document 11-06-1306r0.

| 4529 | 0 |  | General | What is an aggregate PPDU? I think you mean MPDU | replace "PPDU" with "MPDU" | General AdHoc: Transfer to MAC Group See CID 65 |
| --- | --- | --- | --- | --- | --- | --- |

Proposed resolution: proposed counter

This is resolved by the following resolution to CID 65:  “Change "aggregated PPDU" to "A-MPDU". Also "non-aggregated PPDU" should be changed to other appropriate word, such as "single MPDU".”

| 1803 | 2 | 11 | 3 | Term "Initiator" has many uses other than the one in this definition | Use a more specific term than the general one chosen | Gen AdHoc: Initiator vs initiator is being used in the draft. The Capital version should always match the definition, and the lower case should be the generic use, Transfer to the MAC group to ensure consistency. |
| --- | --- | --- | --- | --- | --- | --- |

Proposed resolution: proposed counter:

Submission 11-06-1302r3 related to CID 472 and others, approved in the September TGn session, and incorporated in D1.06 addresses this issue by defining new terms to use where appropriate.  Our use of Initiator in the TGn draft has been replaced with TXOP holder, which is equivalent to the old meaning, but more specific.

| 1808 | 3 | 2 | 3 | Term "Responder" has many uses other than the one in this definition | Use a more specific term than the general one chosen | Gen AdHoc: Responder vs responder is being used in the draft. The Capital version should always match the definition, and the lower case should be the generic use, Transfer to the MAC group to ensure consistency. |
| --- | --- | --- | --- | --- | --- | --- |

Proposed resolution: proposed counter:

Submission 11-06-1302r3 related to CID 472 and others, approved in the September TGn session, and incorporated in D1.06 addresses this issue by defining new terms to use where appropriate.  Our use of Responder in the TGn draft has been replaced with on of three terms:  TXOP responder, MFB responder, Beamformee where appropriate.

| 7607 | 92 |  | 9.10.7 | The BA sections do not work well with the baseline BA sections. They include a re-statement of behaviour which is part of the baseline (i.e. rx reorder buffer operation). It is not clear whether this is intended to modify the behaviour or is an informative description of an implementation. They refer to 802.11e. | Take the original BA sections and merge into this section in a consistent fashion. Clearly identify informative description as such (and move to an informative annex if one exists) | Defer - U |
| --- | --- | --- | --- | --- | --- | --- |

Proposed counter:

Submission 11-06-1345r9 contains a reorganization and rewriting of the related sections.

| 2517 | 100 | 38 | 9.12 | undefined non-terminal | define cf-ack-piggybacked-poll-sequence | Edit Notes (D1.05) ER: <The resolution disagrees with that in 11-06/1306r0, which was: Proposed counter. Term changed to: cf-ack-piggybacked-qos-poll-sequence>, to resolution (D1.05): counter. Insert the following term at the start of TXOP-part-requiring-ack: "Data+individual[+null] ". This allows a txop-sequence to start data/ack if it so wishes. |
| --- | --- | --- | --- | --- | --- | --- |

Proposed resolution: counter

Change reference to cf-ack-piggybacked-poll-sequence to cf-ack-piggybacked-qos-poll-sequence.

This change is present in D1.06.

| 2518 | 100 | 40 | 9.12 | undefined non-terminal | define cf-ack-piggybacked-data-sequence | Edit Notes (D1.05) ER: <The resolution disagrees with that in 11-06/1306r0, which was: "Proposed counter. Term changed to: cf-ack-piggybacked-qos-data-sequence">, to resolution (D1.05): counter. Term changed to: cf-ack-piggybacked-qos-poll-sequence |
| --- | --- | --- | --- | --- | --- | --- |

Proposed resolution: counter

Change reference to cf-ack-piggybacked-data-sequence to cf-ack-piggybacked-qos-data-sequence.

This change is present in D1.06.

| 1296 | 101 | 7 | 9.12 | this font produces a "1" and an "l" which are nearly identical - the "1" is slightly shorter in height than the "l" - is it possible, within the confines of the IEEE 802 standards style formatting rules to switch to some font which more clearly differentiates the two characters? | change fonts to allow visual differentiation between "1" and "l" | Edit Notes (D1.05) ER: <The resolution disagrees with that in 11-06/1306r0, which was: Proposed accept. Change l-sig to L-sig, and use courier new for the 1 preceeding {.>, to resolution (D1.05): counter. Term changed to: cf-ack-piggybacked-qos-data-sequence |
| --- | --- | --- | --- | --- | --- | --- |

Proposed resolution:  counter.

Change l-sig to L-sig, and use courier new for the 1 preceeding {.

This change is present in D1.06

| 2519 | 101 | 7 | 9.12 | notation. Use EBNF notation | 1*{initiator-sequence} | Edit Notes (D1.05) ER: <The resolution disagrees with that in 11-06/1306r0, which was: Proposed reject. This changes the meaning to exactly one occurrance.>, to resolution (D1.05): accept. Change l-sig to L-sig, and use courier new for the 1 preceeding {. |
| --- | --- | --- | --- | --- | --- | --- |

Proposed resolution: reject.

This would change the meaning to “exactly one occurrence”, which is not what is intended.

| 5146 | 101 | 27 | 9.12 | RTS+ CTS is not defined in the syntax | Replace by RTS CTS | Edit Notes (D1.05) ER: <The resolution disagrees with that in 11-06/1306r0, which was: Proposed accept>, to resolution (D1.05): counter. Accept that single-ended L-sig protection is not defined. However, the commenter refers to lines 20 and 21, whereas line 19 contains the offending term. Remove line 19. |
| --- | --- | --- | --- | --- | --- | --- |

Proposed resolution: accept

This change is already present in D1.06.

| 9991 | 101 | 35 | 9.12 | nav-reset is missing double CF-Ends | Change to "nav-reset = CF-End ¦ [CF-End+CF-End]" | Edit Notes (D1.05) ER: <The resolution disagrees with that in 11-06/1306r0, which was: Proposed accept. Added the term: " (CF-End+non-QAP CF-End+QAP);" as an alternate.>, to resolution (D1.05): accept |
| --- | --- | --- | --- | --- | --- | --- |

Proposed resolution: accept.

This change is already present in D1.06.

| 2521 | 101 | 37 | 9.12 | From this point onward there are numerous errors in the EBNF. The fact that this definition was not done correctly indicates the design is too complex and confusing, and needs to be simplified. | Simplify the design | Edit Notes (D1.05) ER: <The resolution disagrees with that in 11-06/1306r0, which was: Proposed reject: The "design" is the result of a selection process that incorporated some features and excluded others from proposals submitted to TGn. The set of features present has received broad support from TGn members.>, to resolution (D1.05): accept. Added the term: " (CF-End+non-QAP CF-End+QAP);" as an alternate. |
| --- | --- | --- | --- | --- | --- | --- |

Proposed resolution: reject:

The "design" is the result of a selection process that incorporated some features and excluded others from proposals submitted to TGn.   The set of features present has received broad support from TGn members.

| 2523 | 101 | 51 | 9.12 | ht-ack-sequence is undefined | define it. Perhaps it should be ht-ack-response (page 102 line 12) | Edit Notes (D1.05) ER: <The resolution disagrees with that in 11-06/1306r0, which was: Proposed accept.>, to resolution (D1.05): counter. Correct BA to BlockAck. |
| --- | --- | --- | --- | --- | --- | --- |

Proposed resolution: accept

This change is already present in D1.06

| 2525 | 102 | 2 | 9.12 | BA is undefined, either as a frame name or as a non-terminal | define it | Edit Notes (D1.05) ER: <The resolution disagrees with that in 11-06/1306r0, which was: "Proposed counter. Correct BA to BlockAck.">, to resolution (D1.05): accept. |
| --- | --- | --- | --- | --- | --- | --- |

Proposed resolution: counter

Correct BA to BlockAck

This change is already present in D1.06

| 2526 | 102 | 3 | 9.12 | burst-BA-RD undefined | define it | Edit Notes (D1.05) ER: <The resolution disagrees with that in 11-06/1306r0, which was: Proposed accept.>, to resolution (D1.05): counter. Correct BA to BlockAck. |
| --- | --- | --- | --- | --- | --- | --- |

Proposed resolution: accept

This change is already present in D1.06

| 2528 | 102 | 18 | 9.12 | notation. Use EBNF notation | 1*{PPDU-not-requiring-response} | Edit Notes (D1.05) ER: <The resolution disagrees with that in 11-06/1306r0, which was: "Proposed reject. This changes the meaning to exactly one occurrance.">, to resolution (D1.05): Accept. |
| --- | --- | --- | --- | --- | --- | --- |
| 2530 | 102 | 46 | 9.12 | notation. Use EBNF notation | 1*{frame-not-requiring-response+ampdu} | Edit Notes (D1.05) ER: <The resolution disagrees with that in 11-06/1306r0, which was: "Proposed reject. This changes the meaning to exactly one occurrance.">, to resolution (D1.05): accept. Also corrected burst-BA-RD-BAR. |

Proposed resolution: reject.

This would change the meaning to “exactly one occurrence”, which is not what is intended.

| 2532 | 102 | 53 | 9.12 | Notation "+(no-ack¦block-ack)" undefined | Change to "(+no-ack¦+block-ack)" | Edit Notes (D1.05) ER: <The resolution disagrees with that in 11-06/1306r0, which was: 'Proposed counter. Define the syntax thus: "The syntax a+(b¦c) where b and c are attributes is equivalent to (a+b) ¦ (a+c).", as the original syntax is more readable.'>, to resolution (D1.05): accept. |
| --- | --- | --- | --- | --- | --- | --- |

Proposed resolution: counter.

Define the syntax thus: "The syntax a+(b|c) where b and c are attributes is equivalent to (a+b) | (a+c).", as the original syntax is more readable

This change is already present in D1.06.

| 2533 | 103 | 5 | 9.12 | What seems to be desired here is an A-MPDU with an implicit BAR somewhere in it | Change to: BlockAckReq ¦ ({Data+QoS+ampdu} 1*{Data+QoS+implicit-bar+ampdu {Data+QoS+ampdu}}) +ampdu-end | Edit Notes (D1.05) ER: <The resolution disagrees with that in 11-06/1306r0, which was: 'Proposed counter. Other comment simplified this because all data frames will carry the implicit-bar.'>, to resolution (D1.05): counter. Define the syntax thus: "The syntax a+(b¦c) where b and c are attributes is equivalent to (a+b) ¦ (a+c).", as the original syntax is more readable. |
| --- | --- | --- | --- | --- | --- | --- |

Proposed resolution: counter

Resolution of another comment has simplified this in D1.06 because all data frames carry the implicit-bar.

| 34 | 103 | 7 | 9.12 | For the Implicit BAR case, the transmitter should unify the Ack policy of all MPDUs in the A-MPDU to "normal Ack". That will give diversity effect to the receiver to know that it shall send a BA frame. | Delete {Data+QoS+ampdu} in line 7. | Edit Notes (D1.05) ER: <The resolution disagrees with that in 11-06/1306r0, which was: Proposed counter. While accepting this specific instance, the same change should be made throughout 9.12 for both implicit BAR and RD cases.>, to resolution (D1.05): counter. Other comment simplified this because all data frames will carry the implicit-bar. |
| --- | --- | --- | --- | --- | --- | --- |

Proposed resolution: counter

While accepting this specific instance, the same change should also be made throughout 9.12 for both implicit BAR and RD cases.

This change has already been implemented for D1.06.

| 7886 | 103 | 7 | 9.12 | 9.10.7.7 says "All frames within an A-MPDU aggregate shall have the same ack policy setting.". This is good to deliver BAR in a robust manner. The definition of PPDU-BAR seems to be inconsistent with this. | Remove {Data+QoS+ampdu} | Edit Notes (D1.05) ER: <The resolution disagrees with that in 11-06/1306r0, which was: Proposed accept. Covered by CID 34.>, to resolution (D1.05): counter. While accepting this specific instance, the same change should be made throughout 9.12 for both implicit BAR and RD cases. |
| --- | --- | --- | --- | --- | --- | --- |

Proposed resolution: accept.  Note this is covered by CID 34. and has been implemented in D1.06

| 2534 | 103 | 13 | 9.12 | Use of "<>" notation incorrect, as there is only one choice within the <>'s | Replace the <> with () | Edit Notes (D1.05) ER: <The resolution disagrees with that in 11-06/1306r0, which was: Proposed counter. The use of this notation has been discontinued.>, to resolution (D1.05): accept. Covered by CID 34. |
| --- | --- | --- | --- | --- | --- | --- |

Proposed counter.  The use of this notation has been discontinued in D1.03

| 7887 | 103 | 17 | 9.12 | 9.10.7.7 says "All frames within an A-MPDU aggregate shall have the same ack policy setting.". This is good to deliver BAR in a robust manner. The definition of PPDU-BA-BAR seems to be inconsistent with this. | Remove {Data+QoS+ampdu} | Edit Notes (D1.05) ER: <The resolution disagrees with that in 11-06/1306r0, which was: Proposed accept. Covered by CID 34.>, to resolution (D1.05): counter. The use of this notation has been discontinued. |
| --- | --- | --- | --- | --- | --- | --- |

Proposed resolution: accept.  Note this is covered by CID 34. and has been implemented in D1.06

| 2535 | 103 | 24 | 9.12 | This definition doesn't allow Data both before and after the BlockAck in the A-MPDU | Change to: BlockAck ¦ (BlockAck+ampdu 1*{Data+QoS(+no-ack¦+block-ack)+ampdu})+ampdu-end ¦ (1*{Data+QoS(+no-ack¦+block-ack)+ampdu} BlockAck+ampdu {Data+QoS(+no-ack¦+block-ack)+ampdu})+ampdu-end | Edit Notes (D1.05) ER: <The resolution disagrees with that in 11-06/1306r0, which was: Proposed reject. Changes being discussed in other ad-hocs limit the order of what goes into an A-MPDU so that any response frame occurs first.>, to resolution (D1.05): accept. Covered by CID 34. |
| --- | --- | --- | --- | --- | --- | --- |

Proposed reject.  Changes being discussed in other ad-hocs limit the order of what goes into an A-MPDU so that any response frame occurs first.

| 2537 | 103 | 36 | 9.12 | Sequence here doesn't allow Data w/o RD both before and after the Data w/RD | Change the <…> to: ({Data+QoS+ampdu} Data+QoS+RD+ampdu {Data+QoS[+RD]+ampdu}) +ampdu-end | Edit Notes (D1.05) ER: <The resolution disagrees with that in 11-06/1306r0, which was: Proposed reject. D1.02 says: "The RDG/More PPDU field shall be set to the same value in all HT Control fields present in a PPDU">, to resolution (D1.05): accept. Change to: ""(BlockAck¦BlockAckReq)"" |
| --- | --- | --- | --- | --- | --- | --- |

Proposed reject.   D1.02 says: "The RDG/More PPDU field shall be set to the same value in all HT Control fields present in a PPDU”

| 2538 | 103 | 47 | 9.12 | This expression reduces to <a b ¦ c d> which is totally undefined by the notation and EBNF rules. | Rewrite this one within the rules. There are three cases: (1) +RD, then +implicit-bar; (2) +implicit-bar, then +RD, (3) +implicit-bar+RD. It would look like this: ({Data+QoS+ampdu} Data+QoS+RD+ampdu {Data+QoS[+RD]+ampdu} Data+QoS+implicit-bar[+RD]+ampdu {Data+QoS[+implicit-bar][+RD]+ampdu} +ampdu-end ¦ ({Data+QoS+ampdu} Data+QoS+implicit-bar+ampdu {Data+QoS[+implicit-bar]+ampdu} Data+QoS[+implicit-bar]+RD+ampdu {Data+QoS[+implicit-bar][+RD]+ampdu}) +ampdu-end ¦ ({Data+QoS+ampdu} Data+QoS+implicit-bar+RD+ampdu {Data+QoS[+implicit-bar][+RD]+ampdu}) +ampdu-end | Edit Notes (D1.05) ER: <The resolution disagrees with that in 11-06/1306r0, which was: Proposed counter. the <> notation has been removed. Where it served a grouping function, the ( ) notation is used.>, to resolution (D1.05): accept. Covered by CID 34. |
| --- | --- | --- | --- | --- | --- | --- |

Proposed counter.  the <> notation has been removed.  Where it served a grouping  function, the ( ) notation is used.  This change has been made in D1.05.

| 2541 | 104 | 7 | 9.12 | Use of "<>" notation incorrect, as there is only one choice within the <>'s | Replace the <> with () | Edit Notes (D1.05) ER: <The resolution disagrees with that in 11-06/1306r0, which was: Proposed accept.>, to resolution (D1.05): counter. the <> notation has been removed. Where it served a grouping function, the ( ) notation is used. |
| --- | --- | --- | --- | --- | --- | --- |

Proposed counter.  the <> notation has been removed.  Where it served a grouping  function, the ( ) notation is used.  This change has been made in D1.05.

| 2543 | 104 | 17 | 9.12 | unmatch parentheses | Suggest the matching right parenthesis belongs at end of line 19 | Edit Notes (D1.05) ER: <The resolution disagrees with that in 11-06/1306r0, which was: Proposed counter. The second term has been simplified, so the unmatched left parenthesis should be removed>, to resolution (D1.05): accept. |
| --- | --- | --- | --- | --- | --- | --- |

Proposed counter.  The second term has been simplified,  so the unmatched left parenthesis should be removed.

This change has already been made in D1.06.

| 2545 | 104 | 20 | 9.12 | The definition of PPDU-BA-RD-BAR doesn't match what the comment says it should be | fix either the comment or the definition | Edit Notes (D1.05) ER: <The resolution disagrees with that in 11-06/1306r0, which was: Proposed accept. Modified definition and comment to match changes proposed elsewhere. See also CID 34.>, to resolution (D1.05): counter. The second term has been simplified, so the unmatched left parenthesis should be removed |
| --- | --- | --- | --- | --- | --- | --- |

Proposed accept.  Modified definition and comment to match changes proposed elsewhere.  See also CID 34.  This change has already been made in D1.06.

| 2546 | 104 | 23 | 9.12 | unreferenced nonterminal psm-sequence | add a production using it | Edit Notes (D1.05) ER: <The resolution disagrees with that in 11-06/1306r0, which was: Proposed accept. Add psmp-sequence as an alternative term at the end of initiator-sequence.>, to resolution (D1.05): accept. Modified definition and comment to match changes elsewhere. See also CID 34. |
| --- | --- | --- | --- | --- | --- | --- |

Proposed accept.  Add psmp-sequence as an alternative term at the end of initiator-sequence.

This change has already been made in D1.06.

| 2547 | 104 | 23 | 9.12 | non-last-psmp should be allowed to repeat | 1*{non-last-psmp} | Edit Notes (D1.05) ER: <The resolution disagrees with that in 11-06/1306r0, which was: Proposed counter. "{non-last-psmp}" - i.e. there is no need for a lower limit of 1. >, to resolution (D1.05): accept. Add psmp-sequence as an alternative term at the end of initiator-sequence. |
| --- | --- | --- | --- | --- | --- | --- |

Proposed counter.  "{non-last-psmp}" - i.e. there is no need for a lower limit of 1.

This change has already been made in D1.06

| 2548 | 104 | 45 | 9.12 | This definition doesn't allow Data both before and after the MTBA in the A-MPDU | fix either the comment or the definition | Edit Notes (D1.05) ER: <The resolution disagrees with that in 11-06/1306r0, which was: Proposed reject. Changes being discussed in other ad-hocs limit the order of what goes into an A-MPDU so that any response frame occurs first.>, to resolution (D1.05): counter. "{non-last-psmp}" - i.e. there is no need for a lower limit of 1. |
| --- | --- | --- | --- | --- | --- | --- |

Proposed reject.  Changes being discussed in other ad-hocs limit the order of what goes into an A-MPDU so that any response frame occurs first.

| 4558 | 104 |  | 9.12 | Frame exchange sequence for link adaptation are not defined | Define frame exchange sequences based on 9.19.1,9.19.2,9.19.3 |  |
| --- | --- | --- | --- | --- | --- | --- |

Proposed accept.  D1.06 defines sequences in 9.12 for link adaptation, implicit and explicit beamforming.

| 4559 | 104 |  | 9.12 | Frame exchange sequence for MIMO power save are not defined | Define frame exchange sequences for MIMO power save | Edit Notes (D1.05) ER: <The resolution disagrees with that in 11-06/1306r0, which was: Proposed reject. These sequences do not attempt to illustrate MLME operation.>, to resolution (D1.05): counter. Exchanges showing aspects of implicit and explicit beamforming have been added as shown in the submission. |
| --- | --- | --- | --- | --- | --- | --- |

Proposed reject.  These sequences do not attempt to illustrate MLME operation

| 9993 | 104 |  | 9.12 | Add CTS-2-Self option to PSMP sequence. |  | Edit Notes (D1.05) ER: <The resolution disagrees with that in 11-06/1306r0, which was: Proposed reject. This is already generated by ht-TXOP-sequence from nav-protected-sequence, from nav-set>, to resolution (D1.05): accept |
| --- | --- | --- | --- | --- | --- | --- |

Proposed reject.  This is already generated by ht-TXOP-sequence from nav-protected-sequence, from nav-set

| 3613 | 149 | 7 | 11.5.2 | Another example of writing reams on a very obvious mechanism. OK, AP and station should establish Block Ack sessions if they have data; what is so special about PSMP that it needs a special mention? | Delete the entire subclause |  |
| --- | --- | --- | --- | --- | --- | --- |

Proposed accept.  (Note the subclause is 11.5.4 (Multi-TID Block Ack) in D1.06.

Submission note: The subclause contains only one normative specification in D1.06:  “A Block Ack agreement shall be set up after the related TS has been established.”.  This statement is unnecessary, because other rules require a BA agreement before data can be exchanged using BA,  and BA is the only mechanism for exchanging acknowledged data under PSMP.

| 1053 | 152 |  | 11.5.2 | change the section from informative to normative | change the section from informative to normative |  |
| --- | --- | --- | --- | --- | --- | --- |

Proposed counter.   Remove the entire subclause as described in CID 3613

| 2855 | 156 | 3 | 11.16 | 11.16 is a MAC layer function, not really MAC Management function. It would be better placed in clause 9 | Make change indicated in comment | Defer- Needs more discussions Ed AdHoc: Transfer to Gen AdHoc GenAdHoc: Agrees that this needs more discussion and transfers to MAC |
| --- | --- | --- | --- | --- | --- | --- |

Proposed accept.  Move the entire 11.17 (D1.06) to a new subclause 9.21

| 10519 | 156 | 3 | 11.16 | 11.16 is a MAC layer function, not really MAC Management function. It would be better placed in clause 9 | as in comment | Defer - Refer to CID-2855 Ed AdHoc: Transfer to Gen AdHoc GenAdHoc: Transfer to MAC |
| --- | --- | --- | --- | --- | --- | --- |

Proposed duplicate of 2855

| 7489 | 156 |  | 11.16 | It seems that the operation of PCO is a bit like the operation of TSF or beaconing, and should be in section 9. | Find it a new home in section 9. | Defer - Needs more discussions Ed AdHoc: Transfer to Gen AdHoc GenAdhoc: Transfer to MAC |
| --- | --- | --- | --- | --- | --- | --- |

Proposed duplicate of 2855