IEEE P802.11
Wireless LANs

| Text Proposal for LB84 Comment Resolution CID 976 Related to the SNR Definition |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-11-09 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Vinko Erceg | Broadcom | 15435 Innovation Dr. San Diego, CA 92128 | 1 858 521 5885 | verceg@broadcom.com |
| Joonsuk Kim | Broadcom |  |  | joonsuk@broadcom.com |
| Assaf Kasher | Intel |  |  | assaf.kasher@intel.com |

Abstract

This document includes proposed text for modification of the TGn 1.05 draft. The text is related to the LB84 Comment IDs 976 related to the SNR definition.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Introduction

This document addresses CID related to the definition of the SNR in Tables n16 and n17 related to the CSI matrices report.

# CID 4006

| SNR in the table is not defined (two places) | Define SNR as: dB value of the linear power average. |
| --- | --- |

Proposed Resolution: Accept

Reason for acceptance:

In Tables n16 and n17 SNR values in channels 1-Nr are not defined. Common approach is to define SNR as a linear power average converted to dB value.

TGn editor: Add the following sentences on pg 48, ln 37, Draft Version 1.05:

“The SNR values in Tables n16 and n17 are defined as dB values of linear power average.  The representation is in 8-bit two’s complement numbers from -64 to 63.5 in 0.5 dB steps.”