IEEE P802.11
Wireless LANs

| Text Proposal for LB84 Comment Resolution Related to the SNR Definition |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-11-08 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Vinko Erceg | Broadcom | 15435 Innovation Dr. San Diego, CA 92128 | 1 858 521 5885 | verceg@broadcom.com |
| Joonsuk Kim | Broadcom | 190 Mathilda Place, Sunnyvale, CA94086 | 1 408 543 3455 | joonsuk@broadcom.com |
| Assaf Kasher | Intel |  |  | assaf.kasher@intel.com |

Abstract

This document includes proposed text for modification of the TGn 1.06 draft. The text is related to the LB84 Comment following CIDs related to the SNR definition.

- CID 976, CID 980, CID 4752

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Introduction

This document addresses CID related to the definition of the SNR in Tables n14, n15, n17, n18, n<xx> and n<yy> related to the explicit feedback report.

# CID 976 & CID 980 & CID 4752

| SNR in the table is not defined (two places) | Define SNR as: dB value of the linear power average. |
| --- | --- |
| Average signal to noise ratio is not defined | Average SNR is defined as: Arithmetic mean of dB values over tones |
| Table n40. First Meaning row contains the phrase 'Average Signal to Noise Ratio' | How is this average SNR calculated? |

Proposed Resolution: Counter

Reason for acceptance:

In Tables n14 and n15 for CSI feedback, Table n17 and n18 for Non-Compressed feedback and Table n<xx> and n<yy> for Compressed feedback, SNR values are not defined. Common approach is to define SNR as a linear power average converted to dB value.

-Note: n<xx> and n<yy> are new Table numbers assigned in the accepted document 06/1626r1

Table <xx> – Compressed Beamforming Matrices Report fixed field (20 MHz)

Table <yy> – Compressed Beamforming Matrices Report fixed field (40 MHz)

TGn editor: Add the following sentences on pg 48, ln 24, Draft Version 1.06:

“The SNR values in Tables n14 and n15 are defined as dB values of linear power average over tones.

The representation is in 8-bits two’s complement value of 4 x (SNR-22) to cover SNR range from -10 to 53.75 in 0.25 dB steps.”

TGn editor: Add the following sentences on pg 50, ln 36, Draft Version 1.06:

“The SNR values in Tables n17 and n18 are defined as dB values of linear power average over tones. The SNR in stream i corresponds to the SNR associated with the ith column of Beamforming matrix V. Each SNR corresponds to the predicted SNR at Beamformee when beamformer applies the matrix V. The representation is in 8-bits two’s complement value of 4 x (SNR-22) to cover SNR range from -10 to 53.75 in 0.25 dB steps.”

TGn editor: Add the following sentences after Table <yy> in 06/1626r1:

“The SNR values in Tables n<xx> and n<yy> are defined as dB values of linear power average over tones.  Each SNR per tone in stream i before averaged corresponds to the SNR associated with the ith column of Beamforming matrix V found at Beamformee. Each SNR corresponds to the predicted SNR at Beamformee when beamformer applies the matrix V. The representation is in 8-bits two’s complement value of 4 x (SNR-22) to cover SNR range from -10 to 53.75 in 0.25 dB steps.”