IEEE P802.11
Wireless LANs

| TGn LB84 Submission to Resolve CID 693: Further clarification of the Shortening Step for the Low-Density Parity Check (LDPC) PPDU Encoding Process |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-11-10 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| George Vlantis | STMicroelectronics, Inc. | 1060 East Brokaw Road, San Jose CA 95131 | +1.408.451.8109 (o) +1.408.893.9357 (m) | George.Vlantis@st.com |

Abstract

This document contains proposed changes to the IEEE P802.11n Draft to address the following LB84 comments:

- CID 693

The changes marked in this document are based on TGn Draft version P802.11n-D1.06.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# CID 693

## Background

At the July2006 Plenary meeting in San Diego, I counter-proposed the following resolution to CID #693, in order to clarify the LDPC shortening step, paragraph (c). While reviewing the entire “LDPC PPDU encoding process”, subclause 21.3.10.6.5 of Draft 1.06, I discovered that this paragraph requires a further clarification, as it does not explicitly specify when the LDPC encoder should be applied within the shortening step (c).  By way of background, when shortening applies, zero bits are appended to the unencoded information bits to provide enough information bits to fill the codeword, the LDPC encoder encodes the codeword, and then the zero bits are removed from the resulting encoded codeword.  When shortening does not apply, paragraph (c) should simply invoke LDPC encoding.  The 802.11n drafts with and without the proposed modification for CID 693 have this defect.  However, the PPDU encoding process figure, Figure n77 in Draft 1.06, does illustrate the process correctly.  Below is the CID with the motioned resolution and Figure n77.

| CID | Name of Commenter(Ed) | Clause Number(C) | Page(C) | Line(C) | Type of Comment T/E(C) | Part of No Vote(Y/N) | Page(Ed) | Line(Ed) | Clause Title(Ed) | Clause(Ed) | Type E/HE/T/ST/DT(Ed) | Duplicate of CID | Resn Status | Assignee | Submission | TGn approval |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 936 | Erceg, Vinko | 20.3.4.3.3.4 | 202 | 5 | E | N | 202 | 5 | LDPCC PPDU encoding process | 20.3.4.3.3.4 | T |  | C | George Vlantis |  |  |

| Comment | Proposed Change | Resolution | Edited in draft |
| --- | --- | --- | --- |
| Description of shortening procedure confusing. | The shortening bits shall be equally distributed over all NCW codewords with the first rem(N_shrt, N_CW) codewords being shortened one more bit than the remaining codewords. Define N_SPCW = floor(N_shrt, N_CW). Then, the shortening is performed by setting information bits (i(k-N_spcw-1),…i(k-1)) to 0 in the first rem(N_shrt, N_CW) codewords and setting information bits (i(k-N_spcw),…,i(k-1)) to 0 in the remaining codewords. The shortened bits shall be discarded after encoding. | Counter: Replace Lines #5-8 with: "When N_shrt <= 0, shortening is not performed, so skip the remainder of this paragraph and continue to paragraph d). The shortening bits shall be equally distributed over all N_CW codewords with the first rem(N_shrt, N_CW) codewords being shortened one more bit than the remaining codewords. Define N_spcw = floor(N_shrt, N_CW). Then, the shortening is performed by setting information bits (i(k-N_spcw-1),…i(k-1)) to 0 in the first rem(N_shrt, N_CW) codewords and, provided N_shrt > 0, setting information bits (i(k-N_spcw),…,i(k-1)) to 0 in the remaining codewords. The shortened bits shall be discarded after encoding." |  |

LDPC Encoding

(concatenate parity)

Data

Bits

Shortened

Bits

Data

Bits

(c)

Parity

Bits

Shortened

Bits

Data

Bits

Parity

Bits

Data

Bits

(Discard Shortened Bits)

(d)

(Discard Punctured Bits)

Parity

Bits

Data

Bits

Repeat

Bits

Parity

Bits

Data

Bits

Copy Repeat Bits

(e)

Figure n77—LDPC PPDU encoding, padding, and puncturing of a single codeword

## Editorial Instructions

Replace Lines 51 to 62 of paragraph (c) in subclause 21.3.10.6.6 “LDPC PPDU encoding process” in TGn Draft version P802.11n-D1.06 as follows:

When N_shrt <= 0, shortening is not performed, so skip the remainder of this paragraph and continue to paragraph d) .  The When N_shrt > 0, shortening bits shall be equally distributed over all N_CW codewords with the first rem(N_shrt, N_CW) codewords being shortened one more bit than the remaining codewords.  Define N_spcw = floor(N_shrt, N_CW).  Then, when N_shrt > 0, the shortening is performed by setting information bits (i(k-N_spcw-1),…i(k-1)) to 0 in the first rem(N_shrt, N_CW) codewords and, provided N_shrt > 0, setting information bits (i(k-N_spcw),…,i(k-1)) to 0 in the remaining codewords.  For all values of N_shrt, encode each of the N_CW codewords using the LDPC encoding technique described in 21.3.10.6.2 through 21.3.10.6.4.  The When N_shrt > 0, the shortened bits shall be discarded after encoding.

## References

## [1] IEEE Std. 802.11, 1999 Edition, "Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications"

## [2] IEEE Std. 802.11a-1999, "Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer in the 5 GHz Band"

## [3] IEEE Std. 802.11-2003, "IEEE Wireless LAN Edition, A compilation based on IEEE Std. 802.11-1999 (R2003) and its amendments"