IEEE P802.11
Wireless LANs

| TGn LB84 Submission to address some CIDs |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-11-10 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Naveen Kakani | Nokia, Inc | 6021 Connection Dr, Irving TX 75039 | +1-214-763-1041 | Naveen.kakani@nokia.com |
| Adrian Stephens | Intel Corporation | 15 JJ Thompson Avenue, Cambridge, CB3 0FD, United Kingdom | +44 1223 763457 | adrian.p.stephens@intel.com |

Abstract

This document contains proposed changes to the IEEE P802.11n Draft to address the following LB84 comments: CID’s 3588, 3589, 7593

The changes marked in this document are based on TGn Draft version D1. 06

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# Introduction

## Interpretation of a Motion to Adopt

A motion to approve this submission means that the editing instructions and any changed or added material are actioned in the TGn Draft.  This introduction, is not part of the adopted material.

Editing instructions formatted like this are intended to be copied into the TGn Draft (i.e. they are instructions to the 802.11 editor on how to merge the TGn amendment with the baseline documents).

TGn Editor:  Editing instructions preceded by “TGn Editor” are instructions to the TGn editor to modify existing material in the TGn draft.   As a result of adopting the changes, the TGn editor will execute the instructions rather than copy them to the TGn Draft.

Summission Note: Notes to the reader of this submission are not part of the motion to adopt.  These notes are there to clarify or provide context.

# Proposed Resolutions

4. Abbreviations and acronyms

Insert the following new abbreviations and acronyms into clause 4 in alphabetical order:

### RMB	Reliable Multicast and Broadcast

### 7.2.1.7 (BlockAckReq) frame format

Replace Figure 31 with the following figure:

- The changes comprise the addition of BAR Ack Policy, Multi-TID and Compressed BA fields from the reserved field and renaming the TID field to TID_INFO.

| B0 | B1 | B2 | B3 | B4 B11 | B12 B15 |
| --- | --- | --- | --- | --- | --- |
| BAR (Ed: CID 1956) Ack Policy | Multi-TID | Compressed BA | MB | Reserved | TID_INFO |
| Bits: 1 | 1 | 1 | 1 | 8 | 4 |

Figure 31: BAR Control field

Table nxx: BlockAckReq Frame MB Field encoding

| MB | BlockACK Req |
| --- | --- |
| 1 | Basic/Multi-TID Block ack req corresponds to a Multicast/Broadcast stream |
| 0 | Basic/Multi-TID Block ack req for unicast streams |

- Block Ack (BlockAck) frame format

Replace Figure 32 (BA control field) with the following figure:

- the changes comprise adding MB field.:

| B0 | B1 | B2 | B3 | B4 B11 | B12 B15 |  |
| --- | --- | --- | --- | --- | --- | --- |
| BA (Ed: CID 1956) Ack Policy | Multi-TID | Compressed BA | MB | Reserved | TID_INFO |  |
| Bits: 1 | 1 | 1 | 1 | 8 | 4 |  |
| - BA Control Field |  |  |  |  |  |  |

Table nyy: BlockAck Frame MB Field encoding

| MB | BlockACK Req |
| --- | --- |
| 1 | Basic/Multi-TID Block ack corresponds to a Multicast/Broadcast stream |
| 0 | Basic/Multi-TID Block ack for unicast streams |

Block Ack (BlockAck) frame format

### 7.3.1.14 Block Ack Parameter Set field

Replace Figure 33.6 (Block Ack Parameter Set fixed field) with the following figure:

- the changes comprise adding MB fiel.:

| B0 | B1 | B2 – B5 | B6 - B15 |
| --- | --- | --- | --- |
| MB | Block ACK policy | TID | Buffer Size |
| Bits: 1 | 1 | 4 | 10 |

Figure 33.6 – Block Ack Parameter Set fixed field

When MB field is set to “1”, the Block ACK Starting Sequence Control field shall indicate the LSB’s of the Multicast /Broadcast Address. When MB field is set to “0”, the Block ACK Starting Sequence Control field shall have the same meaning as defined in the baseline.

7.3.2.46.2 HT Capabilities Info field

- the changes comprise adding RMB field:

The HT Capabilities Info field is 2 octets in length, and contains capability information bits. The structure of this field is defined in HT Capabilities Info field.

| B0 | B1 |  | B2 B3 |  | B4 |  | B5 |  | B6 | B7 |  | B8 B9 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LDPC coding capability | Supported channel width set |  | SM Power Save |  | Greenfield |  | Short GI for 20 MHz |  | Short GI for 40 MHz | Tx STBC |  | Rx STBC |  |
| B10 |  | B11 |  | B12 |  | B13 |  | B14 |  |  | B15 |  |  |
| HT-delayed BlockAck |  | Maximum A-MSDU length |  | DSSS/CCK Rates in 20/40 MHz Mode (Ed: CID 771) |  | PSMP support |  | RMB support |  |  | L-SIG TXOP Protection support |  |  |
| - HT Capabilities Info field |  |  |  |  |  |  |  |  |  |  |  |  |  |

| - Subfields of the HT Capabilities Info fieldinput.doc |  |  |
| --- | --- | --- |
| Subfield | Definition | Encoding |
| RMB | Reliable Multicast Broadcast support | Set to 0 if not supported Set to 1 if supported |

- Add new subsection to include Block ACK for Multicast and Broadcast streams:

    - BlockAck setup and operation for Multicast Broadcast streams

A Block ACK can be setup for  Multicast Broadcast streams only in BSS mode of operation

9.10.9.1 BA setup for Multicast and Broadcast Streams

A non-AP STA shall signal to the AP that it is part of Multicast/Broadcast group by transmitting ADDBA Request frame with “MB” field in the Block ACK parameter set, set to “1”. The AP-STA can initiate a BA set up with the non-AP STA in response to the received ADDBA Request Frame. If the AP initiates a BA setup in response to the received ADDBA Request frame, then the ADDBA request frame from the AP shall have the “MB” field in the Block ACK parameter set, set to “1”.. The ADDBA Response frame from the non-AP STA in response to the received ADDBA Request frame from the AP shall have the “MB” field in the Block ACK parameter set, set to “1”.

9.10.9.2 Block ACK Operation

Rules at the transmitter (AP STA):

  - The starting sequence number is set to the starting sequence number of the first BA/MTBA frame received for the TID

  - BA/MTBA can be requested implicitly or explicitly

  - Explicit request: In the explicit request frame BAR/MTBAR, Bit 3 in the BA control field shall be set to “1” to indicate that the request is for MB stream,, and the TID corresponds to the TID of the MB stream at the RX STA

  - Window updates are done as per the rules of regular BA

  - Decision to retransmit the data (either using multicast address or unicast address) is at the discretion of the transmitter

Rules at the recipient (non-AP STA):

  - - The starting sequence number is set to the sequence number of data frame received after setting up the BA

  - BA/MTBA frames that carry the bitmap of MB stream shall have Bit 3 in BA control field set to “1”, and the TID value corresponds to the MB stream that was signaled for this STA for the specific MB stream

  - When the window is full and there is new data received the starting sequence number of the Bit Map is shift to accommodate the new data

  - Window updates are done as per the rules of regular BA if data is received addressed to the multicast address or if data for the TID is transmitted as a unicast frame

9.15.2.8.1 Rules at the AP

Editorial: Modify the text in the subsection as below:

The DA of the PSMP shall be set to the multicast address if the PSMP contains a

PSMP-DTT for this multicast address and the rest of the PSMP-DTT’s and PSMP-UTT’s are allocated to STAs that are recepients of the Multicast date transmitted in the PSMP-DTT for the multicast address

.