IEEE P802.11
Wireless LANs

| TGn LB84 Submission 40MHz operation in the 2.4GHz Band |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-10-12 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Assaf Kasher | Intel Corporation | Matam Industrial Park Haifa 31015, Israel | +972-4-8651547 | Assaf.kasher@intel.com |

Abstract

This document contains proposed changes to the IEEE P802.11n Draft to address the following LB84 comments:

- 10380, 3109, 10902, 426, 12036, 2056,  7193, 7315

The changes marked in this document are based on TGn Draft version 1.05.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# Introduction

## Interpretation of a Motion to Adopt

A motion to approve this submission means that the editing instructions and any changed or added material are actioned in the TGn Draft.  This introduction, is not part of the adopted material.

Editing instructions formatted like this are intended to be copied into the TGn Draft (i.e. they are instructions to the 802.11 editor on how to merge the TGn amendment with the baseline documents).

TGn Editor:  Editing instructions preceded by “TGn Editor” are instructions to the TGn editor to modify existing material in the TGn draft.   As a result of adopting the changes, the TGn editor will execute the instructions rather than copy them to the TGn Draft.

Summission Note: Notes to the reader of this submission are not part of the motion to adopt.  These notes are there to clarify or provide context.

# CID : 10380, 3109, 10902, 426, 12036, 2056,  7193, 7315.

TGn editor: Add the following fields to figure n44

|  |  |  |  | B0 B1 | B2 | B3 | B4 | B5 B7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Element ID | Length | Primary Channel | Secondary Channel Offset | STA Channel Width | RIFS Mode | PSMP Access only | Service Interval Granularity |
| Octets: | 1 | 1 | 1 | 1 |  |  |  |  |

| B0 B1 | B2 | B3 B6 | B7 B9 | B10 B12 | B13 B15 |
| --- | --- | --- | --- | --- | --- |
| Operating Mode | Non-GF STAs Present | Secondary Channel Activity Threshold | Secondary Channel Activity Detection Period | Secondary Channel Recovery Wait Time | Reserved |
| 2 |  |  |  |  |  |

| B0 B6 | B7 | B8 | B9 | B10 | B11 | B12 B15 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Basic STBC MCS | Dual CTS Protection | Secondary Beacon | L-SIG TXOP Protection Full Support | PCO Active | PCO Phase | Reserved | Basic MCS Set |
| 2 |  |  |  |  |  |  | 16 |

Figure n44—HT Information element format

TGn editor: Add the following lines to table n27

| Field | Definition | Encoding |
| --- | --- | --- |
| Secondary Channel Activity Threshold | Indicates the minimum level of Secondary Channel non-BSS activity that will cause a switch to 20MHz. The level is expressed as a percentage of time active as defined in 11.9.8.4. | Set to 0 for 2% Set to 1 for 4% Set to 2 for 6% Set to 3 for 8% Set to 4 for 10% Set to 5 for 12% Set to 6 for 14% Set to 7 for 16% |
| Secondary Channel Activity Detection Period | Indicates the period over which non-BSS activity in the secondary channel is detected | Set to 0 for 1s Set to 1 for 2s Set to 2 for 4s Set to 3 for 6s Set to 4 for 8s Set to 5 for 10s Set to 6 for 14s Set to 7 for 18s |
| Secondary Channel Recovery Wait Time | Indicates the minimum period that a STA waits after it has moved to 20MHz because of secondary channel non-BSS activity before returning to 40MHz channel width. | Set to 0 for 5s Set to 1 for 10s Set to 2 for 20s Set to 3 for 40s Set to 4 for 80s Set to 5 for 160s Set to 6 for 320s Set to 7 for 640s |

TGn Editor Add the following text after line 15 in page 39 in subclause 7.3.1.23

When the channel width is set to 2 indicating a static change of the channel to the 20MHz primary channel, the station will use the 20MHz channel and not be able to receive 40MHz packets.

TGn editor: Insert the next paragraph at the end of 9.20.1(page 145)

A 40/20MHz capable station operating in the 2.4GHz band shall switch to 20MHz mode according to the rules specified in 11.9.8.4

TGn editor: insert the following subclause after subclause 11.9.8.3 in draft D1.05

Insert the following new subclause:

11.9.8.4 Channel width switching in the 2.4GHz band.

A non-AP STA operating in 20/40 MHz mode in the 2.4 GHz band shall switch to 20 MHz mode if the total  duration of time with PHY_CCA.indicate(BUSY) signaled for the secondary channel exceeds Secondary Channel Activity Threshold percent of the preceeding Secondary Channel Activity Detection Period seconds, where the PHY_CCA.indicate(BUSY) events are caused by transmissions that are not part of a 40 MHz transmission in the same BSS. When switching to 20 MHz mode, the station shall transmit a Notify Channel Width with the channel width set to 2.  After the AP transmits an ACK to the STA, no 40MHz transmission shall be sent to the STA until its set its channel width to 40MHz using Notify Static Channel Width Frame.  When the STA has set its static channel width to 2, it cannot receive or transmit 40MHz transmissions. A STA that has set its static channel width to 2, may only transmit PPDUs that conform to the 20MHz mask specified in 21.3.20.1. The STA may return to transmission using the 40 MHz channel and set its recommended channel width to 40MHz Secondary Channel Recovery Wait Time seconds after switching to 20 MHz operation.

An AP operating a 20/40 MHz BSS in the 2.4 GHz band shall switch the BSS to 20 MHz mode following the procedure of 11.10.7.1 (Selecting and advertising a new channel in an infrastructure BSS) if the total duration of time with PHY_CCA.indicate(BUSY) signaled for the secondary channel exceeds Secondary Channel Activity Threshold percent of the preceeding Secondary Channel Activity Detection Period seconds, where the PHY_CCA.indicate(BUSY) events are caused by transmissions that are not part of a 40 MHz transmission in the same BSS. The AP may return the BSS to 20/40 MHz mode Secondary Channel Recovery Wait Time seconds after switching to 20 MHz mode.

Annex D –

TGn Editor Insert the following into the dot11OperationTable:

dot11SecondaryChannnelActivityThreshold,

dot11SecondaryChannnelActivityDetectionPeriod,

dot11SecondaryChannnelOutBSSWaitTime

TGn editor: insert the following MIB entries to Annex D

dot11SecondaryChannnelActivityThreshold OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“Indicates the minmum percentage of Secondary channel  outside BSS transmission that will cause a switch to 20MHz.  Allowed values are 0-7.  A value of n indicates 2(n+1)%.  Default value is 6, representing 14%”

::= { dot11OperationEntry 33 }

dot11SecondaryChannnelActivityDetectionPeriod OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“Indicates the period over which outside BSS activity in the secondary channel shall be detected. Allowed values are 0-7. The values correspond to [1 2 4 6 8 10 14 18] seconds.  Default value is is 5, representing 10s”

::= { dot11OperationEntry 34 }

dot11SecondaryChannnelRecoveryWaitTime OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“Indicates the minimum period in which a STA should wait after it has moved to 20MHz because of secondary channel Outside BSS activity before returning to 40MHz channel width.. Allowed values are 0-7.   A value of n indicates 50×2(n+1) seconds.  Default value is 5, representing 160s”

::= { dot11OperationEntry 33 }