IEEE P802.11
Wireless LANs

| Proposed text to amend IEEE 802.11 for Medium Congestion Control |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-11-14 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Jason Hunzinger | Denso International America, LA Laboratories | 3252 Business Park Drive Vista, CA 92081 | +1 760 597 7420 | Jason_hunzinger@denso-diam.com |
| Bryan Wells | Denso International America, LA Laboratories | 3252 Business Park Drive Vista, CA 92081 | +1 760 597 7448 | Bryan_wells@denso-diam.com |

Abstract

This document contains proposed normative text to amend IEEE 802.11 for Medium Congestion Control.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Contents

The following sections are amended:

3. Definitions	1

4. Abbreviations and acronyms	1

5. General description	1

5.3 Logical Service Interfaces	1

5.3.1 Station service (SS)	1

5.4 Overview of the services	1

5.4.8 Medium congestion control services	2

5.4.8.1 Measurement	2

5.4.8.2 Control	2

10. Layer Management	2

10.3 MLME SAP Interface	2

10.3.41 Medium congestion control protocol layer	2

10.3.42 Medium Congestion Control	7

10.3.42.1 MLME-LOCALMCC.request	7

10.3.42.1.1 Function	7

10.3.42.1.2 Semantics of the service primitive	7

10.3.42.1.3 When generated	10

10.3.42.1.4 Effect of receipt	10

10.3.42.2 MLME-LOCALMCC.confirm	10

10.3.42.2.1 Function	10

10.3.42.2.2 Semantics of the service primitive	10

10.3.42.2.3 When generated	11

10.3.42.2.4 Effect of receipt	11

10.3.42.3 MLME-LOCALMCC.indication	11

10.3.42.3.1 Function	11

10.3.42.3.2 Semantics of the service primitive	11

10.3.42.3.3 When generated	12

10.3.42.3.4 Effect of receipt	12

11. MLME	13

11.7 Medium Congestion Control	13

11.7.1 Measurement procedures	13

11.7.1.1 Procedures	13

11.7.1.2 Direct Measurements	13

11.7.1.3 Derived Measurements	14

11.7.2 Control procedures	15

11.7.2.1 Default Control Values	15

11.7.2.2 Basic Control Procedures	16

11.7.2.3 Basic Controls	16

11.7.2.4 Adaptive Control Procedures	17

11.7.2.5 Adaptations	17

List of Figures

Insert the following entries and renumber as appropriate:

Figure 1 -	Layer management model: MCC Active Role	3

Figure 2 -	Layer management model: MCC Passive Role	4

Figure 3 -	MCC Active Control	5

Figure 4 -	MCC Passive Control	6

List of Tables

Insert the following entries and renumber as appropriate:

Table 1	AC Mask Encoding	7

Table 2	MCC Measurements	8

Table 3	MCC Condition Types	9

Table 4	MCC Controls	9

Table 5	MCC Control Type	9

Table 6	MCC Control Values	10

Table 7	MCC Direct Measurements	14

Table 8	MCC Derived Measurements	15

Table 9	MCC Default Control Values	16

Table 10	MCC Mandatory and Optional Controls	17

Table 11	MCC Valid Control Types	18

# Definitions

Insert the following clauses and renumber as appropriate:

- 3.v.1 derivative measurement: A measurement calculated based on one or more direct measurements. A derivative measurement is not a direct measurement and therefore is not the value of a MAC or PHY parameter.

- 3.v.2 direct measurement: A measurement of the value of a MAC or PHY parameter.

- 3.v.3 medium congestion control (MCC): A set of services including measurement of parameters potentially indicating channel congestion level, control of parameters potentially impacting channel congestion, and adaptation by applying controls based on measurements.

# Abbreviations and acronyms

Insert the following new acronyms in alphabetical order:

ADR	aggregate data rate

AFT	aggregate frame rate

FDE	frame drop event

FR	frame rate

MCC	medium congestion control

MCW	mean contention window index

MDR	mean drop rate

MFP	MAC frame parameters

MRC	mean retry count

NARL	network activity rate level

QL	queue level

QS	queue size

RC	retry count

SD	source diversity

# General description

## Logical Service Interfaces

Insert the item at the end of the list of architectural services in 5.3:

o)	Medium congestion control

### Station service (SS)

Insert the item at the end of the list of station services (SSs) in 5.3.1:

j)	Medium congestion control

## Overview of the services

Insert the following at the end of the first paragraph:

One of the services is used for medium congestion control.

Insert the following new clause after 5.4.7.

### Medium congestion control services

Two basic services are required for IEEE 802.11 to provide Medium Congestion Control (MCC) functionality: measurement of congestion level and control of congestion level.

A client may request a measurement, receive results, and then determine and request an appropriate control. A client may also request the MAC automate this process by periodically invoking a specified measurement and, depending on the result and specified criteria, applying a specified control.

MCC services do not include flow control.

#### Measurement

To determine appropriate control of congestion, a client requires information about the congestion level. Measurements of parameters relating to congestion control are invoked by a client or automatically as a result of an adaptation requested by a client.

Measurement involves: (1) taking direct measurements within a wireless communication protocol stack, (2) deriving indirect measurements from direct measurements, and (3) reporting those derivative measurements to the client. Derivative measures are measurements that a communication layer computes or determines through additional processing of one or more direct measures. Only derivative measures shall be reportable to upper layers.

Measures either indicate a level of wireless medium congestion or an effect of wireless medium congestion.

MCC measurement is independent of channel measurement for spectrum management.

#### Control

To impact congestion, a client requires a means to request changes at the MAC layer.  Control of MAC parameters relating to functions that potentially impact congestion is invoked by a client or automatically as a result of an adaptation request by a client.

Control involves potential action to address over-utilization or underutilization and potential reporting of action taken to an upper layer. There are two types of controls: basic and adaptive.

Basic controls provide a mechanism for a client to set or adjust parameters that can impact the STA’s contribution to the amount of congestion on the medium. Basic controls thus provide the client with manual controls. However, basic controls can also be operated by an adaptation requested by the client.

To automate congestion control a client may request the MAC adapt parameters. One or more controls can be applied based on the result of one or more measurements. A client may request an adaptation by specifying (1) a derivative measure, (2) a control, (3) an adaptation (how to change the control), (4) a threshold (how to trigger the adaptation to the control). An adaptation is executed by the MAC without requiring further input from a client.

# Layer Management

## MLME SAP Interface

Insert a new clause after Clause 10.3.40.

### Medium congestion control protocol layer

The layer management extensions for Medium Congestion Control assume a portioning of functionality between the MLME and SME where the SME takes an active or passive decision making role.

In the active role, the SME makes each control decision based on a measurement policy. The MLME performs the measurements and delivers the results to the SME. The MLME also executes the control commands from the SME.  Figure 1 illustrates the layer management in the active role.

In the passive role, the SME makes an adaptation decision based on control and measurement policies. The MLME executes measurements and controls based on instructions received from the SME. In the passive role, the SME may instruct the MLME once and leave the management of measurement and control to the MLME.  Figure 2 illustrates the layer management in the passive role.

The SME may operate in active or passive role, both simultaneously, or alternate between roles.

- Layer management model: MCC Active Role

- Layer management model: MCC Passive Role

The informative diagrams within this subclause further illustrate the Medium Congestion Control model adopted. Figures 3 and 4 illustrate a typical exchange between SME and MLME for in an Active Role and Passive Role respectively.

- MCC Active Control

- MCC Passive Control

### Medium Congestion Control

#### MLME-LOCALMCC.request

##### Function

This primitive requests one or more local congestion measurements and or controls.

##### Semantics of the service primitive

The primitive parameters are as follows:

MLME-LOCALMCC.request	(

Dialog Token,

Periodicity,

Report Period,

Channel,

AC Mask,

Number of Measure Control Entry Pairs,

Measure Control Entry Pair(s)

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Dialog Token | Integer | 0 – 255 | The Dialog Token to identify the requested transaction. (0 = cancel all previous requests.) |
| Periodicity | Integer | 0 – 65535 | Periodicity in [ms]. Indicates how frequently to perform the request. 0 = continuously. 65534 = only once. 65535 = continuously while awake. |
| Report Period | Integer | 0 – 255 | Divider. (0 = no reports, 1 = report every period, 2 = report every-other period, etc.) |
| Channel | Integer | As defined in 17.3.8.3.3 | Specifies the channel number |
| AC Mask | AC Mask Encoding | 0-0xf | Specifies the Access Categories |
| Number of Measure Control Entry Pairs | Integer | 0 – 255 | Length of list. 0 = cancel previous request for Channel & AC. |
| Measure Control Entry Pair(s) | List of Measure Control Entry Pair(s) | See below. | See below. |

- AC Mask Encoding

| Bit | AC |
| --- | --- |
| 0 | AC_BE |
| 1 | AC_BK |
| 2 | AC_V1 |
| 3 | AC_V0 |

Measure Control Entry Pair = 	(

Measurement,

Condition Type,

Condition Value,

Control,

Control Type,

Control Value

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Measurement | Enumeration, see Table 2. | See Table 2 | See Table 2. Null = Basic Control. |
| Condition Type | Enumeration, see Table 3. | See Table 3 | Indicates the type of comparison between the Measurement and the Control Value. Ignored if Measurement = null. |
| Condition Value | See Table 2. | See Table 2 | Indicates the value to compare the Measure against. Ignored if Measurement or Condition Type = null. |
| Control | Enumeration, see Table 4. | See Table 4 | See Table 4. Null = Measurement only. |
| Control Type | Enumeration, see Table 5. | See Table 6 | See Table 5. Ignored if Control = null. |
| Control Value | (varies, see Table 4) | (varies, see Table 4) | Indicates the amount to change the Control Value (for increment or decrement), or the new Control Value setting. See Table 4. Ignored if Control or Control Type = null. |

- MCC Measurements

| Measurements | Type | Units |
| --- | --- | --- |
| Null | N/A | N/A (no measurement) |
| Receive Frame Rate (RX/FR) | Integer | Frames / second |
| Transmit Frame Rate (TX/FR) | Integer | Frames / second |
| Aggregate Frame Rate (Utilization) (AFR) | Integer | Frames / second |
| Receive Data Rate (RX/DR) | Integer | Kbits/second |
| Transmit Data Rate (TX/DR) | Integer | Kbits/second |
| Aggregate Data Rate (Utilization) (ADR) | Integer | Kbits/second |
| Queue Level (QL) | Integer | Kbits |
| Mean Retry Count (MRC) | Integer | Retry count |
| Mean Frame Drop Rate (MDR) | Integer | Frames / second |
| Mean Contention Window Index (MCW) | Integer | CW Index |
| Network Activity Rate Level (NARL) | Integer | Fraction of busy slots or TUs (multiplied by 10,000) |
| Source Diversity (SD) | Integer | Unique STAs |

- MCC Condition Types

| Parameter | Description |
| --- | --- |
| Null | No condition. |
| Less Than | Determine if Measure is < Threshold. |
| Less Than or Equal | Determine if Measure is < Threshold. |
| Equal | Determine if Measure is = Threshold. |
| Greater Than or Equal | Determine if Measure is > Threshold. |
| Greater Than | Determine if Measure is > Threshold. |

- MCC Controls

| Control |  | Type |  | Units |
| --- | --- | --- | --- | --- |
| Null | N/A |  | N/A |  |
| CWmin |  | Integer |  | See 802.11e |
| CWmax |  | Integer |  | See 802.11e |
| TXOPLimit |  | Integer |  | See 802.11e |
| AIFSN |  | Integer |  | See 802.11e |
| SuspendAC |  | Boolean |  | 0 (normal), 1 (suspend) |
| SuspendACDuration |  | Integer |  | [ms] |
| FlushACQueue |  | Boolean |  | 0 (normal), 1 (flush, then revert to state 0) |
| Delay |  | Integer |  | [us] |
| Throttle |  | Integer |  | [us] |
| AccessTimeConstraint |  | Integer |  | Numerator in fraction N/255 (percentage) |
| ShortRetryLimit |  | Integer |  | Maximum number of attempts. |
| LongRetryLimit |  | Integer |  | Maximum number of attempts. |
| RTSThreshold |  | Integer |  | [bytes] |
| AlwaysBackoff |  | Boolean |  | 0 (normal), 1 (force backoff) |
| CWReversion |  | Integer |  | Number of successful transmissions before reversion. |
| CWIncrement |  | Integer |  | CW Index Step |
| SuspendChannel |  | Integer |  | (mask, 1 = enabled) |

- MCC Control Type

| Parameter | Description |
| --- | --- |
| Null | No control. |
| Increment | Increment the specified control by the value provided. |
| Decrement | Decrement the specified control by the value provided. |
| Set | Set the control value to the value provided. |

- MCC Control Values

| Control | Valid Control Types | Control Value Type | Control Value Units |
| --- | --- | --- | --- |
| CWmin | All | See aCWmin and 802.11e | See aCWmin and 802.11e |
| CWmax | All | See aCWmax and 802.11e | See aCWmax and 802.11e |
| TXOPLimit | All | See TXOPLimit in 802.11e | See TXOPLimit in 802.11e |
| AIFSN | All | See AIFS in 802.11e | See AIFS in 802.11e |
| SuspendAC | Set | Boolean | None |
| SuspendACDuration | All | Integer | [ms] (0 = indefinitely) |
| FlushACQueue | Set | Boolean | None |
| Delay | All | Integer | [us] |
| Throttle | All | Integer | [us] |
| AccessTimeConstraint | All | Integer | Units of Control. |
| ShortRetryLimit | All | Integer | Number of attempts. |
| LongRetryLimit | All | Integer | Number of attempts. |
| RTSThreshold | All | Integer | [bytes] |
| AlwaysBackoff | Set | Boolean | 0 (false), 1 (true) |
| CWReversion | All | Integer | Number of successful transmissions before reversion (step). |
| CWIncrement | All | Integer | CW Index Step (step) |
| SuspendChannel | Set | Integer | (mask, 1 indicates enabled (suspended), 0 indicates disabled (normal / no change)) |

##### When generated

This primitive is generated by the client to request one or more local congestion measurements and or controls.

##### Effect of receipt

This primitive shall be processed by the STA to determine validity of the parameters. The STA shall send an MLME-LOCALMCC.confirm either indicating validity of the request or rejecting the request as invalid. A valid request must have either (a) at least one non-null Measurement or Control in each Measure Control entry pair or (b) cancel an active request(s).  All parameters of a valid request must be within the Valid Ranges indicated above.  If a request is valid, the STA shall initiate the measurement and or control process according to the Periodicity value indicated in the MLME-LOCALMCC.request, and send an MLME-LOCALMCC.indication according to the Report value indicated in the request.

#### MLME-LOCALMCC.confirm

##### Function

This primitive confirms a measurement control pair capability or rejects a measurement control pair request.

##### Semantics of the service primitive

The primitive parameters are as follows:

MLME-LOCALMCC.confirm	(

Dialog Token,

Status

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Dialog Token | Integer | 0 – 255 | The Dialog Token to identify the measurement request transaction. |
| Status | Boolean | 0 (failed), 1 (success) | Indicates success or failure of a measurement request. |

##### When generated

This primitive is generated by the STA to confirm a MLME-LOCALMCC.request as either valid or invalid.

##### Effect of receipt

This primitive is received by the client. If the confirmation indicates success, the client may expect one or more MLME-LOCALMCC.indication primitives according to the parameters of the request. If the confirmation indicates a failure, the client shall not expect subsequent measurements.

#### MLME-LOCALMCC.indication

##### Function

This primitive indicates a Measurement Control Pair report.

##### Semantics of the service primitive

The primitive parameters are as follows:

MLME-LOCALMCC.Indication	(

Dialog Token,

Number of Measure Control Report Pairs,

Measure Control Report Pair(s)

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Dialog Token | Integer | 1 – 255 | The Dialog Token to identify the measurement request transaction. |
| Number of Measure Control Report Pairs | Integer | 0 – 255 | Length of list |
| Measure Control Report Pair(s) | List of Measure Control Report Pair(s) | See below. | See below. |

Measure Control Report Pair = 	(

Measurement,

Measurement Value,

Control,

Trigger Count,

Control Count

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Measurement | Enumeration | see Table 2 | see Table 2 |
| Measurement Value | see Table 2 | see Table 2 | see Table 2 |
| Control | Enumeration | See Table 4 | See Table 4 |
| Trigger Count | Integer | 0 – 65535 | Number of times triggered. |
| Control Count | Integer | 0 – 65535 | Number of times parameter was changed. |

##### When generated

This primitive is generated by the STA to report a Measurement Control Pair according to the Periodicity indicated in an MLME-LOCALMCC.request.

##### Effect of receipt

This primitive provides the result of a Measurement Control request that may be used by the client to determine and control channel congestion. The client may use the Dialog Token parameter to associate the measurement with a MLME-LOCALMCC.request and determine an appropriate subsequent request or change to an active request.

# MLME

…

Insert a new clause after Clause 11.6.

## Medium Congestion Control

This subclause describes the procedures used for IEEE 802.11 Medium Congestion Control (MCC). The procedures include methods for measurement, control and adaptation.

Upon receipt of a MLME-LOCALMCC.request, the MLME shall determine whether the request is valid and supported. The MLME shall respond to the client with a MLME-LOCALMCC.confirm that reflects the result.

When a STA receives a request to cancel all local MCC requests (Dialog Token = 0), it shall cancel all active requests (any Local MCC requests received by the STA that have not been cancelled, rejected, or completed).  Otherwise, if the Number of Measure Control Entry Pairs = 0, the active request corresponding to the provided Dialog Token is cancelled.  Otherwise, if the Dialog Token or the Channel and AC match an active Local MCC request, the STA shall override the parameter values from the previous request with the parameter values from the new request.

If the request is valid and supported, the MLME shall determine the periodicity of the request and process each Measure Control Pair at the determined periodicity. Each Measure Control Pair may be either a Measurement only (null Control), a Control only (null Measurement), or an adaptation (both Measurement and Control are non-null). Each type is processed according to the appropriate procedure.

The MLME shall send a MLME-LOCALMCC.indication at the requested interval reflecting the result of the requested measure(s), control(s) or adaptation(s) if a report was requested.

### Measurement procedures

This subclause describes the procedures relating to the measurement service.

#### Procedures

The MLME shall invoke the direct measurement(s) corresponding to the requested derivative measurement(s). Upon completion of all direct measurements(s) corresponding to requested derivative measurement(s), the MLME shall compute the requested derivative measurement(s).

#### Direct Measurements

The STA shall support measuring of all of the direct measurements listed in Table 7. The STA shall determine the measurement from the given source on a per-channel, per-AC or per frame basis depending on the particular measurement.

- MCC Direct Measurements

| Measurement | Parameter(s) | Description of Source |
| --- | --- | --- |
| Clear Channel Acquisition (CCA) State | Channel | PHY service (PHY-SAP): PHY-CCA.indication (see 12.3.5) |
| Network Activity Vector (NAV) value | Channel | DCF access procedure (see 9.2.9.4) |
| Contention Window (CW) index value | Channel, AC | DCF backoff procedure (see 9.2.4) |
| Queue size (QS) | Channel, AC | MAC queues |
| MAC Frame Parameters (MFP) | Frame | MAC data services and primitives: MA-UNITDATA.request, MA-UNITDATA.indication, MA-UNITDATA-STATUS.indication (see 6.2.1), MAC control and management frames (see 7.2) and PHY: PLME-TXTIME.request, PLME-TXTIME.confirm (see 10.4). |
| Retry counts (RC) | None | DCF access procedure (see 9.2.5) |
| Frame drop event(s) (FDE) | None | DCF access procedure (see 9.2.5) |
| PHY Characteristics (PHY-CHAR) | None | PLME SAP interface: PLME-CHARACTERISTICS.request, PLME-CHARACTERISTICS.confirm (see 10.4) |

#### Derived Measurements

The STA shall support all mandatory measurements listed in Table 8 and may support the optional measurements listed. The STA shall derive the measurements according to the description provided based on the direct measurements each depends upon.

- MCC Derived Measurements

| Derived Measurement | Dependencies | Mandatory / Optional | Description of derivation |
| --- | --- | --- | --- |
| Receive Frame Rate (RX/FR) | MFP, PHY-CHAR | M | Rate of frames received (destined to the measuring STA). |
| Transmit Frame Rate (TX/FR) |  | M | Rate of frames transmitted (by the measuring STA). |
| Aggregate Frame Rate (Utilization) (AFR) |  | M | Rate of frames received and transmitted (including received frames destined to other STAs). |
| Receive Data Rate (RX/DR) |  | O | Ratio of payload received to burst rate used (destined to the measuring STA). |
| Transmit Data Rate (TX/DR) |  | O | Ratio of payload transmitted to burst rate used (by the measuring STA). |
| Aggregate Data Rate (Utilization) (ADR) |  | O | Ratio of utilization to burst rate used (including received frames destined to other STAs). |
| Queue Level (QL) | QS | M | Queue size relative to maximum size. |
| Mean Retry Count (MRC) | RC | M | Average of retry count or frame drop rate over a predetermined or given time. |
| Mean Frame Drop Rate (MDR) | FDE | M | Fraction of frames dropped over a predetermined or given time. |
| Mean Contention Window Index (MCW) | CW | M | Average of contention window index value over a predetermined or given time. |
| Network Activity Rate Level (NARL) | NAV, CCA | M | Ratio of busy time to total time period(s) considered. |
| Source Diversity (SD) | MFP | O | Number of unique MAC addresses observed over total time period(s). |

### Control procedures

This subclause describes the procedures relating to the control service.

#### Default Control Values

The control parameters shall be set to their default values at initialization and when a control has been cancelled. The default values are given in Table 9.

- MCC Default Control Values

| Control | Default Control Value | Units |
| --- | --- | --- |
| CWmin, CWmax, TXOPLimit, AIFSN | See 9.2.5 and 802.11e. | N/A |
| SuspendAC | Clear | N/A |
| SuspendACDuration | 0 | [ms] |
| FlushACQueue | Clear | N/A |
| Delay | 0 (none) | [us] |
| Throttle | 1 s | [us] |
| AccessTimeConstraint | 0 (unlimited) | [ms] |
| ShortRetryLimit, LongRetryLimit, RTSThreshold | See 9.2.5. | N/A |
| AlwaysBackoff | Clear | N/A |
| CWReversion | 0 (normal) | [index] |
| CWIncrement | 1 (normal) | [index] |
| SuspendChannel | Clear | N/A |

#### Basic Control Procedures

The MLME shall invoke the control(s) requested.

##### Basic Controls

The STA shall support all mandatory controls listed in Table 10 and may support the optional controls listed. The STA shall perform the operation (set the control parameter) as described.

- MCC Mandatory and Optional Controls

| Control | Mandatory / Optional | Description of Control |
| --- | --- | --- |
| CWmin, CWmax, TXOPLimit, AIFSN | M | See 9.2.5 and 802.11e. |
| SuspendAC | M | If the control is set, frames in the given AC shall be suspended. |
| SuspendACDuration | M | Controls duration of suspension of AC activated by SuspendAC. |
| FlushACQueue | M | If the control is set, frames in the given AC shall be deleted. |
| Delay | O | If the control is set, every frame shall be delayed by the given Delay before submitting to the MAC for transmission. |
| Throttle | M | If the control is set, every frame shall be flow controlled at the MAC so that the effective aggregate transmit time for frames submitted from the queue to the contention mechanism for the corresponding channel and AC, per second, is maintained below the given throttle value. |
| AccessTimeConstraint | O | The STA shall not be permitted to access the medium for more than the AccessTimeConstraint fraction of time. |
| ShortRetryLimit, LongRetryLimit, RTSThreshold | M | See 9.2.5. |
| AlwaysBackoff | M | If the control is set, all frames shall enter backoff procedure (first retry) without attempting immediately transmit. |
| CWReversion | O | Instead of reverting the CW index to CWmin upon successful retry, the CW index shall be ratcheted down in steps of CWReversion. |
| CWIncrement | O | Instead of incrementing the CW index by one, the CW index shall be increment in steps of CWIncrement. |
| SuspendChannel | M | If the control is set, the STA shall temporarily suspend TX or RX use of the channel by the STA (including preventing tuning to the channel). |

#### Adaptive Control Procedures

The MLME shall execute the measurement(s) requested and evaluate the condition. If the condition is met, the MLME shall execute the control(s).

##### Adaptations

The STA shall support all mandatory adaptation parameters listed in Table 11. The STA may support the optional adaptation parameters listed. The STA shall perform the operation (set the control parameter) as described.

All adaptation parameters are mandatory if the corresponding control parameter is supported (whether optional or mandatory).

- MCC Valid Control Types

| Control | Description of Valid Control Types |
| --- | --- |
| CWmin | Set, increment by multiplier, decrement by divisor. |
| CWmax | Set, increment by multiplier, decrement by divisor. |
| TXOPLimit | Set, increment by value, decrement by value. |
| AIFSN | Set, increment by value, decrement by value. |
| SuspendAC | Set (or Clear) only. |
| SuspendACDuration | Set, increment by value, decrement by value. |
| FlushACQueue | Set (or Clear) only. |
| Delay | Set, increment by value, decrement by value. |
| Throttle | Set, increment by value, decrement by value. |
| AccessTimeConstraint | Set, increment by value, decrement by value. |
| ShortRetryLimit | Set, increment by value, decrement by value. |
| LongRetryLimit | Set, increment by value, decrement by value. |
| RTSThreshold | Set, increment by value, decrement by value. |
| AlwaysBackoff | Set (or Clear) only. |
| CWReversion | Set, increment by value, decrement by value. |
| CWIncrement | Set, increment by value, decrement by value. |
| SuspendChannel | Set (or Clear) only. |