IEEE P802.11
Wireless LANs

| TGy-Nov-7th-Text-Changes |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-11-07 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Peter Ecclesine | Cisco Systems |  | 408-527-0815 | petere@cisco.com |

Abstract

Text changes to TGy D0.01 discussed in 7 Nov telecom, or subsequently posted to the TGy reflector.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

From 06/1674 TGy Nov 7th 2006 Conference Call Minutes:

- Review 06/864r4 3650 MHz Mobile Service Enablement

- Scott B proposes to replace “base station” with “enabling station” everywhere in the draft

- There is unanimous agreement to do so

- Annex A, p9 A.4.17 should be DSE functions

- Discussion of A.4.17 concludes that MS1 is Fixed station operation, MS2 is Enabling station operation, and MS3 is Dependent station operation, with Status CFy:O,3

- Enabling station creation of BSA, and Enabling station operation with DSE become MS2.1 and MS2.2, with Status MS2:M

- 11.10.3 changes reviewed, and it was agreed to add an additional statement after the last statement:

- -Every dependent STA shall count the sum of the changes to dot11TransmittedFragmentCount, dot11MulticastTransmittedFrameCount and dot11ReceivedFragmentCount, and create a Probe Response DSE having the enabling station’s DSE registered location element and having the Dependent STA bit set to TRUE. This Probe Response shall be scheduled for transmission to self whenever another [256] changes have been counted.

- Review 06/955r3 OFDM PHY 3650 MHz Band

- OF2.19.3.2 is for CCA-ED Mode 2

- Six typos in OF3.3.13, 14, 15,  PF4.14, 15, 16 as REV-ma D9.0 forces numbering change to become OF3.2.7

Editing instructions to be moved:

Editor to construct P802.11y D0.02 from 06/1432r1, 06/864r4 and 06/955r3, then globally replace “base station” with “enabling station” and rectify any resultant grammerical errors.

Insert the following new reference in alphabetical order:

IETF RFC 3825, Dynamic Host Configuration Protocol Option for Coordinate-based Location Configuration Information, Polk, J., Schnizlein, J., Linsner, M., July 2004

Insert the following text after the fourth sentence of the second paragraph of 5.2.7:

Additionally, all stations shall use Multi-Domain capability (dot11MultiDomainCapabilityEnabled true), Spectrum Management capability (dot11SpectrumManagementRequired true), Regulatory Classes (dot11RegulatoryClassesRequired true) and the OFDM PHY.

Replace 11.10.3 text with text as shown:

Dependent STAs shall have dot11RegLocRequired and dot11DSERequired set to TRUE.

For the purpose of DSE, the following statements apply:

—A STA with dot11DSERequired set to TRUE shall not operate in a BSS or IBSS unless it has received a beacon frame or probe response frame from an enabling station with Spectrum Management bit set to 1 in the Capability Information field;

—Every dependent STA that is not associated with an enabling station shall not transmit, except to authenticate and associate with an enabling station, unless such action is as mandated in the regulatory domain (e.g. Emergency Services);

—Every dependent STA shall associate with an enabling station that is sending beacons with DSE registered location element RegLoc DSE set to TRUE, and operate under the control of the enabling station to which the dependent STA is associated;

—Every dependent STA shall create a dependent DSE registered location element with the enabling station’s DSE registered location element and having the Dependent STA bit set to TRUE. Before association, the Dependent Enablement Identifier (DEI) field shall be set to zero. Upon association or reassociation, the DEI field shall be set to the DEI value in the MLME-ASSOCIATE.response or MLME-REASSOCIATE.response, and this DSE registered location element shall be used in constructing Probe Response DSE frames.

—Every dependent STA that is not associated with an enabling station shall only transmit for up to [8] seconds in attempting to associate,

then if it is not associated with an enabling station, it shall not transmit for [512] seconds,

before it can again attempt to authenticate and associate;

—Every associated dependent STA shall cease transmission within [60] seconds if it does not receive either a beacon frame or a probe response frame with RegLoc DSE set to TRUE in the DSE registered location element, and set to zero all fields of the dependent DSE registered location element;

—Every dependent STA shall respond to a Request information Probe Request having the DSE registered location element (asking “what base station’s signal did you receive and decode that enables you to transmit?”) with a Probe Response DSE with the dependent DSE registered location element.

—Every dependent STA shall create a Probe Response frame containing order 23 (DSE), with the dependent DSE registered location element. The dependent STA shall count the sum of dot11TransmittedFragmentCount, dot11MulticastTransmittedFrameCount and dot11ReceivedFragmentCount, and schedule this Probe Response be sent to self using normal frame transmission rules, whenever the sum modulo [256] changes.

In Annex A, A.4.8, in OF2.19.3.2, change ‘Mode 5’ to ‘Mode 2’

In Annex A, A.4.8, in OF3.3.13, 14, 15 and PF4.14, 15, 16 replace ‘OF3.2.6’ with ‘OF3.2.7’

- A.4.13 Regulatory classes extensions

Insert these entries at the end of A.4.13:

| Item | Protocol capability | References | Status | Support |
| --- | --- | --- | --- | --- |
| RC5 | CoverageClass 0-31 | 9.8.4 | CFy:M | Yes □ No □ N/A □ |
| RC6 | Power level, equivalent Max Transmit Power Level and Regulatory Class | 9.8.4 | CFy:M | Yes □ No □ N/A □ |
| RC7 | Power level, different Max Transmit Power Level and Regulatory Class | 9.8.4 | CFy:M | Yes □ No □ N/A □ |

### In Annex A, A.4.17, replace ‘UDE’ in title with ‘DSE’

### A.4.17 UDE functions

Replace A.4.17 Table with

| Item | Protocol Capability | References | Status | Support |
| --- | --- | --- | --- | --- |
| *MS1 | Fixed station operation with RegLoc | 11.10.1 | CFy:O.1 | Yes □ No □ N/A □ |
| *MS2 | Enabling station operation with RegLoc | 11.10.1 | CFy:O.1 | Yes □ No □ N/A □ |
| MS2.1 | Enabling station creation of BSA | 11.10.2 | MS2:M | Yes □ No □ N/A □ |
| MS2.2 | Enabling station operation with DSE | 11.10.1 | MS2:M | Yes □ No □ N/A □ |
| *MS3 | Dependent STA operation with DSE | 11.10.3 | CFy:O.1 | Yes □ No □ N/A □ |
| MS3.1 | Dependent STA Enablement | 11.10.3 | MS3:M | Yes □ No □ N/A □ |
| MS3.2 | Dependent STA DSE Time to Associate | 11.10.3 | MS3:M | Yes □ No □ N/A □ |
| MS3.3 | Dependent STA DSE Time to Not Transmit | 11.10.3 | MS3:M | Yes □ No □ N/A □ |
| MS3.4 | Dependent STA Time to Not Transmit | 11.10.3 | MS3:M | Yes □ No □ N/A □ |
| MS3.5 | Dependent STA DSE Probe Response | 11.10.3 | MS3:M | Yes □ No □ N/A □ |
| MS3.6 | Dependent STA MLME-ASSOCIATE.response DSE | 10.3.6.4 | MS3:M | Yes □ No □ N/A □ |
| MS3.7 | Dependent STA MLME-REASSOCIATE.response DSE | 10.3.7.4 | MS3:M | Yes □ No □ N/A □ |
| MS4 | Channel switch procedure Transmission of extended channel switch announcement and channel switch procedure by an AP Transmission of extended channel switch announcement and channel switch procedure by a STA Reception of extended channel switch announcement and channel switch procedure by a STA | 11.9.7 11.9.7 11.9.7 | (CFy AND CF1):M (CFy AND CF2):M CFy:M | Yes □ No □ N/A □ |
| MS5 | Station Association procedure Transmission of Association Request with SupportedRegulatoryClasses by a STA Transmission of Association Response with SupportedRegulatoryClasses by an AP | 9.8.3, 11.3.2.1 9.8.3, 11.3.2.2 | (CFy AND CF2:M (CFy AND CF1):M | Yes □ No □ N/A □ |
| MS6 | Station Reassociation procedure Transmission of Association Request with SupportedRegulatoryClasses by a STA Transmission of Reassociation Response with SupportedRegulatoryClasses by an AP | 9.8.3, 11.2.3.3 9.8.3, 11.2.3.4 | (CFy AND CF2:M (CFy AND CF1):M | Yes □ No □ N/A □ |

Motion: Move to accept editing instructions above as written and instruct the editor to follow these instructions in the creation of P802.11y D0.02

Moved:

Seconded:

Result:   Yes       No       Abstain

Motion (passes/fails)

References: