IEEE P802.11
Wireless LANs

| Null Beacon Energy Conservation draft text |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-11-13 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Peter Ecclesine | Cisco Systems | MS SJ-10-5 170 W. Tasman Dr., San Jose, Ca 95134-1706 | +1-408-527-0815 | petere@cisco.com |

Abstract

Null Beacon Energy Conservation draft text is aligned with P802.11 REV-ma D9.0 and P802.11v D0.05

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# 7. Frame formats

## 7.1 MAC frame formats

### 7.1.3 Frame fields

### 7.1.3.1 Frame Control field

7.1.3.1.2 Type and Subtype fields

Editor’s note—11 ma D9.0 Table 1 shows 0110-0111 Reserved

Change the seventh entry and insert new eighth entry as shown:

Table 1-Valid type and subtype combinations

| Type value b3 b2 | Type description | Subtype value b7 b6 b5 b4 | Subtype description |
| --- | --- | --- | --- |
| 00 | Management | 0110-0111 | Reserved |
| 00 | Management | 0111 | Null beacon |

## 7.2 Format of individual frame types

### 7.2.3 Management frames

Editor’s note—11 ma D9.0 ended with 7.2.3.12 Action frame format

Insert the following new subclause after 7.2.3.12

### 7.2.3.13 Null beacon frame format

The frame format of a management frame of subtype Null beacon is shown in Figure v35A.

Figure v35A—Null beacon frame format

Frame fields are defined in 7.2.3.

# 11.MLME

## 11.1 Synchronization

### 11.1.1 Basic approach

### 11.1.1.1 TSF for infrastructure networks

Insert the following sentence after the last sentence of the subclause:

Null beacons shall be generated whenever the AP changes SA, DA or BSSID for beacons.

### 11.1.2 Maintaining synchronization

### 11.1.2.1 Beacon generation in infrastructure networks

Change the fourth sentence and insert new text after the fifth sentence as shown.

The AP shall define the timing for the entire BSS by transmitting beacons according to the

dot11BeaconPeriod attribute within the AP. This defines a series of TBTTs exactly dot11BeaconPeriod TUs

apart. Time zero is defined to be a TBTT with the beacon being a DTIM and transmitted at the beginning of

a CFP. At each TBTT, the AP shall schedule either a beacon or a null beacon as the next frame for transmission according to the medium access rules specified in Clause 9. The beacon period is included in Beacon and Probe Response frames, and STAs shall adopt that beacon period when joining the BSS. When dot11WirelessManagementImplemented is true, a null beacon may be sent instead of a beacon when the following statements are true:

—the beacon TIM element length is 4, the DTIM count is zero, the Partial Virtual Bitmap is a single octet equal to zero, and there are no buffered broadcast, multi-cast or unicast frames.

—the beacon does not contain a channel switch announcement element, nor an extended channel switch announcement element.

Insert new subclause after 11.1.2.3 and renumber accordingly:

### 11.1.2.v3A Null beacon reception

When dot11WirelessManagementImplemented is true, STAs in an infrastructure network that receive a null beacon frame whose BSSID is equal to the MAC address currently in use by the STA contained in the AP of the BSS, shall take the actions indicated by a beacon frame with a TIM element whose length is 4, DTIM count is zero,  DTIM period is one, and Bitmap control field and Partial Virtual Bitmap are zero.

In Annex A, clause A.4.14, insert a new final entry

| Item | Protocol capability | References | Status | Support |
| --- | --- | --- | --- | --- |
| RME10 | Null beacon power save | 7.2.3.13, 11.1.2.1, 11.1.2.v3A | CFv:M | Yes □ No □ N/A □ |