IEEE P802.11
Wireless LANs

| The definition of broadcast in mesh |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-11-16 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Kazuyuki Sakoda | Sony Corporation | 2-17-1 Higashi-gotanda, Shinagawa-ku, Tokyo, 141-0022 Japan | +81-3-6409-3201 | sako@wcs.sony.co.jp |
| Joseph Kim | STMicroelecronics | 1060 East Brokaw Road, MS 212, San Jose, CA 95131 | +1-408-621-4913 | Kyeongsoo.kim@st.com |
| W. Steven Conner | Intel Corp. | 2111 NE 25th Ave, Hillsboro, OR 97124 | 1-503-264-8036 | w.steven.conner@intel.com |

Abstract

This document provides the high level normative behaviour of “broadcasting” in WDS, which is a potential resolution to CID50.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

CID: 50

Comment:

The sentence "This frame is typically transmitted in a broadcast manner." illustrates a problem that TGs must address. Namely, the term "broadcast" as used in 802.11 is not always the same as the same term used in TGs. In many cases, 802.11 uses the term broadcast to refer to a local (i.e., a non-relayed) transmission to multiple, indeterminate "neighbors". This is the case for all current 802.11 management frames. The destination address in these cases is "all 1's". However, in TGs, an all 1's destination address is also used to indicate a data frame that is to be sent to all nodes in the mesh (actually, all nodes in the LAN). This means that just looking at the destination address is not sufficient to tell whether or not the frame should be relayed - it is also necessary to see if it is a management frame or a data frame. However, suppose TGs wishes to send a management frame that is intended for all other mesh points. What is the destination address?

Proposed Change:

TGs should define an address (not all 1's) that can be used for defining a local (non-relayed) broadcast. Moreover, it may be necessary to define some other multicast addresses for groups of mesh nodes. For example, multicast addresses indicating that a frame is intended for all MPs, all MAPs, all LWMPs, all routing MPs, etc.

Resolution: Defer-RPS

Resolution Comment (generated by ad-hoc comment resolution groups, related to Resolution):

may need more investigation.  TTL value may be useful here.

Several Multicast addresses may be helpful.

Proposed resolution

Insert the new subclause before the current subclause 7.4.6.1

7.4.6.1 Clarification on transmission of action frames (Informative)

The action frames shown in table s19 utilize the normal 3 address management frame headers and therefore are only transmitted 1 hop for either broadcast or unicast delivery. Note that in some cases the reception and processing of an action frame results in the subsequent transmission of the same type of action frame that may include the propagation of the contents from the received IEs.

References:

[1] Draft Amendment: ESS Mesh Networking.  doc.: IEEE P802.11s/D0.04, October 2006.

[2] Resolution of informal comments received May 1, 2006. doc.: IEEE 802.11-06/0602r17.