IEEE P802.11
Wireless LANs

| IEEE 802.1X Port Control and IEEE 802.11r |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-11-14 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Jouni Malinen | Devicescape Software, Inc. | 900 Cherry Avenue, 6th Floor, San Bruno, CA 94066, USA | +1 650 829 2630 | jkm@devicescape.com |

Abstract

IEEE 802.11r/D3.0 uses ForceAuthorized variable to force IEEE 802.1X port into authorized state in supplicant and authenticator R1KH state machines. This mechanism has issues since it disables EAPOL state machines and do not allow them to used, e.g., for re-authentication. This submission proposes changes to 802.11r/D3.0 to allow IEEE 802.1X port to be set without having to use ForceAuthorized and addresses CID 1619 of LB 87.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

802.11r/D3.0 LB78 CID 1619 / IEEE 802.1X portControl

802.11r/D3.0 uses 802.1X portControl state variable force the 802.1X port into authorized state. This mechanism has a drawback of disabling EAPOL state machines completely since PAE state machine would remain in FORCE_AUTH state. This would disable re-authentication requested by both the authenticator (internally in EAPOL state machine) and by the supplicant (by sending EAPOL-Start). In addition, FORCE_AUTH state sends out a canned EAP-Success message.

In order to resolve this issue, IEEE 802.1X/EAPOL state machines should be used to change port state. This is already happening for the initial association, so use of ForcedAuthorized can be removed from the state machines showing this case. Looking at the state machines defined in IEEE 802.1X-2004, following tasks would need to happen for the FT case to work by using EAPOL state machine to authorize the port. This is based on the authenticator state machine, but supplicant case is similar.

PAE: INITIALIZE

BE: INITIALIZE

PAE: DISCONNECTED

BE: IDLE

authStart = FALSE

PAE: RESTART

eapRestart = TRUE

<something to reset eapRestart to FALSE>

PAE: CONNECTING

reAuthenticate = FALSE

inc(reAuthCount)

<something to set eapSuccess>

PAE: AUTHENTICATING

eapolStart = FALSE

authSuccess = FALSE

authFail = FALSE

authTimeout = FALSE

authStart = TRUE

keyRun = FALSE

keyDone = FALSE

BE: SUCCESS

txReq() (send EAP-Packet _only_ if one is available, i.e.,

nothing sent here)

authSuccess = TRUE

keyRun = TRUE

BE: IDLE

authStart = FALSE

PAE: AUTHENTICATED

authPortStatus = Authorized

reAuthCount = 0

<OK state for waiting session timeout, re-authentication request, or EAPOL-Start>

Annex E (informative) defines PAE SM <-> EAP/AAA interface. EAP is mentioned as an example, i.e., 802.11r could be another “higher layer” for 802.1X. Following proposed changes to 802.11r/D3.0 add this kind of functionality and address CID 1619 from LB87. This is done by removing the unneeded use of ForceAuthorized from initial association (EAPOL state machines are authorizing the port anyway in this case) and by adding a new SKIP-EAP state to the R1KH state machines to act as a very simple “higher layer” for 802.1X.

Proposed changes to 802.11r/D3.0

8A.6.2 R1KH authenticator FT initial association state machine

In Figure 158N (R1KH authenticator FT initial association sm), remove

“802.1X::portControl = ForceAuthorized' from FT-PTK-INIT-DONE state.

8A.6.2.1 R1KH authenticator FT initial association state machine states

Remove the following text from the description of FT-PTK-INIT-DONE: “ and generates the trigger to open the 802.1X port”

8A.6.3 R1KH authenticator FT mechanisms state machine

In Figure 158O (R1KH authenticator FT mechanisms sm), add transition from FT-HANDSHAKE-DONE state to a new state SKIP-EAP with following condition: 802.1X::eapRestart. Add a new state SKIP-EAP with following operations:

802.1X::eapRestart = FALSE

802.1X::eapSuccess = TRUE

802.1X::eapFail = FALSE

8A.6.3.1 R1KH authenticator FT mechanisms state machine states

Remove the following text from the description of FT-HANDSHAKE-DONE: “This state generates the trigger to open the 802.1X port.”

Add the following text into the end of 8A.6.3.1:

SKIP-EAP: This state is entered after successful completion of FT mechanism. This state generates the trigger to get the EAPOL state machines to open the 802.1X port.

8A.6.5 R1KH supplicant initial association state machine

In Figure 158Q (R1KH supplicant FT initial association sm),  remove

“802.1X::portControl = ForceAuthorized” from FT-PTK-INIT-DONE state.

8A.6.5.1 R1KH supplicant initial association state machine states

Remove the following text from the description of FT-PTK-INIT-DONE: “ and generates the trigger to open the 802.1X port”

8A.6.6 R1KH supplicant FT mechanisms state machine

In Figure 158R (R1KH supplicant FT sm), add transition from FT-DONE state to a new state SKIP-EAP with following condition: 802.1X::eapRestart. Add a new state SKIP-EAP with following operations:

802.1X::eapRestart = FALSE

802.1X::eapSuccess = TRUE

802.1X::eapFail = FALSE

802.1X::eapNoResp = TRUE

8A.6.6.1 R1KH supplicant FT mechanisms state machine states

Remove the following text from the description of FT-DONE: “This state generates the trigger to open the 802.1X port.”

Add the following text into the end of 8A.6.6.1:

SKIP-EAP: This state is entered after successful completion of FT mechanism. This state generates the trigger to get the EAPOL state machines to open the 802.1X port.

Motion

Move to instruct the editor to apply the proposed changes from 11-06/1769r1 to the 802.11r draft and to accept the comment 1619 in LB87 with the resolution of “Accepted. Addressed with the acceptance of 11-06/1769r1.”