IEEE P802.11
Wireless LANs

| LB84 CID 12038 submission text |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-11-13 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Matthew Fischer | Broadcom | 190 Mathilda Place, Sunnyvale CA 94040 | +1 408 543 3370 | mfischer@broadcom.com |

Abstract

This document contains proposed changes to the IEEE P802.11n Draft related the following LB84 comments: CID 12038, 1464, which relate to the general issue of coexistence between 20 mhz-only capable devices and 40 mhz capable devices in the 2.4 GHz band and restrictions on transmissions in the secondary channel. The submission proposes a mechanism for transmitting a sequence of two CTS2SELF frames, one in the primary channel and the other in the secondary channel, to preced the transmission of a 40 MHz frame.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# Introduction

## Interpretation of a Motion to Adopt

A motion to approve this submission means that the editing instructions and any changed or added material are actioned in the TGn Draft.  This introduction, is not part of the adopted material.

Editing instructions formatted like this are intended to be copied into the TGn Draft (i.e. they are instructions to the 802.11 editor on how to merge the TGn amendment with the baseline documents).

TGn Editor:  Editing instructions preceded by “TGn Editor” are instructions to the TGn editor to modify existing material in the TGn draft.   As a result of adopting the changes, the TGn editor will execute the instructions rather than copy them to the TGn Draft.

Summission Note: Notes to the reader of this submission are not part of the motion to adopt.  These notes are there to clarify or provide context.

# CID 12038, 1464

TGn Editor:  Change the second paragraph of subclause “9.20.7 STA switching from 40 MHz to 20 MHz in 20/40 MHz BSS” in TGn draft D1.06, page 146, about line 45 as follows:

9.20.7 STA switching from 40 MHz to 20 MHz in 20/40 MHz BSS

When a STA is associated with an AP that declares a Supported Channel Width of 20/40 MHz, the STA may

transmit frames in the 20 MHz wide primary channel as well as in the 40 MHz wide channel. A non-AP STA

that is a member of a 40MHz BSS shall not transmit non-control type 20MHz frames in the secondary channel. (Ed: CID 1464)

Proposed resolutions – the resolution for CID 1464 is a modification of an existing resolution:

| CID | Author | Subclause | Comment | Commentor’s suggestion | TGn proposed resolution | NEW Proposed resolution |
| --- | --- | --- | --- | --- | --- | --- |
| 1464 | Fischer, Matthew | 9.23.9 | we need an explicit statement regarding extension channel transmissions | add a sentence stating that a a STA associated with a 40MHz BSS shall not transmit 20MHz frames in the extension channel | Countered: add the following sentence after line 65 in 9.20.7 (D1.03), "A non-AP STA that is a member of a 40MHz BSS shall not transmit 20MHz frames in the secondary channel." | Counter – change the last sentence of the second paragraph of 9.20.7 (D1.06) to: “A non-AP STA that is a member of a 40MHz BSS shall not transmit non-control type 20 MHz frames in the secondary channel.” |
| 12038 | Douglas, Brett | 20.3.8.2 | 40 MHz in the 2.4 GHz band is a mess. It causes all kinds of interoperability problems with legacy 802.11b/g stations | We probably want to specify that you must scan for beacons across the entire 40 MHz, and if you can't find a clean 40 MHz, you must fall back to 20 MHz modes |  | Counter – allow for mechanisms of coexistence between 40 MHz BSS and 20 MHz BSS as provided for in doc 11-06-1773r0. |