IEEE P802.11
Wireless LANs

| Extended Channel Switch Announcement |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-11-14 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Bjorn A. Bjerke | Qualcomm, Inc. | 9 Damonmill Square, Suite 2A | +1 781-276-0912 | bbjerke@qualcomm.com |

Abstract

This submission contains normative text related to adding an Extended Channel Switch Announcement to the 802.11n draft.  The intention is to extend the DFS capabilities introduced in 802.11h for Channel Switch Announcement to all regulatory classes (including 40 MHz operation, and operation in 2.4 GHz and 5 GHz). This document borrows significantly from the Extended Channel Switch Announcement proposals adopted by TGv and TGy, and authored by Jari Jokela (Nokia), Emily Qi (Intel) and Allan Thomson (Cisco).

The text is aligned with P802.11-REVma/D9.0 and P802.11n-D1.06.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

TGn Editor: The following text should be inserted into the 802.11n draft as instructions to the 802.11 base standard editor.

- 7.2.3.1 Beacon frame format

Insert the following row in Table 8 just before the Vendor Specific IE and insert the appropriate number in the “Order” column:

- Table 8—Beacon frame body

|  | Extended Channel Switch Announcement | Extended Channel Switch Announcement element may be present if dot11HighThroughputOptionImplemented is true. |
| --- | --- | --- |

- 7.3.2.9 Probe Response frame format

Insert the following additional row in Table 15 just before the Vendor Specific IE and insert the appropriate number in the “Order” column:

- Table 15—Probe Response frame body

|  | Extended Channel Switch Announcement | Extended Channel Switch Announcement element may be present if dot11HighThroughputOptionImplemented is true. |
| --- | --- | --- |

- 7.3.2 Information Elements

Insert Element IDs x and x+1 into Table 26 and change the Reserved row accordingly:

Table 26—Element IDs

| Information Element | Element ID | Length (in octets) |
| --- | --- | --- |
| Extended Channel Switch Announcement | x | 6 |
| Supported Regulatory Classes | x+1 | 3 to 34 |

Insert the following new subclauses 7.3.2.36 and 7.3.2.37:

- 7.3.2.36 Extended Channel Switch Announcement element

The Extended Channel Switch Announcement element is used by an AP in a BSS or a STA in an IBSS to advertise

when it is changing to a new channel . The announcement includes both the new regulatory class and the channel number of the new channel, or, in the case of 40 MHz operation, the primary channel of the new channel pair. The format of the Extended Channel Switch Announcement element is shown in Figure 112b.

|  | Element ID | Length | Channel Switch Mode | New Regulatory Class | New Channel Number | Channel Switch Count |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 | 1 | 1 |

Figure 112b-Extended Channel Switch Announcement element

This information element is variable in length. The length of this information element is indicated in the Length field.

The Channel Switch Mode field indicates any restrictions on transmission until a channel switch. An AP in a

BSS or a STA in an IBSS shall set the Channel Switch Mode field to either 0 or 1 on transmission. A Channel

Switch Mode set to 1 means that the STA in a BSS to which the frame containing the element is addressed shall transmit no further frames within the BSS until the scheduled channel switch. A STA in an IBSS may treat a Channel Switch Mode field set to 1 as advisory. A Channel Switch Mode set to 0 does not impose any requirement on the receiving STA.

The New Regulatory Class field shall be set to the number of the regulatory class to which the STA is moving, as defined in Annex J.

The New Channel Number field shall be set to the number of the channel to which the STA is moving. In the case where the STA is moving to a 40 MHz channel (i.e., a pair of 20 MHz channels), the channel number refers to the primary channel. The position of the secondary channel (above or below the primary channel) is uniquely specified by the New Regulatory Class field.

The Channel Switch Count field either shall be set to the number of TBTTs until the STA sending the Channel

Switch Announcement element switches to the new channel or shall be set to 0. A value of 1 indicates that the switch shall occur immediately before the next TBTT. A value of 0 indicates that the switch shall occur at any time after the frame containing the element is transmitted.

TGn Editor: Update the references to section numbers below

The Extended Channel Switch Announcement element is included in Extended Channel Switch Announcement frames, as described in 7.4.1.6, and may be included in Beacon frames, as described in 7.2.3.1, and Probe Response frames, as described in 7.2.3.9. The use of Extended Channel Switch Announcement elements and frames is described in 11.9.7 and 11.9.8.3.

- 7.4.1 Spectrum management action details

Make the following changes to Table 44:

- Table 44—Spectrum management action value field values

| Action Value field value | Description |
| --- | --- |
| 0 | Measurement Request |
| 1 | Measurement Response |
| 2 | TPC Request |
| 3 | TPC Report |
| 4 | Channel Switch Announcement |
| 5 | Extended Channel Switch Announcement |
| 6-255 | Reserved |

Insert the following new subclause:

##### 7.4.1.6 Extended Channel Switch Announcement frame format

The Extended Channel Switch Announcement frame uses the Action frame body format and is transmitted by an AP in a BSS or a STA in an IBSS to advertise a channel switch. The format of the Extended Channel Switch Announcement frame body is shown in Figure 117a

|  | Category | Action value | Extended Channel Switch Announcement element |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | 6 |

Figure 117a-Extended Channel Switch Announcement frame format

The Category field shall be set to 0 (representing spectrum management).

The Action Value field shall be set to 5 (representing an Extended Channel Switch Announcement frame).

The Extended Channel Switch Announcement element shall be set as described 7.3.2.36.

- 10.3 MLME SAP Interface

Insert the following new subclause:

10.3.30 Extended channel switch

The following MLME primitives support the signalling of Extended Channel Switch Announcement.

10.3.30.1 MLME-EXTCHANNELSWITCH.request

10.3.30.1.1 Function

This primitive requests that an Extended Channel Switch Announcement frame be sent by an AP.

10.3.30.1.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-EXTCHANNELSWITCH.request	(

Mode,

Regulatory Class,

Channel Number,

Channel Switch Count,

VendorSpecificInfo

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Mode | Integer | 0,1 | Channel switch mode, as defined for the Extended Channel Switch Announcement element. |
| Regulatory Class | Integer | As defined in Annex J | Specifies the new regulatory class. |
| Channel Number | Integer | As defined in Annex J | Specifies the new channel number. |
| Channel Switch Count | Integer | 0 – 255 | Specifies the number of TBTTs until the channel switch event, as described for the Extended Channel Switch Announcement element. |
| VendorSpecificInfo | A set of information elements | As defined in 7.3.2.26 | Zero or more information elements. |

10.3.30.1.3 When generated

This primitive is generated by the SME to request that an Extended Channel Switch Announcement frame be sent to a non-AP STA that is associated to the AP.

10.3.30.1.4 Effect of receipt

On receipt of this primitive, the MLME constructs an Extended Channel Switch Announcement frame. The AP then attempts to transmit this to the STA that is associated.

10.3.30.2 MLME-EXTCHANNELSWITCH.confirm

10.3.30.2.1 Function

This primitive reports the result of a request to switch channel.

10.3.30.2.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-EXTCHANNELSWITCH.confirm	(

ResultCode,

VendorSpecificInfo

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| ResultCode | Enumeration | SUCCESS, INVALID PARAMETERS or UNSPECIFIED FAILURE | Reports the result of a channel switch request. |
| VendorSpecificInfo | A set of information elements | As defined in 7.3.2.26 | Zero or more information elements. |

10.3.30.2.3 When generated

This primitive is generated by the MLME when a channel switch request completes. Possible unspecified failure causes include an inability to schedule an extended channel announcement.

10.3.30.2.4 Effect of receipt

The SME is notified of the results of the channel switch procedure.

10.3.30.3 MLME-EXTCHANNELSWITCH.indication

10.3.30.3.1 Function

This primitive indicates that an Extended Channel Switch Announcement frame was received from an AP.

10.3.30.3.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-EXTCHANNELSWITCH.indication	(

Peer MAC Address,

Mode,

Regulatory Class,

Channel Number,

Channel Switch Count,

VendorSpecificInfo

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerMAC Address | MACAddress | Any valid individual MAC Address | The address of the peer MAC entity from which the Measurement Report frame was received. |
| Mode | Integer | 0,1 | Channel switch mode, as defined for the Channel Switch Announcement element. |
| Regulatory Class | Integer | As defined in Annex J | Specifies the new regulatory class. |
| Channel Number | Integer | As defined in Annex J | Specifies the new channel number. |
| Channel Switch Count | Integer | 0 – 255 | Specifies the number of TBTTs until the channel switch event, as described for the Channel Switch Announcement element. |
| VendorSpecificInfo | A set of information elements | As defined in 7.3.2.26 | Zero or more information elements. |

10.3.30.3.3 When generated

This primitive is generated by the MLME when a valid Extended Channel Switch Announcement frame is received.

10.3.30.3.4 Effect of receipt

On receipt of this primitive, the SME decides whether to accept the switch.

10.3.30.4 MLME-EXTCHANNELSWITCH.response

10.3.30.4.1 Function

This primitive is used to schedule an accepted extended channel switch.

10.3.30.4.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-EXTCHANNELSWITCH.response	(

Mode,

Regulatory Class,

Channel Number,

Channel Switch Count,

VendorSpecificInfo

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Mode | Integer | 0,1 | Channel switch mode, as defined for the Channel Switch Announcement element. |
| Regulatory Class | Integer | As defined in Annex J | Specifies the new regulatory class. |
| Channel Number | Integer | As defined in Annex J | Specifies the new channel number. |
| Channel Switch Count | Integer | 0 – 255 | Specifies the number of TBTTs until the channel switch event, as described for the Channel Switch Announcement element. |
| VendorSpecificInfo | A set of information elements | As defined in 7.3.2.26 | Zero or more information elements. |

10.3.30.4.3 When generated

This primitive is generated by the SME to schedule an accepted extended channel switch request.

10.3.30.4.4 Effect of receipt

On receipt of this primitive, the MLME schedules the extended channel swtich.  The actual channnel switch can be achieved at the appropriate time through the MLME-PLME interface using the PLME-SET primitive of the dot11CurrentFrequency MIB attribute.

Change clause 11.9.7.1 as follows:

11.9.7.1 Selecting and advertising a new channel in an infrastructure BSS

The decision to switch to a new operating channel in an infrastructure BSS shall be made only by the AP. An

AP may make use of the information in Supported Channel elements, Supported Regulatory Classes elements and the results of measurements undertaken by the AP and other STAs in the BSS to assist the selection of the new channel. The algorithm to choose a new channel is beyond the scope of this standard, but shall satisfy applicable regulatory requirements, including uniform spreading rules and channel testing rules. The AP shall attempt to select a new channel that is supported by all associated STAs, although it should be noted that this might not always be

possible.

In the following text wherever Channel Switch Announcement is referred to, the following rules apply:

- If an AP is switching to a new channel in a different regulatory class then the AP shall use the Extended Channel Switch Announcement elements and frames instead of the Channel Switch Announcement elements and frames.

- If an AP is switching to a channel within the same regulatory class, the AP shall send the Channel Switch Announcement element and frame. Optionally the Extended Channel Switch Announcement element and frame may be used.

An AP shall inform associated STAs that the AP is moving to a new channel and maintain the association by

advertising the switch using Channel Switch Announcement elementsI in Beacon frames, Probe Response frames, either the Channel Switch Announcement frames until the intended channel switch time. The AP may force STAs in the BSS to stop transmissions until the channel switch takes place using the Channel Switch Mode field in Channel Switch Announcement element. If possible, the channel switch should be scheduled so that all STAs in the BSS, including STAs in power save mode, have the opportunity to receive at least one Channel Switch Announcement element before the switch. The AP may send the Channel Switch Announcement frame in a BSS without performing a backoff, after determining the WM is idle for one PIFS period.

A STA that receives a Channel Switch Announcement element may choose not to perform the specified switch, but to take alternative action. For example, it may choose to move to a different BSS.

A STA in a BSS that is not the AP shall not transmit the Channel Switch Announcement element.

Insert new first paragraph in clause 11.9.7.2 as follows:

11.9.7.2 Selecting and advertising a new channel in an IBSS

In the following text wherever Channel Switch Announcement is referred to, the following rules apply:

- If a DFS owner is switching to a new channel in a different regulatory class then the DFS owner shall use the Extended Channel Switch Announcement elements and frames instead of the Channel Switch Announcement elements and frames.

- If a DFS owner is switching to a channel within the same regulatory class, the DFS owner shall send the Channel Switch Announcement element and frame. Optionally the Extended Channel Switch Announcement element and frame may be used.