IEEE P802.11
Wireless LANs

| HWMP Specification |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-11-13 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Avinash Joshi | Motorola | 485 N. Keller Rd., Suite 250, Maitland, FL 32751 | +1-407-659-5398 | avinash.joshi@motorola.com |
| Hrishikesh Gossain | Motorola | 485 N. Keller Rd., Suite 250, Maitland, FL 32751 | +1-407-659-5393 | hrishikesh.gossain@motorola.com |
| Jorjeta Jetcheva | Firetide | 16795 Lark Ave., Los Gatos, CA 95032 | +1-408-355-7215 | jjetcheva@firetide.com |
| Malik Audeh | Tropos | 555 Del Rey Ave, Sunnyvale, CA 94085 | +1-408-331-6834 | audeh@tropos.com |
| Michael Bahr | Siemens | CT IC 2, Otto-Hahn-Ring 6, 81730 München, Germany | +49-89-636-49926 | bahr@siemens.com |
| Jan Kruys | Cisco | Cisco Way Bld 14 San Jose, CA | + 31 348 453719 | jkruys@cisco.com |
| Azman-Osman Lim | NICT | 3-5 Hikaridai, Seika-cho, Soraku-gun, Kyoto 619-0289, Japan | +81-774-98-6868 | aolim@nict.go.jp |
| Shah Rahman | Cisco | Cisco Way, San Jose, CA, USA | +1-408-525-1351 | sharahma@cisco.com |
| Joseph Kim | STMicroelectronics | 1060 East Brokaw Road, Mail Station 212, San Jose, CA 95131 | +1-408-621-4913 | kyeongsoo.kim@st.com |
| Steve Conner | Intel | 2111 NE 25th Ave, M/S JF3-206, Hillsboro, OR 97124 | +1-503-264-8036 | w.steven.conner@intel.com |
| Hang Liu | Thomson | Two Independence Way – Suite 300 Princeton, NJ 08540 | +1 609 987-7335 | hang.liu@thomson.net |
| Guenael Strutt | Motorola | 485 N. Keller Rd., Suite 250, Maitland, FL 32751 | +1-407-659-5350 | guenael.strutt@motorola.com |

Abstract

The HWMP overview section introduces a set of procedures (previously described in the HWMP clause).  We distinguish two modes of operation: (1) on demand mode, (2) proactive tree-based mode.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

- This text provides a resolution to Comments 168, 169, 170, 171, 173, 174, 175, 176, 178, 179, 181, 185, 186, 188, 189, 190, 191, 192.  The following instructions are amendments to Draft-P802.11s-D0.04.

Instruction to Editor: Remove clause “7.3.2.42 HWMP Information elements”

Instruction to Editor: Replace clause 11A.4 with the following:

  - Hybrid Wireless Mesh Protocol

    - Overview

      - Rules shared by all routing modes

The Hybrid Wireless Mesh Protocol (HWMP) is a mesh routing protocol that combines the flexibility of on-demand routing with proactive topology tree extensions. The combination of reactive and proactive elements of HWMP enables optimal and efficient path selection in a wide variety of mesh networks (with or without infrastructure).

HWMP uses a common set of protocol primitives, generation and processing rules taken from Ad Hoc On Demand (AODV) routing protocol [IETF RFC 3561] adapted for Layer-2 address-based routing and link metric awareness. AODV forms the basis for finding on-demand routes within a mesh network while additional primitives are used to proactively set up a distance-vector tree rooted at a single root MP. The root role that enables building of topology tree is a configurable option of an MP.

HWMP supports two modes of operation depending on the configuration.  These modes are:

- On demand mode: this mode allows MPs to communicate using peer-to-peer routes.  The mode is used in situations where there is no root configured.  It is also used in certain circumstances if there is a root configured.

- Proactive tree building mode: this can be performed by using either the RREQ or RANN mechanism.

These modes are not exclusive: on demand and proactive modes may be used concurrently.

All HWMP modes of operation utilize common processing rules and primitives.  HWMP control messages are the Route Request (RREQ), Route Reply (RREP), Route Error (RERR) and Root Announcement (RANN).  The metric cost of the links determines which routes HWMP builds. In order to propagate the metric information between MPs, a metric field is used in the RREQ, RREP and RANN messages.

Routing in HWMP uses a sequence number mechanism to maintain loop-free connectivity at all times.  Each MP maintains its own sequence number, which is propagated to other MPs in the HWMP control messages.

      - On demand routing mode

If a source MP needs to find a route using the on demand routing mode, it broadcasts a RREQ with the destination MP specified in the destination list and the metric field initialized to 0.

When a MP receives a RREQ it creates a route to the source or updates its current route if the RREQ contains a greater sequence number, or the sequence number is the same as the current route and the RREQ offers a better metric than the current route.  If a new route is created or an existing route modified, the RREQ is also forwarded (re-broadcast).  Each MP may receive multiple copies of the same RREQ that originated in the source, each RREQ traversing a unique path from the source to the MP.

Whenever a MP forwards a RREQ, the metric field in the RREQ will be updated to reflect the cumulative metric of the route to the RREQ’s source.  After creating or updating a route to the source, the destination MP sends a unicast RREP back to the source.

Intermediate MPs generate RREPs only if the “Destination Only (DO)” flag is not set, e.g. DO = 0, corresponding destinations, provided that they have routes to these destinations. If the DO flag is set to 1, which is default, only the destination MP can generate a RREP. If an intermediate MP receives a RREQ with the DO flag set to 0 for a destination and this intermediate MP already has a valid route to the destination, it issues a unicast RREP to the source. Furthermore, if the “Reply and Forward (RF)” flag is set to 1 for the destination, this intermediate MP will forward the RREQ with the DO flag for the destination set to 1 (the reason to set the DO flag to 1 is to suppress any RREP messages from the subsequent intermediate MPs). Otherwise, there is no RREQ forwarding at intermediate MP. The purpose of the “Destination Only” and “Reply and Forward” mechanisms is to enable a MP to quickly establish a route using the RREP generated by the intermediate MP and send data frames with a low route discovery delay and buffer requirement, while allowing that the route with the best route metric will be chosen (or validated) after the reverse route establishment procedure has been completed. The source sets the DO flag to 0 and RF flag to 1 for a destination in the RREQ only when it does not have a valid route and wants to discover a new route to this destination. As described below, the DO flag in the maintenance RREQ is always set to 1.

Intermediate MPs create a route to the destination on receiving the RREP, and also forward the RREP toward the source. When the source receives the RREP, it creates a route to the destination. If the destination receives further RREQs with a better metric, then the destination updates its route to the source to the new route and also sends a fresh RREP to the source along the updated route.  Thus a bidirectional, best metric end-to-end route is established between the source and destination.

Note that the RREQ processing when the “Destination Only” flag is set to 0 but the “Reply and Forward” flag is set to 1, as specified in this standard, is different from that of the original AODV specification. Also, note that in HWMP, the RREQ processing at intermediate MPs is controlled per destination.

      - Proactive tree building mode

There are two mechanisms for proactively disseminating routing information for reaching the root MP.  The first method uses a proactive Route Request (RREQ) message and is intended to create routes between the root and all MPs in the network proactively.   The second method uses a Root Annoucement (RANN) message and is intended to distribute route information for reaching the root but the actual routes to the root can be built on-demand.

An MP configured as root would send either proactive RREQ or RANN messages periodically.

        - Proactive RREQ mechanism

The RREQ tree building process begins with a proactive Route Request message sent by the root MP, with the destination address set to all ones (broadcast address), the DO flag set to 1 and the RF flag set to 1. The RREQ contains the distance metric (set to 0 by the root) and a sequence number.  The proactive RREQ is sent periodically by the root, with increasing sequence numbers.

Any MP hearing a proactive RREQ creates or updates its forwarding information to the root MP, updates the metric and hop count of the RREQ, records the metric and hop count to the root, and then transmits the updated RREQ.  Thus, information about the presence of and distance to available root(s) is disseminated to all MPs in the network.

Each MP may receive multiple copies of a proactive RREQ, each traversing a unique path from the root to the MP. A MP updates its current route to the root if and only if the RREQ contains a greater sequence number, or the sequence number is the same as the current route and the RREQ offers a better metric than the current route to the root.  The processing of the proactive RREQ is the same as in the on-demand mode described in 11A.4.1.2.

If the proactive RREQ is sent with the “Proactive RREP” bit set to 0, the recipient MP may send a gratuitous RREP if required (for example, if the MP has data to send to the root and requires establishing bidirectional route with the root). If the RREQ is sent with a “Proactive RREP” bit set to 1, the recipient MP shall send a gratuitous RREP. The gratuitous RREP establishes the route from the root to the MP.  When the route from an MP to a root changes, and the root RREQ was sent with a “Proactive RREP” bit set to 1, it shall send a gratuitous RREP to the root containing the addresses of the MPs which have established a route to the root through the current MP.

        - Proactive RANN mechanism

The root periodically floods a RANN message into the network.  The information contained in the RANN is used to disseminate route metrics to the root.

Upon reception of a RANN, each MP that has to create or refresh a route to the root will send a unicast RREQ to the root via the MP from which it received the RANN.

The unicast RREQ will follow the same processing rules defined in the on demand mode (11A.4.1.2).

The root sends a RREP in response to each RREQ.  The unicast RREQ creates the reverse route from the root to the originating MP, while the RREP creates the forward route from the MP to the root.

When the route from an MP to a root changes, it may send a RREP with the addresses of the MPs which have established a route to the root through the current MP.

    - Definitions

Figure 1: Illustration of definitions

Originator:

The originator is the MP that initiates an HWMP routing message that is flooded through the whole wireless mesh network (e.g. RREQ). The transmission of the corresponding information element by the originator is called “original transmission”.

Forwarding Information

The forwarding information maintained by an MP consists of at least a destination, the Destination Sequence Number (DSN), the next hop to the destination, the route metric and the lifetime of this forwarding information.Stored forwarding information can be active or inactive (invalidated). The latter means that the forwarding information is still known for future reference but not used for forwarding.

Forward Route:

The stored forwarding information of the path from the originator to the destinaton of a route discovery. Set up by route replies.

Reverse Route:

The stored forwarding information of the path from the destination to the originator of a route discovery. Set up by route requests.

Unreachable destination:

A destination is considered unreachable in two cases:

- An MP does not have active forwarding information for the destination and the MP did not receive a RREP in response to its route request for this destination within the set waiting time.

- The destination cannot be reached over the established path, because it contains a broken link.

Destination Sequence Number: An integer sequence number associated with an MP, which is used to distinguish newer from older routing information. See also section XXX(Destination Sequence Number).

Time-to-Live: An integer number that is used to limit the number of hops of frame in the WLAN mesh network. See also 11A.1.1.3.

Unicast/Broadcast RREQ: A “unicast RREQ” is a RREQ IE contained in a management frame that is unicast to the next hop toward the destination of the RREQ.  A “broadcast RREQ” is an RREQ IE contained in a management frame that is broadcast.

Dependent MP: A neighboring MP which has established a route to the root through the local MP.

    - General rules for processing HWMP information elements

This subclause describes the rules for the processing of the following components of the HWMP messages and IEs:

- Destination Sequence Number

- TTL

- Metric

Note: It is assumed that the receiving MP will only receive HWMP messages from MPs with which it has established a secure link. Therefore, all HWMP messages received are presumed to have originated in the same mesh network that the receiving MP belongs to.

      - Re-transmission

Many HWMP IEs are intended to be flooded across a mesh network and therefore are retransmitted by MPs that receive them. Each retransmission is subject to certain rules or limitations as explained in the following subclauses.  Some parameters in the HWMP IEs will be updated before retransmission, e.g. route metric.

      - Destination Sequence Number (DSN)

The DSN is set by the originator of a RANN, RREQ or RREP information element.

The value of the DSN is specific to each originator; it is incremented with each new instance of one of the aforementioned IE types.  In case the DSN reaches its maximum value, the incremented DSN becomes the lowest DSN value.

In general, when an MP receives an information element with a DSN that is less than the last received DSN for that originator, it will discard the received IE. If they are the same, the outcome (IE discarded or not) depends on the type of the information element and some additional conditions. These cases are noted in the applicable IE descriptions.

When the destination increments its sequence number, it shall do so by treating the sequence number value as if it were an unsigned number. To accomplish sequence number rollover, if the sequence number has already been assigned to be the largest possible number representable as a 32-bit unsigned integer (i.e., 4294967295), then when it is incremented it will then have a value of zero (0). On the other hand, if the sequence number currently has the value 2147483647, which is the largest possible positive integer if 2's complement arithmetic is in use with 32-bit integers, the next value will be 2147483648, which is the most negative possible integer in the same numbering system. The representation of negative numbers is not relevant to the increment of HWMP sequence numbers. This is in contrast to the manner in which the result of comparing two HWMP sequence numbers is to be treated.

In order to ascertain that information about a destination is not stale, the MP compares its current numerical value for the sequence number with that obtained from the incoming HWMP message. This comparison shall be done using signed 32-bit arithmetic, this is necessary to accomplish sequence number rollover. If the result of subtracting the currently stored sequence number from the value of the incoming sequence number is less than zero, then the information related to that destination in the HWMP message shall be discarded, since that information is stale compared to the MP's currently stored information.

      - Time-to-Live (TTL)

The TTL is set by the originator of an HWMP IE. It gives the maximum number of hops that the IE is allowed to be forwarded.

A receiving MP will decrement the value of the TTL field before re-transmitting the IE. If the decremented value is zero, the IE will be discarded.

      - Re-transmission delay

In general, retransmission of an HWMP IE is not subject to a delay.  Exceptions exist for the RANN IE as described below.

      - Forwarding Information

The forwarding information maintained by a MP consists at least of a destination, the DSN of the destination, the next hop to the destination, the route metric to the destination and the lifetime of this forwarding information.

Stored forwarding information can be active or inactive (invalidated). The latter means that the forwarding information is still known for future references but not used for forwarding.

      - Creation and Update of Forwarding Information

HWMP path selection information elements create or update the forwarding information in the MPs that process these information elements. The creation and update of forwarding information follows the same rules for RREQ, RREP, and RANN. These rules are given below. “HWMP_IE” stands for the IE under consideration (RREQ, RREP, or RANN). Exceptions are stated in the corresponding sections of the descriptions of the information elements.

- If the MP does not have stored any active forwarding information to the originator of the HWMP_IE (hwmp_ie.originator_address), it creates this information from the field hwmp_ie.originator_address ( destination), the transmitter address of the management frame containing the HWMP_IE ( next hop), the accumulation of the value of field hwmp_ie.metric with the metric of the last link ( path metric), and the value of field hwmp_ie.lifetime ( lifetime).

- If the MP does not have stored any active forwarding information to the transmitter of the HWMP_IE, it creates this information from the transmitter address of the management frame containing the HWMP_IE ( destination and next hop), the metric of the last link ( path metric), and the value of field hwmp_ie.lifetime ( lifetime).

- If the MP has stored any active forwarding information to the originator of the HWMP_IE (hwmp_ie.originator_address), then the MP updates this forwarding information with the transmitter address of the management frame containing the HWMP_IE ( next hop), the accumulation of the value of field hwmp_ie.metric with the metric of the last link ( path metric), and the larger one of the lifetime of the stored forwarding information and the value of field hwmp_ie.lifetime (lifetime).

- If the MP has stored any active forwarding information to the transmitter of the HWMP_IE and if the path metric of this information is larger than the metric of the last link, then the MP updates this forwarding information with the transmitter address of the management frame containing the HWMP_IE ( next hop), the metric of the last link ( path metric), and the larger one of the lifetime of the stored forwarding information and the value of field hwmp_ie.lifetime ( lifetime).

      - Metric of Last Link

The term metric of last link specifies the current link metric between the transmitter of the IE under consideration and the mesh point that received this IE. The latter is the mesh point under consideration.

    - Root Announcement (RANN)

This subclause describes the function, generation and processing of the Root Announcement IE.

      - Function

This IE is used for announcing the presence of a MP configured as Root MP. RANN IEs are sent out periodically by the root MP.

The RANN IE propagates route metric information across the network so that each MP can select a best metric path to the announced root MP. This mechanism allows bidirectional trees to be built, using a robust unicast procedure initiated by the MPs.  This ensures that the root is aware of all MPs in the mesh.

Receiving MPs shall propagate the RANN as described below.

      - RANN information element

Figure s2: RANN  Element

| Octets: 1 | 1 | 1 | 1 | 1 | 6 | 4 | 4 | 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Element ID | Length | Flags | Hopcount | Time to Live | Originator Address | Destination Sequence Number | Lifetime | Metric |

Table s1: RANN Element Fields

| Field | Value/description |
| --- | --- |
| ID | TBD |
| Length | Length of the IE |
| Flags | Bit 0: Portal Role (0 = non-portal, 1 = portal) Bit 1 – 7: Reserved |
| Hop Count | The number of hops from the Originator to the MP transmitting the request |
| Time to Live | Remaining number of times the RANN may be forwarded. |
| Originator Address | Root MAC address |
| Destination Sequence Number | A sequence number specific to the originator (root). |
| Lifetime | The time for which MPs receiving the RANN consider the forwarding information to be valid. |
| Metric | The cumulative metric from the Originator to the MP transmitting the announcement. |

      - Conditions for generating and sending a RANN

An MP will send out a RANN in the following cases:

Case A: Original transmission

All of the following applies:

- The MP is configured as a Root MP

- at every ROOT_ANNOUNCEMENT_INTERVAL

Content:

| Field | Value |
| --- | --- |
| ID | RANN IE ID |
| Length |  |
| Flags | Bit 0: Portal Role (0 = non-portal, 1 = portal) Bit 1 – 7: Reserved |
| Hop Count | 0 |
| Time to Live | Maximum number of hops allowed for this IE |
| Originator Address | Own MAC address |
| Destination Sequence Number | Last used DSN + 1 |
| Lifetime | HWMP_PATH2ROOT_TIMEOUT (shall be greater than ROOT_ANNOUNCEMENT_INTERVAL) |
| Metric | initial metric value (0 for airtime metric) |

Case B:	Re-transmission

All of the following applies:

- the MP has received and accepted a RANN – See 11A.1.2.4

- RANN_PROPAGATION_DELAY has expired – See 11A.1.2.4.2

Content:

| Field | Value |
| --- | --- |
| ID | As received |
| Length | As received |
| Flags | As received |
| Hop Count | As received + 1 |
| TTL | As received – 1 |
| Originator Address | As received |
| Destination Sequence Number | As received |
| Lifetime | As received |
| Metric | As received + link metric to the transmitting MP |

      - RANN Reception

Received RANN IEs are subject to certain acceptance criteria. Processing and actions taken depend on the content of the RANN and the forwarding information maintained by the receiving MP.

See also 11A.4.3: General rules for message processing

        - Acceptance criteria

The RANN will be discarded if any of the following is true:

- The DSN < previous DSN from this originator

- ( DSN = previous DSN ) AND (updated path metric >= previous path metric )

        - Effect of receipt

The following applies only to RANN that has not been discarded.

- The receiving MP shall initiate a timer for RANN_PROPAGATION_DELAY

- The receiving MP may

- initiate a RREQ/RREP exchange with the Root to set up or update a route to the Root.

See RREQ, when generated, case C (CHECK)

- send a gratuitous RREP to the Root. See: RREP, when generated, case D

- The receiving MP shall record the Originator Address, together with the DSN, hopcount, metric.

- The receiving MP shall transmit a RANN as defined in 11A.1.2.3, Case B

- The receiving MP shall maintain one root as its active root—this choice may be changed at any time, e.g. if the metric of the received RANN justifies this.

    - Route Request  (RREQ)

This subclause describes the function, generation and processing of the Route Request IE.

      - Function

This IE is used for three purposes:

- Discovering a route to one or more destinations

- Building a proactive (reverse) routing tree to the root MP

- Confirming a route to a destination (optional)

      - RREQ information element

Figure sRREQ1: RREQ Element

| Octets: 1 | 1 | 1 | 1 | 1 | 4 | 6 | 4 | 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Element ID | Length | Flags | Hopcount | Time to Live | RREQ ID | Originator Address | Originator Sequence Number | Lifetime |

| 4 | 1 |  |  | 6 | 4 |  | 1 |  |  | 6 | 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Metric | Per Destination Flags |  |  | Destination Address #1 | Destination Seq. Num.#1 | …. | Per Destination Flags |  |  | Destination Address #N | Destination Seq. Num.#N |
|  | DO #1 | RF #1 | Reserved |  |  |  | DO #N | RF #N | Reserved |  |  |

Table sRREQ1: RREQ Element Fields

| Field | Value/description |
| --- | --- |
| ID | TBD |
| Length | Length of the IE |
| Flags | Bit 0: Portal Role (0 = non-portal, 1 = portal) Bit 1: (0 = broadcast, 1 = unicast) (see 11A.4.2) Bit 2: RREP (0 = off, 1 = on) Bit 3 – 7: Reserved |
| Hop Count | The number of hops from the Originator to the MP transmitting the request |
| Time to Live | Maximum number of hops allowed for this IE |
| RREQ ID | Some unique ID for this RREQ |
| Originator Address | Originator MAC address |
| Originator’s Destination Sequence Number | A sequence number specific for the originator |
| Lifetime | The time for which MPs receiving the RREQ consider the forwarding information to be valid. |
| Metric | The cumulative metric from the Originator to the MP transmitting the RREQ |
| Destination Count N | Gives the number of Destinations (N) contained in this RREQ |
| Per Destination Flags | Flags Bit 0: DO (Destination Only): If DO=0, an intermediate MP with active forwarding information to the corresponding destination shall respond to the RREQ with a unicast RREP; if DO=1, only the destination can respond with a unicast RREP. The default value is 1. Bit 1: RF (Reply-and-Forward):. The RF flag controls the forwarding of RREQ at intermediate MPs. When DO=0 and the intermediate MP has active forwarding information to the corresponding destination, the RREQ is not forwarded if RF=0 and forwarded if RF=1. The default value is 1. When DO=1, the RF flag has no effect.. Bit 2-7: Reserved |
| Destination Address | MAC address of the destination MP |
| Destination Sequence Number | The latest sequence number received in the past by the originator for any route towards the destination. |

      - Conditions for generating and sending a RREQ

An MP will send out a RREQ in the following cases:

Case A: Original Transmission (Route Discovery)

All of the following applies:

- The MP needs to establish an on-demand route to one or more destinations for which there is no ongoing route discovery initiated by this MP.

- The MP has not sent more than (HWMP_RREQ_RATELIMIT – 1) route request messages during the last second. If this is the case, the transmission of the RREQ has to be postponed until this condition becomes true.

- The MP has not made more than (HWMP_MAX_RREQ_RETRIES – 1) repeated attempts at route discovery towards the destination of the RREQ.

Content:

| Field | Value |  |
| --- | --- | --- |
| ID | Tbd |  |
| Length | 27 + N*11 |  |
| Flags | Bit 0: 0 (no portal role) Bit 1: 0 (broadcast) Bit 2: 0 (no proactive RREP applicable) Bit 3 – 7: Reserved |  |
| Hop Count | 0 |  |
| Time to Live | Maximum number of hops allowed for this IE, e.g., HWMP_NET_DIAMETER. |  |
| RREQ ID | Previous RREQ ID + 1 |  |
| Originator Address | Own MAC address |  |
| Originator’s Destination Sequence Number |  | Previous Originator DSN + 1. See Note 2 |
| Metric |  | 0 |
| Destination Count |  | (N) |
| Per Destination Flags |  | DO flag, RF flag, as required |
| Destination Address |  | MAC address of requested destination |
| Destination Sequence Number |  | The latest sequence number received in the past by the originator for any route towards the destination. |

Note 1: 	Repeated attempts by an MP at route discovery towards a given (set of) destination(s) shall be limited to HWMP_MAX_RREQ_RETRIES and utilize a binary exponential backoff between transmissions. The minimum waiting time for the RREP corresponding to an RREQ is 2 * HWMP_RT_NETDIAMETER_TRAVERSAL_TIME.

Note 2: In order to improve route stability (and further reduce overhead), an MP may use the same Originator Destination Sequence Number (Originator DSN) for a certain time interval.  The Originator DSN is incremented only after at least HWMP_RT_NETDIAMETER_TRAVERSAL_TIME  has elapsed since the previous increment. This mechanism prevents MPs from changing the route frequently to the source every time the source sends a burst of RREQs within a very short time.  This element of the protocol allows a source MP to immediately initiate on-demand route discovery to a new destination without affecting recently refreshed routes to the source in other MPs.

Case B:	Original Transmission (Route Maintenance) (optional implementation enhancement)

All of the following applies:

- the MP has a route to a given destination that is not a Root MP

- the HWMP_ROUTE_MAINTENANCE_INTERVAL has expired

Content:

| Field | Value |  |
| --- | --- | --- |
| ID | Tbd |  |
| Length | As required |  |
| Flags | Bit 0: 0 (no portal role) Bit 1: 0 (broadcast) Bit 2: 0 (no proactive RREP applicable) Bit 3 – 7: Reserved |  |
| Hop Count | 0 |  |
| Time to Live | Maximum number of hops allowed for this IE = HWMP_NET_DIAMETER. |  |
| RREQ ID | Previous RREQ ID +1 |  |
| Originator Address | Own MAC address |  |
| Originator’s Destination Sequence Number |  | Originator DSN + 1. See Note 2 under Case A. |
| Metric |  | 0 |
| Destination Count |  | Gives the number of Destinations contained in this RREQ |
| Per Destination |  | Flags = DO flag = 1, RF flag = 0 |
|  |  | Address = Destination MAC Address |
|  |  | Destination Sequence Number (for this destination) |
| Etc |  |  |

Case C: Root Path Confirmation.

All of the following applies:

- the MP has a route to a Root MP which broadcasts RANNs

- the HWMP_ROUTE_CONFIRMATION_INTERVAL has expired

Content:

| Field | Value |  |
| --- | --- | --- |
| ID | Tbd |  |
| Length | As required |  |
| Flags | Bit 0: 0 (no portal role) Bit 1: 1 (unicast) Bit 2: 0 (no proactive RREP applicable) Bit 3 – 7: Reserved |  |
| Hop Count | 0 |  |
| Time to Live | 1 |  |
| RREQ ID | Not used |  |
| Originator Address | Own MAC address |  |
| Originator’s Destination Sequence Number |  | Originator DSN + 1. See Note 2 under Case A. |
| Metric |  | 0 |
| Destination Count |  | 1 |
| Per Destination |  | Flags: DO flag = 1, RF flag = 0 |
|  |  | Address = Root MP MAC Address |
|  |  | Destination Sequence Number (for this destination) |
| Etc |  |  |

Case D:	RREQ Re-transmission

Case D1 (destination count = 1, no RREP generation):

All of the following applies:

- the MP has received and accepted a RREQ – See 11A.1.1.4

- Destination_count = 1

- the MP is not the destination of the RREQ

- the MP has no active forwarding information for the requested destination

- [Destination Only flag of the destination in the RREQ is ON (DO = 1)] 
OR
[[Destination Only flag of the destination in the RREQ is OFF (DO = 0)] AND [MP has no active forwarding information for the requested destination rreq.destination_address]]

Content for D1:

| Field | Value |  |
| --- | --- | --- |
| ID | Tbd |  |
| Length |  |  |
| Flags | as received |  |
| Hop Count | As received + 1 |  |
| Time to Live | As received – 1 |  |
| RREQ ID | As received |  |
| Originator Address | As received |  |
| Originator’s Sequence Number |  | As received |
| Metric |  | As receiver + own metric towards transmitter of received RREQ |
| Destination Count |  | 1 |
| Per Destination flags #1 |  | as received |
| Destination MAC address #1 |  | as received |
| Destination Sequence Number #1 |  | as received |

Case D2 (destination count = 1, RREP generation as intermediate MP):

All of the following applies:

- the MP has received and accepted a RREQ – See 11A.1.1.4

- rreq.destination_count = 1

- the MP is not the destination of the RREQ

- the MP has active forwarding information for the requested destination rreq.destination_address

- Destination Only flag of the destination in the RREQ is OFF (DO = 0)

- Reply and Forward flag of the destination in the RREQ is ON (RF = 1)

Content for D2:

| Field | Value |  |
| --- | --- | --- |
| ID | Tbd |  |
| Length |  |  |
| Flags | as received |  |
| Hop Count | As received + 1 |  |
| Time to Live | As received – 1 |  |
| RREQ ID | As received |  |
| Originator Address | As received |  |
| Originator’s Sequence Number |  | As received |
| Metric |  | As receiver + own metric towards transmitter of received RREQ |
| Destination Count |  | 1 |
| Per Destination flags #1 |  | Bit 0 (DO): 1 (set to 1 before forwarding because MP sent a RREP) Bit 1 (RF): as received |
| Destination MAC address #1 |  | as received |
| Destination Sequence Number #1 |  | as received |

Case D3 (destination count > 1): All of the following applies

- the MP has received and accepted a RREQ – See 11A.1.1.4

- there is at least one requested destination left after processing the RREQ according to 11A.1.1.4.

Content for D3:

| Field | Value |  |
| --- | --- | --- |
| ID | Tbd |  |
| Length |  |  |
| Flags | as received |  |
| Hop Count | As received + 1 |  |
| Time to Live | As received – 1 |  |
| RREQ ID | As received |  |
| Originator Address | As received |  |
| Originator’s Sequence Number |  | As received |
| Metric |  | As receiver + own metric towards the transmitter |
| Destination Count |  | 1  destination count  received destination count received destination count less the number of requested destinations, for which the processing MP - is the destination or - has active forwarding information for the requested destination and the corresponding Destination Only flag is off (DO=0) and Reply and Forward flag is on (RF = 1) |
| For the per destination fields (per destination flags, destination MAC address, destination sequence number) assume the following: - destination #A: If destination A would have been the only requested destination, it would generate a RREQ for retransmission according to case D1 - destination #B: If destination B would have been the only requested destination, it would generate a RREQ for retransmission according to case D2 |  |  |
| Per Destination Flags #A |  | as received |
| Destination MAC address #A |  | as received |
| Destination Sequence Number #A |  | as received |
| Per Destination Flags #B |  | Bit 0 (DO): 1 (set to 1 because MP sent RREP) Bit 1 (RF): as received |
| Destination MAC address #B |  | as received |
| Destination Sequence Number #B |  | as received |

Case E:	Proactive Root RREQ

All of the following applies:

- The Root MP is configured to send proactive root RREQs

- [The Root Announcement interval has expired]

Content:

| Field | Value |  |
| --- | --- | --- |
| ID | Tbd |  |
| Length |  |  |
| Flags | Bit 0: As needed (portal role) Bit 1: 0 (broadcast) Bit 2: As needed (proactive RREP) Bit 3 – 7: Reserved |  |
| Hop Count | 0 |  |
| Time to Live | Maximum number of hops allowed for this IE= HWMP_NET_DIAMETER. |  |
| RREQ ID | Previous RREQ ID + 1 |  |
| Originator Address | Own MAC address |  |
| Originator’s Destination Sequence Number |  | Previous Originator DSN + 1. See Note 2 |
| Metric |  | 0 |
| Destination Count |  | 1 |
| Per Destination |  | DO = 1, RF = 1 |
|  |  | Address = All ones (broadcast address) |
|  |  | Destination Sequence Number (none) |
| Etc |  |  |

      - RREQ processing

Received RREQ IEs are subject to certain acceptance criteria. Processing and actions taken depend on the contents of the RREQ and the information available to the receiving MP.

See also 11A.4.3: General rules for message processing

        - Acceptance criteria

The RREQ will be discarded if any of the following is true:

- The Originator DSN < previous Originator DSN

- ( DSN = previous DSN ) AND (updated path metric >= previous path metric )

Otherwise, the RREQ IE will be accepted.

See also 11A.4.3: General rules for message processing

        - Effect of receipt

The following applies only to a RREQ that has not been discarded:

- The receiving MP shall record the RREQ ID, the Originator Address, and entries for each destination MAC Address and DSN

- The receiving MP shall update the active forwarding information it maintains for the originator and previous hop MPs of the RREQ (see 11A.1.1.6)

- If the MP is addressed by the RREQ it will initiate the transmission of an RREP to the originator (see tbd)

- If the MP has active forwarding information to any of the requested destinations and the DO flag for such a destination is OFF (DO=0), it will initiate the transmission of an RREP to each of these destinations

- If there are destinations in the RREQ that have been not processed in steps 3 or 4 or that have been processed in step 4 but the corresponding Reply and Forward Flag is ON (RF = 1),  the receiving MP shall retransmit the RREQ as defined in 11A.1.1.3, Case D

    - Route Reply (RREP)

This subclause describes the function, generation and processing of the Route Reply IE.

      - Function

The purpose of the Route Reply Information Element is

- to establish a forward route to a destination and

- to confirm that a destination is reachable.

      - RREP information element

| Octets: 1 | 1 | 1 | 1 | 1 | 6 | 4 |
| --- | --- | --- | --- | --- | --- | --- |
| ID | Length | Mode Flags | Hopcount | Time to Live | Destination Address | Destination Seq.Num. |

| 4 | 4 | 6 | 4 | 1 | 6 | 4 |  | 6 | 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Lifetime | Metric | Source Address #1 | Source Seq. Num. | Dependent MP Count N | Dependent MP MAC Address #1 | Dependent MP DSN #1 | ... | Dependent MP MAC Address #N | Dependent MP DSN #N |

Figure s3: Route Reply Element

Table s2: RREP Element Fields

| Field | Value/description |
| --- | --- |
| ID | TBD |
| Length | Length of the IE |
| Flags | Bit 0 – 7: Reserved |
| Hop Count | The number of hops from the Originator Address to the local MP |
| Time to Live | Maximum number of hops allowed for this IE |
| Destination Address | MAC address of the Destination of the RREP |
| Destination Sequence Number | DSN of the originator of the RREP |
| Lifetime | If applicable: reflects the Lifetime of the RREQ this RREP responds to |
| Metric | The cumulative metric from the Originator Address to the local MP. |
| Dependents | List of addresses of dependent MPs including MAC address and the DSN associated with a given MAC address |

      - Conditions for generating and sending a RREP

An MP will send out a RREP in the following cases:

Case A: Original transmission

A RREP is transmitted if the MP has received a RREQ fulfilling all of the following conditions:

- One of the following applies:

  - The Destination Address of the RREQ is the same as MAC address of the receiving MP

  - The destination address of the RREQ = all 1’s (broadcast) and the RREP flag is set to 1 (”Proactive RREP”)

- One of the following applies:

  - The Originator DSN of the RREQ (rreq.orig_dsn) is greater than the DSN of the last RREQ received from the same originator address (which includes the case that there is no route to the originating MP)

  - The Metric is better than the path selection metric currently associated with the Originator Address and the Originator DSN of the RREQ (rreq.orig_dsn) is equal to the DSN of the last RREQ received from the same originator address

The content of the generated RREP shall be:

| Field | Value |  |
| --- | --- | --- |
| ID | RREP IE ID |  |
| Length | As required |  |
| Flags | Bit 0 – 7: Reserved |  |
| Hop Count | 0 |  |
| Time to Live | Maximum number of hops allowed for this IE |  |
| Destination Address | Own MAC address |  |
| Destination Sequence Number | DSN of the originator of the RREP |  |
| Lifetime | As per the RREQ that triggered the transmission of this RREP |  |
| Metric | 0 |  |
| Dependents |  | As applicable: List of addresses of dependent MPs including MAC address and DSN |

Note: the destination address of the action frame carrying the RREP IE is the Originator Address of the RREQ or RANN that triggered the RREP.

Case B:	Re-transmission (forwarding)

A RREP is transmitted if all of the following applies:

- the MP has received and accepted the RREP – See 11A.1.3.4.1

- the MP is not the destination of the RREP

Content:

| Field | Value |
| --- | --- |
| ID | As received |
| Length | As received |
| Flags | As received |
| Hop Count | As received + 1 |
| Time to Live | As received – 1 |
| Destination Address | As received |
| Destination Sequence Number | As received |
| Lifetime | As received |
| Metric | As received + own metric towards the transmitting MP |
| Dependents | As received |

Note: the destination address of the action frame carrying the RREP IE is the next hop to the Originator Address of the RREQ or RANN that triggered the RREP.

Case C:	Intermediate reply

A RREP is transmitted if the MP has received a RREQ fulfilling all of the following conditions:

- The RREQ Destination Only flag is set to 0

- The receiving MP has  active forwarding information with:

  - A destination which is the same as the Destination Address of the RREQ

  - A DSN which is less than or equal to the DSN of the RREQ (rreq.dest_dsn)

  - A route metric which is greater than or equal to the Metric or the RREQ

  - A non-zero lifetime

The content of the generated RREP shall be:

| Field | Value |
| --- | --- |
| ID | RREP IE ID |
| Length | As required |
| Flags | Bit 0 – 7: Reserved |
| Hop Count | 0 |
| Time to Live | Maximum number of hops allowed for this IE |
| Destination Address | Destination MAC address from the RREQ |
| Destination Sequence Number | DSN of the stored forwarding information of the Destination MAC address of the RREQ |
| Metric | 0 |
| Dependents | As applicable: List of addresses of dependent MPs including MAC address and DSN |

      - RREP processing

Received RREP IEs are subject to certain acceptance criteria. Processing and actions taken depend on the contents of the RREP and the information available to the receiving MP.

        - Acceptance criteria

The RREP will be discarded if any of the following is true:

- The DSN < previous DSN from this originator

- The Time to Live is 1 or less

        - Effect of receipt

The following applies only to a RREP that has not been discarded

- The receiving MP shall record the Originator Address, together with the DSN, hopcount and metric according to the rules defined in 11A.1.1.6.

- The receiving MP may record the list of dependent MPs if present in the RREP.

- If the receiving MP is not the final destination of the RREP, the RREP is re-transmitted as per Case B above.

    - Route Error Information Element (RERR)

This subclause describes the function, generation and processing of the route error IE.

      - Function

The RERR IE is used for announcing a broken link to all traffic sources that have an active path over this broken link.  The active forwading information associated with the unreachable destinations should no longer be used for forwarding.

A RERR IE is propagated by MPs receiving a RERR if certain conditions are fulfilled.

An MP generating or receiving a RERR may decide to establish routes to unreachable destinations using any of the available HWMP mechanisms.

      - Route Error Information Element

| Octets: 1 | 1 | 1 | 1 (or 4) | 6 | 4 |
| --- | --- | --- | --- | --- | --- |
| ID | Length | Mode Flags | Num of Destinations | Destination Address | Destination MP Seq. Num |

Figure sXX: Route Error Element

Table sXX: Route Error Element Fields

| Field | Value/description |
| --- | --- |
| ID | TBD |
| Length | Length of the IE |
| Mode Flags | Bit 0 – 7: Reserved |
| Number of Destinations | Number of announced destinations in RERR (destination address and destination MP sequence number). |
| Destination Address | Detected unreachable destination MAC address |
| Destination Sequence Number | The sequence number of detected unreachable destination MP |

      - Conditions for generating and sending a RERR

A mesh point will send out a RERR in the following cases:

Case A: Original transmission

All of the following applies:

- The MP detects a link break to the next hop of an active path in its stored forwarding information while transmitting packets to it.  Note: the detection may be triggered by the fact that an MP is unable to forward a data frame to a next hop MP.

- The MP did not send more than HWMP_RERR_RATELIMIT – 1 RERR messages during the last second.

Actions before sending the RERR:

- The destination sequence numbers in all valid stored forwarding information of unreachable destinations announced in this RERR is incremented.

- The stored forwarding information for each unreachable destination announced in this RERR is invalidated.

Content:

| Field | Value |
| --- | --- |
| ID | RERR IE ID |
| Length |  |
| Mode Flags | Bit 0 – 7: Reserved |
| Number of Destinations | Number of announced unreachable destinations in RERR. A destination is unreachable if its next hop in the stored forwarding information is an unreachable neighbor. |
| Destination Address | MAC address of detected unreachable destination #1 |
| Destination Sequence Number | Last used DSN for Destination Address #1 + 1 |

Case B:	Re-transmission

All of the following applies:

- The MP received a RERR from a neighbor for one or more of its active paths in its stored forwarding information.

- The MP has not sent or forwarded more than HWMP_RERR_RATELIMIT – 1 RERR messages during the last second.

Content:

| Field | Value |
| --- | --- |
| ID | RERR IE ID |
| Length |  |
| Mode Flags | Bit 0 – 7: Reserved |
| Number of Destinations | Number of announced unreachable destinations in RERR ( received value) A destination is unreachable if its next hop in the corresponding stored forwarding information is the transmitter of the received RERR. |
| Destination Address | MAC address of detected unreachable destination #1 (as received, but maybe at different position in destination list) |
| Destination Sequence Number | as received (but maybe at different position in destination list) |

      - RERR Reception

Received RERR IEs are subject to certain acceptance criteria. Processing and actions taken depend on the contents of the RERR and the information available to the receiving MP.

See also 11A.4.3: General rules for message processing

        - Acceptance criteria

The RERR is not discarded if the following applies:

- The MP that receives the RERR has forwarding information stored where

  - the destination is contained in the list of unreachable destinations of the RERR and

  - the next hop is the transmitter of the received RERR

        - Effect of receipt

The following applies only to RERR that has not been discarded.

- The destination sequence numbers in all valid stored forwarding information of unreachable destinations announced in this RERR is set to the values taken from the corresponding fields of the RERR (rerr.dsnX) if the latter is greater than the destination sequence number currently stored in the forwarding information ( destination sequence number).

- The stored forwarding information of unreachable destinations announced in this RERR is invalidated.

- The receiving MP shall transmit a RERR as defined in 11A.1.2.3, Case B

    - HWMP parameters

To be added.