IEEE P802.11
Wireless LANs

| Miscellaneous comments |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-07-07 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Assaf Kasher | Intel | POB 1659, Haifa 31015, Israel | 97248651547 | assaf.kasher@intel.com |

The documents suggests resolution for the following comments: 7408, 4690, 3493, 220, 224, 233, 236, 3944

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <Error! Hyperlink reference not valid.>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Abstract

This document contains 802.11n D1.05 submission to solve the following comments: CID 3961

| 7408 | 215 | 5 | 20.3.4.5 | The size of vectors/matrices in eq. (20-57) do not match. Perhaps also related to the unclear comment on "square matrices" at line 2, page 212. | Clarify/correct. Better specify structure of Qk matrices for beamforming: they substantially differ from spatial mapping matrices, for instance their entries are (processed) channel coefficients and not just 0s/1s. |
| --- | --- | --- | --- | --- | --- |

Background:

The equivalent complex baseband MIMO channel model is one in which when a vector  is transmitted in subcarrier  the received vector  is modeled as:

(20-1)

where

is channel matrix of dimensions NRX×NTX

is white (spatially and temporally) Gaussian noise. This is illustrated in Figure n1.

- Figure n1—The beamforming MIMO channel model

When beamforming is used, the transmitter replaces  with  so that the received vector is

TGn Editor: Replace lines 21- 23 in page 251 with

When beamforming is used, the transmitter replaces , that in this case may have only elements, with , where has rows and columns, so that the received vector is

| 4690 | Y | 198 | 4 | 20.3.4.3 | DT |  | The text states that one FEC encoder can be used when the PHY rate is less than equal to 300 Mbps. Is this due to some current preformance related issue. If so, this limitation should be removed from this standards document. It should not address optionals based on performance issues. | The text should explain that 1 or 2 FEC encoders can be used, but should not specify a performance criteria, as this value will be a burden in the future. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3493 | N | 198 |  | 20.3.4.3.1 | DT |  | The use of two BCC encoders for 300Mbps and higher is an unncessary complication of the specification. Two encoders was recommended based on an analysis done over 2 years ago. | Specify the use of a single encoder for all rates which will significantly simplify the specification. |

Suggestion: Reject

We believe that two FEC encoders at a rate of 300Mbmps and above are the best technological solutions for the time frame of the standard.

| 220 | Y | 181 | 6 | Non-HT Short Training Field | I think it less confusing if upper tones in (20-6) are rotated for the reader and then it can also simplify the equation, i.e., can use a single summation instead of two summations | Rotate upper tones in (20-6) by +90 degrees and eliminate second summation and change indices on first summation to go from -N_SR to +N_SR |
| --- | --- | --- | --- | --- | --- | --- |
| 224 | Y | 182 | 8 | Non-HT Long Training Field | I think it less confusing if upper tones in (20-9) are rotated for the reader and then it can also simplify the equation, i.e., can use a single summation instead of two summations | Rotate upper tones in (20-9) by +90 degrees and eliminate second summation in (20-10) and change indices on first summation to go from -N_SR to +N_SR |
| 233 | Y | 190 | 14 | The HT STF training symbol | I think it less confusing if upper tones in (20-13) are rotated for the reader and then it can also simplify the equation, i.e., can use a single summation instead of two summations | Rotate upper tones in (20-13) by +90 degrees and eliminate second summation in (20-14) and change indices on first summation to go from -N_SR to +N_SR |
| 236 | Y | 192 | 16 | The HT-LTF long training Field | I think it less confusing if upper tones in (20-16) are rotated for the reader and then it can also simplify the equation, i.e., can use a single summation instead of two summations | Rotate upper tones in (20-16) by +90 degrees and eliminate second summation in (20-17) and (20-18) and change indices on first summation to go from -N_SR to +N_SR |

Suggestion: Reject

These comments refer to transmission formulas of the STF’s and LTF’s.  There are two reasons that the rotation is defined in the formula and not in the sequence:  First to indicate that the rotation should be considred a part of the channel and should be ignored by the receiver.  Second – to make the formulas for the LTF’s STF’s consitent with the formulas of the HT-SIG and L-SIG or data.

Moreover, document 0935r2 simplifies the equations in question and therefore the change is not necessary

| 3944 | N | 188 | 1 | 20.3.3.2.2.2 | T |  | The formula for HT-SIG transmission is not in the same form as the forumula for L-SIG transmission | Change the formula |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

Fomula as it is now:

Suggested change:

TGnEditor: Replace the second formula in table n63 page 221 in draft D1.06 with

Current Formula (HT-SIG in GF):

TGnEditor: Replace the formula in lines 38-50, page 230 in draft D1.06 with