IEEE P802.11
Wireless LANs

| Spectral Mask Measurement Language Change |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-11-15 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Broady Cash | ARINC, Inc. | 2551 Riva Road, Annapolis, MD | 410-266-4413 | bcash@arinc.com |

Abstract

This is a recommended change to the IEEE 802.11 (11p) document.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

“For WAVE operations in the 5.85 – 5.925 GHz frequency band in the USA, the transmit spectrum mask shall be relative to the station power class. As with non-WAVE 10 MHz channel spacing, for all classes the transmit mask shall demonstrate a 0 dBr bandwidth not exceeding 9 MHz (±4.5 MHz). Additional spectral falloff requirements are provided in Table pI.1. The corresponding WAVE Class A through D spectrum masks are shown in Figures pI.1 through pI.4, respectively. In all cases, the transmitted spectral density of the transmitted signal shall fall within the spectral mask. This shall be accomplished by attenuating the transmitted signal by 55 + 10log(P)dBr at ±15 MHz and above, where P is the total transmitted power in watts. The measurements shall be made using a 100 kHz resolution bandwidth and a 30 kHz video bandwidth.”

Replace the preceding text in “I.2.3 Transmit spectrum mask:” with the following:

For WAVE operations in the 5.85 – 5.925 GHz DSRC frequency band in the USA, in accordance with FCC CFR 47, Section 90.377, the transmitted spectrum shall be as follows and summarized in Table pI.1.  The measurements shall be made using a 100 kHz resolution bandwidth and a 30 kHz video bandwidth and where the 0 dBr reference point is the maximum spectral density of the signal:

a) For WAVE Class A operation using 10 MHz channel spacing, the transmitted spectrum shall have a 0 dBr bandwidth not exceeding 9 MHz, and shall not exceed –10 dBr at 5 MHz frequency offset, –20 dBr at 5.5 MHz frequency offset, –28 dBr at 10 MHz frequency offset, –40 dBr at 15 MHz frequency offset and above.  Figure pI.1 shows the spectral mask for WAVE Class A operation.

b) For WAVE Class B operation using 10 MHz channel spacing, the transmitted spectrum shall have a 0 dBr  bandwidth not exceeding 9 MHz, and shall not exceed –16 dBr at 5 MHz frequency offset, –20 dBr at 5.5 MHz frequency offset, –28 dBr at 10 MHz frequency offset, –40 dBr at 15 MHz frequency offset and above.  Figure pI.2 shows the spectral mask for WAVE Class B operation.

c) For WAVE Class C operation using 10 MHz channel spacing, the transmitted spectrum shall have a 0 dBr bandwidth not exceeding 9 MHz, and shall not exceed –26 dBr at 5 MHz frequency offset, –32 dBr at 5.5 MHz frequency offset, –40 dBr at 10 MHz frequency offset, –50 dBr at 15 MHz frequency offset and above. Figure pI.3 shows the spectral mask for WAVE Class C operation.

d) For WAVE Class D operation using 10 MHz channel spacing, the transmitted spectrum shall have a 0 dBr bandwidth not exceeding 9 MHz, and shall not exceed –35 dBr at 5 MHz frequency offset, –45 dBr at 5.5 MHz frequency offset, –55 dBr at 10 MHz frequency offset, –65 dBr at 15 MHz frequency offset and above.  Figure pI.4 shows the spectral mask for WAVE Class D operation.

Also change the footnote of Table pI.1 from “Reduction in Power Spectral Density, dBr” to “Maximum allowed Power Spectral Density, dBr”