IEEE P802.11
Wireless LANs

| AS Attributes to R1KH |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-11-14 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Jouni Malinen | Devicescape Software, Inc. | 900 Cherry Avenue, 6th Floor, San Bruno, CA 94066, USA | +1 650 829 2630 | jkm@devicescape.com |

Abstract

IEEE 802.11r/D3.0 does not send AS authorization attributes to R1KH in case of FT. This breaks use cases like dynamic VLAN assignment which require the new AP to receive information from the AS even if the full IEEE 802.1X authentication is skipped. This submission proposes changes to 802.11r/D3.0 to allow these attributes to be sent to R1KH and addresses CID 20 of LB 87.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

802.11r/D3.0 LB78 CID 20 / AS attributes to R1KH

This issue is about the new R1KH not receiving attributes from the AS in case of FT since it does not go through the full IEEE 802.1X authentication and does not communicate with the AS (e.g., RADIUS

authentication server). One good example of what is lost is RADIUS attributes for assigning VLAN ID (Tunnel-Private-Group-Id RADIUS attribute). In other words, if dynamic VLAN assignment is used, the

current IEEE 802.11r draft does not provide a mechanism for the new R1KH to know which VLAN the STA should be assigned to.

I went through D2.2 in search of good location for handling this issue. After going through all parts that had anything to do with communication between R0KH and R1KH, it looks like 8A.2.1 would be the place to do this. There is not much details for this communication, but at least there is comment about IEEE 802.1X Authenticator receiving "authorization attributes" associated with MSK. After some thought, I think I could interpret this as actually including all RADIUS attributes from Access-Accept message, i.e., this would also include the VLAN ID mentioned above. This information is included in PMK-R0 PMKSA ("authorization information"). However, it is not sent to the R1KH.

It looks like it would be enough to just add this authorization information into PMK-R1 PMKSA and send this to the R1KH with rest of the information related to PMK-R1. Since there is no details on how the

actual communication between R0KH and R1KH is to happen, so this new change should not require much details. In practice, I would assume that implementations would forward RADIUS attributes from Access-Accept as a blob in whatever mechanism is used to send the data. This way the R1KH

would have a way of receiving additional information from the AS and things like dynamic VLAN assignment could actually work with 802.11r.

Proposed changes to 802.11r/D3.0

8A.2.1 Fast BSS Transition Initial Mobility Domain Association in an RSN

Option #1:

Append the following item into the PMK-R1 PMKSA contents (8A.2.1, page 46, line 33 in D3.0):

- authorization attributes

Option #2:

Append the following item into the PMK-R1 PMKSA contents (8A.2.1, page 46, line 33 in D3.0):

- authorization attributes from the initial association

Option #3:

Append the following item, including the informative note, into the PMK-R1 PMKSA contents (8A.2.1, page 46, line 33 in D3.0):

- authorization attributes from the initial association

(Note: this could include RADIUS attributes from Access-Accept

message if RADIUS is used as the Authenticator-to-AS protocol)

Motion

Move to instruct the editor to apply the proposed changes (option #) from 11-06/1805r0 to the 802.11r draft and to accept the comment 20 in LB87 with the resolution of “Accepted. Addressed with the acceptance of 11-06/1805r1.”