IEEE P802.11
Wireless LANs

| Legacy Protection Mechanism – Proposed text changes |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-11-15 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Brian Hart | Cisco Systems, Inc. | 170 West Tasman Drive, San Jose, CA 95134 | 408-525-3346 | brianh@cisco.com |
| Douglas Chan | Cisco Systems, Inc. | 170 West Tasman Drive, San Jose, CA 95134 | 408-527-9344 | dougchan@cisco.com |
| Luke Qian | Cisco Systems, Inc. | 170 West Tasman Drive, San Jose, CA 95134 | 330-523-2051 | lchia@cisco.com |
| Andrew Myles | Cisco Systems, Inc. | 170 West Tasman Drive, San Jose, CA 95134 | 61-2-8446-1010 | amyles@cisco.com |

Abstract

The current legacy protection mechanism given in the text does not adequately address the issue of collisions between Greenfield packets with 11abg packets. To ensure proper interoperability between devices, we propose changes to the current text to (1) change non-HT/GF protection to match the existing non-GF HT/GF protection for same-BSS devices, and (2) allow HT devices to report the presence of an OBSS HT/non-HT situation, and thereby provide nearby HT devices with information that optional counter-measures may be required.

This document contains a text proposed change to the P802.11n-D1.03 Draft to address CIDs 118, 4017, 4035, 4511, 4522, 10292, 12030, 12048, 12049, 12127.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Introduction:

## Motion to Adopt

A motion to adopt the changes defined in this submission means that the editing instructions and any changed or added material are actioned in the current TGn Draft.  This introduction, is not part of the adopted material.

Editing instructions formatted like this are intended to be copied into the TGn Draft (i.e. they are instructions to the 802.11 editor on how to merge the TGn amendment with the baseline documents).

TGn Editor:  Editing instructions preceded by “TGn Editor” are instructions to the TGn editor to modify existing material in the TGn draft.   As a result of adopting the changes, the TGn editor will execute the instructions rather than copy them to the TGn Draft.

Summission Note: Notes to the reader of this submission are not part of the motion to adopt.  These notes are there to clarify or set in context the response to the omission.

- Text Proposal:

TGn Editor:  Change Table n44 in section 7.3.2.47 as indicated by the following changes:

| B0 B1 | B2 | B3 B1215 | B13 | B14 B15 | B0 B6 | B7 | B8 | B9 | B10 | B11 | B12 B15 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Mode | Non-Greenfield STAs Present | Reserved | OBSS Non-HT STAs Present | Reserved | Basic STBC MCS | Dual CTS Protection | Secondary Beacon | L-SIG TXOP Protection Full Support | PCO Active | PCO Phase | Reserved | Basic MCS Set |
| 2 |  |  |  |  | 2 |  |  |  |  |  |  | 16 |
| - HT Information element format |  |  |  |  |  |  |  |  |  |  |  |  |

TGn Editor:  Insert the following row below the eighth row from top of Table n21 in section 7.3.2.47 as indicated by the following additions:

| OBSS Non-HT STAs Present | Indicates if any non-HT STAs are present. Present in Beacon and Probe response frames transmitted by an AP. Reserved otherwise. See 9.13.3.2 | Set to 1 if one or more non-HT STAs are associated. Examples of when this bit may additionally be set to 1 include, but are not limited to, when - a non-HT BSS is overlapping (a non-HT BSS may be detected by the reception of a Beacon where the supported rates only contain clause 15, 17, 18 or 19 rates, or where the Operating Mode equals 1, 2 or 3) - a management frame (excluding a Probe Request) is received where the supported rate set includes only Clause 15, 17, 18 and 19 rates, or where the Operating Mode field in the HT Information element equals 1, 2 or 3. Set to 0 otherwise. |
| --- | --- | --- |

TGn Editor:  Change section 9.13.3.2 as indicated by the following changes:

- Greenfield protection

A STA that is associated with a BSS shall protect Green Field PPDUs using any of the protection mechanisms described in 9.13.5 (Ed: CID 2553) when its AP transmits an HT Information element with the Non-Greenfield STAs Present field set to 1. (Ed: CID 1298). The STA shall also protect Green Field PPDUs using protection mechanisms (such as RTS/CTS or CTS-to-self) when its AP transmits an HT Information element with the Operating Mode field set to 1, 2 or 3. A HT that detects a beacon with the OBSS Non-HT STAs Present field set to 1 may, in conjunction with radio resource measurements or heuristics, protect the HT transmissions of its BSS by setting its Operating Mode field to 2.