IEEE P802.11
Wireless LANs

| Regulatory Measurement Language Change |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-02-26 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Alastair Malarky | MARK IV IVHS | 6030 Ambler Drive Mississauga, Ontario Canada L4W 2P1 | 905-624-3020 | amalarky@ivhs.com |
| Wayne Fisher | ARINC | 2551 Riva Road Annapolis, MD 21401 | 410-266-4958 | wfisher@arinc.com |

Abstract

This is a recommended change to the IEEE 802.11 (11p) document regarding the regulatory measurement language. This revision (r3) includes comment resolution for LB92.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart@ok-brit.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

EXISTING

Draft P802.11p-D1.4 specified changes to Tables I-2 and Table J-1 as :

Change Table I.2- Emissions limits sets, to add the Emissions limits sets for WAVE as shown:

Table I.2—Emissions limits sets

| Emissions limits set | USA | Europe | Japan |
| --- | --- | --- | --- |
| 0 | Not specified | Not specified | Not specified |
| 1 nomadic use | FCC CFR47 [B8], Section 15.407 | ETSI EN 301 389-1 | MIC EO Articles 49.20, 49.21 |
| 2 interference-limited areas | Reserved | Reserved | MIC EO Article 49.21 |
| 3 other interference areas | Reserved | Reserved | MIC EO Article 49.21 |
| 4 ISM bands | FCC CFR47 [B8], Section 15.247 | ETS 300-328 | MPHPT EO Articles 7.49.20 |
| 5 public safety | FCC CFR47 [B8], Section 90.210 | Reserved | Reserved |
| 6 WAVE Mode Class A | FCC CFR 47, Sections 90.375, 90.377, 90.1511, and 90.639 | Reserved | Reserved |
| 7 WAVE Mode Class B | FCC CFR 47, Sections 90.375, 90.377 and 95.1511 | Reserved | Reserved |
| 8 WAVE Mode Class C | FCC CFR 47, Sections 90.375, 90.377 and 95.1511 | Reserved | Reserved |
| 9 WAVE Mode Class D | FCC CFR 47, Sections 90.375, 90.377 and 95.1511 | Reserved | Reserved |
| 10-255 | Reserved | Reserved | Reserved |

In Table I.3- Behavior limits sets, Add to the Behavior limits sets for WAVE as shown:

Table I.3—Behavior limits sets

| Behavior limits set | USA | Europe | Japan |
| --- | --- | --- | --- |
| 11 Wave Mode, Coverage Class 1 | FCC CFR 47, Sections 90.375, 90.377, 90.379, 90.383, and 95.1511 / Coverage Class = 1 | Reserved | Reserved |
| 12 Wave Mode, Coverage Class 2 | FCC CFR 47, Sections 90.375, 90.377, 90.379, 90.383, and 95.1511 / Coverage Class = 0 | Reserved | Reserved |
| 13-255 | Reserved | Reserved | Reserved |

Add  Regulatory classes 12 and 13 to Table J.1 as shown:

Table J.1—Regulatory classes in the USA

| Regulatory class | Channel starting frequency (GHz) | Channel spacing (MHz) | Channel set | Transmit power limit (mW) | Emissions limits set | Behavior limits set |
| --- | --- | --- | --- | --- | --- | --- |
| 13 | 5 | 10 | 172, 174, 176, 178, 180, 182, 184 | 760 | 6-9 | 11 |
| 14 | 5 | 20 | 175, 181 | 100 | 6-9 | 12 |
| 15-255 | Reserved | Reserved | Reserved | Reserved | Reserved | Reserved |

RECOMMENDED CHANGE

The following changes are defined with respect to the text in 802.11-REVma-D9.0.  Change P802.11p entries for Tables I.2, I.3, I.4 and J.1 as follows:

Insert 5 new Emissions limits sets for WAVE in Table I.2- Emissions limits sets, starting at the first available set (currently 6), as shown below, and changing the reserved row accordingly:

| Emissions limits set | USA | Europe | Japan |
| --- | --- | --- | --- |
| 6 WAVE Mode Class A | FCC CFR 47, Sections 90.375, 90.377, 95.1511, and 90.639 | Reserved | Reserved |
| 7 WAVE Mode Class B | FCC CFR 47, Sections 90.375, 90.377 and 95.1511 | Reserved | Reserved |
| 8 WAVE Mode Class C | FCC CFR 47, Sections 90.375, 90.377 and 95.1511 | Reserved | Reserved |
| 9 WAVE Mode Class D | FCC CFR 47, Sections 90.375, 90.377 and 95.1511 | Reserved | Reserved |
| 10 WAVE Mode 20 MHz Channels | FCC CFR 47, Sections 90.375, 90.377 and 95.1511 | Reserved | Reserved |
| 11-255 | Reserved | Reserved | Reserved |

Insert 2 new behaviour limits sets for WAVE in Table I.3- Behaviour limits sets, starting at the first available set (currently 10), as shown below, and changing the reserved row accordingly:

Table I.3—Behavior limits sets

| Behavior limits set | USA | Europe | Japan |
| --- | --- | --- | --- |
| 10 Wave Mode, Coverage Class 1 | FCC CFR 47, Sections 90.375, 90.377, 90.379, 90.383, and 95.1511 / Coverage Class = 1 | Reserved | Reserved |
| 11 Wave Mode, Coverage Class 0 | FCC CFR 47, Sections 90.375, 90.377, 90.379, 90.383, and 95.1511 / Coverage Class = 0 | Reserved | Reserved |
| 12-255 | Reserved | Reserved | Reserved |

Insert a new entry for WAVE into Table I.4 - Transmit Power by Regulatory Domain as shown below:

| Frequency band (GHz) | USA (Maximum output power with up to 6 dBi antenna gain) (mW) | Europe (EIRP) |
| --- | --- | --- |
| 5.850–5.925 | 760 with up to 13 dBi antenna gain | — |

Insert 6 new entries for WAVE into Table J.1- Regulatory classes in the USA, starting at the first available set (currently 12), as shown below, and changing the reserved row accordingly:

Table J.1—Regulatory classes in the USA

| Regulatory class | Channel starting frequency (GHz) | Channel spacing (MHz) | Channel set | Transmit power limit (mW) | Emissions limits set | Behavior limits set |
| --- | --- | --- | --- | --- | --- | --- |
| 12 | 5 | 10 | 172, 174, 176, 178, 184 | 760 Notes 1, 2 | 6-9 | 11 |
| 13 | 5 | 10 | 178, 184 | 760 Notes 1, 2 | 6-9 | 11 |
| 14 | 5 | 10 | 180, 182 | 10 Notes 1, 3 | 6-9 | 11 |
| 15 | 5 | 10 | 180, 182 | 100 Notes 1, 3 | 6-9 | 11 |
| 16 | 5 | 20 | 175, 181 | 10 Notes 1, 3 | 10 | 12 |
| 17 | 5 | 20 | 181 | 100 Notes 1, 3 | 10 | 12 |
| 18-255 | Reserved | Reserved | Reserved | Reserved | Reserved | Reserved |
| Note 1 — FCC CFR 47 Section 90.377 and ASTM E2133-03, which is incorporated per FCC CFR 47 Sections 90.379 and 95.1509, specify additional EIRP restrictions per channel. Note 2 — FCC CFR 47 Section 90.377 and ASTM E2133-03, which is incorporated per FCC CFR 47 Sections 90.379 and 95.1509, specify 2 EIRP restrictions for channels 178 and 184. Regulatory class 12 applies for the lower EIRP and class 13 for the higher EIRP Note 3 — ASTM E2213-03, which is incorporated per FCC CFR 47 Sections 90.379 and 95.1509, specifies different TX power limits but similar EIRP limits for operation in these channels. Classes 14 and 16 are applicable for roadside unit operation and classes 15 and 17 for mobile/portable operation. |  |  |  |  |  |  |

RATIONALE

The existing Table I.4 in REV-ma does not include the DSRC frequency band, however we are defining channels which are in this band.  Therefore an entry is required in Table I.4.

The existing specifications do not appear consistent with the FCC regulatory specifications for Part 90.  The FCC defines 90.377 (b)  :

Frequencies available for assignment to eligible applicants within the 5850–5925 MHz band for RSUs and the maximum EIRP permitted for an RSU with an antenna height not exceeding 8 meters above the roadway bed surface are specified in the table below. Where two EIRP limits are given, the higher limit is permitted only for state or local governmental entities.

------------------------------------------------------------------------------------------------

Frequency range     Max. EIRP \1\

Channel No.                    (MHz)              (dBm)         Channel use

------------------------------------------------------------------------------------------------

170.................................          5850-5855  .................  Reserved.

172.................................          5855-5865                 33  Service Channel.\2\

174.................................          5865-5875                 33  Service Channel.

175.................................          5865-5885                 23  Service Channel.\3\

176.................................          5875-5885                 33  Service Channel.

178.................................          5885-5895            33/44.8  Control Channel.

180.................................          5895-5905                 23  Service Channel.

181.................................          5895-5915                 23  Service Channel.\3\

182.................................          5905-5915                 23  Service Channel.

184.................................          5915-5925              33/40  Service Channel.\4\

------------------------------------------------------------------------------------------------

\1\ An RSU may employ an antenna with a height exceeding 8 meters but not exceeding 15 meters provided the EIRP specified in the table above is reduced by a factor of 20 log(Ht/8) in dB where Ht is the height of the radiation center of the antenna in meters above the roadway bed surface. The EIRP is measured as the maximum EIRP toward the horizon or horizontal, whichever is greater, of the gain associated with the main or center of the transmission beam. The RSU antenna height shall not exceed 15 meters above the roadway bed surface.

\2\ Channel 172 is designated for public safety applications involving safety of life and property.

\3\ Channel Nos. 174/176 may be combined to create a twenty megahertz channel, designated Channel No. 175.  Channels 180/182 may be combined to create a twenty-megahertz channel, designated Channel No. 181.

\4\ Channel 184 is designated for public safety applications involving safety of life and property. Only those entities meeting the requirements of § 90.373(a) are eligible to hold an authorization to operate on this channel.

Any emission class equipment may be used on any channel, at any power level allowed within that class, as long as the equipment, in conjunction with their antenna, complies with the EIRP restrictions for that channel.

Also since we define in appendix I the emission class A through D as 10 MHz only, hence a separate emission class is required for WAVE 20 MHz operations.  Note that since the regulations applicable for WAVE are in 47CFR90, the existing 802.11 20 MHz emission entries in Table I.2, which call up 47CFR15 and have applicable requirements in Europe and Japan, are not correct for WAVE.

ASTM E2213-03 section 8.9 defines specific TX power limits and EIRP restrictions per channel, in particular +10 dBm (10mW) antenna input power for channel 175 and for RSU operation on channels 180, 181, and 182, with +20dBm (10mW) antenna input power for OBU operation on channels 180, 181, and 182.  Since this revision is explicitly invoked by the FCC, until ASTM E2213 is changed and the reference in 47CFR90 updated by the FCC, 802.11p should be compliant.  Thus multiple regulatory classes are applicable.

Further since there are 2 EIRP categories for channels 178 and 184, they should have separate regulatory classes.

Further since ASTM E2213-03 specifies different antenna input powers for channels 180, 181 and 182 these require separate regulatory classes.