IEEE P802.11
Wireless LANs

| TGp Submission |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-11-15 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| J. McNew | TechnoCom | 16133 Ventura Blvd Suite 640 Encino, CA 91436 | 818-501-1903 | jmcnew@technocom-wireless.com |
| R. Roy |  |  | 650-861-3351 | dickroy@alum.mit.edu |

Abstract

This document describes the changes to 802.11p draft 1.4

This document uses REVma D9.0 for reference

`

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

|  | Submissions |
| --- | --- |

Motion:

Move to accept the proposed changes to the 11p draft (D1.4) as presented in this document and to instruct the TGp editor to make these changes with the appropriate formatting.

Motion by: ____________________Date: _________________

Second:  ______________________

| Approve: | Disapprove: | Abstain: |
| --- | --- | --- |

Proposed changes to the body of the 11p amendment:

Change the Introduction as follows:

# Introduction

WAVE is a mode of operation for use by IEEE 802.11 devices in environments where the physical layer properties are rapidly changing and where very short-duration communications exchanges are required.  The purpose of this standard is to provide the minimum set of specifications required to ensure interoperability between wireless devices attempting to communicate in potentially rapidly changing communication environments and in situations where transactions must be completed in time frames much shorter than the minimum possible with infrastructure or ad-hoc 802.11 networks.  In particular, time frames that are shorter than the amount of time required to perform standard authentication and association to join a BSS are accommodated in this amendment.

This introduction is not part of IEEE P802.11p, Draft Amendment to STANDARD FOR
Wireless Access in rapidly-varying RF Environments (WAVE) – LAN/MAN Specific Requirements - Part 11: Wireless Medium Access Control (MAC) and physical layer (PHY) specifications

This specification accomplishes the following:

- Describes the functions and services required by WAVE-conformant stations to operate in a rapidly varying environment and exchange messages either without having to join a BSS or within a WAVE BSS.

- Defines the WAVE signaling technique and interface functions that are controlled by the IEEE 802.11 MAC

This amendment to IEEE 802.11 is based on extensive testing and analyses of wireless communications in a mobile environment.  The results of these efforts were documented in ASTM E 2213-03, “Standard Specification for Telecommunications and Information Exchange Between Roadside and Vehicle Systems — 5.9 GHz Band Wireless Access in Vehicular Environments (WAVE) / Dedicated Short Range Communications (DSRC) Medium Access Control (MAC) and Physical Layer (PHY) Specifications”.  This document is available from: ASTM International, 100 Barr Harbor Drive, PO Box C700, West Conshohocken, PA, 19428.  This amendment to IEEE 802.11  is based on the ASTM E 2213-03 document.

# Change the list of participants as follows:

# Participants

The following is a list of officers in the 802.11 Working Group.

Stuart J. Kerry, Chair

Al Petrick and Harry Worstell, Vice Chairs

Tim Godfrey, Secretary

At the time this amendment to the standard was submitted to Sponsor Ballot, the WAVE task group had the following officers:

Lee Armstrong, Chair

Filip Weytjens, Secretary

Wayne Fisher, Editor

Major contributions were received from the following individuals:

Guillermo Acosta

Lee Armstrong

Broady Cash

Ken Cook

Susan Dickey

Peter Ecclesine

Tim Godfrey

Mary Ann Ingram

Daniel Jiang

Carl Kain

Doug Kavner

Keiichiro Koga

Thomas Kurihara

Jerry Landt

Sheung Li

Jason Liu

Justin McNew

Andrew Myles

Rick Noens

Satoshi Oyama

Ed Ring

Randy Roebuck

Jon Rosdahl

Richard Roy

Robert Soranno

Lothar Stibor

Bryan Wells

Filip Weytjens

Jeffrey Zhu

Change the table of contents as follows:

Contents

Introduction	ii

Participants	iv

2. Normative references	1

3. Definitions	1

4. Abbreviations and acronyms	1

5. General Description	2

5.2 Components of the IEEE 802.11 architecture	2

5.2.2 STA membership in a BSS is dynamic	2

5.2.2A WAVE mode and WAVE BSSs	2

5.2.3 Distribution system (DS) concepts	2

5.2.3.2 RSNA	2

5.2.3.3 DS concepts in a WSS	3

5.4 Overview of the services	3

5.4.1  Distribution of messages within a DS	3

5.4.1.1 Distribution	3

7. Frame formats	3

7.1 MAC frame formats	3

7.1.3 Frame fields	3

7.1.3.1.2 Type and Subtype fields	3

7.1.3.3 Address fields	3

7.1.3.3.3 BSSID field	3

7.3 Management frame body components	4

7.3.1 Fields that are not information elements	4

7.3.1.4 Capability Information field	4

7.3.1.11 Action field	4

7.3.2 Information elements	4

7.3.2.21 Measurement Request element	4

7.3.2.21.2 CCA request	4

7.3.2.22 Measurement Report element	4

7.3.2.22.2 CCA report	4

7.3.3 WAVE Information elements	4

7.4 Action frame format details	4

7.4.8 WAVE management action details	4

7.4.8.1 WAVE Announcement frame format	4

9. MAC sublayer functional description	4

9.1.4 Combined use of DCF, PCF, and HCF	4

9.15 WAVE mode operations	4

10. Layer management	5

10.3 MLME SAP interface	5

10.3.9 Reset	5

10.3.9.1 MLME-RESET.request	5

10.3.9.1.4 Effect of receipt	5

10.3.25 Management of direct links	5

10.3.25A Get TSF timer	5

10.3.25A.1 MLME-GETTSFTIMER.request	5

10.3.25A.1.1 Function	5

10.3.25A.1.2 Semantics of the service primitive	5

10.3.25A.1.3 When generated	5

10.3.25A.1.4 Effect of receipt	5

10.3.25A.2  MLME-GETTSFTIMER.confirm	5

10.3.25A.2.1 Function	5

10.3.25A.2.2 Semantics of the service primitive	5

10.3.25A.2.3 When generated	5

10.3.25A.2.4 Effect of receipt	5

10.3.25B Set TSF timer	5

10.3.25B.1 MLME-SETTSFTIMER.request	5

10.3.25B.1.1 Function	5

10.3.25B.1.2 Semantics of the service primitive	5

10.3.25B.1.3 When generated	5

10.3.25B.1.4 Effect of receipt	5

10.3.25B.2  MLME-SETTSFTIMER.confirm	5

10.3.25B.2.1 Function	5

10.3.25B.2.2 Semantics of the service primitive	5

10.3.25B.2.3 When generated	5

10.3.25B.2.4 Effect of receipt	5

10.3.25B.3 MLME-SETTSFTIMER.indication	5

10.3.25B.3.1 Function	5

10.3.25B.3.2 Semantics of the service primitive	5

10.3.25B.3.3 When generated	5

10.3.25B.3.4 Effect of receipt	5

10.3.25C Increment TSF timer	5

10.3.25C.1 MLME-INCTSFTIMER.request	5

10.3.25C.1.1 Function	5

10.3.25C.1.2 Semantics of the service primitive	5

10.3.25C.1.3 When generated	5

10.3.25C.1.4 Effect of receipt	5

10.3.25C.2  MLME-INCTSFTIMER.confirm	5

10.3.25C.2.1 Function	5

10.3.25C.2.2 Semantics of the service primitive	5

10.3.25C.2.3 When generated	5

10.3.25C.2.4 Effect of receipt	5

10.3.35 MLME SAP interface for remote requests	5

10.3.36 WAVE Annoucement	5

10.3.36.1 MLME-WAVEANNOUNCEMENT.request	5

10.3.36.1.1 Function	5

10.3.36.1.2 Semantics of the service primitive	5

10.3.36.1.3 When generated	6

10.3.36.1.4 Effect of receipt	6

10.3.36.2 MLME-WAVEANNOUNCEMENT. confirm	6

10.3.36.2.1 Function	6

10.3.36.2.2 Semantics of the service primitive	6

10.3.36.2.3 When generated	6

10.3.36.2.4 Effect of receipt	7

10.3.36.3 MLME-WAVEANNOUNCEMENT. indication	7

10.3.36.3.1 Function	7

10.3.36.3.2 Semantics of the service primitive	7

10.3.36.3.3 When generated	8

10.3.36.3.4 Effect of receipt	8

10.3.37 WAVE Join	8

10.3.37.1 MLME-WAJOIN.request	8

10.3.37.1.1 Function	8

10.3.37.1.2 Semantics of the service primitive	8

10.3.37.1.3 When generated	9

10.3.37.1.4 Effect of receipt	9

10.3.37.2 MLME-WAJOIN.confirm	9

10.3.37.2.1 Function	9

10.3.37.2.2 Semantics of the service primitive	9

10.3.37.2.3 When generated	9

10.3.37.2.4 Effect of receipt	9

11. MLME	10

11.1 Synchronization	10

11.1.3 Acquiring synchronization, scanning	10

11.14 WAVE mode management	10

11.14.1 Initializing a WSS	10

11.14.2 Joining a WSS	10

17. Orthogonal frequency division multiplexing (OFDM) PHY specification	11

for the 5 GHz band	11

17.3 OFDM PLCP sublayer	11

17.3.8 PMD operating specifications (general)	11

17.3.8.8 Transmit and receive operating temperature range	11

17.3.9.4 Transmit center frequency tolerance	11

17.3.9.5 Symbol clock frequency tolerance	11

17.3.10  PMD receiver specifications	11

17.3.10.1 Receiver minimum input sensitivity	11

17.3.10.2 Adjacent channel rejection	11

17.3.10.3 Nonadjacent channel rejection	11

17.4 OFDM PLME	11

17.4.1 PLME_SAP sublayer management primitives	11

Annex A	11

Protocol Implementation Conformance Statement (PICS) proforma	11

A.4 PCIS proforma—IEEE Std 802.11™, 2006 Edition	11

A.4.3  IUT Configuration	11

A.4.8 OFDM PHY function	11

Annex D	11

ASN.1 encoding of the MAC and PHY MIB	11

Annex I	11

Regulatory classes	12

I.1 External regulatory references	12

I.2.3 Transmit spectrum mask	13

Annex J	15

Country information element and regulatory classes	15

Change the list of tables as follows:

List of figures

List of Tables

Table 24—Category values	4

Table p1—WAVE elements	4

Table p2—WAVE Action field values	4

Table p3—WAVE announcement contents	4

Table p4― EDCA parameter set in the absence of a WSS	4

Table p5—WAVE receiver performance requirements	11

Table 146—MIB attribute default values/ranges	11

Table I.1—Regulatory requirement list	12

Table I.2—Emissions limits sets	12

Table I.3—Behavior limits sets	13

Table pI.1—WAVE spectrum mask	14

Table J.1—Regulatory classes in the USA	15

# Change the definitions as follows:

# 3. Definitions

Insert the following new definitions at the appropriate locations in clause 3, renumbering as necessary:

Note:  802.11ma,D9.0 ends with 3-172, 11kD6.0 adds 17, and 11rD3.0  adds 16.

3.173A   Wireless Access in Vehicular Environments (WAVE): The mode of operation of a station that complies with the MAC and PHY requirements specified in this amendment.

3.174A  WAVE basic service set (WBSS):  A set of cooperating stations operating in WAVE mode consisting of a single WBSS announcer and 0 or more WSS users.

3.175A  WAVE service information (WSI): A WSI contains information regarding WAVE services being offered. It is a field in a WAVE Announcement action frame.

# Change the abbreviations as follows:

# 4. Abbreviations and acronyms

Insert the following new abbreviations and acronyms at the appropriate locations in clause 4:

WAVE		wireless access in vehicular environments

WBSS		WAVE basic service set

WSI		WAVE service information

# Change section 5 as follows:

# 5. General Description

# 5.2 Components of the IEEE 802.11 architecture

# 5.2.2 STA membership in a BSS is dynamic

Insert a new subclause (5.2.2A) after Clause 5.2.2 renumbering as necessary:

# 5.2.2A WAVE mode and WAVE BSSs

WAVE mode supports data exchange between STAs without having to first establish a BSS.  STAs in WAVE mode shall be able to transmit data to arbitrary valid MAC addresses, including broadcast addresses, without the presence of a BSS.  These data exchanges cannot be transmitted directly to a MAC address on a DS (the To DS and From DS bits are both 0 as defined in Table 7).

WAVE also supports data exchange between STAs, and between STAs and a DS, when a WAVE mode BSS is present. The WAVE Mode BSS (WBSS) provides for rapid establishment of a LAN, a capability required in rapidly changing communication environments.  Such environments typically involve mobile stations that move much faster than mobile or portable stations participating in an 802.11 infrastructure or ad-hoc BSS.  A WAVE mode STA that announces the presence of WBSSs is termed a WBSS provider.  A WAVE mode STA that elects to join a WBSS announced by aWBSS provider is termed a WBSS user.

STAs in WAVE mode become members of a WBSS in one of two ways, either as a WBSS provider or as a WBSS user.  A STA announces a WBSS as described in 11.14.1 and thereby is a member of the WBSS.   STAs join a WBSS and optionally synchronize with that WBSS as described in 11.1.3.4 and 11.14.2 respectively.  Unlike infrastructure and ad-hoc 802.11 BSS types, WBSSs do not require MAC sublayer authentication and association (as described in 11.3) prior to being allowed to transmit data to a DS.  Only the first step in infrastructure or ad-hoc BSS membership, joining the BSS (i.e., unauthenticated, unassociated state), is required; the authenticated and associated states do not apply to WAVE mode.  The impact of this WAVE-specific architecture is that operation in WAVE mode does not require the DSS of an AP within the WBSS provider STA in order to access a DS connected to that STA.

WBSSs may operate in frequency bands dedicated for their use.  These bands may require licensing depending on the regulatory domain.

# 5.2.3 Distribution system (DS) concepts

# 5.2.3.2 RSNA

Insert the following new subclause (5.2.3.3) after subclause 5.2.3.2:

# 5.2.3.3 DS concepts in a WBSS

WAVE mode BSSs may utilize a DS without requiring authentication or association.  In WAVE mode, authentication and association services may be implemented in a station management entity or in protocol layers above the MAC.

# 5.4 Overview of the services

# 5.4.1  Distribution of messages within a DS

# 5.4.1.1 Distribution

Insert after the last sentence of the first paragraph:

WBSSs do not use the DSS provided by the 802.11 MAC (see 5.2.3.3).

# Change section 7.1.3.3.3 as follows:

# 7.1.3.3.3 BSSID field

Insert after the second paragraph of 7.1.3.3.3

The value of the BSSID field in a WBSS shall be the MAC address of the WBSS provider. The value of this field may be a locally administered IEEE MAC address as defined in 5.2 of IEEE Std 802-1990.

Change the last sentence of the last paragraph of 7.1.3.3.3 to:

A wildcard BSSID shall not be used in the BSSID field except for management frames of subtype probe request and data frames transmitted in WAVE mode while not a member of a BSS.

Action field values

# Change section 9 subsections as follows:

# 9.1.4 Combined use of DCF, PCF, and HCF

Insert the following sentence at the end of the first paragraph of 9.1.4:

Because beacons are not transmitted in WAVE mode (see 11.14), the PCF is not supported in a WBSS.

Insert the following new subclause (9.15) after subclause 9.14 adjusting the table numbers as necessary:

Editorial Note:  802.11ma,D9.0 ends with 9.13 and .11r,D3.0 adds 9.14.

# 9.15 WAVE mode operations

A STA announces the presence of a WBSS by issuing a MLME-WAVEANNOUNCEMENT.request containing the parameters necessary to announce the presence of the WBSS.  Broadcast, multicast and unicast destination addresses are allowed.  The initial and continued presence of a WBSS shall be announced by transmission of a WAVE Announcement action frame. STAs shall not join a WBSS unless a WAVE Announcement action frame has been received (see 11.14).

Prioritized access in WAVE mode operations uses the EDCA mechanism. The default EDCA parameter set used in the WAVE Announcement is defined in Table p4. The default EDCA parameter set shall be used for all STAs when transmitting data frames in the absence of a WSS.  For data exchanges within a WSS, the EDCA parameter set received in the WAVE Announcement action frame shall be used.

Table p4― EDCA parameter set in the absence of a WBSS

| ACI | CWmin | CWmax | AIFSN | TXOP Limit OFDM PHY (WAVE mode) |
| --- | --- | --- | --- | --- |
| 0 | aCWmin | aCWmax | 9 | 0 |
| 1 | (aCWmin+1)/2 - 1 | aCWmin | 6 | 0 |
| 2 | (aCWmin+1)/4 - 1 | (aCWmin+1)/2 - 1 | 3 | 0 |
| 3 | (aCWmin+1)/4 - 1 | (aCWmin+1)/2 - 1 | 2 | 0 |

# )

# )

# )

# )

# Change section 10 subsections as follows:

# 10.3.25B.3.3 When generated

This primitive shall be generated by the MLME and sent to the SME whenever the TSF timer is changed other than normal increments and changes in response to MLME-SETTSFTIMER.request, MLME-INCTSFTIMER.request or MLME-RESET.request commands.

# )

# )

# 10.3.36 WAVE Annoucement

This mechanism supports the WAVE announcement function to announce the presence of a WBSS.

# 10.3.36.1 MLME-WAVEANNOUNCEMENT.request

# 10.3.36.1.1 Function

This primitive is generated by the SME to request the MLME to announce the presence of  a WBSS.

# 10.3.36.1.2 Semantics of the service primitive

This primitive provides the following parameters:

MLME-WAVEANNOUNCEMENT.request	(

SSID,

Capability Information,

BSSBasicRateSet,

OperationalRateSet,

EDCA parameter set,

WSI,

Receiver MAC address

)

SSID, Capability Information, BSSBasicRateSet, and EDCA parameter set are defined in 10.3.2.2.2. The other parameters are defined in the table below.

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| OperationalRateSet | Set of Integers | 1 – 127 inclusive (for each integer in the set) | The set of data rates that the STA desires to use for communication within the BSS. The STA must be able to receive at each of the data rates listed in the set. This set is a superset of the BSS basic rate set advertised by the BSS. |
| WSI | As defined in 7.4.8.1 | As defined in 7.4.8.1 | WAVE Service Information |
| Receiver MAC address | MAC address | Any valid address | Specifies the desired destination MAC address (either unicast, multicast or broadcast) |

# 10.3.36.1.3 When generated

This primitive is generated by the SME to announce the presence of  a WAVE BSS.

# 10.3.36.1.4 Effect of receipt

Upon the receipt of this primitive, the MLME announces the presence of a WBSS by transmitting a WAVE Announcement action frame.

# 10.3.36.2 MLME-WAVEANNOUNCEMENT. confirm

# 10.3.36.2.1 Function

This primitive is generated by the MLME and sent to the SME to report the result of a MLME-WAVEANNOUNCEMENT.request

# 10.3.36.2.2 Semantics of the service primitive

This primitive provides the following parameters:

MLME-WAVEANNOUNCEMENT.confirm (

ResultCode

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| ResultCode | Enumeration | SUCCESS, INVALID PARAMETERS | Reports the result of a MLME-WAVEANNOUNCEMENT.request. |

# 10.3.36.2.3 When generated

This primitive is generated by the MLME in response to a MLME-WAVEANNOUNCEMENT.request.

# 10.3.36.2.4 Effect of receipt

The SME is notified of the result of a WAVE Announcement request.

# 10.3.36.3 MLME-WAVEANNOUNCEMENT. indication

# 10.3.36.3.1 Function

This primitive is sent by the MLME to the SME to indicate the reception of a WAVE Announcement action frame.

# 10.3.36.3.2 Semantics of the service primitive

This primitive provides the following parameters:

MLME-WAVEANNOUNCEMENT.indication(

BSSID,

SSID,

Timestamp,

Local Time,

Capability Information,

BSSBasicRateSet,

OperationalRateSet,

EDCA parameter set,

WSI,

Transmitter MAC address

RCPI

)

BSSID is defined in 7.1.3.3.3.  SSID, Timestamp, Capability Information, BSSBasicRateSet, and EDCA parameter set are defined in 10.3.2.2.2. Local Time is the value of a WAVE station’s TSF timer at the start of reception of the first octet of the timestamp field of the received WAVE announcement action frame.  The other parameters are defined in the table below.

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| OperationalRateSet | Set of Integers | 1 – 127 inclusive (for each integer in the set) | The set of data rates that the STA desires to use for communication within the BSS. The STA must be able to receive at each of the data rates listed in the set. This set is a superset of the BSS basic rate set advertised by the BSS. |
| WSI | As defined in 7.4.8.1 | As defined in 7.4.8.1 | WAVE Service Information |
| RCPI | Integer | 0 - 255 | RCPI is the measured value of received channel power on the received WAVE Announcement frame. |
| Transmitter MAC address | MAC address | Any valid address | The address of the MAC entity that transmitted the WAVE announcement that was received. |

10.3.36.3.3 When generated

This primitive is generated by the MLME when a WAVE Announcement frame is received.

# 10.3.36.3.4 Effect of receipt

Upon the receipt of this primitive, the SME is notified that a WAVE announcement action frame has been received.

# 10.3.37 WAVE Join

# 10.3.37.1 MLME-WAJOIN.request

# 10.3.37.1.1 Function

This primitive requests joining a WBSS.

# 10.3.37.1.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-WAJOIN.request             (

BSSID,

SSID,

OperationalRateSet,

Timestamp,

Local Time,

EDCA parameter Set

)

BSSID is defined in 7.1.3.3.3.  SSID, OperationalRateSet, Timestamp,  and EDCA parameter set are defined in 10.3.2.2.2. Local time is the value of a WAVE station’s TSF timer at the start of reception of the first octet of the timestamp field of the received WAVE announcement action frame.

# 10.3.37.1.3 When generated

This primitive is generated by the SME to instruct the MLME to join a WBSS.

# 10.3.37.1.4 Effect of receipt

Upon receipt of this primitive, a MLME sets the WAVE join parameters. Optionally, the MLME may synchronize its timing with the specified WBSS based on the time provided in the Local Time parameter and subsequently exchange data frames within the corresponding WBSS. The MLME subsequently issues a MLME-WAJOIN.confirm that reflects the results.

# 10.3.37.2 MLME-WAJOIN.confirm

# 10.3.37.2.1 Function

This primitive confirms joining a WBSS.

# 10.3.37.2.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-WAJOIN.confirm(

ResultCode

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| ResultCode | Enumeration | SUCCESS, INVALID PARAMETERS | Reports the result of a MLME-WAJOIN.request. |

# 10.3.37.2.3 When generated

This primitive is generated by the MLME as a result of an MLME-WAJOIN.request to join a WBSS.

# 10.3.37.2.4 Effect of receipt

The SME is notified of the results of the join procedure.

# Change section 11 as follows:

# 11. MLME

# 11.1 Synchronization

Add the following sentence to the end of subclause 11.1:

Synchronization is not required for STAs operating in WAVE mode.

11.1.3 Acquiring synchronization, scanning

Add the following to the beginning of the first sentence:

Except in WAVE mode (see 11.1 and 11.14),

Insert the following new subclauses (11.14 through 11.14.2) after subclause 11.13 adjusting the figure  numbers as necessary:.

# 11.14 WAVE mode management

Operation in WAVE mode shall not use active or passive scanning as prescribed for IBSSs and infrastructure BSSs, nor shall it use authentication and association procedures. STAs in WAVE mode that are not transmitting WAVE Announcement action frames shall listen for WAVE Announcement action frames.  These frames contain all the information necessary to join a WBSS.  The channel specified by the value of dot11CurrentFrequency shall be used in announcing a WBSS, and STAs in WAVE mode shall listen for WAVE Announcements on this channel only.

# 11.14.1 Initializing a WBSS

STAs in WAVE mode initialize WBSSs by issuing a first MLME-WAVEANNOUNCEMENT.request containing the necessary parameters for the WBSS and receiving a subsequent confirmation that the request was successful.  Note that subsequent WAVE Announcement.requests associated with the WBSS continue to announce the presence of the WBSS.  When a WBSS provider initializes or announces a WBSS, it shall set the BSS's BSSID (as described in 7.1.3.3.3), assemble the WAVE Announcement action frame, and transmit the WAVE Announcement action frame.

# 11.14.2 Joining a WBSS

STAs in WAVE mode that receive WAVE Announcement action frames and elect to join the WBSS are termed WBSS users. When a STA receives a WAVE Announcement action frame, the following steps are taken if the MIB attribute dot11WAVEServicesRequired is TRUE:

- The MLME issues an MLME-WAVEANNOUNCEMENT.indication to the SME.

- If the SME elects to join the WBSS, the SME issues a MLME-WAJOIN.request (see 10.3.36.1) to the MLME. The user shall join the WBSS by adopting the parameters received in the corresponding WAVE Announcement.

The WAVE Announcement action frame shall be ignored if the MIB attribute dot11WAVEServicesRequired is FALSE.  This process is shown in Figure p3.  After joining a WBSS, a STA may transmit data to another STAs or to a DS.

Figure p3—WAVE Announcement process

# A.4.8 OFDM

# Change Annex I as follows:

Annex I

(informative)

# Regulatory classes

# I.1 External regulatory references

In Table I.1- Regulatory requirement list, add to the list of documents for the United States Geographic Area the following: “Section 90.371-383, Section 95.639, Section 95.1501-1511”:

Change Table I.2- Emissions limits sets, to add the Emissions limits sets for WAVE as shown:

Table I.2—Emissions limits sets

| Emissions limits set | USA | Europe | Japan |
| --- | --- | --- | --- |
| 0 | Not specified | Not specified | Not specified |
| 1 nomadic use | FCC CFR47 [B8], Section 15.407 | ETSI EN 301 389-1 | MIC EO Articles 49.20, 49.21 |
| 2 interference-limited areas | Reserved | Reserved | MIC EO Article 49.21 |
| 3 other interference areas | Reserved | Reserved | MIC EO Article 49.21 |
| 4 ISM bands | FCC CFR47 [B8], Section 15.247 | ETS 300-328 | MPHPT EO Articles 7.49.20 |
| 5 public safety | FCC CFR47 [B8], Section 90.210 | Reserved | Reserved |
| 6 WAVE Mode Class A | FCC CFR 47, Sections 90.375, 90.377, 90.1511, and 90.639 | Reserved | Reserved |
| 7 WAVE Mode Class B | FCC CFR 47, Sections 90.375, 90.377 and 95.1511 | Reserved | Reserved |
| 8 WAVE Mode Class C | FCC CFR 47, Sections 90.375, 90.377 and 95.1511 | Reserved | Reserved |
| 9 WAVE Mode Class D | FCC CFR 47, Sections 90.375, 90.377 and 95.1511 | Reserved | Reserved |
| 10 WAVE Mode 20 MHz Channels | FCC CFR 47, Sections 90.375, 90.377 and 95.1511 | Reserved | Reserved |
| 11-255 | Reserved | Reserved | Reserved |

In Table I.3- Behavior limits sets, Add to the Behavior limits sets for WAVE as shown:

Table I.3—Behavior limits sets

| Behavior limits set | USA | Europe | Japan |
| --- | --- | --- | --- |
| 11 Wave Mode, Coverage Class 1 | FCC CFR 47, Sections 90.375, 90.377, 90.379, 90.383, and 95.1511 / Coverage Class = 1 | Reserved | Reserved |
| 12 Wave Mode, Coverage Class 2 | FCC CFR 47, Sections 90.375, 90.377, 90.379, 90.383, and 95.1511 / Coverage Class = 0 | Reserved | Reserved |
| 13-255 | Reserved | Reserved | Reserved |

Change Annex J as follows:

# Annex J

(normative)

# Country information element and regulatory classes

Add a sentence to the sixth paragraph in Annex J changing it from:

The transmit power limit shall be the maximum transmit power that is legal for a regulatory domain and

class.

To:

The transmit power limit shall be the maximum transmit power that is legal for a regulatory domain and

class. The definition of transmit power used in that regulatory domain shall apply.

Add  Regulatory classes 12 and 13 to Table J.1 as shown:

Table J.1—Regulatory classes in the USA

| Regulatory class | Channel starting frequency (GHz) | Channel spacing (MHz) | Channel set | Transmit power limit (mW) | Emissions limits set | Behavior limits set |
| --- | --- | --- | --- | --- | --- | --- |
| 13 | 5 | 10 | 172, 174, 176, 178, 184 | 760 Notes 1, 2 | 6-9 | 11 |
| 14 | 5 | 10 | 178, 184 | 760 Note 2 | 6-9 | 11 |
| 15 | 5 | 10 | 180, 182 | 10 Note 1, 3 | 6-9 | 11 |
| 16 | 5 | 10 | 180, 182 | 100 Note 1, 3 | 6-9 | 11 |
| 17 | 5 | 20 | 175, 181 | 10 Notes 1, 3 | 10 | 12 |
| 18 | 5 | 20 | 181 | 100 Notes 1, 3 | 10 | 12 |
| 19-255 | Reserved | Reserved | Reserved | Reserved | Reserved | Reserved |

Note 1 : 47CFR90.377 specifies additional EIRP restrictions per channel.

Note 2: 47CFR90.377 specifies 2 EIRP restrictions for channels 178 and 184.  Regulatory class 13 applies for the lower EIRP and class 14 for the higher EIRP

Note 3: ASTM E2213-03 specifies different TX power limits but similar EIRP limits for RSU and OBU operation in these channels.  Classes 15 and 17 are applicable for RSU operation and classes 16 and 18 for OBU operation.

Available from U.S. Government Printing Office Superintendent of Documents, 732 N. Capitol St., NW, Mail Stop: SDE, Washington, DC 20401.