IEEE P802.11
Wireless LANs

| Admission Control Traffic Request |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-11-16 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Subbu Ponnuswamy | Aruba Networks | 1322 Crossman Ave Sunnyvale, CA 94089 | +1 408-754-1213 | subbu@arubanetworks.com |
| Dorothy Stanley | Aruba Networks | 1322 Crossman Ave Sunnyvale, CA 94089 | +1 630-363-1389 | dstanley@arubanetowrks.com |

Abstract

This document describes a mechanism to enable a STA to indicate its QoS traffic requests to an AP in a Probe Request frame. In the returned Probe Response frame, the AP indicates whether or not it can service the requested traffic. The information provided is meant to be an indication only; it is non-binding on both the STA and AP.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Background:

A STA is able to request TSPEC resources after association. While means exist for an AP to indicate its overall load, no means is currently defined for it to provide an indication of the AP’s ability to service the requests of an individual STA prior to association.

This submission provides a method for a STA to indicate its non-binding traffic request to an AP in a Probe Request frame. In the returned Probe Response frame, the AP indicates whether or not it can service the requested traffic. The AP indication is also non-binding.

Changes from r0 and r1:

- Changed element name from “Admission Control Traffic Request” to “Admission Control Traffic Query”

- Changed reason code definitions to be declarative rather than describe future behaviour

- Changed from “shall” to” may” in second sentence of 11.15.8.

Modify Clause 7.2.3.8 as follows:

##### 7.2.3.8 Probe Request Frame

The frame body of a management frame of subtype Probe Request contains the information shown in Table 14. If the dot11MultiDomainCapabilityEnabled attribute is true, a STA may include a Request information element in the Probe Request frame. If the dot11WirelessManagementImplemented attribute is true, a STA may include an Admission Control Traffic Query element in the Probe Request frame. The format of the Request information element is specified in 7.3.2.12.

[Insert the following row in Table 14]

| Order | Information | Notes |
| --- | --- | --- |
| 5 | Admission Control Traffic Query | The Admission Control Traffic Query element is optionally present if dot11WirelessManagementImplemented is true. |

##### 7.2.3.9 Probe Response frame body

[Insert the following row in Table 15]

| Order | Information | Notes |
| --- | --- | --- |
| next available | Admission Control Traffic Query | The Admission Control Traffic Query element is optionally present if dot11WirelessManagementImplemented is true and the Admission Control Traffic Query element was present in the received Probe Request frame. |

### 7.3.2 Information elements

[Insert the following row into Table 26]

| Information element | Element ID | Length (in octets) |
| --- | --- | --- |
| Admission Control Traffic Query | x+14 |  |
| Reserved | (x+15) - 126 |  |

Insert a newClauses 7.3.2.47 as follows:

- 7.3.2.47Admission Control Traffic Query element

The Admission Control Traffic Query element defines information about the traffic streams that the non-AP STA may present to an AP. The format of the Admission Control Traffic Query element is shown in Figure v1.

|  | Element ID | Length | QoS Request Field 1 | QoS Request Field…… | QoS Request Field n |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | variable | 4 | ….. | 4 |

- Figure v1 – Admission Control Traffic Query Element

The Length field is set to 4n, where n is the number of QoS Request fields included in the element.

|  | ACI | Medium Time | Reason Code |
| --- | --- | --- | --- |
| Octets: | 1 | 2 | 1 |

- Figure v1 – QoS Request Field

The QoS field ACI contains the Access Category, as defined in Table 36.

The QoS Field Medium Time is a 16-bit unsigned integer and contains the amount of time the non-AP STA is indicating for access to the medium, in units of 32 μs/s. The derivation of this field is described in K.2.2.

The QoS Field Reason Code value is reserved and set to 0 when the Traffic Indication Query element is included in a Probe Request frame. When the Traffic Indication Query element is included in a Probe Response frame, the values of the Reason Code field are as indicated in Table vx.

| Reason Code | Definition |
| --- | --- |
| 0 | Reserved |
| 1 | Requested resources may be available |
| 2 | Suggested resources may be available |
| 3-7 | Reserved |
| 8 | Requested resources not available |
| 9 | Denied due to administrative policy |
| 10 | Denied due to other reasons |
| 11-255 | Reserved |

Table vx: Reason Code

Insert the following text as Clause 11.15.8

11.15.8 Admission Control Traffic Query Element Processing

When a STA sends a Probe Request frame, it may include an Admission Control Traffic Query element to indicate the number and priority of flows being requested and the desired medium time. If the AP has sufficient resources to accommodate the request, it may include the same IE with appropriate non-zero medium time values in the returned Probe Response frame. The AP need not reserve any resources based on the data provided in the received Probe Request frame. If the AP does not have resources to support a new request, the AP sets the medium time to zero in the Admission Control Traffic Query element QoS field in the returned Probe Response frame, indicating unavailability of resources.

The Admission Control Traffic Query element included in a Probe Request frame may request medium time for more than one traffic flow. The AP may return a medium time that is different from (typically lower than) the one requested by STA in the Admission Control Traffic Query element included in the Probe Request frame. The AP may include a reason code in the QoS Request field in the Admission Control Traffic Query element included in the Probe Response frame.

References: