IEEE P802.11
Wireless LANs

| TGn LB84 Some More MAC Resolutions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-01-05 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Adrian Stephens | Intel Corporation |  | +1 503 616 3800 | adrian.p.stephens@intel.com |

Abstract

This document contains proposed changes to the IEEE P802.11n Draft to address the following LB84 comments:

- (Assigned to Srinivas Kandala) 3623, 3846, 1463, 3144, 6943

- (Assigned to Matthew Fischer) 1054, 743, 7174, 11965

- (Assigned to this author) 3739, 10316

- (Unassigned) 3823, 7228, 2396, 2743, 11739, 3512, 1476, 6787

- (Others) 3589, 7593, 3588, 10366

The changes marked in this document are based on TGn Draft version D1.07 (equivalent to D1.08 as the differences between D1.07 and D1.08 do not affect the resolutions proposed here)..

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart@ok-brit.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# Introduction

## Interpretation of a Motion to Adopt

A motion to approve this submission means that the editing instructions and any changed or added material are actioned in the TGn Draft.  This introduction, is not part of the adopted material.

Editing instructions formatted like this are intended to be copied into the TGn Draft (i.e. they are instructions to the 802.11 editor on how to merge the TGn amendment with the baseline documents).

TGn Editor:  Editing instructions preceded by “TGn Editor” are instructions to the TGn editor to modify existing material in the TGn draft.   As a result of adopting the changes, the TGn editor will execute the instructions rather than copy them to the TGn Draft.

Summission Note: Notes to the reader of this submission are not part of the motion to adopt.  These notes are there to clarify or provide context.

# Comments assigned to Srini

Submission Note:  these comments are assigned to Srinivas Kandala.   This author has reviewed these proposed resolutions with Srini, and he supports them.

## CID 3623

| 3623 | 3.x | 2 | 13 | Definitions | The concept of "Long NAV" is alreay present in the base standard. The definition appears to be unnecessary. | Either remove it or rewrite the parts of the base standard which refer to this mechanism. | mac adhoc 061019 - Fails a straw poll to reject 8-3 -- Proposed reject. What's new about LongNAV is deliberate overestimation and truncation. This is not present in the baseline. Ed: reclassified as technical Gen AdHoc:Transfer to MAC |
| --- | --- | --- | --- | --- | --- | --- | --- |

### Proposed resolution

Counter.  While the concept of LongNAV is not new, the subclause in which it is descrbed does contain two normative statements that are not made elsewhere.   The resolution is to delete LongNAV and to move these two normative statements into a new subclause to replace the LongNAV subclause.   This is effected by the following editing instructions (D1.07):

1.  Delete the definition of LongNAV (clause 3)

2.  Move and modify as shown the two sentences:  "The Duration/ID field in all frames within an A-MPDU shall contain the remaining duration of TXOP (referenced to the end of the PPDU carrying the frame). (Ed: CID 1324) The Duration/ID field of all (Ed: CID 1323) frames sent in the TXOP by the TXOP holder or TXOP responder shall contain the time remaining in the TXOP."  to a new subclause 9.13.6.x Duration/ID rules for A-MPDU and TXOP

3.  Delete the remainder of 9.13.6.1

4.  in 9.13.6.2 delete the phrase: "and uses LongNAV to protect a duration value"

5. in 9.14.1 replace "for LongNAV in 9.13.6.1 (Long NAV)" with "for TXOP truncation in 9.13.6.2"

6. in Annex S (frame exchange sequences), replace "sequence protected using the LongNAV protection feature, which" with "that"

## CID 3846

| 3846 | 9.16 | 107 |  | Protection mechanisms for Aggregation Exchange Sequences | The description in this subclause is sort of an overkill. The mechanism to set NAV covering more than one MPDU is not new. Usage of Cf-End to reset the NAVs is not new either. The only thing that is really added is that any station may transmit a CF-End and reset the NAVs of the stations in the vicinity. | Move all the examples and descriptions to an informative annex. In this subclause just indicate that any station may reset the NAV of other stations in its vicinity by sending a CF-End. | Deferred for later submission or discussion with the author. Ref: CID2572 addresses the second sentence of proposed change. |
| --- | --- | --- | --- | --- | --- | --- | --- |

### Proposed resolution

Counter.    Remove LongNAV as specified in the response to CID 3623.

## CID 1463

| 1463 | 9.23.8 | 131 |  | ACK and BlockAck in non-HT duplicated mode | these rules are overly restrictive - if legacy long NAV protection is in force, then these rules are not needed | add a qualifier that says that these rules are only needed when long NAV protection has not been employed | Transferred to MAC Pending the resolution of CID6800 in MAC. |
| --- | --- | --- | --- | --- | --- | --- | --- |

### Background

The commenter is referring to the following text:

9.23.8 ACK and BlockAck in non-HT duplicated mode

An HT STA shall behave in the following way if it uses non-HT duplicated mode for protection.

Non-aggregated 40MHz frames shall be acknowledged by ACK in non-HT duplicate mode.

A non-aggregated BlockAckReq shall be transmitted in non-HT duplicate mode if the STA is operating in a

40MHz mode.

A non-aggregated BlockAck shall be sent in non-HT duplicated mode if it is a response to a PPDU that is a

40MHz type.

Comment 6800 was countered by submission 11-06/1557r4, edited in D1.08, which supplied material that became a 9.6.7.  These rules have the unintended effect of requiring a BA to be non-HT or non-HT duplicate (i.e. it is not covered by the exclusions in 9.6.7).

### Proposed Resolution

Counter.  The rules in 9.20.6 (D1.08) permit the aggregation of BA/BAR and data.  When transmitting an isolated BA/BAR,  the rules require non-HT duplicate mode.  However, because these are short frames and the non-HT duplicate PHY overhead is less than for HT frames,  there is no value in using HT frames for this purpose.   The restriction costs nothing from a performance point of view and may improve protection if the frames initially setting up the TXOP were not heard.   It also provides improved protection if an APSD power saver awakes,  and is able to protect the remainder of the TXOP, given the Duration/ID of a (non-HT duplicate) BA/BAR received in the middle of the TXOP.

However, 9.6.7 (D1.08) requires BA (and MTBA) to be non-HT or non-HT duplicate,  which is inconsistent.   Therefore modify 9.6.7 as follows to remove this inconsistency:

At the end of 9.6.7 add the following text: "A BlockAck may be aggregated with data (using A-MPDU) within a HT PPDU. NOTE-If improved protection from  non-HT STAs is desired, aggregation of BlockAck should not be used.”

### Notes from mac meeting:

Straw poll:  do we support the aggregation of data and BA?

yes 11

no 3

Straw poll:  do we support the addition of the following: A BlockAck may be aggregated with data (using A-MPDU) within a HT PPDU. NOTE-If improved protection from  non-HT STAs is desired, aggregation of BlockAck should not be used.”

Yes 11

No 1

## CID 3144

| 3144 | A4.15.1 | 267 | 0 | MAC Enhancements for Higher Throughput | HTM8 should be mandatory | Make change indicated in comment | see CID 3623 GenAdHoc: Long NAV Protection: Use of a protection method is necessary, but use of a particular method is not mandated. The receiver rules for the NAV are defined in the Baseline 802.11 (TGma rev7) and it is mandatory behaviour on how to deal with the NAV. The AP behaviour is mandatory if it supports PSMP (see 9.9.4), 9.16.2 indicates that this bit should be optional. Transfer to MAC group to discuss and determine a consistent response and to consider clarifying the text in 9.9.4. |
| --- | --- | --- | --- | --- | --- | --- | --- |

### Proposed Resolution

Counter.   See resolution of 10316,  which removes LongNAV,  and separates the TXOP truncation PICS entry into optional transmitter and mandatory recevier parts.

Submission note:  a resolution to 10316 is proposed later in this submission.

## CID 6943

| 6943 | 9.23.6 | 130 | 21 | Protection in 40/20MHz BSS | Should these non-HT control frame transmission be duplicate? | Indicate that these shall be "duplicate non-HT" frames | Deferred See CID1519 |
| --- | --- | --- | --- | --- | --- | --- | --- |

### Proposed Resolution

Counter.  D1.07 already has this requirement, although in a different form. Subclause 9.6.7 contains the following:  "A control frame shall be carried in a non-HT PPDU or an HT PPDU with the TXVECTOR parameter CH_BANDWIDTH equal to NON_HT_CBW40 except under the conditions listed below." This requires any 40MHz transmission with FORMAT set to NON_HT to be non-HT duplicate (with exceptions as called out in this subclause).

Submission note:  the exceptions are incomplete, however this need not affect the resolution of this particular comment.  This issue is addressed in the resolution to 1463 proposed above.

# Comments assigned to Matthew Fischer

Submission Note:  these comments are assigned to Matt Fischer.   This author has reviewed these proposed resolutions with Matt, and he supports them.

## CID 1054

| 1054 | 11.5.2 | 153 |  | Multi-TID Block Ack | the table herein contains information which is unqualifiedly assertive in construction and which does not appear elsewhere in the document, and therefore needs to be expressed in a normative manner | find the unqualifiedly assertive sentences and change them to normative statements (change the verb tense to use "shall") e.g. the statements about beacons are all candidates for this treatment -- if any of the statements are actually found elsewhere, then do NOT make this a duplicate occurrence of that pre-existing rule |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

### Proposed Resolution

Counter.   Submission 11-06/1600r5 completely reworks this section and removes the offending table.   Implemented in D1.07

## CID 743

| 743 | 9.14.1 | 105 | 4-5 | RIFS Protection | The clause states that STAs shall protect RIFS sequences, but it does not state how it shall protect them. | Define how RIFS Protection will work. | Deferred: for later submission by Matt Fischer |
| --- | --- | --- | --- | --- | --- | --- | --- |

### Proposed Resolution

Accept.  Submission 11-06/1572r1 provides the necessary description.

## CID 7174

| 7174 | 8.8 | 82 | 8 | Security for HT STA | Referencing security here is inconsistent with the structure of 802.11n, which is otherwise agnostic. The special requirement is driven by the current state of technology and not fundamental. | Remove this section. | Defer - U - need to request clarification on whether this text is within the scope of the TGn PAR. |
| --- | --- | --- | --- | --- | --- | --- | --- |

### Proposed Resolution

Reject.   The goal of the standard is to define something that can be implemented in a timely fashion.   Similar constraints/design choices exist in the baseline today for the same reason.

Note – the PAR allows changes to both MAC and PHY,  and clause 8 is part of the MAC.

Note, the specific subclause has been removed from D1.07, however equivalent statements are now made elswhere in clause 8.

## CID 11965

| 11965 | 8.8 | 82 | 8 | Security for HT STA | 1) This encryption requirement is beyond the scope of 11n which is supposed to focus on things that improve throughput. 2) Currently, the excessive latency from the TKIP restarting makes 11n devices hard to implement A-MPDUs with TKIP for all vendors. So what is required in the spec is actually the only practical choice for A-MPDU implementations at this moment. Having that specified does not add any values. 3) Whether there will be innovative ways to have TKIP and A-MPDU work together in the future in unknown. If it happens, it does not help to prevent people from using TKIP if it works fine. 4) That requirement can lead to many unnecessary confusions and discussions that may potentially slow-down the 11n process. | Remove the single sentence specification on security altogether. | Defer - U - need to request clarification on whether this text is within the scope of the TGn PAR. |
| --- | --- | --- | --- | --- | --- | --- | --- |

### Proposed Resolution

Reject.   The goal of the standard is to define something that can be implemented in a timely fashion.   Similar constraints/design choices exist in the baseline today for the same reason.

Note, the specific subclause has been removed from D1.07, however this equivalent statements are now made elswhere in clause 8.

Note also that TKIP is explicitly in 802.11 to support a transition from pre-RSN hardware.  Given that a .11n device has to support CCMP, it makes no sense to permit the less secure TKIP encyption mechanism to be used between .11n devices.

Note, this does not prevent TKIP being used between a .11n device and a non-HT device.

# Comments assigned to this author

The following comments are assigned to this author.

## CID 3739

| 3739 | 7.3.1.14 | 36 | 20 | Block Ack Parameter Set field | While I like the concept of aggregating both at the upper MAC level and the lower MAC level, I do not believe the receivers put away large-sized buffers away for this mechanism. On some platforms you can only choose the size of each of the buffers and at an AP even if you maintain a few Block Ack streams with moderate buffer sizes. Having this set to 4K or 8K will introduce serious resource crunch on the AP. | Delete the added changes to the fourth paragraph of the subclause | Defer - 10-0 - The draft text does not change the resource requirements at a receiver, but instead, simply changes the degree of resolution of which the device may express its resources. Accept - 0 - 9 - Delete the line as suggested. Reject - 5-4 - the cited language is not so restrictive as to disallow the sharing of buffers among multiple TIDs. And hence, the real resource requirement is not increased from previous versions of the specification. |
| --- | --- | --- | --- | --- | --- | --- | --- |

### Proposed Resolution

Counter: Submission 11-06/1843r2 enables the use of A-MSDU to be negotiated per BA agreement, thereby enabling a recipient STA to control whether it provides 2K or 4K BA buffers.

## CID 10316

| 10316 | Annex A | 267 | HTM8 |  | Capability should be mandatory for all STAs.. | Modify Status column accordingly. | Gen AdHoc: Transfer to MAC PICS should match feature in resultant text provide in Draft. |
| --- | --- | --- | --- | --- | --- | --- | --- |

### Proposed Resolution

Counter.  The commenter refers to LongNAV.  The resolution to comment 3623 deletes the term and the subclause referenced by HTM8 from the draft, but introduces a new subclause that is not referenced from the PICS containing normative requirements.

The edits shown below rename change the old LongNAV requirement to refer to the new 9.13.6.1 and show CF-End as an optional transmitter behaviour

These are achieved by the following edits (D1.07):

1. Rename HTM8 (Long NAV protection mechanism) as HTM8 (Duration/ID rules for A-MPDU and TXOP)

2. Promote HTM8.1 to HTM8a and modify as follows: (Truncation of TXOP as TXOP holder) (9.13.6.2) (CF15:O)

# Unassigned Comments

The following comments are not assigned to any author.  This submission proposes a resolution.

## CID 3823

| 3823 | 9.10.6 | 92 | 20 | Use of Compressed bitmap | It appears that uncompressed bitmap is not allowed - however, this is not in line with allowing fragmenting of A-MSDUs. | Disallow the fragmenting of A-MSDUs, at least when sent at HT rates. | Defer - U |
| --- | --- | --- | --- | --- | --- | --- | --- |

### Proposed Resolution

Counter.

D1.08 reads: 7.3.1.14 :"Fragmentation is not allowed under Compressed block ack"

9.1.5: "An MSDU or A-MSDU transmitted under HT-immediate or HT-delayed BlockAck agreement shall not be fragmented”

CID 7228

| 7228 | 7.2.1.8 | 26 | Table n13 | Block Ack (BlockAck) frame format | The ACK policy table is referencing its relevance to N-Delayed BlockAck scheme. Although, in the definitions within the table, i.e. Normal Acknowledgement it implies to apply to all cases requiring immediate acknowledgement, i.e. N-Immediate BlockAck. | Please specify in this table (or add a statement right after this table) that the ACK policy bit is reserved (set to zero) for N-Immediate BlockAck scheme. | Edit Notes (D1.05) ER: <This change conflicts with 11945 that explicitly refers to the table only for the HT-delayed case. Likewise it conflicts with 11947, 11948 and 11949. It also conflicts with 1738.>, to resolution (D1.05): Counter - U - within the table entry for Normal Acknowledgement, add the two sentences: "Normal Acknowledgement is the only setting allowed for BARAckPolicy and BAAckPolicy bits within BAR and BA frames transmitted under the N-immediate Block Ack mechanism. In the case of the N-immediate Block Ack mechanism, the addressee returns an Block Ack, Block Ack (compressed) or MTID Block Ack, as appropriate." Also delete the words " for N-Delayed BlockAck" from the caption for table n13. Add the sentence: "Normal Acknowledgement is the only setting allowed for BARAckPolicy and BAAckPolicy bits within BAR and BA frames transmitted under the Block Ack mechanism." |
| --- | --- | --- | --- | --- | --- | --- | --- |

### Proposed Resolution

Counter: See resolution of CID 11945 and 11949 (Implemented in D1.07).  These add a statement to 7.2.1.7 and 7.2.1.8 that clearly indicate that the BAR Ack Policy and BA Ack Policy fields are only defined for HT-delayed BlockAck, and reserved otherwise.

This achieves the intent of the commenter's proposal in a slightly different form.

## CID 2396

| 2396 | 9.2.3 | 83 | 1 | IFS | Replace Figure 157 with one showing RIFS | Make change indicated in comment | Edit Notes (D1.03) ER: <"a new figure showing RIFS" is inadequate editing instructions. It is not clear from the instruction what relationships are intended to be shown, specifically because RIFS does not relate to SIFS minus any number of slot times. The title of the figure "Some IFS relationships" hints that this is the purpose of the figure. BTW, the figure number quoted is in REVma D6.0.>, to resolution (D1.03): Accept - U - editor to include RIFS in a new figure to replace figure 157 |
| --- | --- | --- | --- | --- | --- | --- | --- |

### Proposed Resolution

Reject.   The commenter provides no justification for the change, and the instruction is insufficiently detailed to be able to action it.

## CID 2743

| 2743 | 9.23.6 | 130 | 17 | Protection in 40/20MHz BSS | This text belongs in the same clause as normal protection measures | Make change indicated in comment | mac adhoc 060920 Defer - U - COEX Deferred then passed to MAC for later submission and discussion |
| --- | --- | --- | --- | --- | --- | --- | --- |

### Proposed Resolution

Counter. The text is addressing issues of protection in the context of 20/40 operation.   It is appropriate to include under the 20/40 operation subclause.  Submission 11-07/0004r0 and 11-06/1572r2 move some of the referenced material into protection subclauses.

## CID 11739

| 11739 | 9.23.6 | 130 | 17 | Protection in 40/20MHz BSS | This text belongs in the same clause as normal protection measures | as in comment | Deferred for later submission and discussion. Ref CID2743 |
| --- | --- | --- | --- | --- | --- | --- | --- |

### Proposed Resolution

Counter. The text is addressing issues of protection in the context of 20/40 operation.   It is appropriate to include under the 20/40 operation subclause.  Submission 11-07/0004r0 and 11-06/1572r2 move some of the referenced material into protection subclauses.

CID 3512

| 3512 | 20.7 |  |  |  | ACK rate for unequal constellation per stream needs to be defined | Send legacy rate less than or equal to 36Mbps or the stream 1 modulation. | PHY adhoc: transferred to MAC. Reason for transfer: Comment refers to rate selection. |
| --- | --- | --- | --- | --- | --- | --- | --- |

### Proposed Resolution

Counter. D1.07 9.6.6 describes the selection of an MCS in response to an MCS for control frames.  9.6.8 describes how to select a non-HT basic rate, given an MCS, and includes the following:  "In the case of an MCS with unequal modulation, the modulation of stream 1 is used. (Ed: CID 11996)".

## CID 1476

| 1476 | 9.9.1.2 | 90 | 8 | EDCA TXOPs | Be more inclusive | Change the word "or" to "and/or" | Defer - U - see CID 12001 |
| --- | --- | --- | --- | --- | --- | --- | --- |

### Proposed Resolution

Counter.  The commenter refers to: "in addition to a possible RTS/CTS exchange or CTS to itself, may be transmitted at any rate for each TXOP."   The only case when both are permitted is during Dual CTS.

Replace the quoted text with (D1.07):  "in addition to a possible RTS/CTS exchange or CTS to itself (or a dual CTS sequence as defined in 9.2.6.6a), may be transmitted at any rate for each TXOP."

## CID 6787

| 6787 | 9.15.3 | 106 | 34 | L-SIG TXOP Protection Rules at Third Party HT | "An HT receiver that asserted the L-SIG TXOP Protection support bit upon association, and that receives an L-SIG protected PPDU in which it can not decode a MAC duration shall update it’s NAV to a value equal to L-SIG duration – HT-SIG duration." To be fully effective, NAV update should be done at all HT STA. | Remove; "that asserted the L-SIG TXOP Protection support bit upon association, and" from the sentence. | 061114 mac adhoc Reject - 9/5 - Accept - 7/4 |
| --- | --- | --- | --- | --- | --- | --- | --- |

### Proposed Resolution

Reject: the proposed text requires third party HT STA to update their NAV based on the L-SIG value.  However they can set their nav based on the PPDU contents,  which will be sent using a basic MCS (9.6.3.1), and so mandating this additional step provides little additional protection at an unnecessary (even though it might be small) increase of complexity.

Yuichi:  see doc 11-07/0004.

Deferred until discussion of 11-07/0004.

# Other comments

## CID 3589, 7593, 3588 – Multicast

These comments are assigned to the author and Naveen Kakani.   They have been reviewed with Naveen, and he supports these resolutions.

| 3589 | 7.4.4.2 | 57 | 2 | ADDBA Response frame format | Add capability for multicast transmission | Enhance ADDBA response frame to signal the response for ADDBA request frame | Defer - U - |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 7593 | 7.4.7.10 |  |  | PSMP | "Multicast data is transmitted using PSMP by setting STA_ID in the STA_INFO field to a specific value 0. The ULT startoffset and ULT duration fields of the STA_INFO field shall be set to the Least Significant Bits of the multicast address." The whole area of multicast is not adequately described or served in this draft. | Modify the PSMP mechanism to achieve reliable multicast. | Transfer to MAC Adhoc as this is part of discussion related to Reliable Multicast - U Defer - Assigned volunteer to bring a submission - U |
| 3588 | 7.4.4.1 | 57 | 1 | ADDBA Request frame format | Add capability for multicast transmission | Enhance ADDBA request frame to signal Block ACK setup by STA for a multicast/broadcast data stream. STA needs to signal: Multicast/broadcast address (complete address, hash, LSB's), TID at the STA that corresponds to the multicast address. | Defer - U - |

### Proposed Resolution

Reject.   While not rejecting the concept of improved support for multicast, this is a topic of ongoing discussion between members of TGn and TGv.  No concrete proposal has been brought that has yet demonstrated a substantial level of support in TGn.

## CID 10366

| 10366 | 5.2.7.2 | 8 | 1 | General description of HT Features | PSMP should be mandatory in APs under all cases. Power save support is critical to wide scale deployment of low power mobile STAs. While this support can be optional in the mobile STAs (depending on battery capacity) it should be supported in all APs. | Make PSMP mandatory in APs under all conditions. Apply the requisite changes here and throughout the draft as needed. | Edit Notes (D1.08) ER: <Note, response is a "reject", not a "counter".>, to resolution (D1.08): Counter - U (PSMP Adhoc Nov 10th) Leave PSMP as it is. Gen AdHoc: Transfer to PSMP to allow PSMP CID grouping. TG: 9-19-06: During the motion to accept the resolution, this comment was excluded and the resolution was requested to address PSMP rather than clause 5.2.7. Gen AdHoc: Please Refer to 11-06-755 for an update to 5.2.7 and all subclauses and 5.2.8 for replacement text. |
| --- | --- | --- | --- | --- | --- | --- | --- |

### Background Information

This comment resolution “went round the houses” a bit. Gen transferred it to PSMP.   PSMP approved a “counter” resolution.   The editor recycled the comment because it should have been marked “reject” as it was not attempting to address the commenter’s concern in any way. The following proposed resolution rejects the comment with an accompanying reason.

This comment is (at the time of writing) owned by the PSMP ad-hoc.  However they only have one comment to address, so the MAC chair has agreed with the PSMP chair to take ownership of this comment.

### Proposed Resolution

Reject. Based on a straw poll in the MAC ad-hoc (1-10) there is not enough support to have PSMP as a mandatory feature of the HT AP.

Straw poll:  should some form of PSMP be mandatory at the HT AP?

Yes 1

No 10