IEEE P802.11
Wireless LANs

| Proposed Spectrum Mask for 20 MHz PPDUs transmitted in 40 MHz |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-01-11 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| John Ketchum | Qualcomm Incorporated | 9 Damonmill Square Suite 2A Concord, MA 01742 | +1-781-276-0915 | johnk@qualcomm.com |

Abstract

This document proposes a resolution to CIDs 11918, 3445, which consists of a new spectral mask for transmission of a 20 MHz PPDU in 40 MHz operating channel width.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart@ok-brit.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# Introduction

## Interpretation of a Motion to Adopt

A motion to approve this submission means that the editing instructions and any changed or added material are actioned in the TGn Draft.  This introduction, is not part of the adopted material.

Editing instructions formatted like this are intended to be copied into the TGn Draft (i.e. they are instructions to the 802.11 editor on how to merge the TGn amendment with the baseline documents).

TGn Editor:  Editing instructions preceded by “TGn Editor” are instructions to the TGn editor to modify existing material in the TGn draft.   As a result of adopting the changes, the TGn editor will execute the instructions rather than copy them to the TGn Draft.

Summission Note: Notes to the reader of this submission are not part of the motion to adopt.  These notes are there to clarify or provide context.

| 11918 | R | Statement incorrect: "The transmit spectral mask for 20MHz transmission in upper or lower 20Mhz channels of a 40MHz is the same mask as that used for the 40MHz channel." | Change the text to: "The transmit spectral mask for 20MHz transmission in upper or lower 20Mhz channels of a 40MHz channel is the same mask as that used for the 20MHz channel." | As per 1929r0 |
| --- | --- | --- | --- | --- |
| 3445 | R | The 20MHz transmit spectral mask should be required for a lower-20MHz or upper-20MHz transmission. Otherwise, any 20MHz transmission by an 11n device can severely impact 802.11a/g devices in the extension channel even when only transmitting in the control channel. | All 20MHz transmissions, whether upper-20, lower-20, or 20, should be restricted to the 20MHz spectral mask. | As per 1929r0 |

Proposed resolution: Counter with the following text and figure.

TGn Editor: replace the following text on page 278 lines 32 – 34 (D1.08)

The transmit spectral mask for 20 MHz transmission in upper or lower 20 MHz channels of a 40 MHz is the

same mask as that used for the 40 MHz channel.

With the following text and figure:

In the absence of other regulatory restrictions, when transmitting a 20 MHz PPDU in 40 MHz operating channel width, the transmitted spectrum shall have a 0 dBr bandwidth not exceeding 18 MHz (-9 MHz to +9 MHz frequency offset from the center frequency of the 20 MHz PPDU), –20 dBr at 11 MHz frequency offset from the center frequency of the 20 MHz PPDU, -28 dBr at 20 MHz frequency offset, -38 dB at 30 MHz frequency offset, and –45 dBr at 50 MHz frequency offset. The transmitted spectral density of the transmitted signal shall fall within the spectral mask, as shown in Figure nxx (Transmit spectral mask for a 20 MHz PPDU in 40 MHz operating channel width).

Figure nxx: Transmit spectral mask for 20 MHzz PPDU in 40 MHz operating channel width