IEEE P802.11
Wireless LANs

| Calibration using NDP |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-01-15 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Vincenzo Scarpa | STMicroelectronics | Via per Arnesano KM5 c/o Distretto Tecnologico – 73100 Lecce, Italy | +390832353124 | vincenzo.scarpa@st.com |

Abstract

This submission covers CID 7357 and is based on draft 1.08

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart@ok-brit.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

| CID | Part of No Vote(Y/N) | Page(Ed) | Line(Ed) | Clause(Ed) | Type E/HE/T/ST/DT(Ed) | Duplicate of CID | Comment | Proposed Change | Resolution | Edited in draft | Topic lookup | Topic Group Lookup |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7357 | Y | 17 |  | 7.1.3.8, 9.20 | T |  | Clarify the use of ZLF outside Antenna Selection procedure (i.e. Calibration and TX Beamforming). | See submission 11-06-0573-00-000n-submission-use-of-ZLF-sounding on the server. | Counter: a calibration procedure using NDP for sounding is defined in 06/1411r7, which has been accepted by the beam and la ad hoc. |  |  | Beamforming & Adaptation |

TGn Editor: Change the text of section 9.17.2.4.2  “Procedure”, on page 144, of TGn D1.08, as follows:

(Ed: CID 7357) STA A transmits a Calibration Start frame (Calibration Position field set to 1) with TRQ field in HT Control field set to 1, NDP Announcement, RDG/More PPDU fields in the HT Control field set to 1, and CSI/Steering subfield in HT Control field set to 1. This frame initiates a calibration procedure. The Calibration Start frame shall have a frame type of QoS Null+HTC, with the ACK Policy set to Normal ACK. The Duration/Id field in this Calibration Start frame shall cover the Calibration Sounding Response frame plus the following NDP from STA B plus a SIFS time plus the Calibration Sound Complete frame.

(Ed: CID 7357) In response, STA B shall transmit a Calibration Sounding Response frame (Calibration Position field set to 2) with NDP Announcement field set to 1, after a SIFS interval. STA B shall send an NDP as a sounding PPDU within SIFS after the Calibration Sounding Response frame. This allows STA A to estimate the MIMO channel from STA B to STA A. In this Calibration Sounding Response frame, the Calibration Sequence subfield in HT Control field shall be set to the same value that is indicated in the Calibration Start frame. The Calibration Response frame shall have a frame type of ACK+HTC. In this frame, RDG/More PPDU field in HTC shall not be set to 1.

(Ed: CID 7357) According to the NDP Announcement in the Calibration Start frame, STA A shall transmit NDP as a sounding PPDU after PIFS SIFS interval. This NDP allows STA B to estimate the MIMO channel from STA A to STA B.  The NDP shall be followed by a Calibration Sounding Complete frame (Calibration Position field set to 3) a SIFS after the NDP from STA A.  In the Calibration Sounding Complete frame, the Calibration Sequence subfield in the HT Control field shall be set to the same value that is indicated in the Calibration Response frame. The Calibration Sounding Complete frame shall be either a QoS Null+HTC or a QoS Data+HTC frame, and shall have the ACK Policy field set to No ACK.

TGn Editor: Replace the figure n55 of 9.17.2.4.2 “Procedure”, on page 144, of TGn D1.08, with the following one:

Cal Position: 1

Cal Start

Cal Position: 2

Sound Response

Cal Position: 3

Sound Complete

NDP

NDP

Step 1

Step 2 is not shown here

- Frame Type: QoS Null

- ACK Policy: Normal

- +HTC: NDP, TRQ, CSI feedback req

- Frame Type: ACK

- +HTC: NDP

- Frame Type: QoS Null (Data)

- ACK Policy: No ACK

TGn Editor: Change the text of section 9.19.1 “NDP Rules”, on page 150, of TGn D1.08, as follows:

Only the current TXOP holder or RD responder a Calibration responder or RD responder may transmit an NDP. A TxOP holder shall not set to 1 the NDP Announcement and RDG/More PPDU fields simultaneously, except during a calibration sounding exchange (see 9.17.2.4 (Calibration)). An RD responder shall not set to 1 the NDP Announcement and set to 0 the RDG/More PPDU fields simultaneously.

TGn Editor: Add the following  text at line 65 of 9.19.1 “NDP Rules”, on page 150, of TGn D1.08:

From a MAC perspective a +HTC frame with NDP announcement set and No-ACK policy followed by an NDP within a SIFS is not a burst (i.e. the NDP is not processed by the MAC).

TGn Editor: modify the section 9.19.3 “Determination of NDP source”, on page 152, of TGn D1.08 as follows:

The source of an NDP is determined at the NDP receiver by examining the frame immediately previous (within SIFS) to the NDP as follows: (Ed: CID 1442)

- If the immediately previous frame at the NDP receiver was a transmission by the TXOP holder or the current RD responder or the Calibration responder or RD responder, the source of the NDP is equal to the TA of that immediately previous frame.

- If the immediately previous frame at the NDP receiver was a transmission by a STA that is not the TXOP holder or RD responder, the source of the NDP is equal to the RA of that immediately previous frame.

If the PPDU a SIFS before the reception of an NDP is also an NDP, the source of the NDP is equal to the destination of the previous NDP.

References:

IEEE P802.11n/D1.08 December 2006