IEEE P802.11
Wireless LANs

| Normative Text Proposal for WLAN Paging and Idle Mode |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-03-12 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Sunghyun Choi | Seoul National University | Kwanak P.O.Box 34, Seoul 151-600, Korea | +82-2-880-1753 | schoi@snu.ac.kr |
| Kyunghun Jang | Samsung Advanced Institute of Technology |  |  | khjang@samsung.com |
| Moo Ryong Jeong | DoCoMo USA Labs | 3240 Hillview Avenue, Palo Alto CA | 650-496-4761 | jeong@docomolabs-usa.com |
| Sunggeun Jin | ETRI | Kwanak P.O.Box 34, Seoul 151-600, Korea | +8228801755 | sgjin@mwnl.snu.ac.kr |
| Youngsoo Kim | Samsung Advanced Institute of Technology |  |  | kimyoungsoo@samsung.com |
| Mee-Suk LEE | Korea Telecom | 17, Woomyenon-dong Seocho-gu Seoul Korea | +8225266174 | kitlee@kt.co.kr |
| Sudheer Matta | Trapeze Networks | 5753 W. Las Positas Blvd, Pleasanton, CA | 9254742290 | matta@trapezenetworks.com |
| R. R. Miller | AT&T | 180 Park Ave, Florham Park NJ | 973-236-6920 | rrm@att.com |
| Subbu Ponnuswamy | Aruba Networks | 1322 Crossman Ave Sunnyvale, CA | 408-754-1213 | Subbu@arubanetworks.com |
| Henry Ptasinski | Broadcom Corp. | 190 Mathilda Place Sunnyvale, CA, 94086 | 408-543-3316 | henryp@broadcom.com |
| Emily Qi | Intel Corp. | 2111 NE 25th Ave Hillsboro OR | 503-264-7799 | Emily.H.Qi@intel.com |
| Dorothy Stanley | Aruba Networks | Warenville, IL | 630-363-1389 | dstanley@arubanetworks.com |
| James Tsai | Intel Corp. | 2111 NE 25th Ave Hillsboro OR | 503-712-5509 | James.tsai@intel.com |
| Jesse Walker | Intel Corp. | 2111 NE 25th Ave Hillsboro OR | 503-712-1849 | Jesse.Walker@intel.com |
| Fujio Watanabe | DoCoMo USA Labs | 240 Hillview Avenue, Palo Alto, CA | 650-496-4726 | watanabe@docomolabs-usa.com |
| Xavier Perez Costa | NEC Network Laboratories | Kurfürsten-Anlage 36, D-69115 Heidelberg, Germany | +49 (6221) 4342 148 | perez@netlab.nec.de |
| Angelo Centonza | Roke Manor Research | Old Salisbury Lan Romsey, Hampshire SO51 0ZN, UK | +44 1794 833185 | angelo.centonza@roke.co.uk |
| James Yee | Marvell | 5488 Marvell Lane, Santa Clara, CA | 408-222-0868 | jyee@marvell.com |
| Joe Epstein | Meru Networks | 1309 S Mary Ave, Sunnyvale CA | 408-215-5345 | jepstein@merunetworks.com |

Abstract

This document contains normative text for a Paging and Idle Mode proposal to meet the TGv Power Saving objective, REQ2010, improving mobile client power efficiency and battery life. The text is aligned with P802.11-REVma/D9.0 and P802.11v D0.07.

Idle Mode is an extended power save mode for non-AP STAs. While in Idle Mode, a STA needs not to listen for every Beacon frame, may traverse multiple BSSs in a Paging Group and still be reachable within the Paging Group. Paging Services within a Paging Group are managed by a Paging Server. The Paging Server maintains paging service information for STAs in Idle Mode within a Paging Group. Paging service information includes the STA MAC Address, Paging Group ID, Paging Server ID, and Home AP.  A STA in a Paging Group moves among BSSs in a Paging Group and stays in Idle Mode.  When a STA moves from one Paging Group to a new Paging Group, the STA performs a reassociation or BSS transition operation to associate with an AP in the new Paging Group.

Idle Mode and Paging operation comprises the following procedures: Paging Capability Discovery and  Paging Service Information Advertisement,  Enter Idle Mode,  Paging, and Idle Mode Notification and Exit Idle Mode.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart@ok-brit.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Contents

3. Definitions	5

4. Abbreviations and Acronyms	6

7. Frame formats	7

7.2.3.1 Beacon frame format	7

7.2.3.8 Probe Request frame format	7

7.2.3.9 Probe Response frame format	7

7.3.2 Information Elements	7

7.3.2.41	Multiple BSSID element	9

7.3.2.49 Paging Service element	9

7.3.2.50 Paging Indication element	10

7.3.2.51 Idle Mode Request element	10

7.3.2.52 Idle Mode Response element	11

7.4 Action frame format details	12

7.4.6 Wireless Network Management action details	12

7.4.6.13 Idle Mode Request frame format	12

7.4.6.14 Idle Mode Response frame format	13

10. Layer management	14

10.3 MLME SAP Interface	14

10.3.41 Paging Indication	14

10.3.41.1 MLME-PIREQUEST.request	14

10.3.41.1.1 Function	14

10.3.41.1.2 Semantics of the service primitive	14

10.3.41.1.3 When generated	14

10.3.41.1.4 Effect of receipt	14

10.3.41.2 MLME-PIREQUEST.confirm	14

10.3.41.2.1 Function	14

10.3.41.2.2 Semantics of the service primitive	14

10.3.41.2.3 When generated	15

10.3.41.2.4 Effect of receipt	15

10.3.41.3 MLME-PIREQUEST.indication	15

10.3.41.3.1 Function	15

10.3.41.3.2 Semantics of the service primitive	15

10.3.41.3.3 When generated	15

10.3.41.3.4 Effect of receipt	15

10.3.42 Idle Mode request	16

10.3.42.1 MLME-IMREQUEST.request	16

10.3.42.1.1 Function	16

10.3.42.1.2 Semantics of the service primitive	16

10.3.42.1.3 When generated	16

10.3.42.1.4 Effect of receipt	16

10.3.42.2 MLME-IMREQUEST.confirm	16

10.3.42.2.1 Function	16

10.3.42.2.2 Semantics of the service primitive	17

10.3.42.2.3 When generated	17

10.3.42.2.4 Effect of receipt	17

10.3.42.3 MLME-IMREQUEST.indication	17

10.3.42.3.1 Function	17

10.3.42.3.2 Semantics of the service primitive	17

10.3.42.3.3 When generated	18

10.3.42.3.4 Effect of receipt	18

10.3.43 Idle Mode response	18

10.3.43.1 MLME-IMRESPONSE.request	18

10.3.43.1.1 Function	18

10.3.43.1.2 Semantics of the service primitive	18

10.3.43.1.3 When generated	19

10.3.43.1.4 Effect of receipt	19

10.3.43.2 MLME-IMRESPONSE.confirm	19

10.3.43.2.1 Function	19

10.3.43.2.2 Semantics of the service primitive	19

10.3.43.2.3 When generated	19

10.3.43.2.4 Effect of receipt	20

10.3.43.3 MLME-IMRESPONSE.indication	20

10.3.43.3.1 Function	20

10.3.43.3.2 Semantics of the service primitive	20

10.3.43.3.3 When generated	20

11. MAC sublayer management entity	21

11.3 Relationships between services	21

11.15 Wireless Network Management Procedures	21

11.15.8 Idle Mode and Paging	21

11.15.8.1 Paging Service Capability Discovery	21

11.15.8.2 Paging Service Advertisement	21

11.15.8.3 Enter Idle Mode	22

11.15.8.4 Paging	22

11.15.8.4.1 AP Paging operation	22

11.15.8.4.2  STA operation for receiving a Paging Indication	22

11.15.8.5 Exit Idle Mode	23

11.15.8.6 Idle Mode Notification	23

Annex A	24

A.4 PICS proforma–IEEE Std. 802.11, 1999 Edition	24

A.4.3 IUT configuration	24

A.4.14 Wireless Network Management extensions	24

Annex D	24

# 3. Definitions

Insert the following new definitions:

3.v.3 Idle Mode: An extended power save mode for non-AP STAs. While in Idle Mode, a STA does not listen for every Beacon frame, may traverse multiple BSSs in an ESS without maintaining the associated state with APs, and still be reachable from the ESS.

3.v.4 Home AP: The AP to which a non-AP STA is associated when it enters Idle Mode.

3.v.5 Paging Group: A group of APs in which a non-AP STA can be paged.

3.v.6 Paging Group Identifier: A one octet value that identifies a Paging Group.

3.v.7 Paging Interval field: The Paging Interval field indicates the number of Beacon intervals between successive DPIMs

3.v.8 Paging Server: An entity that provides the paging service for non-AP STAs. The Paging Server maintains paging service information for non-AP STAs in Idle Mode. A Paging Server serves one or more Paging Groups.

3.v.9 Paging Server Identifier: A 48-bit value that identifies a Paging Server.

3.v.10 Paging Service Bit: The Paging Service bit indicates whether or not the STA supports Paging Service and Idle Mode operation.

# 4. Abbreviations and Acronyms

Insert the following new acronym in alphabetical order:

DPIM		Delivery Paging Indication Message

# 7. Frame formats

## 7.2.3.1 Beacon frame format

Insert new element in Table 8 as indicated below:

Table 8—Beacon frame body

| Order | Information | Notes |
| --- | --- | --- |
| 31 | Paging Service | The Paging Service element is present only within Beacon frames generated by APs when dot11PagingServiceEnabled is true. |
| 32 | Paging Indication | The Paging Indication element is present only within Beacon frames generated by APs when dot11PagingServiceEnabled is true and the DPIM Count field in the Paging Service element is 0. |

#### 7.2.3.8 Probe Request frame format

Insert a new row into table 14 as follows:

Table 14—Probe Request frame body

| Order | Information | Notes |
| --- | --- | --- |
| 7 | Paging Service | The Paging Service element is present only within Probe Request frames generated by non-AP STAs when dot11PagingServiceEnabled is true. |

#### 7.2.3.9 Probe Response frame format

Insert a new row into table 15 as follows:

Table 15—Probe Response frame body

| Order | Information | Notes |
| --- | --- | --- |
| 28 | Paging Service | The Paging Service element is present only within Probe Response frames generated by APs when dot11PagingServiceEnabled is true. |
| 29 | Paging Indication | The Paging Indication element is present only within Probe Response frames generated by APs when dot11PagingServiceEnabled is true. |

### 7.3.2 Information Elements

Insert new rows into Table 26 and change the Reserved row accordingly:

Table 26—Element IDs

| Information Element | Element ID |
| --- | --- |
| Paging Service | x+14 |
| Paging Indication | x+15 |
| Idle Mode Request | x+16 |
| Idle Mode Response | x+17 |
| Reserved | x+18, 220 |

Change Clause 7.3.2.35 as shown below:

7.3.2.35 Wireless Network Management Capability Information element

The Wireless Network Management Capability Information element contains information about the wireless network management capabilities of a STA as shown in Figure v2.

|  | Element ID | Length | Wireless Network Management Capabilities |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | variable |

- Figure v2 —Wireless Network Management Capability Information element format

The Element ID field is equal to the Wireless Network Management Capability value in Table 26.

The value of the length field is variable and depends on the length of the Wireless Network Management Capabilities field.  The minimum value of the Length field is 2.

The Wireless Network Management Capabilities field is a bit field indicating the advertised management capabilities of the STA. The Wireless Network Management Capabilities field is shown in Figure 3.

|  | B0 |  | B1 |  | B2 |  | B3 |  | B4 |  | B5 |  | B6 |  | B7 |  | B8 |  | B15 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Event Log |  | Diagnostics |  | Multicast Alert |  | Presence |  | FBMS |  | Proxy ARP Service |  | Co-located Interference Reporting |  | Paging Service |  | Reserved |  |  |
| Bits: |  | 1 |  | 1 |  | 1 |  | 1 |  | 1 |  | 1 |  | 1 |  | 1 |  | 8 |  |  |

- Figure v3 —Wireless Network Management Capabilities

- The Event Log bit set to 1 indicates the STA supports Event Log as described in 11.15.2. The Event Log bit set to 0 indicates that the STA does not support this service.

- The Diagnostics bit set to 1 indicates the STA supports Diagnostics as described in 11.15.3. The Diagnostics bit set to 0 indicates that the STA does not support this service.

- The Multicast Alert bit set to 1 indicates the STA supports Multicast diagnostics as described in 11.15.1.1. The Multicast Alert bit set to 0 indicates that the STA does not support this service.

- The Presence bit set to 1 indicates that the STA supports Presence as described in 11.15.4. The Presence bit set to 0 indicates that the STA does not support this service.

- The FBMS bit set to 1 indicates the STA supports FBMS as described in section 11.2.1. The FBMS bit set to 0 indicates the STA does not support FBMS.

- The Proxy ARP Service bit set to 1 indicates the AP is providing proxy ARP service.  If Proxy ARP service is enabled, then the AP responds to broadcast ARP request on behalf of the STA. The Proxy ARP Service bit set to 0 indicates the AP is not providing proxy ARP service for any associated client.

- The Paging Service bit set to 1 indicates that the STA supports Paging Service and Idle mode operation as described in 11.15.8. The Paging Service bit set to 0 indicates that the STA does not support this service.

- All other bits are reserved, and are set to 0 on transmission and ignored on reception.

The lack of a Wireless Network Management Capability element is interpreted as the STA having no advertised Wireless Network Management Capabilities.

Modify the 6th paragraph and Table V45 in Clause 7.3.2.41 as indicated below.

      - Multiple BSSID element

As shown in Table V45, the Non-Transmitted BSSID profile may include the Capability, Supported Rates, CF Parameter Set, Power Constraint, Quiet, TPC Report, Extended Supported Rates, RSN, Paging Service, Paging Indication and Vendor Specific information element fields for each non-transmitted BSSID. The Timestamp, Beacon Interval, DS Parameter Set, FH Parameter Set, IBSS Parameter Set, Country, FH Parameters, FH Pattern Table, Channel Switch Assignment, IBSS DFS, and ERP Information element values for each non-transmitted BSSID is not included in the Multiple BSSID element, and is always the transmitted BSSID element values.

Insert rows for Paging Service and Paging Indication elements as showm below:

- V45—Non-Transmitted-BSSID Profile

| Information | Notes |
| --- | --- |
| IBSS Parameter Set | Not Included |
| Country | Not Included |
| Paging Service | Optional |
| Paging Indication | Optional |
| Vendor Specific | Optional |

Add new clauses after Clause 7.3.2.48 as shown below:

### 7.3.2.49 Paging Service element

The Paging Service information element contains paging service and paging indication delivery information. The format of the Paging Service Information element is shown in Figure v-1.

|  | Element ID | Length | Paging Group ID | Paging Interval | DPIM Count |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 | 1 |

- Figure v-1—Paging Service element format

The Element ID field is equal to the Paging Service information element value in Table 26.

The value of the Length field is set to 4.

The Paging Group ID field contains a two octet value that identifies the Paging Group.

The Paging Interval field indicates the number of Beacon intervals between successive DPIMs. If the Paging Interval equals the Beacon interval, the Paging Interval field has the value 1. The Paging Interval field is a single octet. The Paging Interval is set to 0 when the Paging Service element is contained in the Probe Request frame. The Paging Interval value 0 is reserved when the Paging Service element is contained in the Beacon and Probe Response frame.

The DPIM Count field indicates the number of Beacon frames to (including the current frame) appear before the next DPIM. A DPIM Count of 0 indicates that the current Beacon is a DPIM. The DPIM count field is a single octet. If the Paging Service information element is present in a Probe Response frame, the DPIM Count field is set to the value that appears in the next beacon. The DPIM Count is set to 0 in the Paging Service element contained in the Probe Request.

### 7.3.2.50 Paging Indication element

The Paging Indication information element contains a Paging ID List field. The format of the Paging Indication element is shown in Figure v-2.

|  | Element ID | Length | Paging ID List |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | 1– 252 |

- Figure v-2— Paging Indication element format

The Element ID field is equal to the Paging Indication value defined in Table 26.

The Length field for this element indicates the length of the information field

The Paging ID List is a variable length field containing from 1-126  2-octet Paging IDs.

### 7.3.2.51 Idle Mode Request element

The Idle Mode Request element is used by the non-AP STA to enter or exit Idle Mode operation, and to provide notification that the non-AP STA is in Idle Mode. The format of the Idle Mode Request element is shown in Figure v-5.

|  | Element ID | Length | Request Type | STA Address | Paging Group ID |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 6 | 1 |

- Figure v-5— Idle Mode Request element format

The Element ID field is equal to the Idle Mode Request value defined in Table 26.

The value of the Length field is set to 14.

The Request Type is set to a number that identifies a Idle Mode request. The Request Types for Idle Mode requests are shown in Table v-2.

- Table v-2—Request Type Definitions

| Name | Request Type |
| --- | --- |
| Exit Idle Mode | 0 |
| Enter Idle Mode | 1 |
| Idle Mode Notification | 2 |
| Reserved | 3 – 255 |

The STA Address field contains the MAC address of the STA sending the Idle Mode Request element.

The Paging Group ID field contains a two octet value that identifies the Paging Group.

The Idle Mode Request element is included in Idle Mode Request frame, as described in 7.4.6.13.

### 7.3.2.52 Idle Mode Response element

The Idle Mode Response element contains the response to a STA’s Idle Mode operation request. The format of the Idle Mode Response sub-element is shown in Figure v-3.

|  | Element ID | Length | Response Status | STA Address | Paging Server ID | Paging Group ID | Paging ID |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 6 | 6 | 1 | 2 |

- Figure v-3— Idle Mode Request element format

The Element ID field is equal to the Idle Mode Response value defined in Table 26.

The value of the Length field is set to 17.

The STA Address field contains the STA address from  the received Idle Mode Request element.

The Response Status is set to a value in Table v-4, indicating the Paging Server’s response to the Idle Mode Request.

- Table v-4—Response Status values

| Status Value | Description |
| --- | --- |
| 0 | Successful |
| 1 | Failed |
| 2 | Refused - Reason Unspecified |
| 3 | Incapable - Reason Unspecified |
| 4 | Refused - The STA is using FBMS |
| 5 | Incapable – Insufficient service capacity in the Paging Server |
| 6 | Incapable – Insufficient service capacity in the Paging Server |
| 7–255 | Reserved |

The Paging Server ID field contains a 6-octet value that identifies the entity that provides the Paging Service.

The Paging Group ID field contains a two octet value that identifies the Paging Group.

The Paging ID field is a value assigned by the Paging Server to specify the indentifier of a STA for the paging service. The Paging ID value is a unique 2 octet value within a Paging Group, and is in the range of 1–65536.

The Idle Mode Response element is included in Idle Mode Response frame, as described in 7.4.6.14.

## 7.4 Action frame format details

### 7.4.6 Wireless Network Management action details

Insert the following rows into Table v47 and change the reserved row accordingly:

Table v47—Wireless Network Management Action field values

| Action field value | Description |
| --- | --- |
| 13 | Idle Mode Request |
| 14 | Idle Mode Response |
| 15-255 | Reserved |

Insert the following new clause after Clause 7.4.6.12 as indicated below:

#### 7.4.6.13 Idle Mode Request frame format

The Idle Mode Request frame uses the Action frame body format and is transmitted by a non-AP STA to enter or exit Idle Mode operation. The format of the Idle Mode Request frame body is shown in Figure v-8.

|  | Category | Action | Dialog Token | Idle Mode Request Element |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 16 |

Figure v-8— Idle Mode Request frame body format

The Category field is set to the value indicating the Wireless Network Management category, as specified in Table 24 in 7.3.1.11.

The Action field is set to the value indicating Idle Mode Request, as specified in Table v7 in 7.4.6.

The Dialog Token field is a nonzero value which identifies the Idle Mode Request/Response transaction. The dialog token is unique for each Idle Mode request frame sent to a given destination MAC address.

The Idle Mode Request Element field contains the Idle Mode elements, described in 7.3.2.51.

#### 7.4.6.14 Idle Mode Response frame format

The Idle Mode Response frame uses the Action frame body format and is transmitted by a AP STA in response to the Idle Mode Request frame with a Request type of “1” (Enter Idle Mode).  The format of the Idle Mode Request frame body is shown in Figure v-9.

|  | Category | Action | Dialog Token | Idle Mode Response Element |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 21 |

Figure v-9— Idle Mode Response frame body format

The Category field is set to the value indicating the Wireless Network Management category, as specified in Table 24 in 7.3.1.11.

The Action field is set to the value indicating Idle Mode Response, as specified in Table v7 in 7.4.6.

The Dialog Token field is set to the nonzero value received in the Idle Mode Request frame to identify the request/response transaction.

The Idle Mode Response Element field contains the Idle Mode Response element, described in 7.3.2.52.

# 10. Layer management

#### 10.3 MLME SAP Interface

Add new clauses after clause 10.3.40 as shown below:

    - Paging Indication

This set of primitives supports the advertising of Paging information from an SME to peer SMEs.

      - MLME-PIREQUEST.request

        - Function

This primitive requests the transmission of a Paging Indication element to peer entities.

        - Semantics of the service primitive

The primitive parameters are as follows:

MLME-PIREQUEST.request	(
Paging Indication Element
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Paging Indication Element | Paging Indication element | Paging Indication element | A Paging Indication element describing the paged STAs. See clause 7.3.2.48. |

        - When generated

This primitive is generated by the SME to request that a Beacon frame containing Paging Indication element be sent to a peer entities to convey paging indication information at the next time the beacon is transmitted.

        - Effect of receipt

On receipt of this primitive, the MLME constructs a Beacon frame containing a Paging Indication element specified. This frame is then scheduled for transmission.

      - MLME-PIREQUEST.confirm

        - Function

This primitive reports the result of the request to include a Paging Indication element in the Beacon frame.

        - Semantics of the service primitive

The primitive parameters are as follows:

MLME-PIREQUEST.confirm	(
ResultCode
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| ResultCode | Enumeration | SCCESS, INVALID PARAMETERS, or UNSPECIFIED FAILURE | Reports the outcome of a request to send a Beacon frame containing a Paging Indication element. |

        - When generated

This primitive is generated by the MLME when the request to transmit a beacon frame containing the Paging Indication element completes.

        - Effect of receipt

On receipt of this primitive, the SME evaluates the result code.

      - MLME-PIREQUEST.indication

        - Function

This primitive indicates that a Beacon frame containing the Paging Indication element has been received.

        - Semantics of the service primitive

The primitive parameters are as follows:

MLME-PIREQUEST.indication	(
Paging Indication Element
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Paging Indication Element | Paging Indication Element | Paging Indication element | A Paging Indication element describing the paged STA information. See 7.3.2.48. |

        - When generated

This primitive is generated by the MLME when a valid Beacon frame containing a Paging Indication element is received.

        - Effect of receipt

On receipt of this primitive, the SME searchs Paging Indication element and checks if the non-AP STA is being paged. If the non-AP STA is being paged, the non-AP STA wakes up and reassociates with an AP. Otherwise, the non-AP STA stays in Idle mode.

    - Idle Mode request

This set of primitives supports the initiation and temination of Idle Mode operation between peer SMEs.

      - MLME-IMREQUEST.request

        - Function

This primitive requests the transmission of Idle Mode Request action frame to a peer entity.

        - Semantics of the service primitive

The primitive parameters are as follows:

MLME-IMREQUEST.request	(
Peer MAC Address,
Dialog Token,
Idle Mode Request,
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Peer MAC Address | MACAddress | Any valid individual MAC Address, or the Broadcast MAC address | The address of the peer MAC entity to which the Idle Mode Request shall be sent. |
| Dialog Token | Integer | 0 – 255 | The dialog token to identify the Idle Mode Request transaction. |
| Idle Mode Request | Idle Mode Request element | Idle Mode Request element | A Idle Mode Request element describing the STA Idle Mode Request information. See 7.3.2.51. |

        - When generated

This primitive is generated by the SME to request that an Idle Mode Request frame be sent to a peer entity to convey idle mode request information.

        - Effect of receipt

On receipt of this primitive, the MLME constructs a Idle Mode Request frame containing a paging parameter element specified. This frame is then scheduled for transmission.

      - MLME-IMREQUEST.confirm

        - Function

This primitive reports the result of an idle mode request.

        - Semantics of the service primitive

The primitive parameters are as follows:

MLME-IMREQUEST.confirm	(
Dialog Token,
ResultCode
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Dialog Token | Integer | 0 – 255 | The dialog token to identify the Idle Mode Request transaction. |
| ResultCode | Enumeration | SUCCESS, INVALID PARAMETERS, or UNSPECIFIED FAILURE | Reports the outcome of a request to send an Idle Mode Request frame. |

        - When generated

This primitive is generated by the MLME when the request to transmit an Idle Mode Request frame completes.

        - Effect of receipt

On receipt of this primitive, the SME evaluates the result code.

      - MLME-IMREQUEST.indication

        - Function

This primitive indicates that an Idle Mode Request frame has been received requesting an Idle Mode Request transaction.

        - Semantics of the service primitive

The primitive parameters are as follows:

MLME-IMREQUEST.indication	(
Peer MAC Address,
Dialog Token,
Idle Mode Request,
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Peer MAC Address | MACAddress | Any valid individual MAC Address or the broadcast MAC address | The address of the peer MAC entity to which the Idle Mode Request shall be sent. |
| Dialog Token | Integer | 0 – 255 | The dialog token to identify the Idle Mode request transaction. |
| Idle Mode Request | Idle Mode Request element | Idle Mode Request element | A Idle Mode Request element describing the STA Idle Mode Request information. See 7.3.2.51. |

        - When generated

This primitive is generated by the MLME when a valid Idle Mode Request frame is received.

        - Effect of receipt

On receipt of this primitive, the SME either rejects the request or commences the Idle Mode Request transaction.

    - Idle Mode response

This set of primitives supports the exchange of Idle Mode parameters between peer SMEs.

      - MLME-IMRESPONSE.request

        - Function

This primitive requests the transmission of Idle Mode Response action frame to a peer entity, in response to a received Idle Mode Request frame.

        - Semantics of the service primitive

The primitive parameters are as follows:

MLME-IMRESPONSE.request	(
Peer MAC Address,
Dialog Token,
Idle Mode Response,
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Peer MAC Address | MACAddress | Any valid individual MAC Address, or the Broadcast MAC address | The address of the peer MAC entity to which the Idle Mode Response shall be sent. |
| Dialog Token | Integer | 0 – 255 | The dialog token to identify the Idle Mode transaction. |
| Idle Mode Response | Idle Mode Response element | Idle Mode Response element | A Idle Mode Response element describing the STA Idle Mode Response information. See 7.3.2.51. |

        - When generated

This primitive is generated by the SME to request that an Idle Mode Response frame be sent to a peer entity to convey idle mode response information.

        - Effect of receipt

On receipt of this primitive, the MLME constructs a Idle Mode Response frame containing a paging parameter element specified. This frame is then scheduled for transmission.

      - MLME-IMRESPONSE.confirm

        - Function

This primitive reports the result of an idle mode request.

        - Semantics of the service primitive

The primitive parameters are as follows:

MLME- IMRESPONSE.confirm	(
Dialog Token,
ResultCode
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Dialog Token | Integer | 0 – 255 | The dialog token to identify the Idle Mode transaction. |
| ResultCode | Enumeration | SCCESS, INVALID PARAMETERS, or UNSPECIFIED FAILURE | Reports the outcome of a request to send an Idle Mode Response frame. |

        - When generated

This primitive is generated by the MLME when the request to transmit an Idle Mode Response frame completes.

        - Effect of receipt

On receipt of this primitive, the SME evaluates the result code.

      - MLME-IMRESPONSE.indication

        - Function

This primitive indicates that an Idle Mode Response frame has been received.

        - Semantics of the service primitive

The primitive parameters are as follows:

MLME-IMRESPONSE.indication	(
Peer MAC Address,
Dialog Token,
Idle Mode Response,
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Peer MAC Address | MACAddress | Any valid individual MAC Address or the broadcast MAC address | The address of the peer MAC entity to which the Idle Mode Response shall be sent. |
| Dialog Token | Integer | 0 – 255 | The dialog token to identify the Idle Mode transaction. |
| Idle Mode Response | Idle Mode Response element | Idle Mode Response element | A Idle Mode Response element describing the STA Idle Mode Response information. See clause 7.3.2.52. |

        - When generated

This primitive is generated by the MLME when a valid Idle Mode Response frame is received.

# 11. MAC sublayer management entity

#### 11.3 Relationships between services

Change list item a)2)vi as follows:

vi) Spectrum Management action, Wireless Network Management Presence Request, Presence Response, Idle Mode Request action frames

Within an IBSS, action frames are class 1.

## 11.15 Wireless Network Management Procedures

Add new clauses after clause 11.15.7 of TGv D0.05 as shown below:

- 11.15.8 Idle Mode and Paging

Idle Mode is an extended power save mode for non-AP STAs. While in Idle Mode, a non-AP STA need not listen for every Beacon frame, may traverse multiple BSSs in a Paging Group and still be reachable within the Paging Group.

Paging Services within a Paging Group are managed by a Paging Server. The Paging Server maintains paging service information for non-AP STAs in Idle Mode within a Paging Group. Paging service information includes the STA MAC Address, Paging Group ID, Paging Server ID, and Home AP.

The Paging Group is an identifier for a set of APs. A non-AP STA in a Paging Group moves among BSSs in a Paging Group and stays in Idle Mode.  When a non-AP STA moves from one Paging Group to a new Paging Group, the non-AP STA shall perform a reassociation or BSS transition operation to associate with an AP in the new Paging Group.

Idle Mode and Paging operation comprises the following procedures:

—  Paging Capability Discovery and  Paging Service Information Advertisement

— Enter Idle Mode

- Paging

—  Exit Idle Mode

— Idle Mode Notification

- 11.15.8.1 Paging Service Capability Discovery

A non-AP STA discovers the paging capability via the Wireless Network Management Capability and the Paging Service element included in the Beacon and Probe Response frames. When the Paging Service bit of the Wireless Network Management Capabilities Information Element is set to ‘1’, Paging Service and Idle Mode operation are supported.

- 11.15.8.2 Paging Service Advertisement

APs which support the Paging Service shall include the Paging Service Information Element defined in  7.3.2.48 in Beacon frames and in Probe Response frames.  The Paging Group ID in the Paging Service information element is included in the Idle Mode Request element by non-AP STAs.  The non-AP STA uses the Paging Interval and DPIM Count to determine how often the Beacon frame contains the Paging Indication information element and when the next Paging Indication information element will be sent by an AP.  The Paging Interval shall be uniform within a Paging Group.

- 11.15.8.3 Enter Idle Mode

A non-AP STA signals its intent to enter Idle Mode by sending an Idle Mode Request frame to the Home AP with a Request Type of “Enter Idle Mode” (0x01), STA address, and Paging Group ID.  Upon receiving an Enter Idle Mode Request frame from the non-AP STA, the AP notifies the Paging Server by means which are outside the scope of this standard.

The Paging Server responds to the Home AP with a unique Paging ID within the Paging Group, if the result of Idle Mode Request is successful.  The Paging Server creates, and maintains paging service information for the Idle Mode non-AP STA. The Home AP sends an Idle Mode Response element in an Idle Mode Response frame to the non-AP STA. If the value of Response Status is not “Successful”, the Paging ID shall be set to 0.

If the non-AP STA does not receive an Idle Mode Response frame within 60 seconds after it sends the Idle Mode Request frame, the non-AP STA shall retransmit the Idle Mode Request frame until the transmission is successful or until the retry limit defined in dot11PagingServiceRequestRetryLimit is reached.

If the non-AP STA successfully enters Idle Mode, then the Paging Server pages the non-AP STA to exit Idle Mode, for example, when frames are received for the Idle Mode non-AP STA.

- 11.15.8.4 Paging

Paging is the procedure used by the Paging Server and the APs to notify the non-AP STA in Idle Mode that directed frames are buffered for delivery. Broadcast and Multicast frames are not buffered for delivery to a non-AP STA in Idle Mode.

- 11.15.8.4.1 AP Paging operation

When the Paging Server determines that directed frames are buffered for a non-AP STA in Idle mode, by means which are outside the scope of this standard, or at any time, the Paging Server pages the non-AP STA by updating the Paging Indication element used by the APs in the Paging Group. The Paging Server provides the  Paging ID list to the APs, by means which are outside the scope of this standard.

The Paging Indication element shall identify the STAs in Idle Mode that are being paged.

An AP that supports the Paging Service shall send a Beacon frame containing a Paging Indication information element when the DPIM count field in the Paging Service element is 0.  The AP shall only include a Paging Indication element in a Beacon in which the DPIM count field has a value of 0.

An AP that supports the Paging Service shall include the Paging Indication element in Probe Response frames, except when no non-AP STA is being paged. If no non-AP STA is paged, the Paging Indication element is omitted from the Beacon frame. Multiple Paging Indication elements may be included in a Probe Response or Beacon frame depending on the number of non-AP STAs being paged.

- 11.15.8.4.2  STA operation for receiving a Paging Indication

A non-AP STA in Idle Mode shall wake up prior to the end of the Paging Interval and perform time synchronization with the received Beacon frame.  When the non-AP STA detects that the DPIM count field is 0, the non-AP STA shall determine if its Page ID is included in the Page ID List field of the Paging Indication element.

When the non-AP STA detects that its Paging ID is present in the Page ID list field, the non-AP STA shall exit Idle Mode by sending an Idle Mode Request frame with a Request Type of “1” (Exit Idle Mode) to the Home AP, if the non-AP STA is still associated to the Home AP. If the non-AP STA is no longer associated to its Home AP, the non-AP STA shall perform a reassociation or BSS transition operation to associate with an AP in its Paging Group, and send an Idle Mode Request frame to the AP with a Request Type of “1” (Exit Idle Mode).

If a non-AP STA’s Paging ID is not present in the Page ID list, the non-AP STA continues in Idle Mode operation.

A non-AP STA in Idle Mode may send a Probe Request frame containing a Paging Service element on supported channels in the Regulatory Class where the channel is valid for the current regulatory domain. The Paging Interval and DPIM Count shall be set to 0 in the Paging Service element contained in the Probe Request frame. An AP which supports the Paging Service shall respond with a Probe Response frame that includes the Paging Indication element.

- 11.15.8.5 Exit Idle Mode

A non-AP STA exits Idle Mode by sending an Idle Mode Request frame with a Request Type of “Exit Idle Mode” (0x01). A non-AP STA shall exit Idle Mode if the non-AP STA has a frame to transmit or if the non-AP STA receives a Paging Indication element indicating that the non-AP STA is being paged.

Upon receipt of an  Idle Mode Request frame with a Request Type of “Exit Idle Mode”, the AP shall indicate the request to the Paging Server specified in Idle Mode Request frame, by means that are outside the scope of this standard. The Paging Server  terminates the paging service for the requesting non-AP STA and notifies the non-AP STA’s Home AP that the non-AP STA has exited Idle Mode, by means which are outside the scope of this standard.

- 11.15.8.6 Idle Mode Notification

A non-AP STA provides notification that it is in Idle Mode by sending an Idle Mode Request frame with a Request Type of “ Idle Mode Notification” (0x02) to an AP in the Paging Group. A non-AP STA shall send an Idle Mode Request frame with a Request Type of “ Idle Mode Notification” (0x02) as defined by the dot11PagingServiceUpdateInterval MIB variable.

Upon receipt of the Idle Mode Request element with a Request Type of “Idle Mode Notification (0x02)”, the AP indicates the request to the Paging Server by means which are outside the scope of this standard. The Idle Mode Request frame may be sent as a broadcast or unicast frame.

.

Insert the following entries into A.4.14 in Annex A as follows:

# Annex A

## A.4 PICS proforma–IEEE Std. 802.11, 1999 Edition

### A.4.3 IUT configuration

### A.4.14 Wireless Network Management extensions

| Item | Protocol Capability | References | Status | Support |
| --- | --- | --- | --- | --- |
| RME10 | Paging Service | 11.15.7, 7.3.2.49 | CFv:O | Yes, No. N/A |
| RME10.1 | Idle Mode Request | 7.3.2.51, 7.4.6.13 | (CFv AND RME10):M | Yes, No, N/A |
| RME10.2 | Idle Mode Response | 7.3.2.52, 7.4.6.14 | (CFv AND RME10):M | Yes, No, N/A |
| RME10.3 | Paging Indication | 7.3.2.50 | (CFv AND RME10):M | Yes, No, N/A |
| RME10.4 | Paging Request/Response | 7.2.1.10, 7.2.1.11, 11.15.8.5.2 | (CFv AND RME10):M | Yes, No, N/A |

Add the following MIB attributes in Annex D:

# Annex D

In the dot11StationConfig table of Annex D, insert the following entries at the end of the dot11StationConfigEntry sequence list:

Dot11StationConfigEntry ::=

SEQUENCE {

…

dot11PagingServiceEnabled TruthValue

dot11PagingServiceRequestRetryLimit INTEGER }

Insert the following elements at the end of the dot11StationConfigTable element definitions:

dot11PagingServiceEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that the station

implementation is supports the Wireless Network Management Paging Service.

The default value of this attribute is FALSE."

::= { dot11StationConfigEntry TBD }

dot11PagingServiceRequestRetryLimit OBJECT-TYPE

SYNTAX INTEGER (1..100)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the number of times the non-AP STA may retry an Idle Mode Request frame for which it does not receive an Idle Mode Response frame. The default value of this attribute is 0.”

dot11PagingServiceUpdateInterval OBJECT-TYPE

SYNTAX Unsigned32 (0..4294967295)

UNITS “seconds”

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute indicates the number of seconds the non-AP STA waits before it sends an Idle Mode Request frame of Type – 2 [Update]. A value of 0 indicates that the non-AP STA does not send an Idle Mode Request Frame with Type = 2. The default value of this attribute is 0.”

::= { dot11StationConfigEntry TBD }